3GPP TSG-RAN WG2 Meeting #129	R2-2500649

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.3

Source:	HONOR

Title: 	Discussion on measurement event evaluation and report

Document for:	Discussion and decision

1	Introduction

The following agreements related to measurement event evaluation and measurement report for purpose of supporting LTM were made during RAN2#128 meeting.

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

In this contribution, we further discuss the L1 measurement event evaluation and report and provide proposals accordingly.

- Discussion

2.1 L1 measurement activation

For L1/L2 triggered measurement, L1 measurement is one of the important parts in the preparation phase before the LTM cell switch execution. Unlike L3 measurement, L1 measurements could result in more signalling overhead and power consumption. Especially for the CSI-RS based measurement, dynamic activation/de-activation solution is introduced to reduce the overhead from perspectives of both serving cell and UE in the current specification. It is not necessary to perform the L1 measurements for all candidates when the serving cell becomes good enough or for all RS types or when the candidate cell is not good enough.

RAN1 has discussed the activation of the CSI-RS measurement of the neighbour cells and RAN3 confirmed they would design the interaction of the NW nodes. Thus, RAN2 needs to discuss the design of the MAC CE for CSI-RS activation of neighbour cells. The difference between the legacy CSI-RS activation, the CSI-RS are unified numbered across the candidate cells. The legacy MAC CE could not be reused without modification. Further information needed for the L1 measurement activation. The activation command could be a new MAC CE or reuse the CSI-RS activation MAC CE with additional indication related to the LTM. RAN2 could also discuss the how to enable effective activation, for example, per CSI-RS activation or per candidate cell activation or per resource set activation.

Proposal 1: RAN2 to design the MAC CE for activation of the CSI-RS measurement of the candidate cells.

2.1 L1 MR MAC CE

Signalling burden

For the report of the L1 MR MAC CE, the signalling burden of the L1 MR report is more severe since the number of the measured SSB/CSI-RS is much bigger than the legacy. Several companies mentioned the signalling burden brought by the SSB/CSI-RS based L1 MR for candidate cells. It is straight forward to introduce a prohibit timer for the report, which may reduce effectiveness of the LTM caused by the improper timer configuration. Considering the motivation of the L1 MR is to switch the cell, more factors could be considered when design the mechanism to reduce the signalling burden of L1 MR.

Proposal 2: RAN2 to discuss how to reduce the signalling burden caused by L1 MR: prohibit timer, LTM2~5 combined with other conditions (early-sync status), the number of satisfying beams and so on.

Additional information of the serving cell

For UE-triggered L1 measurement report for the LTM, RAN2 needs to further discuss which beam is included in the report. Beam(s) of neighbour satisfying the event is the basic option since it is useful to help the NW determine the target beam. Additionally, during the RAN2#127-bis meeting, it was agreed that the current beam could be reported based on the NW configuration. This provides up-to-date information of the current beam to assist the NW for target beam/cell of the LTM cell switch. In addition, on one hand, beam management within the serving cell causes less communication interruption than the cell switch to the candidate cell; on the other hand, if there is a good enough beam with the serving cell upon the LTM cell switch, the ping pong between the previous serving cell and target candidate cell may occurs. Thus, it is beneficial to provide more information of the current cell.

Proposal 3: RAN2 to discuss what additional information of the serving cell needs to be reported in the MR MAC CE.

Additional information of the candidate cell

During the RAN2#128 meeting, it is already agreed that additional beam(s) of the candidates could be reported based on the NW control which is stated as below.

Agreement

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

The definition of the N is not that clear. N could be the fixed number of beams to report or the maximum number of beams in the MR MAC CE. Based on the limitation introduced by N, Furthermore, it still needs discuss which beam not satisfying the event of the candidate cell is selected be included in the L1 MR MAC CE. As the current L3 measurement and report mechanism, the UE measures several beams to derive the cell quality to ensure that UE could receive good service under the cell with more good beams. Even the LTM is based on beam measurement, it provides more robust if the target cell has multiple good beams within the cell or CU, even the good beams have not met the L1 measurement event.

Proposal 4: RAN2 to discuss which beam and how many beams not satisfying the condition could be reported in the MR MAC CE.

In addition, UE may perform DL/UL sync phase before the LTM execution. Especially, the UE may obtain the TA based on its own measurement. It is beneficial to report additional information of the candidate cells like UL sync status.

Proposal 5: RAN2 to discuss what additional information (e.g., UL sync status) of the candidate cell needs to be reported in the MR MAC CE.

Measurement result in the MR

During RAN1#118-bis meeting, they further clarified the agreement made in RAN1#118 meeting and made the following agreement.

Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

Based on RAN1 agreement, it is observed that L1 filter may not be introduced. To take the robust into account, RAN2 assumed filtering was needed during RAN2#126 meeting.

Agreement

RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

There may be rapid variation of L1 measurement. It may lead to misunderstanding of the beam quality if the latest measurement result of the beam is reported when the event is met. Thus, if L1 specified filter is not introduced, RAN2 needs to further discuss what measurement results to report to further improve the robust without L1 filtering.

Proposal 6: RAN2 to discuss whether the reported L1 result needs to be filtered/averaged in MAC.

3	Conclusions

In this contribution, we discussed the L1 measurement enhancement for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1: RAN2 to design the MAC CE for activation of the CSI-RS measurement of the candidate cells.

Proposal 2: RAN2 to discuss how to reduce the signalling burden caused by L1 MR: prohibit timer, LTM2~5 combined with other conditions (pre-sync status), the number of satisfying beams and so on.

Proposal 3: RAN2 to discuss what additional information of the serving cell needs to be reported in the MR MAC CE.

Proposal 4: RAN2 to discuss which and how many beam not satisfying the condition could be reported in the MR MAC CE.

Proposal 5: RAN2 to discuss what additional information (e.g., UL sync status) of the candidate cell needs to be reported in the MR MAC CE.

Proposal 6: RAN2 to discuss whether the reported L1 result needs to be filtered/averaged in MAC.

4	References