3GPP TSG-RAN WG2 Meeting #129	R2-2500650

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.4

Source:	HONOR

Title: 	Discussion conditional LTM

Document for:	Discussion and decision

1	Introduction

For the conditional LTM, UE evaluates conditions for triggering LTM where the design of the evaluation condition could refer to that of event triggered L1 measurement reporting. The discussion about measurements related enhancements for purpose of supporting LTM was already started several meetings ago and the following agreements were made during last RAN2 meeting.

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this contribution, we discuss the general concept of the conditional LTM like execution trigger events and the early UL synchronization. Then we propose some solutions/way-forwards accordingly.

- Discussion

Event triggered L1 measurement activation

During the previous discussion, RAN2 already confirmed that the event triggered L1 measurement is designed for LTM early synchronization and LTM cell switch procedure. Considering cell switch procedure of L3 cell switch, L3 measurement is the only pre-operation before the cell switch. L3 evaluation could be performed upon the measurement configuration. For L1/L2 triggered measurement, L1 measurement is one of the important parts in the preparation phase before the LTM cell switch execution. Unlike L3 measurement, L1 measurements could result in more signalling overhead and power consumption. Especially for the CSI-RS based measurement, dynamic activation/de-activation solution is introduced to reduce the overhead from perspectives of both NW and UE. It is not necessary to perform the L1 measurements for all candidates when the serving cell becomes good enough or for all RS types or when the candidate cell is not good enough.

Proposal 1: Event triggered L1 measurement should be designed for the one additional LTM purposes: trigger L1 measurement without NW signalling (e.g., UE perform CSI-RS measurement based the triggered L1 measurement event) for conditional LTM.

Early-sync for CLTM

During L3 mobility like CHO, UE synchronizes to that candidate cell after detaching the source gNB. For the LTM with less latency, the UE needs to perform the early synchronization (DL and/or UL) before the LTM cell switch execution. During previous discussion, RAN2 already confirmed that the event triggered L1 measurement is designed for LTM early synchronization. This enables that the early synchronization could be obtained earlier than the cell switch by performing synchronization upon the satisfaction of an easily satisfied condition and executing the cell switch upon a stricter condition. What is more, we still need to discuss the early synchronization of C-LTM is triggered by the UE or the NW based on L1 measurement result. On one hand, we could reuse the PDCCH order triggered early TA requisition defined in Release 18 LTM with less specification impacts. But it is possible that UE has not received PDCCH order before it triggers the execution of the LTM based on the L1 measurement results. On the other hand, the early synchronization is also trigger by the UE based on the preconfigured L1 measurement event. For example, UE could send the preamble or perform the UE-based TA measurement with a lower threshold and triggered the execution with a higher threshold.

Observation: The UE could send the preamble for early TA acquisition or perform the UE-based TA measurement with a lower threshold and triggered the execution with a higher threshold to enable RACH-less which reduce the latency caused by RACH.

Proposal 2: Without the PDCCH order triggered, the event triggered early synchronization of C-LTM could be introduced to conditional LTM.

For Rel-18 LTM, the TA is carried on the LTM switch command to trigger the execution. Since the execution for the conditional LTM is triggered by UE when the L1 measurement results meet the pre-configured condition, the TA needs to be sent to the UE ahead to enable the RACH-less switch. RAN2 has agreed to introduce a new MAC CE to convey the TA of candidate cells. For this new MAC CE, at least the TA value and the information of the candidate cell should be sent to the UE. In addition, other related information like the TAT, TAG ID could also be considered to provide up-to-date configuration/information of this TA.

Proposal 3: RAN2 to discuss which info is needed in the new MAC CE for the early sync: TA value, configuration ID and TAG ID, TAT.

With the early UL synchronization designed in Rel-18, UE could perform the RACH-less LTM on the triggered candidate. To avoid signalling and power overhead resulted from frequent TA acquisition, TA could be maintained in the UE after pre-acquisition before the execution and UE could use the corresponding TA when executing the switch to the candidate if the TA of this candidate is still valid. Furthermore, whether the former received TA could be reused after the cell switch is another question. If the TA could be maintained, we could further discuss how to guarantee the validity of the TA based on the legacy mechanism like  TAT and RSRP threshold.

Proposal 4: UE could maintain the TA for the candidates after receiving the TA. FFS the TA could be maintained after the serving cell switch. FFS how to guarantee the validity of the TA.

3	Conclusions

In this contribution, we discussed the conditional LTM for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1: Event triggered L1 measurement should be designed for the one additional LTM purposes: trigger L1 measurement without NW signalling (e.g., UE perform CSI-RS measurement based the triggered L1 measurement event) for conditional LTM.

Observation: The UE could send the preamble for early TA acquisition or perform the UE-based TA measurement with a lower threshold and triggered the execution with a higher threshold to enable RACH-less which reduce the latency caused by RACH.

Proposal 2: Without the PDCCH order triggered, the event triggered early synchronization of C-LTM could be introduced to conditional LTM.

Proposal 3: RAN2 to discuss which info is needed in the new MAC CE for the early sync: TA value, configuration ID and TAG ID, TAT.

Proposal 4: UE could maintain the TA for the candidates after receiving the TA. FFS the TA could be maintained after the serving cell switch. FFS how to guarantee the validity of the TA.

4	References