3GPP RAN WG2 Meeting #129	R2-2500651

# Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.3

Source:	HONOR

Title:	Discussion on RRM measurement

Document for:	Discussion and Decision

1. Introduction

In RAN2#128, the following agreements were agreed [1]:

No MG-specific enhancements is needed on DSR operation. RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped No MG-specific enhancements is needed for DRX operation.

In this contribution, we will focus on discussion of RRM enhancements.

2 Discussion

RAN1 has agreed that introducing an indication in DCI to skip a measurement gap. The DCI-based method is suitable for the DL since the network knows when the DL data arrives. For the UL, the network can dynamically schedule PUSCH based on the SR-BSR/DSR procedure. When the scheduled resources overlap with the MG, the MG can be skipped through the new indication of the DCI. Therefore, the DCI-based method is more applied to dynamic scheduling.

In the Rel-18 XR, multi-PUSCH configured grant and UTO-UCI mechanism are introduced to cover the jitter of XR services. Meanwhile, the problem of non-integer cycles can be solved by configuring multiple CGs. Therefore, the current CG can well cover the arrival time of XR periodic services. With knowing the periodicity of MG, the network can indicate which CG can be used to transmit when overlapping with MG. This method is simpler and with less signalling overhead.

Observation 1: The DCI-based MG skipping is more applied to dynamic scheduling.

Proposal 1: The network can configure the CG that is prior to MG, whose data can be transmitted while overlapping with the MG.

The current MAC spec restricts the UE behavoir for TX/RX in an activated measurement gap. If the MG is indicated as skipping, the UE should relax the restricting regardless of the scheduling type.  The changes in current specification are as following:

5.14 Handling of measurement gaps During an activated measurement gap which is not indicated as skipping, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]: 1> not perform the transmission of HARQ feedback, SR, and CSI; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.5, and 5.7. 1> else: 2> not monitor the PDCCH; 2> not receive on DL-SCH.

Proposal 2: If the MG is indicated as skipping, the UE should relax the restrictions regardless of the scheduling type (e.g. HARQ feedback, SR, CSI, SRS…).

Skipping measurement gap can avoid transmission delays due to measurement. If the UE does not receive the new indication, it will perform measuring rather than TX/RX. In one case, the network sends the DCI containing new indication, but the UE does not receive or successfully decode, which will cause TX/RX failure. Similar as SPS deactivation/activation, a confimation can be introduced to confirm whether the MG is skipped.

Proposal 3: RAN2 can discuss whether a confimation is needed for the MG skipping.

3 Conclusions

Proposal 1: The network can configure the CG that is prior to MG, whose data can be transmitted while overlapping with the MG.

Proposal 2: If the MG is indicated as skipping, the UE should relax the restrictions regardless of the scheduling type (e.g. HARQ feedback, SR, CSI, SRS…).

Proposal 3: RAN2 can discuss whether a confimation is needed for the MG skipping.

4. References

[1]	RAN2 #128 Chairman notes