3GPP RAN WG2 Meeting #129	R2-2500654

# Athens, Greece, Feb 17th – 21st, 2025

Agenda Item:	8.7.5

Source:	HONOR

Title:	Discussion on RLC enhancements

Document for:	Discussion and Decision

1. Introduction

In RAN2#128, the following agreements were agreed for enhancing RLC retransmissions [1]:

Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced - Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report. Timely RLC retransmissions – autonomous versus polling Compromise solutions: - If the remaining time drops below a threshold, poll is triggered. If SR is not received within a time, then Tx can autonomously retransmit the delay critical packet. - Have both autonomous and polling enhancement. Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

In this contribution, we discuss the potential solutions related to RLC enhancements.

2. Discussion

2.1 Avoiding unnecessary RLC retransmissions

How the TX side determines the outdated SDU

In the current specification, the SDU discarded mechanism is as following:

TS38.323 v18.3.0 5.3 SDU discard If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers. TS38.322 v18.1.0 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

If the discarded indication of an RLC SDU is received from PDCP, the RLC will not discard the RLC SDU or a segment thereof what has been submitted to the lower layers for avoiding SN gap. However, the resources will be wasted to retransmit the outdated SDUs. Therefore, RAN2 discusses the enhancements on avoiding unnecessary RLC retransmission. It is agreed that TX side stops transmissions of an outdated SDU. The current indication can be reused for the TX side to determine which SDUs are outdated. When receiving the discarded indication, the RLC discards the SDU regardless of whether it is submitted to lower layer. To differentiate from existing handling, the network configures an enabled or disabled indication for the discarding.

Proposal 1: The Network enables the RLC to stop transmissions of an outdated SDU notified by the PDCP regardless of whether the SDU is submitted to the lower layer.

How the new RLC timer for the outdated SDU works in the RX side

For the convenience of description, the new RLC timer may be called t-Outdated. In the RX RLC, the t-Reassembly is also related to the SN gap, which is used to trigger Status report and notify TX to retransmit incompletely received SDU. To ensure that incompletely received SDU has a chance to be retransmitted before it is abandoned in RX, the t-Outdated should be set longer than the t-Reassembly.

Proposal 2: The new RLC timer should be set longer than the t-Reassembly.

In the last meeting, some companies propose the operation of the new RLC timer at the Rx side, such as per RLC entity timer and per RLC SDU timer.

- Per RLC entity timer

It is similar to t-Reassembly, i.e. only one t-Outdated is running at a given time. The start and stop conditions of t-Outdated can be the same as t-Reassembly.

When the SN gap occurs in RX window, the t-Outdated starts if it is not running and associated with this gap. Such as RX_Next_Status_Trigger associated with t-Reassembly, a state variable should be introduced to hold the value of the SN following the SN of the RLC SDU which triggered this timer. The t-Outdated stops when all the SDUs in this SN gap are received. When t-Outdated expires, the SDU in this SN gap will be discarded and the RX window will be updated. Per RLC entity timer is convenient for RLC entity to maintain.

Observation 1: If per RLC entity timer is adopted, a state variable is introduced to hold the value of the SN following the SN of the RLC SDU which triggered this timer.

- Per RLC SDU timer

For per RLC SDU timer, the t-Outdated is associated with one SDU which is lost at lower layer.

When the SN gap occurs in RX window, start the multiple t-Outdateds associated with every SDU this gap, no matter whether there are other t-Outdateds are running. The t-Outdated stops when the associated SDU is received. When t-Outdated expires, the associated SDU will be discarded and the RX window will be updated.

For per RLC entity timer, if one t-Outdated is running, no matter how many new gaps occur, new timer will not start for these new gaps. The SDUs in these new gaps are not discarded in time. Per RLC SDU timer can discard obsolete SDUs more accurately than per RLC entity timer, but its implementation is complex.

Because the SDUs in one SN gap are discarded at the same time, they can associate one timer, that is, per RLC SN gap timer.

- Per RLC SN gap timer

When the SN gap occurs in RX window, start the t-Outdated associated with this gap. The t-Outdated stops when all the SDUs in this SN gap are received. When t-Outdated expires, the SDU in this SN gap will be discarded and the RX window will be updated.

It has the same advantages as per RLC SDU timer, but maintains fewer timers.

Proposal 3: RAN2 discusses per RLC entity timer and per RLC SN gap timer, excludes per RLC SDU timer.

SR trigger

In some cases, when t-Outdated expires, the SDUs whose SN is less than RX_Next_Status_Trigger are obsolete and no longer need to be retransmitted. It can be considered that SR is triggered when t-Outdated expires, to stop or avoid retransmission of incompletely received SDUs whose SN is less than RX_Next_Status_Trigger.

Proposal 4: RAN2 needs to discuss the expiry of new RLC timer to trigger SR.

2.2 Timely RLC retransmissions

In previous meetings, some autonomous retransmission and polling enhancement solutions have been proposed, but no conclusion has been reached. After confirming that both directions can be used as solutions for timely RLC retransmission, RAN2 should continue to discuss the specific implementation of autonomous retransmission, polling enhancement, and even the combined solution of autonomous retransmission and polling.

Autonomous retransmission solution

The autonomous retransmission solutions are as following:

Option1: UE autonomously triggers retransmission of an RLC PDU after the remaining time of a PDU has dropped below a configured threshold.

It is the most supported solution, because its triggering condition most directly meets the short delay requirement of XR service.

Option2: Trigger autonomous retransmission of an SDU with unknown status if all SDUs of the PDU Set are sent or have unknown status.

Option3: The spare PUSCH resources can be used for autonomous retransmission.

The triggering conditions in Option2 and Option3 are not related to the remaining time of PDU, and these triggering conditions cannot achieve timely retransmission of PDU with insufficient remaining time.

Option4: The RLC SDU (segment) can be retransmitted when the channel quality is deteriorated, to ensure the transmission reliability of the associated RLC SDU.

This option is theoretically effective way to trigger autonomous retransmission. When the channel deteriorates, the transmission of PDUs are likely to be unsuccessful, so they need to be retransmitted. However, it is not reasonable in implementation, because it is difficult for UE to get the uplink quality.

Option5: Autonomous retransmission can be activated/deactivated by the network or can be triggered when the remaining time of the data becomes lower than a threshold.

Option5 is based on network implementation. The network controls the UE to perform autonomous retransmission based on the remaining time of the PDU, which effectively reduces the possible congestion caused by autonomous retransmission. In addition, there are some other conditions, such as PSI, and only high-PSI PDUs can be autonomously retransmitted.

Proposal 5: RAN2 considers Autonomous retransmission based on the remaining time of SDU being lower than a threshold.

Proposal 6: Consider adding constraints on autonomous retransmissions to reduce the possible congestion caused by autonomous retransmission.

Polling enhancement solution

The Polling enhancement solutions are as following:

Option1：The remaining time could be considered to enhance the polling trigger mechanism, such as the delay criticality of PDU.

It is triggered based on the remaining time of the PDU, hoping to obtain SR in time, and retransmit PDU if it is NACK. This solution can accurately detect PDUs with insufficient remaining time.

Option2：the network can configure a new set of shorter value for pollPDU and pollByte to trigger the polling for timely RLC retransmission.

It reuses the existing triggers and is simple to implement by network, but the submission of SR is still prohibited by the t-StatusProhibit. Frequent triggering of poll by TX may not necessarily achieve timely acquisition of SR.

Option3：Use dedicated POLL for PDUs with short PBD. The receiving entity distinguishes between dedicated polling and normal polling and handles reassembly and inhibit timers differently.

This option is similar to Option2, but it requires RX to cooperate with proprietary polling to enable timely submission of SR, which involves the enhancement of status reporting. The enhancement of SR has been excluded in RAN2#127bis meeting, so Option3 is not suitable.

Option4：Retransmission of delay-critical RLC SDUs upon expiration of t-PollRetransmit.

In Option4, relying on t-PollRetransmit to trigger retransmission cannot achieve timely retransmission of PDU with insufficient remaining time.

Proposal 7: RAN2 considers polling enhancement based on the remaining time of SDU.

Autonomous retransmission and Poll enhancement

The major motivation on timely RLC retransmission is to reduce RLC ARQ retransmission delay. From this perspective, autonomous retransmission can minimize retransmission delay to the greatest extent. If status report of the SDU is not received within a long period of time, the autonomous retransmission should be performed. However, the subsequent operation of the SDU after autonomous retransmission needs to be based on status report. Therefore, further optimizing on autonomous retransmission can be considered by autonomous retransmission with polling enhancement.

The enhancement based on autonomous retransmission is considered for timely RLC retransmissions. The enhancement for timely RLC retransmission primarily relies on autonomous retransmission. When the remaining time of the SDU meets a certain threshold, the autonomous retransmission is triggered. The retransmitted SDU carries a poll indication, which is used to query the status of the SDU, allowing the RX to promptly provide a status report for subsequent actions in TX. If the status of the SDU is ACK, the retransmission will not continue. If the status of the SDU is NACK, retransmission continues based on the status report until the SDU is successfully received or discarded due to outdated.

Timely feedback of status reports allows TX to retransmit SDU within a limited time, improving the complete reception rate of the SDU. In some cases, timely status reports can also help TX to advance the window, potentially avoiding the retransmission of other nearly expired ACK SDUs. This autonomous scheme not only reduces the delay in waiting for status report but also controls unnecessary retransmissions, making it an optional timely retransmission solution.

Proposal 8：Enhance autonomous retransmission of SDU with poll indication.

In addition, the poll enhancement or autonomous retransmission can be selected to perform according to the timing of the indication, such as delayed critical indication. The initially transmitted delayed critical SDU triggers poll. After the initial transmission, autonomous retransmission is considered if it becomes a delayed critical SDU.

NW configure either or both

In RAN2#128, the network can configure one or both of autonomous retransmission and polling enhancement for the UE. NW can configure the parameters related to the enhancement solution in the RLC entity when the entity is established. The UE executes the corresponding enhancement solution according to the configuration. Another way is that the network also configures the parameters related to the enhancement solution in the RLC entity, and indicates whether and which enhancement scheme to execute, i.e., timely RLC retransmission activation/deactivation. The indication can activate one or both schemes, or deactivate the enhancement scheme.

Proposal 9: RAN2 considers introducing the indication for timely RLC retransmission activation/deactivation.

3. Conclusions

- Avoiding unnecessary RLC retransmissions

Proposal 1: The Network enables the RLC to stop transmissions of an outdated SDU notified by the PDCP regardless of whether the SDU is submitted to the lower layer.

Proposal 2: The new RLC timer should be set longer than the t-Reassembly.

Observation 1: If per RLC entity timer is adopted, a state variable is introduced to hold the value of the SN following the SN of the RLC SDU which triggered this timer.

Proposal 3: RAN2 discusses per RLC entity timer and per RLC SN gap timer, excludes per RLC SDU timer.

Proposal 4: RAN2 needs to discuss the expiry of new RLC timer to trigger SR.

- Timely RLC retransmissions

Proposal 5: RAN2 considers Autonomous retransmission based on the remaining time of SDU being lower than a threshold.

Proposal 6: Consider adding constraints on autonomous retransmissions to reduce the possible congestion caused by autonomous retransmission.

Proposal 7: RAN2 considers polling enhancement based on the remaining time of SDU.

Proposal 8：Enhance autonomous retransmission of SDU with poll indication.

Proposal 9: RAN2 considers introducing the indication for timely RLC retransmission activation/deactivation.

4. References

[1]	RAN2 #128 Chairman Notes