3GPP TSG-RAN WG2 Meeting #129	R2-2500656

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.8.2

Source:	HONOR

Title: 	Discussions on downlink coverage enhancement

Document for:	Discussion and decision

1	Introduction

During the previous meetings for Rel-19 NTN, RAN2 discussed the downlink coverage enhancement in NTN system and made some agreements as follows.

Agreements:

1. As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

2. RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

3. For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states.

In this contribution, we would further discuss downlink coverage enhancement based on the previous agreements and provide proposals accordingly.

2	Discussion

As mentioned in the WID, the issues/impacts could be cell search complexity, initial cell selection, latency, and success rate. For RRC_IDLE and RRC_INACTIVE UEs, the initial access is one of the most severe problems caused by the extension. Considering the limited number of simultaneously active beams, it is hard to keep one satellite beam active for a long time due to limited power and limited feeder link bandwidth. That is, it is also hard for RRC_CONNECTED UEs receiving long-term service from one specific NR beam/cell. Thus, enhancements for RRC_CONNECTED UEs may be needed due to the satellite beam state transition.

Considering the DL CE mechanism has impacts on both RRC_IDLE/INACTIVE UEs and RRC_CONNECTED UEs, the broadcasted SIB could be the baseline approach to deliver the information related to the cell/beam operating with DL CE.

Proposal 1: Information related to the cell/beam operating with DL CE could be broadcasted to the UE by SIB.

Observation 1: In addition to RRC_IDLE/INACTIVE UEs, the SSB periodicity extension also has impacts on RRC_CONNECTED.

In legacy, mobility management is important for the cellular network to provide consecutive service when the UE moves around. In another word, the mobility occurs at least when the UE prepares or executes the switch from one beam/cell to another. In the current specification, the mobility includes the mobility in all RRC states (e.g., handover, cell selection and cell reselection, etc.), measurement, radio link failure, beam failure detection and recovery and so on. In the scenario of DL coverage enhancement, the mobility occurs not only when the UE or NW node moves around but also during beam hopping. From perspective of both CONNECTED and IDLE/INACTIVE mode mobility, mobility management needs to perform measurement, cell selection, cell reselection, handover, radio link failure, beam failure detection and recovery and so on. Thus, we propose that:

Observation 2: There are potential impacts on mobility related aspects (e.g., measurement, handover, etc.) and failure handling cause by SSB periodicity extension.

Since NW is aware of whether the beam/cell is operating with DL CE, NW could send the beam/cell switch command to the suitable beam/cell to the RRC_CONNECTED UE. However, for the conditional handover which is triggered by the UE based on the measurement results and the Ephemeris, the UE may execute the handover to the beam/cell which is not available for a while which may result in unnecessary failure. Thus, we propose that:

Proposal 2: RAN2 to study the impacts on conditional handover considering the beam/cell operating with DL CE.

The RRC_IDLE and RRC_INACTIVE UE shall perform measurements for cell selection and reselection purposes and try to select a suitable cell based on the measurements and the NW-configured criteria. Cell Reselection evaluation process composes of several steps like reselection priorities handling, measurement rules for cell re-selection, cell ranking/selection and so on. The SSB periodicity extension would prolong the time of performing measurement and the initial access. Thus, we propose:

Proposal 3: RAN2 to study the impacts on the measurement and evaluation for the cell selection/reselection handover considering the beam/cell operating with DL CE.

As can be seen from the RAN1’s agreement above, there may be another potential enhancement for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint. Although, the satellite beam footprint is not visible to the UE, NR beam which is composed of satellite beam(s) could be divided to two transmission modes: only DL common channels, DL/UL dedicated channels. That is, the N2 state with only DL common channel could be applied in the SSB of the wider beam footprint. UE may be not able/supposed to initiate random access to these NR beams with wider beam footprint.

If the R19 UE not supporting DL coverage enhancement is not barred based on the SIB broadcasted by the N2 mode beam mentioned above, the UE could attempt random access ending up with failure. Thus, if RAN1 considers the beam with only DL common channel, further enhancement needs to be done.

Proposal 4: RAN2 to study the potential impacts (e.g., access control, DL only indication) brought by the beam with only DL common channel.

3	Conclusions

In this contribution, we discussed the issues related to DL coverage enhancements in NTN. Based on the discussion, the following proposals are concluded:

Proposal 1: Information related to the cell/beam operating with DL CE could be broadcasted to the UE by SIB.

Observation 1: In addition to RRC_IDLE/INACTIVE UEs, the SSB periodicity extension also has impacts on RRC_CONNECTED.

Observation 2: There are potential impacts on mobility related aspects (e.g., measurement, handover, etc.) and failure handling cause by SSB periodicity extension.

Proposal 2: RAN2 to study the impacts on conditional handover considering the beam/cell operating with DL CE.

Proposal 3: RAN2 to study the impacts on the measurement and evaluation for the cell selection/reselection handover considering the beam/cell operating with DL CE.

Proposal 4: RAN2 to study the potential impacts (e.g., access control, DL only indication) brought by the beam with only DL common channel.

4	References