3GPP TSG-RAN WG2 Meeting #129	R2-2500657

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.8.4

Source:	HONOR

Title: 	Discussions on the support of broadcast service

Document for:	Discussion and decision

1	Introduction

During the previous meetings for Rel-19 NTN, RAN2 clarified use cases of the broadcast service in NTN system and made some basic agreements during RAN2#128 meeting as follows.

Agreements: 1. The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. 2. The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. 3. The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed) 4. A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s). 5. RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. 6. Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) 7. The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

In this contribution, we would further discuss how to provide the broadcast service via NTN based on the previous agreements.

- Discussion

In addition, we need to discuss the node to perform the association of the intended service area and the broadcast service. Unlike the interest of area, the intended service area is to indicate the region for the broadcast service. For example, only the people in nation A could watch the news from the channel of the nation A. Thus, the higher layer could make the association based on the policy/strategy provided by the service providers. In another hand, since the intended service area is divided by the RAN, the CN could forward the policy/strategy provided by the service providers to RAN and RAN to perform the association. Although the final decision would be made in SA2 or RAN3, RAN2 as the leading group for this topic could trigger the discussion based on our observation, especially when SA2 has no TU for this topic.

Proposal 1: RAN2 to discuss the association of the intended service area and the broadcast service is provided by CN or gNB to UE. LS to SA2/RAN3 may be needed.

During the last meeting, RAN2 agreed to have the list of the intended service areas and related pointer, but left with the FFS about what is this pointer. To reduce the signalling burden, the index associated with the service area could be provided one by one. In this way, UE could derive the ID related to the area where it located and compare it with the ID which is carried in the MCCH.

Proposal 2: List of ISAs and related index is provided in the new SIB, i.e., there is a corresponding relationship between service area and the index.

Considering the size of the new SIB, if the UE already obtained the list of some area across cells, it could save it to the variable part and use it based on the info provided in the MCCH when moving to another cell. In this way, UE is not required to re-obtain the new SIB before it establishes/releases the MRB if there is no update (reuse the legacy systemInformationAreaID of the SIB is not changed) when it is still within the area in the obtained area info.

Proposal 3：The index of the intended service areas is unified assigned across cells.

Although the issue is more severe in the NTN scenario, the intended area is defined since the NTN cell may be too big to provide MBS service within the proper area. However, the TN cell is related to the deployment of the gNB and the intended service area is based on the policy provided by the service provider. This means the border of the intended service area may be not aligned with the border of the TN cell. Thus, TN UEs could also utilize the info to understand whether it can obtain the specific MBS service based on its location.

Proposal 4: The MBS intended service area mechanism should not be restricted to neither NTN cells nor NTN-capable UEs.

3	Conclusions

In this contribution, we discussed the issues related to intended service area in NTN. Based on the discussion, the following proposals are concluded:

Proposal 1: RAN2 to discuss the association of the intended service area and the broadcast service is provided by CN or gNB to UE. LS to SA2/RAN3 may be needed.

Proposal 2: List of ISAs and related index is provided in the new SIB, i.e., there is a corresponding relationship between service area and the index.

Proposal 3：The index of the intended service areas is unified assigned across cells.

Proposal 4: The MBS intended service area mechanism should not be restricted to neither NTN cells nor NTN-capable UEs.

4	References