3GPP TSG RAN WG2 #129          						R2-2500660

Athens, Greece, Feb 17th – 21st, 2025

Source:	Tejas Networks Ltd.

Title:	Discussion on A-IoT paging message

Agenda item:	8.2.2

Document for:     Discussion and Decision

# Introduction

As discussed in TR38.769, the A-IoT devices receive Msg0 paging signal from reader or base station (BS), upon receiving Msg0, multiple A-IoT devices transmit Msg1 which carries a unique random ID of the device in response to Msg0. In response to Msg1, the reader transmits Msg2 with scheduling information for the device to transmit the actual device ID and group ID. Based on the resource allocated to the devices, they send their data consists of device ID and group ID. Fig. 1 shows the timing diagram of A-IoT communication between the reader and a device. At every communication slot or occasion, the reader initiates the communication by sending a paging signal and the slot ends when the device received acknowledgement of msg3 or the command response.

Fig. 1: Timing diagram of communication from Msg0 to Msg4 between reader and device.

# A-IoT Paging Message Content and Paging Identifier Details

As to the A-IoT paging message, the identifier may be required to identify the device/group of devices in this trigger message (e.g., for the case of reaching a single or a group of devices). Following cases are studied:

-	The A-IoT paging message containing an identifier of a single A-IoT device

-	The A-IoT paging message containing a group ID that maps to multiple A-IoT devices

-	The A-IoT paging message that does not contain any identifier, i.e., indicating all A-IoT devices that can receive the A-IoT paging message need to respond

-	The A-IoT paging message containing multiple identifiers of A-IoT devices. The need for this use case is still to be confirmed/dependent according to the conclusion in [151]. From RAN2 perspective, it is feasible to support paging multiple identifiers of A-IoT devices, pending on TB size and multiplexing design of A-IoT paging message.

The following has been studied in TR 38.769, section 6.3.3 for A-IoT paging message content [1]:

It needs to be further discussed for the scenario that different readers may send A-IoT paging messages, which are associated with the same service request from the CN, to the same device for response. If this scenario is in the scope, it needs to be further discussed, by considering the progress from all the WGs.

As presented above, in case the A-IoT paging message does not contain any identifier, it indicates that all the devices receiving the A-IoT paging message need to respond. In such scenario, if devices receive paging message signal from multiple readers, they have to decide on to whether to response to all the readers or to prioritize one reader. In related to that condition, the following has been studied in TR 38.769, section 6.3.3 for A-IoT paging identifier details [1]:

Fig. 2: System-level deployment scenario of D1T1-topology.

Consider the scenario depicted in Fig. 2, where multiple BSs are equally spaced and the devices are randomly distributed in the region. If the A-IoT devices are deployed at the cell edge or corner of a cell, such devices can receive and decode DL signals from more than one BSs.

The message signals may contain the following information to handle the inventory command from multiple base stations:

- In order for the device to identify the message type, some indication should be provided as part of the message structure either explicitly as a message ID or implicitly by using a different preamble sequence for each of the messages.

- For the device to identify from which reader the message was sent from, the messages should also contain a base station ID or reader ID.

Observation 1: Based on the deployment position, the A-IoT devices may get one or multiple service requests from one or multiple base stations.

Proposal 1: In the case of A-IoT paging message does not contain any identifier, the following information can be implicitly or explicitly conveyed to the device through msg0 to handle the service request from multiple base stations:

- Msg ID

- BS or reader ID

# Subsequent Paging in A-IoT

It is supported that the reader can send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. The duplicated response from devices for the same service request should be avoided. The A-IoT paging message can include information to avoid this duplicated response from the device to a reader. It needs to be further discussed on how to design this information in A-IoT paging message (e.g., including stage-3 details and considering the related aspects from other WGs). Then, based on this information, the device determines whether to skip sending the response to A-IoT paging message or not (if the device had successfully responded the same service before). This information should be short and simple. This information is one ID, while it needs to be further discussed whether the ID is generated by the reader or by the core network. It needs to be further discussed on the size of this information.

The following has been studied in TR 38.769, section 6.3.3 for subsequent paging in A-IoT [1]:

The A-IoT paging message can carry a unique service ID generated at the reader to convey the information to the device if it is the same service request within the same inventory cycle. The device when activates and receives a paging message, it stores the service ID to identify the same service request and take decision on whether to send a service response or not.

Observation 2: The reader can send multiple subsequent A-IoT paging message signals associated with the same service request.

Proposal 2: The A-IoT paging message can implicitly or explicitly carry a unique service ID generated at the reader to convey the information to the device if it is the same service request. The device when activates and receives a paging message, it stores the service ID to identify the same service request and take decision on whether to send a service response or not.

# A-IoT Communication Process Between Multiple Base stations and a Device

The timing diagram of the method to handle the inventory and command procedure between multiple BSs and single device is shown in Fig. 3. Figs. 3a and 3b shows the successful and unsuccessful completion of the inventory and command procedure, respectively.

The msg0 signal should contain the communication slot duration T to provide this information to the device. This information is required for the device to decide on the sleep mode duration when the device is not scheduled by the BS to send msg3.

Fig. 3a: Device receives DL signals from multiple BSs/readers and msg3 is scheduled by R1.

Fig. 3b: Device receives DL signals from multiple BSs/readers and msg2 is not received from R1.

The inventory and command service request(s) received from one or multiple base station(s) at the device can be handled using msg ID, base station or reader ID, service ID, communication slot duration T, etc. as follows:

- If the energy available at the device is above certain threshold, the device activates the receiver part and operates at listening mode.

- If the device receives any signal, it decodes the signal to find the msg ID, BS ID, and service ID.

- If the msg ID is zero or such that it indicated msg0, the device decodes the BS ID and service ID.

- If the current BS ID or service ID (any of them) is not same as the previous BS ID or service ID, the device switches to transmitting mode and transmits msg1 signal.

- The msg1 signal contains the msg ID, BS ID (received in msg0 signal), unique random ID associated with the device, energy status of the device, etc.

- If the device receives multiple msg0 signals during listening mode within that communication slot, as shown in Fig. 4(a), it responds to the initial msg0 signal only. The subsequently decoded msg0 signals within the slot are discarded by the device.

- In order to discard the msg signals from other BSs in the same communication slot, the device decodes the BS ID. If the BS ID is not same as the BS ID of the first msg0 signal received in the current slot, the device discards the msg signals.

- Consider the device D receives first msg0 signal from BS/reader R1, as shown in Fig. 3a, it will not respond to the msg0 signals received from reader R2 or reader R3.

- If the msg1 signal is successfully received by the corresponding BS (R1), as shown in Fig. 3a,

- Reader R1 can schedule the device according to the availability of time and frequency domain resources, and the energy status of the device.

- If the msg1 signal is not successfully received by the corresponding BS (R1), as shown in Fig. 3b,

- Reader R1 does not schedule the device.

- After transmitting msg1, device switches to listening/receiving mode and waits for msg2 signal from R1.

- If the device does not receive any msg2 signal within the msg2 duration,

- it computes the time (TSleep = T - t), where t is the current time (t = 0 is set at the beginning of slot) and goes to sleep mode if TSleep > Tth.

- After the duration TSleep, the device again activates and starts from step 1.

- If device receives one or multiple msg2 signals from the same reader R1, it decodes one or multiple msg2 signals.

- If resources are allocated to the device, it transmits msg3 containing msg ID, BS ID, actual device ID, device type, etc.

- If the msg2 signals do not carry any resource for the device, it computes the time (TSleep = T - t) and goes to sleep mode if TSleep > Tth. Alternatively, if the reader is using multiple msg2 to schedule the devices, the msg2 also indicates if the received msg2 is the last msg2 transmission for this current slot. If the received msg2 is not the last msg2 in this slot and the device is not scheduled by the msg2, the device listens for the next msg2. If the device determines that the received msg2 is the last msg2 transmitted by the reader in this slot and the device is not scheduled by that msg2, and it moves to sleep mode if TSleep > Tth.

- After the duration TSleep, the device again activates and starts from step 1.

- When the device receives resource from BS in msg2, it transmits msg3. After transmitting msg3, the device switches to listening mode and waits for acknowledgement from R1. If it receives an ACK, it considers that the inventory is successfully completed.

- The device responds to next msg0 signal only if either of the next BS ID and the service ID carried by msg0 signal is not same as the current BS ID and service ID. Else, it goes to sleep mode.

Proposal 3: The estimated communication slot duration T can be implicitly or explicitly conveyed to the device through msg0 signal. This information is required for the device to decide on the sleep mode duration when the device is not scheduled by the BS to send msg3.

Observation 3: Device may receive multiple msg0 signal when the device is operating at listening mode or receiving mode.

Proposal 4: If the device receives multiple msg0 signals during listening mode or receiving mode within one communication slot, it responds to the initial msg0 signal only. The subsequently decoded msg0 signals within that slot are discarded by the device.

Observation 4: In 4-step CBRA, the device transmits msg1 to get resource from the base station to transmit msg3 to send the data such as, device ID, group ID, etc.

Proposal 5: The device can transmit the device energy status to the reader through msg1 to inform the energy status of the device, which the reader can use for prioritizing/ deprioritizing the device for scheduling.

Observation 5:  If the msg1 is not successfully received at the base station or the base station does not schedule the device in that communication slot or the device do not receive acknowledgement after transmitting msg3, the inventory process is considered unsuccessful.

Proposal 6: When the inventory process is not successful, it computes TSleep which is the remaining time to complete the communication slot and receive next msg0 paging signal. The device can go to sleep mode for TSleep, if TSleep > Tth, where Tth is the minimum sleep time required to save energy of the device.

Proposal 7: If reader sends multiple msg2 signals, the msg2 signal can carry an indication to convey the device if it is the last msg2 signal in this communication slot.

# Conclusion

The A-IoT paging message content and paging identifier are discussed in this work in the context of one or multiple service request(s) received by the device from one or multiple base stations. This work also discusses handling subsequent paging signals from the same base station associated with the same service. We have made the following observations and proposals related to the above-mentioned topics:

Observation 1: Based on the deployment position, the A-IoT devices may get one or multiple service requests from one or multiple base stations.

Proposal 1: In the case of A-IoT paging message does not contain any identifier, the following information can be implicitly or explicitly conveyed to the device through msg0 to handle the service request from multiple base stations:

- Msg ID

- BS or reader ID

Observation 2: The reader can send multiple subsequent A-IoT paging message signals associated with the same service request.

Proposal 2: The A-IoT paging message can implicitly or explicitly carry a unique service ID generated at the reader to convey the information to the device if it is the same service request. The device when activates and receives a paging message, it stores the service ID to identify the same service request and take decision on whether to send a service response or not.

Proposal 3: The estimated communication slot duration T can be implicitly or explicitly conveyed to the device through msg0 signal. This information is required for the device to decide on the sleep mode duration when the device is not scheduled by the BS to send msg3.

Observation 3: Device may receive multiple msg0 signal when the device is operating at listening mode or receiving mode.

Proposal 4: If the device receives multiple msg0 signals during listening mode or receiving mode within one communication slot, it responds to the initial msg0 signal only. The subsequently decoded msg0 signals within that slot are discarded by the device.

Observation 4: In 4-step CBRA, the device transmits msg1 to get resource from the base station to transmit msg3 to send the data such as, device ID, group ID, etc.

Proposal 5: The device can transmit the device energy status to the reader through msg1 to inform the energy status of the device, which the reader can use for prioritizing/ deprioritizing the device for scheduling.

Observation 5:  If the msg1 is not successfully received at the base station or the base station does not schedule the device in that communication slot or the device do not receive acknowledgement after transmitting msg3, the inventory process is considered unsuccessful.

Proposal 6: When the inventory process is not successful, it computes TSleep which is the remaining time to complete the communication slot and receive next msg0 paging signal. The device can go to sleep mode for TSleep, if TSleep > Tth, where Tth is the minimum sleep time required to save energy of the device.

Proposal 7: If reader sends multiple msg2 signals, the msg2 signal can carry an indication to convey the device if it is the last msg2 signal in this communication slot.

# 4. References

[1] TR 38.769, Study on solutions for ambient IoT (Internet of Things) in RAN, V2.0.0.