3GPP TSG-RAN WG2 Meeting #129										R2- 2500663

Athens, Greece, Feb 17th– 21nd, 2025

Agenda item		: 8.4.2

Title	: On IDLE/INACTIVE procedures for LP-WUS

Source				: Tejas Networks Limited

Document for		: Discussion and Decision

# Introduction

In RAN2#128 meeting, RAN2 agreed:

- FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session. - FFS on the following options - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - Option 3: The formula for UE_ID based PEI subgrouping is reused.

This contribution further addressed the procedure and configuration of LP-WUS for IDLE/INACTIVE modes.

# Discussion

## 2.1 LP-WUS Subgrouping:

In order to reduce false alarm for UEs in a particular PEI subgroup after network indicates UEs ins a LP-WUS subgroup to wake up. the LP-WUS subgroup ID should be different from PEI subgroup. In order to achieve this, the following formula for calculating LP-WUS subgroup ID is proposed:

LP-WUS Subgroup ID = (floor (UEID/(N*Ns*peiSubgroupNum)) mod lpwusSubgroupsNumUEID) + (lpwusSubgroupsNumPerPO – lpwusSubgroupsNumUEID). Where,

- UEID is derived from 5G-TMSI

- N is number of Paging Frames in a DRX cycle

- Ns is number of Paging occasion per PO

- peiSubgroupNum is total number of UEID based PEI subgroups, if UE does not support PEI and/or Network does not broadcast support PEI, this is taken as 1

- lpwusSubgroupsNumUEID is number of UEID based LP-WUS subgroup

- lpwusSubgroupsNumPerPO is total number of LP-WUS subgrouping supported in the cell.

Proposal 1: RAN2 to agree the following formula for LP-WUS subgroup ID calculation:

LP-WUS Subgroup ID = (floor (UEID/(N*Ns*peiSubgroupNum)) mod lpwusSubgroupsNumUEID) + (lpwusSubgroupsNumPerPO – lpwusSubgroupsNumUEID),

## 2.2 Impact of UE not supporting LR on all bands as MR

In RAN Plenary RAN#106, the issue of LR and MR not supporting same bands was discussed. The following way forward was agreed:

For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered. Detailed discussions and final solution selection is left up to WGs.

Solution 1: MR (re-)selects a cell to camp on which operates in same band as LR. No reselection needed after reception of LP-WUS. In Solution 1, the UE based on indication which can be part of SIB message, performs cell (re)selection to a cell which is operating on a band which is supported by both LR and MR. This could result in a case where crowding of UEs in one cell/band and might result in additional signaling by network to redirect the UEs for load balancing.

Solution 2: MR (re-)selects a cell to camp for reception of SIB, LP-WUS and paging but performs legacy reselection to perform further MR operation such as RACH. In solution 2. there can be additional delay after the reception of LP-WUS for UE to perform cell reselection if the current cell where UE received LP-WUS is not the best cell.

Solution 3: Network controlled redirection to a cell which is operating in band supported by LR. This solution has the minimal specification impact as the UE just follow legacy reselection procedure and also gives control to network on handling crowding of UEs across bands.

Proposal 2: RAN2 to agree that the issue of LR and MR not supporting same bands (intra-frequency case) is handled by network implementation with no spec impact.

## 2.3 Entry/Exit criteria for using LR:

The main advantage of using LR to monitor wake up indication is that MR can move to ultra-deep sleep/ deep sleep state to save power. The currently agreed entry and exit conditions only consider the signal qualities of LP-SS/SSB measured by LR/MR. Considering just the thresholds might lead to ping-pong effect of moving between use of LR and MR resulting in UE not having sufficient time to move to between sleep states. This can even result in adverse effect as there is power penalty associated with transition between sleep states. In order to overcome this, we propose to reuse the similar principle used in Rel16 relaxed measurement criteria where there is a time period configured for which the evaluation criteria need to be satisfied to consider the condition as fulfilled.

Proposal 3: RAN2 to consider evaluation time period for which the entry/exit conditions needs to be satisfied to switch between LR/MR.

## 2.4 LO and PO association

RAN1 in R1#118bis agreed to discuss the following options for determining the association of LO with a PO:

- Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset

The drawback with Option 1 is that it considers a single offset value which has to consider the worst case delay value of all the UEs in the cell. Since the NW will not have the knowledge of the UEs currently within its coverage, it will not be able to pick an optimal offset value. So picking an offset value becomes an issue. One more drawback if the NW picks a smaller offset value is that there will be additional delay incurred by the UEs which had wake-delay greater than the offset value as they will miss the associated PO and will have to monitor the next valid PO. Alternatively, if the NW picks a larger offset value, the UE receiving the LP-WUS will be moved out of sleep state soon even if its LR has a smaller the minimum wake up delay.

Observation 1: NW indicating a single offset value (Option 1 from RAN1) has many drawbacks.

Considering Option 2: From the network perspective, there can be multiple UEs having different wake-up delays in the cell, so NW indicates multiple offset values from the reference PO/PF. The UE monitors an LO by selecting an offset value which is at least greater than or equal to the wake delay of the LR.

Proposal 4: RAN2 to consider NW broadcasting multiple offset as baseline and further study the alternatives within option 2.

## 2.5 Supporting LP-WUS for MBS group paging:

Group paging was designed as an efficient mechanism for notifying UEs in IDLE/INACTIVE mode which had already joined an MBS session about the start/resume of the session. The group paging uses TMGI as the group paging identity and is indented to notify all the UEs in the cell which has joined in the session. If the UE is using LR to monitor wake up indication for paging reception, the network has to signal LP-WUS group ID to wake up that UE which can cause all UEs which haven’t subscribed to any MBS session to also wake up. This also adds in additional load at network to signal all the LP-WUS group IDs which has at least one UE mapped to it and which has subscribed to the MBS session.

Proposal 5: RAN2 to study mechanism to handle LP-WUS notification for signaling wake up indication for group paging reception

# Conclusion

LP-WUS subgrouping:

Proposal 1: RAN2 to agree the following formula for LP-WUS subgroup ID calculation:

LP-WUS Subgroup ID = (floor (UEID/(N*Ns*peiSubgroupNum)) mod lpwusSubgroupsNumUEID) + (lpwusSubgroupsNumPerPO – lpwusSubgroupsNumUEID),

Impact of LR not supporting all bands as MR

Proposal 2: RAN2 to agree that the issue of LR and MR not supporting same bands (intra-frequency case) is handled by network implementation with no spec impact.

Entry/Exit criteria for using LR

Proposal 3: RAN2 to consider evaluation time period for which the entry/exit conditions needs to be satisfied to switch between LR/MR.

LO and PO association

Observation 1: NW indicating a single offset value between LO and PO (Option 1 from RAN1) has many drawbacks.

Proposal 4: RAN2 to consider NW broadcasting multiple offset values between a PO and LOs (Option 2 from RAN1) as baseline and RAN2 to further study how UE selects the offset value.

LP-WUS for Group paging:

Proposal 5: RAN2 to study mechanism to handle LP-WUS notification for signaling wake up indication for group paging reception