3GPP TSG-RAN WG2#129											        R2-2500664

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.9.3

Source: 	NEC

Title:	        Discussion on UL Capacity Enhancement for IoT-NTN

Document for:	 Discussion and Decision

- Introduction

In this contribution, we follow the RAN2#128 agreements and further discuss the FFS points for CB-msg3 EDT on the following topics:

- CB-msg3 resource configuration: RSRP thresholds, DSA window, and corresponding OCC impacts

- Contention resolution: single RNTI / multiple RNTIs, single PDCCH monitoring window / multiple PDCCH monitoring windows

- CB-msg4 design

- CB-msg3 EDT re-attempt and fallback handling

- Discussion

RAN1 has replied to RAN2 LS on OCC for CB-msg3 NPUSCH and confirmed that the OCC solution that RAN1 is working on might be applied to CB-msg3 NPUSCH format 1 single tone at least for OCC length 2, for the subcarrier spacing (s) for which CB-msg3 is supported when the power imbalance is small [1].

Considering OCC, Fig. 1 below illustrates the CB-msg3 EDT with multiple resources in the time, frequency, and code domains.

Fig. 1 CB-msg3 NPUSCH

  - CB-msg3 PUSCH resources configuration

For the legacy NB-IoT random access procedure, the number of RUs and TBS for EDT are defined in Table 16.3.3-2 and Table 16.3.3-3 in [2], and EDT-TBS-InfoList-NB-r15 in [3] as below:

Table 16.3.3-2: EDT TBS for Msg3 NPUSCH with edt-SmallTBS-Enabled set to 'true' [2]

| edt-TBS | edt-SmallTBS-Subset | Allowable TBS values |
| --- | --- | --- |
| 408 | not configured | 328, 408 |
| 504 | not configured | 328, 408, 504 |
| 504 | enabled | 408, 504 |
| 584 | not configured | 328, 408, 504, 584 |
| 584 | enabled | 408, 584 |
| 680 | not configured | 328, 456, 584, 680 |
| 680 | enabled | 456, 680 |
| 808 | not configured | 328, 504, 680, 808 |
| 808 | enabled | 504, 808 |
| 936 | not configured | 328, 504, 712, 936 |
| 936 | enabled | 504, 936 |
| 1000 | not configured | 328, 536, 776, 1000 |
| 1000 | enabled | 536, 1000 |

Table 16.3.3-3: MCS index for Msg3 NPUSCH and EDT [2]

| MCS Index | Number of RUs |  |  |
| --- | --- | --- | --- |
|  | edt-TBS = 328, 408, 504, or 584 | edt-TBS = 680 | edt-TBS = 808, 936, or 1000 |
| '011' | 3 | 3 | 4 |
| '100' | 4 | 4 | 5 |
| '101' | 5 | 5 | 6 |
| '110' | 6 | 8 | 8 |
| '111' | 8 | 10 | 10 |

EDT-TBS-InfoList-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF EDT-TBS-NB-r15

EDT-TBS-NB-r15 ::=	SEQUENCE {

edt-SmallTBS-Enabled-r15		BOOLEAN,

edt-TBS-r15						ENUMERATED {b328, b408, b504, b584, b680, b808, b936, b1000}

}

Observation 1: The candidate RU numbers for EDT are 3, 4, 5, 6, 8, and 10, with each RU number covering various TBSs in the range of 328 to 1000.

The duration of a RU in different SCS and tone configurations for frame structure type1 and NPUSCH format 1 are defined in Table 10.1.2.3-1 of [4] as below:

Table 10.1.2.3-1: Supported combinations of , , and  for frame structure type 1[4]

| NPUSCH format |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 | 7 |
|  | 15 kHz | 1 | 16 |  |
|  |  | 3 | 8 |  |
|  |  | 6 | 4 |  |
|  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1 | 4 |  |
|  | 15 kHz | 1 | 4 |  |

Observation 2: The duration of one EDT depends on the required number of RUs, the SCS, and the number of subcarriers.

As RAN2 has already agreed that CB-msg3 PUSCH resources are cell-specific [6], the SCS and the number of subcarriers are cell-specific. The duration of one CB-msg3 transmission depends on the required RU. For one particular CE level, the time duration of the CB-msg3 PUSCH resource should meet the required repetition number multiplex the single shot CB-msg3 duration. Then, a preconfigured resource for one CE level could support a maximum number of RUs for the CB-msg3 NPUSCH transmission.

The network could configure CB-msg3 PUSCH with different MCS, TBS, or RU numbers for different CE levels to enhance the overall performance. For example, 1) higher CE levels with smaller TBS, then lower MCS could be applied to enhance the coverage performance; 2) higher CE level with smaller RU number, then the CB-msg3 transmission delay could be better controlled as shown in Fig. 2, which is essential to the CB-msg3 feature, especially under GEO scenario.

Fig. 2 Different CE levels with different RU numbers and repetition numbers

Proposal 1: The network could configure different maximum TBSs for different CE levels; this optimizes the overall CB-msg3 EDT performance.

Proposal 2: For NB-IoT NTN, the network could configure different RU numbers for different CE level CB-msg3 NPUSCH resources to control the maximum TBS and the overall CB-msg3 EDT performance.

The legacy CE level determination is based on the RSRP threshold for PRACH (i.e. rsrp-ThresholdsPrachInfoList [3]). As the decoding SNR for a PRACH signal and a PUSCH are very different and have different coverage performances, the exact threshold for CB-msg3 should be different. The legacy threshold only determines 3 different CE levels with different repetition numbers. Under the CB-msg3 EDT scenario, the UE needs to know when to switch to the legacy EDT procedure when the link quality cannot support CB-msg3 EDT.

Proposal 3: Network configures separate RSRP thresholds for UEs to determine the CB-msg3 EDT CE level.

Proposal 4: Network configures the lowest RSRP threshold for UE to switch between CB-msg3 EDT and legacy EDT.

Based on the RAN1 response, the power imbalance is important for enabling CB-msg3 NPUSCH transmission with OCC. To narrow down the power imbalance, a smaller GAP between different RSRP thresholds could help the CB-msg3 NPUSCH transmission with OCC from different UEs within a proper power imbalance range.

Observation 3: The network could control the CB-msg3 EDT power imbalance between different UEs through the RSRP threshold configuration.

Proposal 5: The network configures, at least, two RSRP thresholds with a small RSRP gap for UEs to determine a CE level for CB-msg3 EDT with OCC.

Fig. 3 DSA CB-msg3 occasion window starting point

RAN2 agreed there is a configurable time window for DSA CB-msg3 occasion selection [6]. The window could start periodically or sporadically as illustrated in Fig. 3.

If the window starts periodically, the network has to configure the window length and the first starting point (e.g. same as the resource starting point). UEs will randomly choose n occasions from the m occasions in the window to do DSA CB-msg3 EDT.

If the window starts sporadically, the network has to configure the window length and UE will determine the window starting point based on the configured CB-msg3 occasions and data arrival time. UE will choose the first available CB-msg3 occasion as the window starting point and transmit the first CB-msg3 EDT, then choose the other (n-1) occasions from the left (m-1) occasions in the DSA window to finish the DSA CB-msg3 EDT.

Proposal 6: The window starting point for the DSA CB-msg3 occasion selection could be:

- Option 1: Periodically configured by the network

- Option 2: Sporadically determined by the UEs

Note: For Option 2, UE will choose the first available CB-msg3 occasion as the window starting point and transmit the first CB-msg3 EDT, then choose the other (n-1) occasions from the left (m-1) occasions in the DSA window to finish the DSA CB-msg3 EDT.

  - Contention resolution

For the DSA RNTI, it can be either a single RNTI or multiple RNTIs as illustrated in Fig. 4 below.

Fig. 4 Single RNTI or Multiple RNTIs for DSA

Regardless of single RNTI or multiple RNTIs are used, multiple UEs may select the PUSCH resources associated with the same RNTI. Then, all these UEs will monitor the same RNTI, similar to Msg2 of 4-step RACH and MSGB of 2-step RACH. CB-Msg4 from multiple UEs with the same RNTI may be multiplexed into one MAC PDU, such that CB-Msg4 may be efficiently delivered.

Proposal 7: CB-msg4 for multiple UEs monitoring the same RNTI could be multiplexed into one MAC PDU for efficient delivery.

For multiple RNTIs for one DSA window, the RNTI can be determined based on the UE selection of the jth CB-msg3 Occasion, which is illustrated as option 1. The Msg4 PDCCH will first filter by the RNTI corresponding to the chosen CB-msg3 occasion. For any further contentions resolution, the UE has to decode and check the Msg4 PDSCH. RAN2 has agreed that the maximum replica number is 4 [6], thus the worst case is one UE has to detect four different RNTIs to find the right Msg4 PDCCH. If the network successfully decodes the CB-msg3 in the first replica transmission, the UE could stop monitoring Msg4 PDCCH with other RNTIs. As the RNTI already resolves partial or all of the contentions, the contention resolution in PDSCH is reduced. The decoding complicity of the PDSCH can be reduced. As the decoding reliability of a control channel is always higher than the data channel, option 1 ensures the miss detection rate via the PDCCH detections.

For a single RNTI for one DSA window, the RNTI can be determined based on the UE selection of the ith DSA window, which is illustrated as option 2. The Msg4 PDCCH will firstly filter by the RNTI corresponding to the chosen CB-msg3 DSA window. For any further contentions resolution, the UE has to decode and check the Msg4 PDSCH. As all the CB-msg3 EDT in one DSA window share one RNTI, the UE has to decode the PDSCH to resolve the contentions in one CB-msg3 DSA window. Due to the multiplexing of different UEs from one DSA window in one Msg4 PDSCH message, the Msg4 PDSCH payload size of option 2 will be multiple of the payload size of option 1. The PDSCH decoding complexity grows up as well as the Msg4 miss detection rate. Plus, in some Msg4 monitoring window configurations, a UE might need to detect the PDSCH several times and bring more decoding complexity.

For a single RNTI for one DSA window, the RNTI can also be determined based on the UE selection of the kth CE Level or just related to CB-msg3 EDT, which is illustrated as option 3. Then all the contention resolutions have to be solved by the Msg4 PDSCH. Then the PDSCH decoding complexity is higher than option 2 as well as the miss detection rate. However, option 3 simplify the RNTI design and the PDCCH detection.

Observation 4: Single RNTI solutions mainly or fully rely on PDSCH decoding to solve the contention resolution, while multiple RNTI solutions mainly rely on PDCCH decoding. The decoding complexity and decoding reliability of the PDCCH and PDSCH are different. Different options have different impacts on the UE power consumption and CB-msg3 contention resolution reliability.

Proposal 8: RAN2 asks RAN1 whether the following RNTI solutions for DSA CB-msg3 EDT response are acceptable in the UE decoding complexity and UE miss detection rate perspectives:

- Option 1: RNTI determined by the jth CB-msg3 occasion transmitting the CB-msg3 EDT

- Option 2: RNTI determined by the ith DSA window transmitting the CB-msg3 EDT

- Option 3: RNTI determined by the kth CE Level transmitting the CB-msg3 EDT or the CB-msg3 EDT behavior

RAN2 has agreed that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) and FFS when the response window(s) is/are started (or restarted) and stopped.

For NB-IoT, only type-B half-duplex FDD operation is supported [4]. In this type of operation, guard periods (GP, illustrated in Fig. 5), each referred to as a half-duplex guard subframe, are created by the UE by

-	not receiving a downlink subframe immediately preceding an uplink subframe from the same UE, and

-	not receiving a downlink subframe immediately following an uplink subframe from the same UE.

Fig. 5 Type-B half-duplex FDD

The legacy RA Response window starts at the subframe that contains the end of the last preamble repetition plus X subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined based on the used preamble format and the number of NPRACH repetitions[5].

For DSA, if the CB-msg3 EDT Response window starts at the subframe that contains the end of the last NPUSCH repetition plus X subframes plus UE-eNB RTT, it might 1) conflict with the terminal’s UL subframes with a small X value; or 2) introduce additional latency with a large X value as illustrated in Fig. 6.

Fig. 6 Legacy response window

Observation 5: The legacy response window design has issues with half-duplex FDD subframe conflicts in DSA CB-msg3 EDT.

For DSA, one response window could start with:

- each replica transmission

- each DSA window

When the window starts with each replica transmission, the relation between the CB-msg3 occasion and the response window can be illustrated in Fig. 7.

To prevent the misdetection of the Msg4 due to the type-B half-duplex FDD constraint, each CB-msg3 occasion should have at least with periodicity larger than the UE-eNB RTT + response window size + guard period. Under the GEO scenario, the UE-eNB RTT would be above 480 ms. This means for the system the delay of one CB-msg3 EDT could be more than 2 s even without counting the duration of each CB-msg3 occasion in different CE levels. Starting the response window with each replica transmission is obvious against the design target of CB-msg3 EDT to cutoff unnecessary delay in RTT level.

Fig. 7 The Response window starts with each replica transmission

Observation 6: The response window starts with each replica transmission will result in a huge system delay which is against the working target of CB-msg3 EDT to cutoff the RTT level delays.

When the window starts with each DSA window, the relation between CB-msg3 occasion and the response window can be illustrated in Fig. 8.

Fig. 8 Response window starts with each DSA window

The window starts with each DSA window having no constraint to the CB-msg3 occasion periodicity within one DSA window. Each CB-Msg4 occasion can be configured close to each other. For the system, the delay of one CB-msg3 EDT could be one UE-eNB RTT (480 ms), the duration of each CB-msg3 occasion in different CE levels, guard period, and potential processing time. There are no constraints on the periodicity of the DSA window, as one UE continuously transmits CB-msg3 EDT in the continuous DSA window is out of the scope of CB-msg3 EDT.

Proposal 9: For each DSA CB-msg3 EDT, there is only one response window and the window starts with the corresponding DSA window.

FFS: The exact window starting point and the window length.

  - CB-msg4 delivery

For CB-msg4, besides Contention Resolution ID, for CP-CIoT optimizations, if there is NAS signalling or DL data, RRCEarlyDataComplete message including NAS signalling or DL data can be transmitted to the UE. For UP-CIoT optimizations, RRCConnectionRelease message and possible DL data can be transmitted to the UE. If CB-msg4 from multiple UEs are multiplexed into one MAC PDU, considering there may be DL data besides RRC message for some UEs, CB-msg4 will be very large and the multiplexing is very complex. Besides, this multiplexing can also cause some decoding delay. Hence, RAN2 should discuss whether to support multiplexing from RRC message and/or DL data from multiple UEs. If RRC message and/or DL data is included in CB-msg4, RAN 2 also should discuss RRC message and/or DL data of how many UEs can be included in CB-msg4.

- Proposal 10: RAN2 to discuss whether RRC message and DL data can be included in CB-msg4 considering limited TB size and decoding latency.

- Proposal 11: If RRC message and/or DL data are included in CB-msg4, RRC message and/or DL data of how many UEs can be included.

Even If RRC message and/or DL data are allowed to be included in CB-msg4, considering limited TB size and decoding latency, it is possible not all UE’s RRC message and/or DL data are included in CB-msg4. If RRC message and/or DL data for some UEs are not included in CB-msg4, once contention resolution is successful, the UE may further receive RRC message and/or DL data if exists. Moreover, NW may would like to move UE into RRC_CONNECTED mode. For both cases, C-RNTI should be included in CB-msg4. Once the contention resolution succeeds, the UE will monitor PDCCH scrambled by the indicated C-RNTI in the subsequent time similar to MSGB of 2-step RACH procedure.

- Proposal 12: C-RNTI could be included in CB-msg4.

  - Failure handling and fallback

The behavior of the UE needs to be defined if it fails to receive the corresponding contention resolution/response in the response window, similar to the 2 or 4-step RACH procedure. Firstly we could allow the UE to re-attempt CB-msg3 transmission for a configured number of times before declaring complete failure or moving to next step. Whether and how to implement power ramping for CB-msg3 re-attempt could be further investigated. Secondly, The network must control and distribute the load over the CB-PUSCH resources, especially for the re-attempts. For instance, when the load is temporarily too high and NW has detected high collision, the network could instruct the UE to apply a backoff for upcoming re-attempt over CB-msg3 resources to avoid that almost the same set of UEs repeatedly re-attempt at the same time and fail.

- Proposal 13: NW configures a maximum number of times a UE can re-attempt CB-msg3 transmission on the same CE.

- Proposal 14: Msg4 could include a backoff indication for UE to determine the backoff value for re-attempt.

If CB-msg3 transmission completely fails after maximum time of re-attempt, the UE can fall back to the normal RACH-based solution, similar to the 2-step RACH falling back to 4-step RACH. In addition, it might be possible for NW to ask UEs to fallback to legacy RACH procedure e.g., when the NW detects high load and would like to offload UEs from re-attempting over CB-POs.

- Proposal 15: UE falls back from CB-msg3 to 4-step RACH procedure after the maximum times of re-attempts or a fallback MAC CE is received.

- Conclusion

This contribution provides our observation and proposals for UL capacity enhancement as listed below:

Observation 1: The candidate RU numbers for EDT are 3, 4, 5, 6, 8, and 10, with each RU number covering various TBSs in the range of 328 to 1000.

Observation 2: The duration of one EDT depends on the required number of RUs, the SCS, and the number of subcarriers.

Observation 3: The network could control the CB-msg3 EDT power imbalance between different UEs through the RSRP threshold configuration.

Observation 4: Single RNTI solutions mainly or fully rely on PDSCH decoding to solve the contention resolution, while multiple RNTI solutions mainly rely on PDCCH decoding. The decoding complexity and decoding reliability of the PDCCH and PDSCH are different. Different options have different impacts on the UE power consumption and CB-msg3 contention resolution reliability.

Observation 5: The legacy response window design has issues with half-duplex FDD subframe conflicts in DSA CB-msg3 EDT.

Observation 6: The response window starts with each replica transmission will result in a huge system delay which is against the working target of CB-msg3 EDT to cutoff the RTT level delays.

Proposal 1: The network could configure different maximum TBSs for different CE levels; this optimizes the overall CB-msg3 EDT performance.

Proposal 2: For NB-IoT NTN, the network could configure different RU numbers for different CE level CB-msg3 NPUSCH resources to control the maximum TBS and the overall CB-msg3 EDT performance.

Proposal 3: Network configures separate RSRP thresholds for UEs to determine the CB-msg3 EDT CE level.

Proposal 4: Network configures the lowest RSRP threshold for UE to switch between CB-msg3 EDT and legacy EDT.

Proposal 5: The network configures, at least, two RSRP thresholds with a small RSRP gap for UEs to determine a CE level for CB-msg3 EDT with OCC.

Proposal 6: The window starting point for the DSA CB-msg3 occasion selection could be:

- Option 1: Periodically configured by the network

- Option 2: Sporadically determined by the UEs

Note: For Option 2, UE will choose the first available CB-msg3 occasion as the window starting point and transmit the first CB-msg3 EDT, then choose the other (n-1) occasions from the left (m-1) occasions in the DSA window to finish the DSA CB-msg3 EDT.

Proposal 7: CB-msg4 for multiple UEs monitoring the same RNTI could be multiplexed into one MAC PDU for efficient delivery.

Proposal 8: RAN2 asks RAN1 whether the following RNTI solutions for DSA CB-msg3 EDT response are acceptable in the UE decoding complexity and UE miss detection rate perspectives:

- Option 1: RNTI determined by the jth CB-msg3 occasion transmitting the CB-msg3 EDT

- Option 2: RNTI determined by the ith DSA window transmitting the CB-msg3 EDT

- Option 3: RNTI determined by the kth CE Level transmitting the CB-msg3 EDT or the CB-msg3 EDT behavior

Proposal 9: For each DSA CB-msg3 EDT, there is only one response window and the window starts with the corresponding DSA window.

FFS: The exact window starting point and the window length.

- Proposal 10: RAN2 to discuss whether RRC message and DL data can be included in CB-msg4 considering limited TB size and decoding latency.

- Proposal 11: If RRC message and/or DL data are included in CB-msg4, RRC message and/or DL data of how many UEs can be included.

- Proposal 12: C-RNTI could be included in CB-msg4.

- Proposal 13: NW configures a maximum number of times a UE can re-attempt CB-msg3 transmission on the same CE.

- Proposal 14: Msg4 could include a backoff indication for UE to determine the backoff value for re-attempt.

- Proposal 15: UE falls back from CB-msg3 to 4-step RACH procedure after maximum times of re-attempts or a fallback MAC CE is received.

- References

[1] R1-2410895

[2] TS 36.213

[3] TS 36.331

[4] TS 36.211

[5] TS 36.321

[6] RAN2#128 chairman notes.