3GPP TSG-RAN WG2#129												    R2-2500665

Athens, Greece, Feb. 17th – 21st 2025

Agenda Item:	8.8.3

Source: 	NEC

Title:	        Uplink Capacity Enhancement for NRNTN

Document for:	 Discussion

- Introduction

Based on limited RAN1 progress, we discuss possible RAN2 aspects related to OCC and provide our views.

- Discussion

As below, RAN1 agreement made in last meeting [1]:

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. - RAN1 assumes that RAN4 does not define new UE requirements for CFO.

Inter-slot OCC with OCC length 2 and OCC length 4 for PUSCH has been supported in RAN1. Then in RAN2, we will discuss signalling issues related to OCC, such as OCC signalling configuration and OCC indication.

As we know, OCC has been supported for PUCCH. The related configuration information in [2], such as OCC length and OCC index, is below:

Then for PUSCH, OCC configuration of PUCCH above may be a baseline. Considering the different mechanisms for CG type 1, CG type 2 and DG, we shall discuss them separately. For CG type 1, since all information including time/frequency domain resources are configured by RRC message, OCC related parameters should also be configured by ConfiguredGrantConfig, such as OCC length and OCC index.

Proposal 1: For CG type 1, OCC length and OCC index can be provided to the UE by ConfiguredGrantConfig.

For CG type 2, in the current specification, some parameters are configured by ConfiguredGrantConfig, such as periodicity and repetition number. And some time/frequency domain resources related information are provided to the UE by DCI. Then for OCC related parameters, OCC length can be configured to the UE by dedicated signalling. Regarding OCC index, the following two options may be considered:

- Option 1: OCC index is configured to the UE by dedicated signalling

- Option 2: OCC index is provided to the UE by DCI

The above option 2 has very good flexibility compared to option 1.

Besides, for DG, similar to CG type2, OCC length can be configured to the UE by dedicated signalling and OCC index can be provided to the UE by one of the above two options.

Proposal 2: For DG and CG type 2, OCC length can be configured to the UE by dedicated signalling. For OCC index, the following two options may be considered:

- Option 1: OCC index is configured by dedicated signalling

- Option 2: OCC index is provided by DCI

The corresponding TPs for OCC related parameter configuration for CG type 1 and CG type 2 are provided in TP #1 and TP #2 of Annex, and the TPs for DG are provided in TP #3 and TP #4 of Annex based on the different options above.

- Conclusion

In summary, we would like to share our view with following proposals:

Proposal 1: For CG type 1, OCC length and OCC index can be provided to the UE by ConfiguredGrantConfig.

Proposal 2: For DG and CG type 2, OCC length can be configured to the UE by dedicated signalling. For OCC index, the following two options may be considered:

- Option 1: OCC index is configured by dedicated signalling

- Option 2: OCC index is provided by DCI

- References

[1] Chair notes RAN1#119.

[2] TS 38.331 i40.

- Annex

TP #1 (CG type 1 and CG type 2 (option 1))

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                       OPTIONAL,   -- Need S

cg-DMRS-Configuration               DMRS-UplinkConfig,

...

[[

disableCG-RetransmissionMonitoring-r18  ENUMERATED {true}                                           OPTIONAL,   -- Need R

nrofSlotsInCG-Period-r18            INTEGER (2..32)                                                 OPTIONAL,   -- Need R

uto-UCI-Config-r18                      SEQUENCE {

nrofBitsInUTO-UCI-r18               INTEGER (3..8),

betaOffsetUTO-UCI-r18               INTEGER (0..31),

...

}                                                                                                   OPTIONAL    -- Need R

]]

[[

occ-r19                               SEQUENCE {

occ-Length-r19                 ENUMERATED {n2,n4}                                            OPTIONAL, -- Need R

occ-Index-r19                  ENUMERATED {n0,n1,n2,n3}                                      OPTIONAL  -- Need R

}

]]

occ-Index Indicates the orthogonal cover code index.

occ-Length Indicates the orthogonal cover code length.

TP #2 (CG type 1 and CG type 2 (option 2))

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                       OPTIONAL,   -- Need S

cg-DMRS-Configuration               DMRS-UplinkConfig,

...

rrc-ConfiguredUplinkGrant           SEQUENCE {

timeDomainOffset                    INTEGER (0..5119),

timeDomainAllocation                INTEGER (0..15),

...

[[

...

applyIndicatedTCI-State-r18         ENUMERATED {first, second, both, spare1}                               OPTIONAL    -- Need R

]]

[[

occ-Index-r19                  ENUMERATED {n0,n1,n2,n3}                                      OPTIONAL  -- Need R

]]

}

[[

disableCG-RetransmissionMonitoring-r18  ENUMERATED {true}                                           OPTIONAL,   -- Need R

nrofSlotsInCG-Period-r18            INTEGER (2..32)                                                 OPTIONAL,   -- Need R

uto-UCI-Config-r18                      SEQUENCE {

nrofBitsInUTO-UCI-r18               INTEGER (3..8),

betaOffsetUTO-UCI-r18               INTEGER (0..31),

...

}                                                                                                   OPTIONAL    -- Need R

]]

[[

occ-Length-r19                 ENUMERATED {n2,n4}                                            OPTIONAL, -- Need R

]]

occ-Index Indicates the orthogonal cover code index.

occ-Length Indicates the orthogonal cover code length.

TP #3 (option 1)

PUSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPUSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

txConfig                                ENUMERATED {codebook, nonCodebook}                                  OPTIONAL,   -- Need S

...

[[

occ-r19                               SEQUENCE {

occ-Length-r19                 ENUMERATED {n2,n4}                                            OPTIONAL, -- Need R

occ-Index-r19                  ENUMERATED {n0,n1,n2,n3}                                      OPTIONAL  -- Need R

}

]]

}

occ-Index Indicates the orthogonal cover code index.

occ-Length Indicates the orthogonal cover code length.

TP #4 (option 2)

PUSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPUSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

txConfig                                ENUMERATED {codebook, nonCodebook}                                  OPTIONAL,   -- Need S

...

[[

occ-Length-r19                 ENUMERATED {n2,n4}                                            OPTIONAL, -- Need R

]]

}

occ-Length Indicates the orthogonal cover code length.