3GPP TSG-RAN WG2 #129	R2-2500667

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.6.2

Source: 	Ericsson

Title:  	Discussion on remaining FFSs for inter-CU LTM

Document for:	Discussion and Decision

# Introduction

In this contribution we are planning to address the remaining FFSs for inter-CU LTM according to the RRC running CR in [1].

# Discussion

## Terminology

One of the open issues in the RRC running CR for LTM and CLTM in [1], but also in the other specifications impacted by this work item is what terminology to use for LTM and conditional LTM. Since the differences between an LTM candidate cell configuration and a conditional LTM candidate configuration is just the configuration of CLTM execution conditions, it would be good to use “LTM” as general terminology for common aspects of intra-CU, inter-CU LTM, and CLTM, and use “CLTM” just for those aspects which apply only to conditional LTM. According to this, we propose:

Use “LTM” for common aspects of intra-CU LTM, inter-CU LTM, and CLTM whereas “CLTM” is used only for aspects which apply only to conditional LTM.

Delete from the RRC running CR for inter-CU LTM and CLTM the following editor’s note: Editor’s Note: FFS whether we use “LTM” for both LTM and CLTM (CLTM will only be used when something applies only to conditional LTM).

## Use of legacy reestablishPDCP for SRBs

In the context of inter-CU LTM, the following has been agreed for the handling of the PDCP entity when a security key change is executed for the SRBs:

Agreements from RAN2#128:

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

However, a similar discussion happened when LTM was specified during Rel-18 and the outcome, for the handling of the RLC entity when a L2 reset is performed, has been that the legacy reestablishRLC flag is used.

Agreement from RAN2#125:

- Use reestablishRLC in the LTM candidate cell configurations for RLC entities serving SRBs.

Our understanding is that the Rel-19 agreement was taken by having CPAC in mind, but for intra-CU LTM a different approach was taken in Rel-18. Since the behavior in such a case is similar for both the PDCP and RLC entity when a security key change is performed, it would be good to have a unified solution and re-use the legacy re-establishment flag, same as it was agreed in Rel-18.

Rel-18 agreements for intra-CU LTM are more applicable than sticking to the baseline of subsequent CPAC.

Thus, we propose:

When a security key change is needed for an LTM cell switch execution procedure, the legacy reestablishPDCP field is used for the SRBs to indicate the re-establishment of the PDCP entity.

Delete from the RRC running CR for inter-CU LTM and CLTM the following editor’s note: Editor’s Note: FFS whether for the SRBs we use the field reestablishPDCP within the LTM candidate configuration (same as for the RLC re-establishment in Rel-18).

## Relationship between Rel-18 L2 reset IDs and Rel-19 security change IDs

For Rel-19, since security key change is supported for LTM, a new set of IDs within the LTM candidate configuration has been agreed, which are used to indicate whether the PDCP entity should be re-established or not (in addition to whether the security should be changed).

Agreement from RAN2#127:

If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Something that need to be clarified is how the new Rel-19 IDs interact with the existing Rel-18 IDs which indicate whether a L2 reset is needed or not. In principle, when a security key change is needed, the PDCP is re-established but also the RLC entity. In this case, even if we have different IDs to indicate the handling of the PDPC and RLC entity, network is perfectly capable of setting the two IDs consistently and make sure that the correct actions are enforced at the UE. Therefore, the UE will fist check the Rel-19 IDs and then the Rel-18 IDs (as it is currently captured in the RRC running CR in [1]). According to this, we propose:

The UE first check the Rel-19 IDs to determine whether the security needs to be changed and the PDCP entity re-established, and then it checks the Rel-18 IDs to determine whether the RLC entity needs to be re-established.

Delete from the RRC running CR for inter-CU LTM and CLTM the following editor’s notes: Editor’s Note: FFS about the relationship between the Rel-18 IDs used for the L2 reset and the Rel-19 IDs used for the security key change/PDCP re-establishment and Editor’s Note: FFS is the field ltm-ReferenceConfiguration can be part of ltm-ConfigSCG in case it containes both MCG and SCG configurations.

## How to signal the reference configuration in inter-SN LTM

For the case when a UE is configured with one or more inter-SN LTM candidate configuration, as agreed by RAN2, the MN is responsible for the generation of the reference configuration:

Agreements from RAN2#127:

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

According to the agreement, it is also clear that the inter-SN LTM candidate cell configuration can be delivered to the UE only via the MN. In the RRC running CR in [1], such SN LTM configuration has been modelled via the new field ltm-ConfigSCG (which is of type LTM-Config). Therefore, it make sense that the reference configuration for the one or more inter-SN LTM candidate configuration would be included within the ltm-ConfigSCG field. According to this, we propose:

The reference configuration related to inter-SN LTM is included within the new ltm-ConfigSCG field.

Delete from the RRC running CR for inter-CU LTM and CLTM the following editor’s note: Editor’s Note: The interaction about when the UE receives an ltm-ReferenceConfiguration in ltm-Config and ltm-ConfigSCG is FFS.

## Configuration and release of inter-SN LTM at the SN

In one of the very first RAN2 meeting, we have agreed that an inter-CU LTM candidate configuration can be configured either at the MN or at the SN. This basically means that one of the two nodes need to make a decision about whether to configure an inter-CU LTM candidate configuration and need to inform the other node. Usually, from the beginning on when DC was specified for NR, the MN is responsible to specify restrictions at the SN and here the same logic can be applied. Since this information is UE-specific, following what has been done so far this information should be included within the inter-node messages used for DC. Therefore, we propose:

The MN indicates to the SN whether it can configure an inter-SN LTM candidate configuration via the inter-node messages.

Delete from the RRC running CR for inter-CU LTM and CLTM the following editor’s note: Editor’s Note: FFS whether the indication whether the SN is allowed to configure an inter-CU LTM configuration is provided in the Xn signaling.

As for the case whether the SN is allowed to configure an inter-SN LTM candidate configuration, it would make sense to support also the case in which it is indicated that the inter-SN LTM candidate configuration is released. This is because this can help the other node to decide whether to configure inter-CU LTM or not. According to this, we propose:

Support the indication on when inter-CU LTM is released by the SN and this is indicated in the inter-node RRC messages.

Delete from the RRC running CR for inter-CU LTM and CLTM the following editor’s note: Editor’s Note: FFS whether the SN can signal to the MN what has released inter-CU SN LTM candidate cells.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Rel-18 agreements for intra-CU LTM are more applicable than sticking to the baseline of subsequent CPAC.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Use “LTM” for common aspects of intra-CU LTM, inter-CU LTM, and CLTM whereas “CLTM” is used only for aspects which apply only to conditional LTM.

Proposal 2	Delete from the RRC running CR for inter-CU LTM and CLTM the following editor’s note: Editor’s Note: FFS whether we use “LTM” for both LTM and CLTM (CLTM will only be used when something applies only to conditional LTM).

Proposal 3	When a security key change is needed for an LTM cell switch execution procedure, the legacy reestablishPDCP field is used for the SRBs to indicate the re-establishment of the PDCP entity.

Proposal 4	Delete from the RRC running CR for inter-CU LTM and CLTM the following editor’s note: Editor’s Note: FFS whether for the SRBs we use the field reestablishPDCP within the LTM candidate configuration (same as for the RLC re-establishment in Rel-18).

Proposal 5	The UE first check the Rel-19 IDs to determine whether the security needs to be changed and the PDCP entity re-established, and then it checks the Rel-18 IDs to determine whether the RLC entity needs to be re-established.

Proposal 6	Delete from the RRC running CR for inter-CU LTM and CLTM the following editor’s notes: Editor’s Note: FFS about the relationship between the Rel-18 IDs used for the L2 reset and the Rel-19 IDs used for the security key change/PDCP re-establishment and Editor’s Note: FFS is the field ltm-ReferenceConfiguration can be part of ltm-ConfigSCG in case it containes both MCG and SCG configurations.

Proposal 7	The reference configuration related to inter-SN LTM is included within the new ltm-ConfigSCG field.

Proposal 8	Delete from the RRC running CR for inter-CU LTM and CLTM the following editor’s note: Editor’s Note: The interaction about when the UE receives an ltm-ReferenceConfiguration in ltm-Config and ltm-ConfigSCG is FFS.

Proposal 9	The MN indicates to the SN whether it can configure an inter-SN LTM candidate configuration via the inter-node messages.

Proposal 10	Delete from the RRC running CR for inter-CU LTM and CLTM the following editor’s note: Editor’s Note: FFS whether the indication whether the SN is allowed to configure an inter-CU LTM configuration is provided in the Xn signaling.

Proposal 11	Support the indication on when inter-CU LTM is released by the SN and this is indicated in the inter-node RRC messages.

Proposal 12	Delete from the RRC running CR for inter-CU LTM and CLTM the following editor’s note: Editor’s Note: FFS whether the SN can signal to the MN what has released inter-CU SN LTM candidate cells.

# References

R2-25xxxxx- Running RRC CR for inter-CU and conditional LTM, Ericsson, RAN2#129, February 2025.