3GPP TSG-RAN WG2 #129	R2-2500669

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.6.4

Source: 	Ericsson

Title:  	Discussion on DC and remaining FFSs for CLTM

Document for:	Discussion and Decision

# Introduction

In the following contribution we discuss the support of DC for CLTM and some remaining FFSs which has been captured in the RRC running CR for LTM and CLTM in [1].

# Discussion

## Support of DC for conditional LTM

In the last RAN plenary, it was discussed whether to support conditional LTM in DC scenarios and, if this was the case, in which particular scenarios. If we go back to Rel-18, when LTM was specified RAN2 supported the coexistence of LTM with DC but only according to some restrictions. In fact, LTM with DC was supported only according to the following:

- Only intra-CU LTM is supported at the MN and SN

- LTM is configured independently at the MN and SN

- In case, of LTM at the MN, the SN can either be kept or released

- PCell change with PScell change with LTM is not possible

- No security change is supported with LTM

These are the principles that we took into account in Rel-18 and, since the scenarios which are going to be supported for conditional LTM are basically the same what we have done for the normal LTM, a straightforward way to support conditional LTM in DC scenarios would be to re-use the following principles. According to this, we propose:

Conditional LTM is supported with DC according to the following principles (same as in Rel-18 for intra-CU LTM):

  - Only intra-CU conditional LTM is supported at the MN and SN

  - Conditional LTM is configured independently at the MN and SN

  - In case of a conditional LTM cell switch at the MN, the SN can either be kept or released

  - PCell change with PSCell change is not supported with conditional LTM

  - No security change is supported with conditional LTM

FFS whether NR-DC enhancements for inter-CU LTM which are applicable for intra-CU LTM can also be considered.

## Number of CLTM configurations

Conditional LTM can be seen as an enhancement of the LTM feature specified in Rel-18 and in line with this it would be good to discuss how the legacy LTM candidate configurations relate to Conditional LTM candidate configuration. We see at least two options:

- Option 1:  A maximum of 8 conditional and non-conditional LTM candidate configurations can be configured by the network. This means that the 8 LTM candidate configurations which have been specified in Rel-18 is the total number of both non-conditional (Rel-18 LTM) and conditional LTM candidate configurations.

- Option 2: A maximum of 8 conditional LTM candidate configurations can be configured by the network, which are independent from the 8 non-conditional LTM candidate configurations.

Either option work but of course both have pro and cons. For instance, with Option 1 there is less flexibility in configuring LTM and CLTM as the number of candidate configuration will be limited. On the other side, Option 2 give more flexibility in configuring LTM and CLTM, but it would require extensive effort in adapting the existing procedure that have been specified for LTM since Rel-18. Since in Rel-19 the time is limited to finish the objective, in order to limit the standardization efforts, it would be nice to re-use what has been done for LTM in Rel-18. According to this, we propose:

A maximum number of 8 candidate configurations (LTM plus CLTM candidate configurations) can be configured.

The LTM-Candidate IE is re-used to model an CLTM candidate cell configuration.

Delete the following editor’s node from the RRC running CR: Editor’s Note: FFS whether the 8 is the number of LTM plus CLTM candidate configurations or we have 8 LTM candidate configurations and 8 CLTM candidate configurations.

## Beam for the L1 events evaluation

One issue that was discussed last meeting was for the case on how many beams that should be considered for comparison and decision on when to execute conditions.

Proposal 11: RAN2 to consider at least one of the following approaches for event evaluation based on multiple beams: - Approach1) UE derives a consolidated quality of multiple beams of candidate cell based on a method similar to the cell quality derivation as specified in 5.5.3.3a of 38.331 and use the consolidated result for evaluation CLTM execution condition. - Approach2) Network configures the minimum number of good candidate cell beams for CLTM event (event 3/4/5), say N. If N is configured for Event CLTM, and UE considers the CLTM execution condition to be met if at least N beams of the candidate cell satisfy the CLTM condition respectively. The value range of N is {1, 2, 3, 4}. - Will revisit it next meeting.

On the one hand, some companies think that methods similar to cell quality derivation (Approach 1 above) from L3 should be considered where cell-level measurements are considered. Another approach (Approach 2 above) that was proposed during the meeting is that the network configures how many beams that should be considered for the evaluation.

Our view is, that L3 measurements are based on cell, and from that point it makes sense to apply a cell quality aspect for the measurements. But L1 measurements considers a beam, and for that reason a single beam should be considered. Otherwise, the risk is that a number of non-optimal beams generate a cell switch. Worth noting is that with the “N beams to consider” approach, the network can of course apply N=1 to achieve that behavior, but it seems like unnecessary configuration in our view. This is also in line with what was decided within the event-triggered L1 measurements objective, and thus it make sense to apply the same logic here. Therefore, for the evaluation of L1 event the UE shall consider only one beam (i.e., any beam) of an LTM candidate cell and the indicated beam of the serving cell.

Considering more beams than the best beam, could result in poor cell switch decisions for the UE.

If a cell quality aspect is the wanted behavior, then L3 measurements are better suited.

For the evaluation of L1 events, the UE considers only one beam (i.e., any beam) of an LTM candidate cell and the indicated beam of the serving cell.

## Support of DG within a CLTM candidate configuration

With the agreement from RAN2#127bis, it is possible for the network to allocate a configured grant in the candidate cell. However, whether a dynamic grant can be used is still open.

RAN2#127bis:
For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

Supporting only the configuration of the CG for the first UL transmission for a CLTM candidate configuration represents a big burden for the network. In this case, the network needs to reserve resource for very long time, or anyway for a time which is uncertain, since the LTM cell switch for the case of CLTM will be triggered autonomously by the UE. To ease the burden on the network the use of the dynamic grant would be particularly helpful. This means that the UE can send an SR in case a RRCReconfigurationComplete message is not enough, and the network would reply with a DG (this is the same we have for LTM in Rel-18). Therefore, we propose:

For RACH-less conditional LTM, dynamic grant is supported for the first UL transmission (i.e., UE sends an SR and network reply with a DG).

## Beam selection for the CLTM cell switch

A further open issue to be addressed when the UE performs a conditional LTM cell switch execution procedure, is what beam of the target cell to select. Indeed, this is different from LTM where the beam is received within the LTM cell switch MAC CE. Without clarifying this aspect, the network may take a long time to detect the UE on one of its beams and this may cause the conditional LTM cell switch execution procedure to fail or be delayed.

This problem could be partially solved by capturing explicitly what beam the UE should select for a conditional LTM candidate configuration when a conditional LTM cell switch execution needs to be performed. A simple rule which could be agreed is that the UE should select, as first priority, a TCI state of the conditional LTM candidate configuration (to which the switch needs to be done) which is currently active. Another solution, if there is a configured grant within the conditional LTM candidate configuration, the UE, as a second priority, could select a TCI state which is related to any of the reference signal which are included in the configured grant configuration. According to this, we propose:

If a RACH-less conditional LTM cell switch is executed, the UE select which TCI state to use according to the following priorities:

  - As a first priority, the UE selects a TCI state of the conditional LTM candidate configuration which is activated.

  - As a second priority, the UE selects a TCI state which is related to the reference signal received in the PDCCH order for the early UL synchronization.

  - As a third priority, the UE selects a TCI state which is part of a configured grant configuration (if present) of the conditional LTM candidate configuration.

  - As a fourth priority, the UE selects a TCI state up to its implementation.

## Handling of the TA in CLTM

When comparing the mechanism for TA handling between non-conditional and conditional LTM some things become apparent:

- For LTM, the TA value is delivered by the network, which is also responsible for its validity. This means that when the TA value is received within the LTM cell switch MAC CE the UE will consider such TA value always valid.

- For conditional LTM, the network delivers the TA value to the UE before a conditional LTM cell switch execution and this means that the check on the validity of the TA value falls to the UE.

- In legacy operation with the serving cell, if the TA value is not valid anymore the UE has solutions to inform the network that a new TA value is needed (e.g., UE will trigger the random access procedure to the serving cell).

- In case of conditional LTM, once the TA validity has expired, the UE has no other choice to perform a random access procedure if a conditional LTM cell switch execution needs to be performed (unless the network triggers the early UL sync procedure again).

Also, consider the following agreements in RAN2:

RAN2#128 agreements:

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

According to this analysis, one issue that needs to be also addressed (and which is different from LTM) is how to maintain the validity of the received TA value. In LTM, since it is the serving DU which delivers the TA within the LTM cell switch MAC CE, RAN2 has agreed that it is also the serving DU which check the validity of the TA value before sending it to the UE. However, since in CLTM the UE receives the TA value before a CLTM cell switch procedure may be triggered, in this case it would make sense that the UE checks and maintain the validity of the received TA. Therefore, we propose:

For CLTM, the validity of the TA value received from the serving DU is maintained by the UE with the following principles:

  - When a TA value is received, the UE starts a Time Alignment Timer (TAT) which is specific for the CLTM candidate configuration to which the received TA value belongs. How the TAT value is signaled is FFS.

  - If the TAT has expired before the execution of a CLTM cell switch, the UE executes a RACH-based CLTM cell switch.

  - if the TAT is still running upon the execution of a CLTM cell switch, the UE executes a RACH-less CLTM cell switch and the TAT is not re-started.

  - If a new TA value is received before the execution of a CLTM cell switch, the UE starts or restarts the TAT for the CLTM candidate configuration to which the TA value belongs.

An additional aspect to be considered is, once the TAT expires before a CLTM cell switch procedure is initiated, how a new TA value could be provided to the UE. In few words, this means how the early UL synchronization procedure can be restarted. According to this, there are basically two options for the network to know that a new TA value should be provided to the UE:

    - When to trigger again the early UL synchronization procedure (in case TA value at the UE is not valid anymore) is up to the network implementation.

    - In case TA value at the UE is not valid anymore, UE will inform the network that the TAT has expired, and it is up to the network whether to trigger a new early UL synchronization procedure.

For simplicity, for CLTM we can adopt solution 1 as is the simplest and basically with no standardization impact. Thus, we propose:

When to re-trigger the early UL synchronization procedure (in case TA value at the UE is not valid anymore) is up to network implementation.

A further aspect that needs to be considered is whether to support the extension of the TA validity by the UE. Relying solely on the network to trigger continuously the early UL synchronization procedure to restart the TAT of the TA of an CLTM candidate cell may not realistic, since the network doesn’t know the actual channel condition of the UE towards the CLTM candidate cell. Because of this, it would be good to specify also a UE solution for extending the validity of the TA. In this sense, different options can be considered but some of them are more promising than others. In particular, the following options can be considered:

- A solution similar to CG-SDT is used to extend the validity of the TA

- UE can indicate to the network when the TAT of a CLTM candidate cell has expired

- UE can estimate a new TA value by itself, if configured by the network

Therefore, we propose:

RAN2 to discuss whether UE-solutions to extend the validity of a TA value of a CLTM candidate cell are needed. If needed, RAN2 to select one or more of the following options:

  - A solution similar to CG-SDT is used to extend the validity of the TA

  - UE can indicate to the network when the TAT of a CLTM candidate cell has expired

  - UE can estimate a new TA value by itself, if configured by the network

## Configuration of simultaneous L1 and L3 execution conditions

During the discussion of the RRC running CR for LTM and CLTM, an issue discussed was whether the network is allowed to configure simultaneously L1 and L3 execution condition for the same CLTM candidate cell.

From the RRC running CR:

Editor’s Note: FFS whether L1 and L3 execution conditions can be configured simultaneously or not.

Generally speaking, there is no benefit for the network to configure simultaneously L1 and L3 execution condition for the same CLTM candidate cell and we fail to understand why such a configuration should be allowed. Further, to simplify the implementation in RRC, it would be reasonable, at least in Rel-19, to configure the same type of execution condition (L1 or L3) to all CLTM candidate cell. Thus, we propose:

Network configures either L1 execution conditions or L3 execution condition for a CLTM candidate cell.

Network configures the same execution conditions (either L1 or L3) for all configured CLTM candidate cells.

Delete from the RRC running CR the following editor’s note: Editor’s Note: FFS whether L1 and L3 execution conditions can be configured simultaneously or not.

## Multiple CLTM candidate cell fulfilling execution conditions

There is a probability that more than one CLTM candidate cell fulfil the execution conditions to trigger a CLTM cell switch execution procedure. This scenario is not something new, as the same could happen for the case of CHO. About CHO, the following has been captured in TS 38.331, clause 5.3.5.13.5:

1> if more than one pair of triggered PCell and associated triggered PSCell exist: 2> select one of the triggered PCell(s) and the associated triggered PSCell(s) as the selected cells for conditional reconfiguration execution; 1> else if only one pair of triggered PCell and associated triggered PSCell exists: 2> consider the triggered PCell and the associated triggered PSCell as the selected cells for conditional reconfiguration execution; 1> else if more than one triggered cell exists: 2> select one of the triggered cells as the selected cell for conditional reconfiguration execution; 1> else: 2> consider the triggered cell as the selected cell for conditional reconfiguration execution; <EXCLUDED TEXT FOR SIMPLIFICATION> NOTE: If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution.

As we can see, for CHO and CPAC in case multiple cells fulfill the execution conditions, it is up to the UE implementation to select one on which to perform the conditional reconfiguration execution. According to this, the same principles can be applied also for the case of CLTM. Thus, we propose:

In the case of two different cells fulfilling the execution conditions at the same time, it is up to UE implementation which cell to switch to.

## Fast Recovery

The fast recovery procedure for RLF is supported for Rel-18 LTM, where the LTM cell switch is only allowed to be intra-CU. Since conditional LTM in Rel-19 is also limited to the intra-CU case, we see no problem is supporting the fast RLF recovery, as most of the aspects specified in Rel-18 can be actually re-used. Thus, we propose:

Fast RLF recovery is supported for conditional LTM.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Considering more beams than the best beam, could result in poor cell switch decisions for the UE.

Observation 2	If a cell quality aspect is the wanted behavior, then L3 measurements are better suited.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Conditional LTM is supported with DC according to the following principles (same as in Rel-18 for intra-CU LTM):

a.	Only intra-CU conditional LTM is supported at the MN and SN

b.	Conditional LTM is configured independently at the MN and SN

c.	In case of a conditional LTM cell switch at the MN, the SN can either be kept or released

d.	PCell change with PSCell change is not supported with conditional LTM

e.	No security change is supported with conditional LTM

Proposal 2	FFS whether NR-DC enhancements for inter-CU LTM which are applicable for intra-CU LTM can also be considered.

Proposal 3	A maximum number of 8 candidate configurations (LTM plus CLTM candidate configurations) can be configured.

Proposal 4	The LTM-Candidate IE is re-used to model an CLTM candidate cell configuration.

Proposal 5	Delete the following editor’s node from the RRC running CR: Editor’s Note: FFS whether the 8 is the number of LTM plus CLTM candidate configurations or we have 8 LTM candidate configurations and 8 CLTM candidate configurations.

Proposal 6	For the evaluation of L1 events, the UE considers only one beam (i.e., any beam) of an LTM candidate cell and the indicated beam of the serving cell.

Proposal 7	For RACH-less conditional LTM, dynamic grant is supported for the first UL transmission (i.e., UE sends an SR and network reply with a DG).

Proposal 8	If a RACH-less conditional LTM cell switch is executed, the UE select which TCI state to use according to the following priorities:

a.	As a first priority, the UE selects a TCI state of the conditional LTM candidate configuration which is activated.

b.	As a second priority, the UE selects a TCI state which is related to the reference signal received in the PDCCH order for the early UL synchronization.

c.	As a third priority, the UE selects a TCI state which is part of a configured grant configuration (if present) of the conditional LTM candidate configuration.

d.	As a fourth priority, the UE selects a TCI state up to its implementation.

Proposal 9	For CLTM, the validity of the TA value received from the serving DU is maintained by the UE with the following principles:

a.	When a TA value is received, the UE starts a Time Alignment Timer (TAT) which is specific for the CLTM candidate configuration to which the received TA value belongs. How the TAT value is signaled is FFS.

b.	If the TAT has expired before the execution of a CLTM cell switch, the UE executes a RACH-based CLTM cell switch.

c.	if the TAT is still running upon the execution of a CLTM cell switch, the UE executes a RACH-less CLTM cell switch and the TAT is not re-started.

d.	If a new TA value is received before the execution of a CLTM cell switch, the UE starts or restarts the TAT for the CLTM candidate configuration to which the TA value belongs.

Proposal 10	When to re-trigger the early UL synchronization procedure (in case TA value at the UE is not valid anymore) is up to network implementation.

Proposal 11	RAN2 to discuss whether UE-solutions to extend the validity of a TA value of a CLTM candidate cell are needed. If needed, RAN2 to select one or more of the following options:

a.	A solution similar to CG-SDT is used to extend the validity of the TA

b.	UE can indicate to the network when the TAT of a CLTM candidate cell has expired

c.	UE can estimate a new TA value by itself, if configured by the network

Proposal 12	Network configures either L1 execution conditions or L3 execution condition for a CLTM candidate cell.

Proposal 13	Network configures the same execution conditions (either L1 or L3) for all configured CLTM candidate cells.

Proposal 14	Delete from the RRC running CR the following editor’s note: Editor’s Note: FFS whether L1 and L3 execution conditions can be configured simultaneously or not.

Proposal 15	In the case of two different cells fulfilling the execution conditions at the same time, it is up to UE implementation which cell to switch to.

Proposal 16	Fast RLF recovery is supported for conditional LTM.

# References

R2-25xxxxx- Running RRC CR for inter-CU and conditional LTM, Ericsson, RAN2#129, February 2025.