3GPP TSG-RAN WG2 Meeting #129	R2-2500670

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.5.2

Source:	Panasonic

Title:	On-demand SSB SCell operation

Document for:	Discussion/Decision

Introduction

In this contribution, we share our views on configuration of OD-SSSB and L3 measurement.

Discussion

  - OD-SSB configuration

Following was agreed in RAN2#127.

Agreements on OD-SSB SCell - No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. - Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Referring to RAN1 progress, multiple candidate values need to be configured by RRC. Whether the parameters are configured by reusing legacy RRC parameters or new RRC parameters was discussed.

Agreement in RAN1#118 For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement in RAN1#118bis For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters Agreement in RAN1#119 - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives. - Alt A: Same as for Case #1 - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

- RRC configuration of OD-SSB

For OD-SSB configuration, sCellConfigCommon and sCellConfigDedicated can support multiple candidate values of OD-SSB parameters. In RRC to initial configuration, sates (activation/ deactivation) are indicated.

- Activation/Deactivation by RRC and MAC

OD-SSB is used for L1/L3 measurements. We consider OD-SSB triggering timing is up to network implementation irrespective of UE situation. Even after the SCell is activated, on-demand SSB may also be further triggered and indicated by network, even if the “always-on” SSB in Case 2 is already available [1]. Therefore, independent signaling from Scell activation is also useful. For RRC, RAN2 agreed that no need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. SCell modification indicates activation/deactivation of OD-SSB. For MAC CE, the new MAC-CE for OD-SSB includes a bitmap for the SCell(s) with OD-SSB to be activated and the indication of the activated OD-SSB configuration ID for each SCell was proposed in RAN1#128 [2]. We support this proposal. OD-SSB configuration ID can indicate any parameter that RAN1 concludes to support multiple candidate values.

Proposal 1: sCellConfigCommon and sCellConfigDedicated support multiple candidate values of OD-SSB parameters.

Proposal 2: Scell modification indicates activation/deactivation of OD-SSB.

Proposal 3: The new MAC-CE for OD-SSB includes a bitmap for the SCell(s) with OD-SSB to be activated and the indication of the activated OD-SSB configuration ID for each Scell.

  - L3 RRM

In RAN2#127bis meeting, the L3 RRM framework on OD-SSB measurement has been discussed and two options are listed as follows.

Option1: Based on different measurement configuration when OD-SSB is transmitted. Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted.

LS on SSB adaptation from RAN4[3], RAN4 asks RAN2 to answer the SMTC impact question due to SSB adaptation.

In RAN1, always on SSB and OD=SSB are located on same frequency and different frequency are under discussion.

Agreement in RAN1#119 Down-select at least one of the following alternatives. - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster. Down-select at least one of the following alternatives. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. - Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

For same frequency case, a SMTC window could cover both always on SSB and OD-SSB. However, longer/dense SMTC window would restrict the network scheduling flexibility and would lead to more UE power consumption for measurement. Therefore, configuration of two SMTCs would be reasonable. It can support different periodicity for always on SSB and OD-SSB. In the current specification, up to 4 SMTCs can be configured. As similar as smtc2, smtc2-LP, smtc3list and smtc4list, additional smtc can be configured for OD-SSB. It could be additional UE capability. When UE has capability to support additional SMTC for OD-SSB, UE measures both always on SSB and OD-SSB. If UE doesn’t have this capability, SMTC is configured for one of among 1) always on SSB, 2) OD-SSB or 3) one SMTC cover both SSBs with longer/dense window.

For different frequency case, different SMTCs are necessary to measure different SSBs. When UE doesn’t have the capability to support additional SMTCs, one of always on SSB or OD-SSB should be measured at one time. When additional SMTC is supported, whether multiple SMTC can be active in same time need to be concluded. When one of SMTC is active in a time, SMTC patterns can be configured for not to be overlapped in time domain.

Proposal 4: For same frequency of always on SSB and OD-SSB, additional SMTC can be configured for OD-SSB depending on the UE capability.

Proposal 5: For different frequency of always on SSB and OD-SSB, additional SMTC can be configured for OD-SSB depending on the UE capability. When one of SMTC is active in a time, SMTC patterns are configured as not to be overlapped in time domain.

In addition, the support of adaptive SSB would also impact the SSB L3 measurement similarly as discussed above.

Proposal 6: L3 RRM framework on adaptive SSB measurement should be discussed.

Conclusion

- Based on the discussion, the following proposals are highlighted:

Proposal 1: sCellConfigCommon and sCellConfigDedicated support multiple candidate values of OD-SSB parameters.

Proposal 2: Scell modification indicates activation/deactivation of OD-SSB.

Proposal 3: The new MAC-CE for OD-SSB includes a bitmap for the SCell(s) with OD-SSB to be activated and the indication of the activated OD-SSB configuration ID for each Scell.

Proposal 4: For same frequency of always on SSB and OD-SSB, additional SMTC can be configured for OD-SSB depending on the UE capability.

Proposal 5: For different frequency of always on SSB and OD-SSB, additional SMTC can be configured for OD-SSB depending on the UE capability. When one of SMTC is active in a time, SMTC patterns are configured as not to be overlapped in time domain.

Proposal 6: L3 RRM framework on adaptive SSB measurement should be discussed.

Reference

[1] R1-2500489 Discussion on on-demand SSB SCell operation, Panasonic

[2] R2-2410255 On demand SSB handling, Nokia, Nokia Shanghai Bell

[3] R2-2409520	LS on SSB adaptation (R4-2416911; contact: Apple)	RAN4	LS