3GPP TSG-RAN WG2 Meeting #129	R2-2500671

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.5.3

Source:		Lenovo

Title:			RAN Plenary outcome and other remaining issues

Document for:	Discussion and Decision

# Introduction

Whether Cell A can transmit its own UL WUS config and then, at a later point, turn into a NES cell is found useful by some operators but some discussion on this will be necessary for its implementation. Besides, some basic agreements with regards to avoiding impact to legacy UEs and to receive SIB1 on-demand by R19 NES UEs are still required to be made. This document examines the related areas.

# Discussion

## Self-WUS Case

Whether Cell A can transmit its own UL WUS config and then, at a later point, turn into a NES cell is found useful by some operators. A proposal towards the same was discussed in RAN2#128 [2], but still no conclusion could be made:

R2-2410842	Considerations on OD-SIB1 feature	Deutsche Telekom, Vodafone, Lenovo, Fraunhofer IIS   	discussion	Rel-19

Proposal 1: Clarify that Cell A can also transmit the WUS configuration for itself in case it intends to become a SIB1-less cell.

Noted.

Discussion:

[IDC]: Specification will not specify cell A or NES cell, but since we already agreed NES cell can include its own WUS configuration, in principle agree with proposal 1. [Huawei]: Agree with IDC. [NTTDCM]: Having the clarification is good to other WGs since there are some confusion there.

[Qualcomm]: We should make sure the proposal 1 does not mean standalone case is supported. [Ericsson]: With the proposal, cell A needs to provide its own WUS configuration. [Nokia]: In addition, NES cell also needs to provide any neighboring cells’ WUS configuration (no restriction to neighboring cell’s NES cell). [Huawei]: We do not need to agree any further thing except what we already agreed. [Qualcomm]: Concerns on agreeing with the proposal. We already finished SI based on the terms and now we’re changing the terms. [Apple]: It was discussed in RAN1 and no conclusion was made. Would like to suggest to discuss in RAN.

This issue was further discussed in RANP, and a way forward was achieved in TSG RAN Meeting #106:

- RAN confirms that Case 2 also includes the case where a particular Cell A can switch to become a NES Cell. This means that Cell A can provide UL-WUS configuration for itself before becoming a NES cell. - No further optimizations will be considered for NES UEs for this scenario. - No further optimizations to handle non-NES capable UEs will be considered for this scenario. It is assumed that these UEs can be handled by careful deployment. - No WID update is required

RAN2 previously agreed that a Cell may provide SIB1 request WUS configuration for itself and on top RAN confirmed the case where a particular Cell A can switch to become a NES Cell. The question is after “becoming” NES Cell how long is the WUS configuration obtained previously considered valid?

Relevant to this discussion, RAN2 made the following agreement which affects this discussion:

RAN2#128 9. UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

If the above agreement (9. from RAN2#128) is implemented ‘as is’, leaving a (NES) cell even for a very short time shall force a UE to reacquire WUS configuration upon return – which is not only wasting UE battery but is also against network energy saving. A WUS configuration must be stable in real deployments and should not be changing every millisecond/ second. We therefore think that RAN2 should revert “valid in directly succeeding cell reselection” decision and rather agree to SI validity area as basis for determining WUS configuration validity.

Proposal 1: RAN2 considers reverting “valid in directly succeeding cell reselection” decision and agree to SI validity area as basis for determining WUS configuration validity.

The above however still does not solve the self-WUS case. In addition, if the current serving cell is not a Cell-A for the said NES Cell in question, even the SI validity area concept would not be of use. For this reason, RAN2 should think about a time validity of the WUS configuration enabling the UE to initiate request for on-demand SIB as long as the WUS configuration remains valid. If there would be an urgent/ important reason to change, e.g., due to emergency situation, likely the network will come out of energy saving mode and start transmitting SIB1 regularly.

Proposal 2: Besides SI validity area feature, time validity of an acquired WUS configuration should also be addressed to avoid too many SIB1 Re-Requests.

## SIB1 on-demand indication and barring legacy UEs

In RAN1#118 meeting following agreements were made:

Agreement For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

In RAN1#129 meeting following agreement was made:

Agreement On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, down-select from the following options: - Option 1: Use it to indicate frequency assistance information to search SSB for Cell A - FFS: Details on the range/granularity of the frequency assistance information - FFS: Potential impact to RAN3 - Option 2: Use it to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1 - Option 3: Above feature is not supported.

RAN1 agreements (copied above) use the MIB information to indicate legacy UEs that SIB1 is not being provided in the cell and they are still discussing if specific kSSB values >23/11 (for FR1 and FR2 respectively) can be used to indicate another GSCN where the legacy UE find SIB1. Except for kSSB values = 30/14, the other values in this range will lead legacy UEs to look for SIB1 “elsewhere” – whereas the NES on-demand SIB1 Cell should not be transmitting SIB1 at this “elsewhere” location – otherwise, how is it saving energy!

So, RAN2 should avoid agreements that make legacy UEs un-necessarily look for SIB1. When SIB1 is not being broadcasted regularly by a R19 NES Cell, only kSSB value 30 in NES MIB can bar legacy UEs without making them look for SIB1 transmission. In addition, kSSB value 30 in NES MIB will signal the R19 NES UEs that this cell provides SIB1 on-demand, without requiring first to find a Cell-A and erroneously barring the NES cell in the meanwhile.

Observation: Using kSSB value 30 in FR1 or 14 in FR2 in NES cell MIB is necessary to bar legacy UEs on a on demand SIB1 cell without making them look for SIB1 transmission when SIB1 is not being broadcasted.

Proposal 3: RAN2 restricts kSSB to values 30 in FR1 or 14 in FR2 in R19 on-demand SIB1 Cell MIB.

RAN2 needs to discuss how a R19 NES UE acquires SIB1 from a NES Cell. UE needs to check if SIB1 is currently broadcasted before it could go ahead and make SIB1 Request from the NES cell, as agreed in the last meeting:

8. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Therefore, if the ssb-SubcarrierOffset (as part of kSSB), is used for special purpose and can’t really provide a valid SSB Subcarrier offset, the kSSB should be provided along with the WUS configuration by the Cell A. We assume here that the pdcch-ConfigSIB1 can be used directly from the MIB of the NES cell i.e., RAN1 has no need to repurpose this IE.

Proposal 4: The kSSB value is provided in the new agreed SIB of Cell A along with already agreed WUS Configuration.

## SIB1 reception after making Request

RAN1 has agreed to the following with regards to on-demand SIB1 transmission:

Agreement The reference time point to determine the window starting time for on-demand SIB1 is based on the RAR window for UL WUS wherein UE successfully received a RAR. - FFS: Use starting or ending slot of the RAR window as reference time Agreement The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration. - Unit of the duration is in slot - FFS: Starting time

After having made the SIB1 request, UE just attempts receiving SIB1 broadcast as in the previous releases i.e., the UE can use SI-RNTI to decode PDCCH and use kSSB and pdcch-ConfigSIB1 received in MIB of the NES Cell.

Proposal 5: UE uses SI-RNTI to decode PDCCH in the time window signalled in the RAR for UL WUS to receive on-demand SIB1 of the NES Cell.

Proposal 6: KSSB value from Cell-A and the pdcch-ConfigSIB1 received in MIB of a NES Cell is used to receive on-demand SIB1 of the NES Cell.

Finally, since some of the proposals affect RAN1 progress, if agreed an LS to RAN1 should be sent to inform them of our progress.

Proposal 7: An LS to RAN1 should be sent to inform them of our progress.

# Conclusion

Remaining essential issues were discussed in the paper for the on-demand SIB1 topic. Following proposals are made:

Proposal 1: RAN2 considers reverting “valid in directly succeeding cell reselection” decision and agree to SI validity area as basis for determining WUS configuration validity.

Proposal 2: Besides SI validity area feature, time validity of an acquired WUS configuration should also be addressed to avoid too many SIB1 Re-Requests.

Observation: Using kSSB value 30 in FR1 or 14 in FR2 in NES cell MIB is necessary to bar legacy UEs on a on demand SIB1 cell without making them look for SIB1 transmission when SIB1 is not being broadcasted.

Proposal 3: RAN2 restricts kSSB to values 30 in FR1 or 14 in FR2 in R19 on-demand SIB1 Cell MIB.

Proposal 4: The kSSB value is provided in the new agreed SIB of Cell A along with already agreed WUS Configuration.

Proposal 5: UE uses SI-RNTI to decode PDCCH in the time window signalled in the RAR for UL WUS to receive on-demand SIB1 of the NES Cell.

Proposal 6: KSSB value from Cell-A and the pdcch-ConfigSIB1 received in MIB of a NES Cell is used to receive on-demand SIB1 of the NES Cell.

Proposal 7: An LS to RAN1 should be sent to inform them of our progress.

# References

[1]: RAN1 meeting minutes from RAN2#127

[2]: RAN1 meeting minutes from RAN2#118