3GPP TSG-RAN WG2 #129	R2-2500674

Athens, Greece, Feb. 17th – 21st, 2025

Source:		NEC

Agenda item:	8.10.2.2	CHO with candidate SCGs

Title:	MRO for CHO with candidate SCG(s)

Document for:		Discussion and decision

- Introduction

During last RAN2 meeting, the following agreements were made for MRO of CHO with candidate SCG(s)

Agreements - RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements. - Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

In this paper, we provide our further consideration on MRO for the CHO with candidate SCG(s).

- Discussion

Currently, the UE only logs the ID of the target PCell in RLF-report and SHR. In case of CHO with candidate SCG(s), it is natural that the UE also report the ID of the target PSCell of the CHO with candidate SCG(s) procedure, such that the network has complete information of the target cells.

Proposal 1. UE logs the ID of target PSCell of the CHO with candidate SCG(s) procedure in RLF-report and SHR.

For execution condition of one candidate cell, at most two triggering conditions can be configuration. Currently, the UE can store information related to trigger condition(s) for candidate PCell in measurement results for candidate PCells of RLF-report, i.e., the first fulfilled trigger condition, and the time gap between the fulfilment of the two trigger conditions. In this way, the network can further optimize the two trigger conditions associated with the PCell. At previous meeting, it was agreed that measurement results of PCells and PSCells is included in the RLF-report. For CHO with candidate SCG(s), it is beneficial that the UE also collects the information related to the two trigger conditions for the candidate PSCells, such that the network can optimize the trigger condition(s) of the candidate PSCells in case of CHO with candidate SCG(s).

Proposal 2. UE includes in the measurement result of the candidate PSCell in the RLF-report, information related to the triggering condition(s) for the candidate PSCell, which comprises

- the first fulfilled triggering condition for the candidate PSCell

- the time gap between the fulfilment of the two triggering conditions for the candidate PSCell

# 3. Conclusion

In this paper, we discuss on further consideration on the MRO for CHO with candidate SCG(s), and have the following proposals:

Proposal 1. UE logs the ID of target PSCell of the CHO with candidate SCG(s) procedure in RLF-report and SHR.

Proposal 2. UE includes in the measurement result of the candidate PSCell in the RLF-report, information related to the triggering condition(s) for the candidate PSCell, which comprises

- the first fulfilled triggering condition for the candidate PSCell

- the time gap between the fulfilment of the two triggering conditions for the candidate PSCell

# Reference

[1] Meeting report of RAN2 #128