3GPP TSG-RAN WG2 Meeting #129 	R2-2500676

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.2

Source:	Panasonic

Title:	Discussion on A-IoT paging

Document for:	Discussion

# Introduction

Recent RAN plenary meeting has approved Rel-19 work item for A-IoT (RP-243326), according to which the RAN2 scope has been set as follows.

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

This paper is intended to discuss some remaining issues on paging, including subsequent paging for the same service type, whether to indicate service type and skip response due to energy constraint.

# Discussion

## Subsequent paging for the same service

During the study item phase, it has been identified the necessity that reader sends multiple (subsequent) A-IoT paging messages that are associated with the same service request from CN. Further, it has been agreed that the A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. Based on this information the device determines whether to skip sending the response to paging (if the device has successfully responded to the same service before).

In the previous meeting, it is clarified that this information is an ID, while it needs to be further discussed in whether the ID is generated by the reader or by the CN. It needs to be further discussed the size of this information.

It is true that SA2 is discussing candidate solutions that involve providing an ID to distinguish different service from CN. This ID is referred to as session ID, service ID, or transaction ID in different candidate solutions [1]. One might think to reuse this ID for the purpose of avoiding duplicate response as mentioned above. However, this seems not a good idea due to the following reasons.

First of all, it is unclear whether this ID from CN is embedded in the paging message (and therefore visible to the device) or is a separate signaling visible to the reader only. Due to the expected long length of this ID, incorporating it into the paging message would increase the size of paging message and device cost unnecessarily. Therefore, the latter case (visible to reader only) would be a better option. After reader uses this ID to identify the same or different service request from CN, reader can generate a paging ID with a shorter length. One possibility would be how to generate the paging ID can be up to the reader. It can be based on part of the service ID from CN or independently from it, as long as the reader can avoid the paging ID collision in the air interface. Another possibility would be to specify how to shorten the ID from CN to a paging ID with a shorter length.

Proposal 1. RAN2 to discuss paging ID in the air interface without waiting for SA2 progress. The paging ID should be short length and generated by reader to assist the device to determine whether to respond to the paging or not.

Device behavior

Device needs to remember the paging ID (i.e. store it) after successful completion of the access procedure triggered by this paging. On the other hand, if the access procedure has failed, device would not store the corresponding paging ID (i.e. discard it). Then, whenever device receives a paging, it compares the paging ID with the one stored (if any). If the new paging has the same paging ID as the stored one, it means that device has successfully responded to the same service request before and therefore device should skip responding to avoid duplicated responses. Otherwise, if new paging has a different paging ID than the store one, or there is no stored paging ID in device, the device should respond to this paging. Note that the successful completion of access procedure can be known to the device explicitly (e.g. by acknowledgement indication) or implicitly (e.g. by a timer). This is a separate discussion within agenda 8.2.3 (A-IoT random access).

Another point is that device is not expected to respond to more than one paging simultaneously due to the complexity restriction. This means that, if there is ongoing access procedure triggered by one paging that device has responded and before the success or fail of the access procedure is determined, device is not required to respond to another paging, regardless of a new paging or subsequent paging.

Proposal 2. Device needs to maintain the corresponding paging ID in the memory after successful completion of the access procedure triggered by a paging.

Proposal 3. If a paging is associated with a paging ID that is the same as the one stored in the device, device ignores the paging; otherwise, device performs the requested procedure.

Timer associated with the paging ID

The paging ID need not be maintained by device for infinite time. After some time (e.g. all subsequent paging rounds associated with the same service request from CN have been completed), the paging ID can be released from the device’s memory. As another motivation to release the paging ID, the reader may need to reuse the previously allocated paging ID for new service request from CN, if the ID space is exhausted.

To ensure the same understanding of the validity time of the paging ID between reader and device, a validity timer can be associated with the paging ID. The timer can have a pre-configured value to cover a typical number of subsequent paging rounds associated with the same CN request. As long as timer is running, device does not need to respond to the paging with the same paging ID. After timer expires, device will release the stored paging ID. Note that the maximum value of the timer should be limited to the device power availability.

Different service requests from CN can be handled by reader in parallel, meaning that reader can trigger new service round (with one or multiple subsequent paging corresponding to new paging ID) before the previous service round (one or multiple subsequent paging corresponding to previous paging ID) has completed. In this case, the validity timer is per paging ID. It means that even if the device has stored a previous paging ID with timer still running, the device needs to respond to the paging with a new paging ID. It is FFS on how many paging IDs (and the associated timers) that a device can maintain. But from the complexity point of view, the baseline could be that device only needs to maintain one paging ID and one associated timer. In this case, after the device has successfully responded to the new paging, the device would overwrite the old paging ID with the new one and re-start the timer.

Proposal 4. Device maintains a validity timer for the stored paging ID. After timer expires, the paging ID is released.

Proposal 5. As a baseline, device needs to maintain only one paging ID and the associated validity timer.

Multi-reader scenario

During the study item phase, there was some discussion on whether to support the scenario that different readers may page the same device for same service request. It was concluded to further discuss this in work item phase.

In our view, it is impractical to assume that the coverage of different readers is always non-overlapping. In other words, it is inevitable that a device may be located in a multi-reader overlapping area. See Fig.1. as an example.

Fig.1. Multi-reader overlapping area

In this case, device may receive paging messages from different readers with the same or different service request from CN. Depending on the service requirements, it may be preferable for device to respond to one single reader only, or multiple readers (duplicated responses). For example, for inventory service, it may be sufficient for device to respond to one (“best”) reader and ignore the paging from other readers to save the energy and signaling overhead. However, for proximity determination, it is useful for the network to locate the device if the device responds to multiple readers. Therefore, network should be able to control whether a device should respond to single reader or multiple readers.

If the paging ID is generated by the reader (see proposal 1), different readers would generate paging ID independently. It is likely that different paging IDs will be used by different readers even if the paging is associated with the same CN request. If no other information to control the duplicated responses is included in paging except the paging ID, it is effectively requested that the device needs to respond to multiple readers. On the other hand, if network would like to control the device to avoid duplicated responses to multiple readers, reader ID can be included in paging. Similar to the paging ID, device now needs to maintain the reader ID after a successful access procedure (i.e. store it in the memory). If the new paging has a different reader ID, device should ignore the paging, regardless the same or different paging ID. In this way, the device is required to respond to one reader only. If the new paging has the same reader ID as the store one, device then compares the paging ID to decide whether it is the subsequent paging or new paging, follows the behavior in proposal 3.

Proposal 6. In additional to paging ID, a reader ID can be optionally indicated in paging message. Device needs to maintain the reader ID (if indicated) and paging ID in the memory after successful completion of the access procedure triggered by a paging.

Proposal 7. On deciding whether or not to respond to a paging, device compares the stored IDs with the IDs in the paging as follows:

| New Paging ID | New Reader ID | Device behavior | Intended scenario |
| --- | --- | --- | --- |
| Same as stored one | Not included | Not respond | Subsequent paging from same reader |
| Different than stored one | Not included | respond | device is requested to respond to multiple readers (assuming each reader generates paging ID independently) |
| Same as stored one | Same as stored one | Not respond | Subsequent paging from same reader |
| Different as stored one | Same as stored one | respond | New paging from same reader |
| Same or different | Different than stored one | Not respond | Device is requested to respond to one reader (assuming reader-device association is fixed) |

Note that the above proposal 7 assumes fixed reader-association. In reality, such association can change, for example, due to mobility of the device. Such aspect can be taken into account by the validity timer associated with the ID. See following proposal 8 which is similar to proposal 4.

Proposal 8. Device maintains a validity timer for the stored paging ID and reader ID. After timer expires, the paging ID and reader ID are released.

## Whether to indicate service/command type in paging

In the study item, there has been some discussion on whether the paging message should include the information to indicate service type/command type or to indicate whether there will be subsequent messages. The conclusion is to further discuss this in work item phase.

From the previous discussion, the motivation is to let device know whether device will expect further communication between reader, e.g. after successfully delivery of Msg3. We don’t see such need as device needs to monitor R2D anyway. More specifically, after sending Msg3, device will still monitor for R2D reception. If there is further need for communication, reader can send R2D to schedule further D2R. If there is no further data (e.g. inventory use case ends at Msg3), reader can send explicit feedback to confirm that Msg3 is successful. Or alternatively, a timer is defined after that no explicit feedback means success of Msg3. In both cases, device can know the procedure triggered by the paging is completed or not.

In the previous discussion, there was a view that although device may need to monitor R2D in any case, it is beneficial for device to know which ID to monitor. It is assumed that subsequent command (after Msg3) can address to AS ID, but paging is addressed to paging ID (e.g. a group ID). If device is not expecting subsequent command data after successfully completing Msg3, the device may monitor only paging ID; otherwise device need to monitor more IDs. However, it is not clear to us how the mentioned benefit can be achieved. RAN1 has agreed that PRDCH is the only physical channel for R2D communication. Although it is still open whether the addressing of device is carried by control part (if any) or data part of PRDCH, the device needs to identify any PRDCH by a “start indicator” and then start to receive PRDCH. Only after PRDCH is decoded, device can know the address (group ID or AS ID). Therefore, it seems no difference regarding the decoding effort between monitoring one ID or two IDs.

Proposal 9. There is no need for reader to explicitly indicate service/command type in the paging message.

## Skip response to paging due to energy constraint

In the study item phase, it has been discussed that if the device does not have sufficient energy to perform the remaining or follow-up procedure, the device can send energy status report to reader in a D2R message (e.g. MSG1, MSG3 and command response message). However, according to the WID, energy status reporting is not included in the scope (see section 1).

If device can identify lack of energy for the follow-up communication, instead of sending energy status report, another option is that the device can simply skip responding to the paging in the first place, since sending a D2R message itself also consumes non-negligible amount of energy and negatively impacts the energy harvesting at the device. Therefore, a more important aspect is that the A-IoT device should be allowed to skip responding if the device does not have sufficient energy to complete the remaining access procedure triggered by the paging. Whether the energy is sufficient can be judged by device implementation. This procedure can be illustrated using Fig.2. below.

Fig.2. Device skips responding based on judgement of its energy

Proposal 10. Device is allowed to skip responding if device judges its remaining energy cannot sustain to complete whole access procedure triggered by this A-IoT paging.

# Conclusion

In this paper, we have discussed some remaining issues on paging, including subsequent for the same service type, whether to indicate service type and skip response due to energy constraint. The proposals are as follows.

Proposal 1. RAN2 to discuss paging ID in the air interface without waiting for SA2 progress. The paging ID should be short length and generated by reader to assist the device to determine whether to respond to the paging or not.

Proposal 2. Device needs to maintain the corresponding paging ID in the memory after successful completion of the access procedure triggered by a paging.

Proposal 3. If a paging is associated with a paging ID that is the same as the one stored in the device, device ignores the paging; otherwise, device performs the requested procedure.

Proposal 4. Device maintains a validity timer for the stored paging ID. After timer expires, the paging ID is released.

Proposal 5. As a baseline, device needs to maintain only one paging ID and the associated validity timer.

Proposal 6. In additional to paging ID, a reader ID can be optionally indicated in paging message. Device needs to maintain the reader ID (if indicated) and paging ID in the memory after successful completion of the access procedure triggered by a paging.

Proposal 7. On deciding whether or not to respond to a paging, device compares the stored IDs with the IDs in the paging as follows:

| New Paging ID | New Reader ID | Device behavior | Intended scenario |
| --- | --- | --- | --- |
| Same as stored one | Not included | Not respond | Subsequent paging from same reader |
| Different than stored one | Not included | respond | device is requested to respond to multiple readers (assuming each reader generates paging ID independently) |
| Same as stored one | Same as stored one | Not respond | Subsequent paging from same reader |
| Different as stored one | Same as stored one | respond | New paging from same reader |
| Same or different | Different than stored one | Not respond | Device is requested to respond to one reader (assuming reader-device association is fixed) |

Proposal 8. Device maintains a validity timer for the stored paging ID and reader ID. After timer expires, the paging ID and reader ID are released.

Proposal 9. There is no need for reader to explicitly indicate service/command type in the paging message.

Proposal 10. Device is allowed to skip responding if device judges its remaining energy cannot sustain to complete whole access procedure triggered by this A-IoT paging.

# Reference

[1] 3GPP TR 23.700-13: Study on Architecture support of Ambient power-enabled Internet of Things