3GPP TSG-RAN WG2#129												R2-2500687

Athens, Greece, Feb. 17th – 21st 2025

Agenda Item:	8.8.6 LTE to NR NTN mobility

Source: 	NEC

Title:	        Redirection from E-UTRN TN to NR NTN

Document for:	 Discussion

# Introduction

In addition to support cell reselection from LTE TN to NR NTN, RAN2 has discussed redirection from LTE TN to NR NTN, and there are following controversial questions:

- Whether to support redirection from LTE TN to NR NTN, i.e. whether an NR NTN frequency can be configured as redirectedCarrierInfo in RRCConnectionRelease message

- whether UE behaviour should be clarified if NR NTN frequency can be configured as redirectedCarrierInfo in RRCConnectionRelease message

- Any extra assistance information should be provided to UE to support the redirection

- whether to support this with or without a new capability bit

In this contribution we share our view.

# Discussion

For mobility from E-UTRAN TN to NR NTN, idle mode cell reselection will be supported in Rel19. With the same deployment scenario, there would be connected mode UEs supporting both LTE TN and NR NTN. However, according to last RANP conclusion, connected mode handover will not be supported in Rel19. Redirection would be a last and simple solution to use while these UEs in connected mode move from LTE TN coverage into NR NTN coverage. Even though supporting redirection seems out of the WI scope, it would be nice to support redirection if not much specification change.

Observation: For LTE TN to NR NTN mobility, in addition to support cell reselection for idle mode UEs, it would be nice to support redirection for connected mode UEs if the specification can be kept to a minimum.

According to 36.331[3], in a LTE TN cell, NW could indicate a NR carrier for redirection:

RedirectedCarrierInfo ::= CHOICE { eutra ARFCN-ValueEUTRA, geran CarrierFreqsGERAN, utra-FDD ARFCN-ValueUTRA, utra-TDD ARFCN-ValueUTRA, cdma2000-HRPD CarrierFreqCDMA2000, cdma2000-1xRTT CarrierFreqCDMA2000, ..., utra-TDD-r10 CarrierFreqListUTRA-TDD-r10, nr-r15 CarrierInfoNR-r15, nr-r17 CarrierInfoNR-r17 }

And the corresponding field is as following:

redirectedCarrierInfo The redirectedCarrierInfo indicates a carrier frequency (downlink for FDD) and is used to redirect the UE to an E-UTRA or an inter-RAT carrier frequency, by means of the cell selection upon leaving RRC_CONNECTED as specified in TS 36.304 [4]. The value geran can only be included after successful security activation when UE is connected to 5GC.

Firstly, NW should only redirect a UE to a NR NTN frequency if the UE supports the NR NTN frequency.  Because UE could report its supported NR bands including NR NTN bands. This is known by NW with legacy capability procedure already.

Note that UE will perform cell selection rather than cell reselection to the indicated NR NTN frequency. According to NR cell selection procedure specified in 38.304, no extra assistance information is required for a UE to select a NR NTN cell.

Cell selection is performed by one of the following two procedures: a) Initial cell selection (no prior knowledge of which RF channels are NR frequencies): 1. The UE shall scan all RF channels in the NR bands according to its capabilities to find a suitable cell. 2. On each frequency, the UE need only search for the strongest cell, except for operation with shared spectrum channel access where the UE may search for the next strongest cell(s). 3. Once a suitable cell is found, this cell shall be selected. b) Cell selection by leveraging stored information: 1. This procedure requires stored information of frequencies and optionally also information on cell parameters from previously received measurement control information elements or from previously detected cells. 2. Once the UE has found a suitable cell, the UE shall select it. 3. If no suitable cell is found, the initial cell selection procedure in a) shall be started. NOTE: Priorities between different frequencies or RATs provided to the UE by system information or dedicated signalling are not used in the cell selection process.

This means that reselection to NR NTN capability, including reading SIB24 and SIB33 for NR NTN assistance information, is not pre-condition to support redirection to NR NTN:

If above observation on no need of assistance information for cell selection to NR NTN is correct, NW could redirect UE from LTE TN to NR NTN without specification change.

If above observation on need of assistance information for cell selection to NR NTN is incorrect, i.e. UE do need assistance information including satellite assistance information to cell select to NR NTN cell, there are two different alternatives to get this information:

- Alt1: NW provides this NR NTN assistance information in RRC release message via a new IE: this requires a capability bit, for NW to know if a UE will be able to decode the new IE and execute redirection to NR NTN.

- Alt2: UE reuses the assistance information provided in SIB24 and SIB33 for cell reselection purpose. Alt2 does not require new IE. However, reading assistance information from SIB24 and SIB33 to assist the cell reselection from LTE TN to NR NTN is a feature without capability signalling. If a UE support NR NTN band but not support cell reselection from LTE TN to NR NTN, it cannot read the corresponding assistance information provided in SIB24 and SIB33, UE behaviour is unknown

Proposal 1: RAN2 discuss if redirection from LTE TN to NR NTN can be executed without extra assistance information for cell selection to indicated NR NTN carrier

Proposal 2a: Support redirection from LTE TN to NR NTN without specification change, if answer to p1 is Yes

Proposal 2b: Redirection from LTE TN to NR NTN is not pursued in Rel19, if answer to p1 is No

# Conclusion

In summary, we would like to share our view with following observation and proposals:

Observation: For LTE TN to NR NTN mobility, in addition to support cell reselection for idle mode UEs, it would be nice to support redirection for connected mode UEs if the specification can be kept to a minimum.

Proposal 1: RAN2 discuss if redirection from LTE TN to NR NTN can be executed without extra assistance information for cell selection to indicated NR NTN carrier

Proposal 2a: Support redirection from LTE TN to NR NTN without specification change, if answer to p1 is Yes

Proposal 2b: Redirection from LTE TN to NR NTN is not pursued in Rel19, if answer to p1 is No

# References

[1] R2-2409672	Discussion on remaining issue for LTE TN to NR NTN IDLE mode mobility	CATT

[2] R2-2410439	On LTE TN to NR NTN reselection using redirection in Release message	 Nokia,

[5]	RP-240846 New WID: Inter RAT mobility support from E-UTRAN TN to NR-NTN

[3]	3GPP TS 36.331 E-UTRAN RRC

[4]	3GPP TS 36.304 User Equipment (UE) procedures in idle mode