3GPP TSG-RAN WG2 Meeting #129	R2-2500689

Athens, Greece, 17 - 21 Feb, 2025

Source: 	Huawei, HiSilicon, Turkcell

Title: 	Discussion on DL coverage enhancements

Agenda Item:	8.8.2

Document for:	Discussion and decision

- Introduction

In the past meetings, RAN2 discussion on DL coverage enhancements is focused on SMTC enhancements, DTX/DRX, and cell barring:

- On SMTC enhancements, RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list).

- On DTX/DRX, RAN2 has agreed only two states (on/off) will be considered, further details are pending on RAN1 replies on the RAN2 LS [2] (RAN1 has provided answers on Q1 and Q4 in [3]).

- On cell barring, RAN2 has agreed to introduce new barring bit in SIB1 to bar UEs supporting DL coverage enhancement if it will be justified to use existing barring bit to bar UEs not supporting DL coverage enhancement.

In this contribution, we will provide further analysis on the remaining issues.

- Discussion

  - SMTC adjustment

According to current specification, the SMTCs (configured by smtc1 and smtc4list) on the same frequency are linked to a shared periodicity and duration, differing only in their SMTC offsets. Based on RAN1 progress, the SSB periodicities can range from 20ms to 160ms:

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms.

To allow different SSB periodicities for neighbour cells on the same frequency, the existing signalling for smtc4list needs to be extended. This could involve supporting multiple smtc4lists, each having its own SSB periodicity and duration, or introducing a separate SMTC periodicity list to correspond with the existing SMTCs in smtc4list.

Proposal 1: The network should be allowed to configure different periodicities for the multiple SMTCs on a single frequency.

In beam hopping scenarios, neighbouring cell SSBs may arrive at different time. As proposed in [4] increasing the number of SMTCs configured per frequency and enabling a location-based UE autonomous SMTC selection could address this issue.

Based on the agreed RAN1 simulation assumptions, up to 1058 beam footprints could be served by a single satellite, with only 16 (in Set 1-2) or 106 (in Set 1-1 or 1-3) active simultaneously. Before deciding on whether SMTC enhancement is needed, we need to figure out how the beam hopping is performed across different footprints.

Assuming 4 beams per cell, there are about 265 cells per satellite. With a beam diameter of 50 km, as per the RAN1 assumptions, the signal quality across different beam and cells may vary. As a result, neighbour cell measurements are still necessary.

- Issue 1: Whether the 4 beams within the cell will be considered as active beam simultaneously?

Figure 1 Beam hopping pattern within one cell

If so, only 4 cells will be simultaneously active cells out of 265 cells, making the issue for unaligned neighbouring cell SSBs will be even more severe;

If not, there will be 16 cells simultaneously active out of the 265 cells.

- Issue 2: Whether the active cells are selected as a cluster or scattered randomly among the footprints?

If the cells are clustered, the active cells will be neighbouring, and the existing 4 SMTCs per frequency should be sufficient. However, if the cells are scattered, it is likely that neighbour cell SSBs will not fall into the same SMTC window, and the current SMTC framework may not function properly. Either some assistance information needs to be provided by the UE to properly select the neighbour cells to measure, or alternative methods can be considered.

Figure 2 Beam hopping pattern across different cells

From our perspective, the answers to the above issues should be provided by RAN1 before deciding on any RAN2 solutions. And a draft LS is provided in the annex.

Proposal 2: Send an LS to RAN1 asking the following questions:

Q1: Whether the SSB beams within the cell will be considered as active beam simultaneously?

Q2: Whether the active cells are selected as a cluster or scattered randomly among the footprints?

- Issue 3: SMTC/gap configuration

In R17, the NW configures SMTC/gap based on UE assistance of PDD (propagation delay difference) reporting:

Figure 3 Existing SMTC/gap configuration based on PDD reporting

However, the PDD reporting mechanism does not work in cases of beam hopping in R19, as the PDD between serving cell and neighbouring cells will always be zero, given that they are served by the same satellite.

In previous meeting, RAN2 discussed associating SMTCs with reference locations, allowing for UE autonomous selection of SMTCs based on proximity to the closest reference location. In our understanding, for UEs in RRC connected, it should be ensured that both the UE and the NW have the same understanding of SMTC. This is important because SMTCs not only impact gap configuration but may also impose scheduling restriction. RAN2 could consider UE assistance information reporting, similar to PDD reporting, while still leaving the SMTC/gap configuration to the NW.

Proposal 3: RAN2 should consider UE assistance reporting (e.g. based on reference location), while the responsibility for configuring SMTCs and gaps to the UE remains with the network.

On extending the number of SMTCs, it should be noted that the SMTC configuration should be aligned with gap configuration (for the measurements that require gaps). Increasing the number of gaps will add UE complexity and cause performance degradation considering the data transmission will be stopped during measurement gaps. In R17, RAN4 has discussed increasing the number of gaps configured to the UE, and the combinations are listed in TS 38.133:

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap | Per-FR2 measurement gap | Per-UE measurement gap |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| 6 | 2 | 0 | 0 |
| 7 | 0 | 2 | 0 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 are only applicable when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, UE Rx-Tx time difference measurement and PRS-RSRPP measurement defined in TS 38.215 [4]. [Note 2]: For UE capable of [Concurrent Pre-MG], up to 2 measurement gap patterns can be configured as Pre-MG in one FR, regardless of whether they are per-UE or per-FR configuration. Otherwise, the gaps can only be configured as Gap(s) configured via GapConfig without suffix or Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17. Note 3: In Gap Combination Configuration Id #0, #1, #6, #7, one per-FR measurement gap in an FR can be associated to measure PRS for any RSTD, PRS-RSRP, PRS-RSRPP, RSCP, RSCPD and UE Rx-Tx time difference measurement defined in TS 38.215 [4] provided that UE supports independentGapConfigPRS-r17. |  |  |  |

It can be observed that, if PRS is not considered, at most 2 gaps will be configured to the UE concurrently. We believe the number of maximum gaps should not be increased further, and it is the network’s responsibility to configure appropriate SMTC/gaps based on necessary UE assistance information.

Proposal 4: The number of configured SMTCs/gaps will not be increased further.

2.2 DTX/DRX

In the latest RAN plenary meeting, the revised WID was approved [1]. The system level DL coverage enhancement now only contains SSB periodicity extension, and the description related to R18 NES has been removed:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

Based on the WID, cell/beam DTX/DRX is no longer in the WID scope, and RAN2 discussion should only be initiated if DTX/DRX enhancement is proven to be essential as a result of SSB periodicity extension.

Proposal 5: RAN2 should no longer discuss DTX/DRX enhancement unless it is proven to be essential due to the extension of SSB periodicity.

2.3 Cell Barring

In previous meetings, RAN2 has achieved some agreements on cell barring, with the pre-requisites of “If we need to bar UEs…” “If it turns out that there is a need to bar UEs …”

Agreements @RAN2#128: 1. If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.) Agreements @RAN2#127bis: 1. If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE) 2. If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit) Agreements @RAN2#127: 1. If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

In RAN1#119 meeting, the following conclusion has been made:

Observation Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan. Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

RAN1 agreement is focused on the case of “periodicity larger than 160ms”. In RAN plenary #106 meeting, SSB periodicity larger than 160ms was ruled out.

When SSB periodicity is equal to 160ms, according to the above RAN1 conclusion, some UEs may be able to operate in the cell while some other UEs may not: “Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan.”.

From RAN2 perspective, the safest approach is to introduce the signalling for barring legacy UEs and leave it to NW implementation to decide whether to enable the cell barring mechanism. Furthermore, the cell barring mechanism only applies to SSB periodicity extension, while other enhancements (e.g. possible SMTC enhancements, link level enhancements like SIB1/msg4 PDSCH repetition) will not cause backward compatibility issues.

Proposal 6: It is up to NW implementation to set cellBarredNTN-r17 to barred to bar UEs not supporting R19 extended SSB periodicity. A new field cellBarredNTN-r19 (value is set to barred or notBarred) is introduced for future use. UEs supporting extended SSB periodicity ignore cellBarredNTN-r17 if the new cellBarredNTN-r19 is present, and follow cellBarredNTN-r17 if the new cellBarredNTN-r19 is absent.

- Conclusion

This contribution discussed the DL coverage enhancements in Rel-19 NR NTN. Proposals are summarized as follows:

Proposal 1: The network should be allowed to configure different periodicities for the multiple SMTCs on a single frequency.

Proposal 2: Send an LS to RAN1 asking the following questions:

Q1: Whether the SSB beams within the cell will be considered as active beam simultaneously?

Q2: Whether the active cells are selected as a cluster or scattered randomly among the footprints?

Proposal 3: RAN2 should consider UE assistance reporting (e.g. based on reference location), while the responsibility for configuring SMTCs and gaps to the UE remains with the network.

Proposal 4: The number of configured SMTC/gaps will not be increased further.

Proposal 5: RAN2 should no longer discuss DTX/DRX enhancement unless it is proven to be essential due to the extension of SSB periodicity.

Proposal 6: It is up to NW implementation to set cellBarredNTN-r17 to barred to bar UEs not supporting R19 extended SSB periodicity. A new field cellBarredNTN-r19 (value is set to barred or notBarred) is introduced for future use. UEs supporting extended SSB periodicity ignore cellBarredNTN-r17 if the new cellBarredNTN-r19 is present, and follow cellBarredNTN-r17 if the new cellBarredNTN-r19 is absent.

- References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, THALES, CATT

- R2-2405762, LS on DL coverage enhancements, CMCC

- R1-2407538, Draft Reply LS on DL coverage enhancements, CMCC

- R2-2410804, Downlink coverage enhancement SMTC impacts, Sequans Communications

- Annex – draft LS to RAN1

# 1. Overall Description:

Regarding DL coverage enhancement, RAN2 has discussed the impacts on L3 measurements, and would like to ask RAN1 the following questions on beam hopping:

Q1: Whether the SSB beams within the cell will be considered as active beam simultaneously?

Q2: Whether the active cells are selected as a cluster or scattered randomly among the footprints?

# 2. Actions:

To RAN1

ACTION: 	RAN2 kindly asks RAN1 to consider the above question and provide feedback if any.

# 3. Date of Next RAN2 Meetings:

3GPP TSG RAN WG2#129bis	7 - 11 Apr, 2025		TBD, CN

3GPP TSG RAN WG2#130	19 - 23, May, 2025		Malta, MT