3GPP TSG-RAN WG2 Meeting #129	R2-2500698

Athens, Greece, Feb 17th – 21st, 2024

Agenda item:		8.7.5

Source:		Samsung

Title:			RLC Enhancements for XR

Document for:	Discussion & Decision

# 1 Introduction

In RAN2#128, following agreements were made:

Agreements on RLC timely retransmissions - Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them. Agreements on avoiding unnecessary retransmissions - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced - Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

# 2 Discussion

We discuss the autonomous retransmission and polling enhancements based solutions individually, as the network may possibly configure only one of them. Even when both of them are configured concurrently, it is reasonably simpler and effective to not involve any cross-interactions among these solutions, and let them operate individually.

## 2.1 Timely RLC AM Retransmissions – Autonomous Retransmissions

TABLE I

| Approach | Pros | Cons |
| --- | --- | --- |
| Upon remaining time of the PDU becomes lesser than a threshold | - Pre-emptive retransmission - Effective as specifically linked with delay consideration | - Transmission overhead but manageable |
| Upon failure of a number of HARQ transmissions | - Early determination & Local NACK | - Not straight-forward to map a RLC PDU and HARQ TB - Involves complicated and cumber-some cross-layer interactions - Large specification impact as currently no explicit HARQ information is indicated to RLC |
| Upon availability of spare transmisison resources | - Opportunistic approach | Not full-proof and non-deterministic solution |

Different approaches of autonomous retransmission are compared in Table I. It is clear that it is not easy to regulate such autonomous retransmissions based on specific conditions of HARQ transmisions or availability of spare transmission resources. Rather, it is quite reasonable and deterministic to rely on the approach based on the remaining time of the RLC PDU becoming less than a configured threshold triggering the autonomous retransmision. Hence, we propose:

Proposal 1: UE triggers an autononous retransmission when the remaining time of a RLC PDU becomes less than a configured threshold. (Refer to TP1 in annex)

Proposal 2: Do not adopt the following as triggers for autonomous retransmission of RLC PDU:

- Number of HARQ retransmisions exceeding a configured threshold

- The availability of spare PUSCH resources for transmission

## 2.2 Timely RLC AM Retransmissions – Polling Enhancements

As shown in Excerpt 1, there are three cases in legacy RLC AM to trigger a polling at the transmitting side:

- Case 1: the number of initially transmitted RLC SDUs or the bytes thereof, have been accumulated up to a certain thresholds (pollPDU or pollByte), respectively, whichever comes first

- Case 2: there is no data to (re)transmit

- Case 3: transmitting window stalling happens

Excerpt 1:

TS 38.322 5.3.3 Polling ... Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below.

Among the three cases, a special care needs to be taken for Case 1, in reducing the polling latency, especially, when there are delay-critical RLC SDUs involved. For example, dedicated thresholds (i.e., pollPDU and pollByte) for delay-critical RLC SDUs and the associated counters can be introduced, in order to reduce the polling latency for delay-critical RLC SDUs. This is illustrated in TP 2 in annex.

Proposal 3: Dedicated polling thresholds and the associated counters are introduced for delay-critical RLC SDUs. (Refer to TP2 in annex)

## Unnecessary RLC AM Retransmission Avoidance

### 2.3.1 Impacts on transmitter RLC

Transmitter RLC entity needs to determine outdated status of the RLC SDU before it can undertake stopping or abandonment of the transmision. Basing this determination on the discard indication from PDCP is straightforward and maps directly with the latency requirement. Specification impact is quite small given that such indication is already present. In our opinion, any other basis of this determination of outdated SDU (e.g., maxReTxThreshold) is not appropriate and brings unnecessary complexity with regard to radio link falure behaviour.

Proposal 4: Transmitter RLC abandons the transmission of the AMD PDU containing RLC SDU or a segment thereof when discard for RLC SDU is indicated from PDCP. (Refer to TP3 in annex)

Further, once abandonment of the transmission is determined for outdated SDU, some of the existing procedural description in spec would need to reflect the same. In particular, transmitter RLC excludes the RLC SDU, for which transmisison is abandoned, for the following considerations:

- Considering the RLC SDU or the RLC SDU segment for which NACK was received for retransmission

- Considering to select the RLC SDU for retransmission, when both transmission buffer and the retransmission buffer are empty or no new RLC SDU or SDU segment can be transmitted

Proposal 5: Transmitter RLC excludes the RLC SDU, for which transmisison is abandoned, for the following considerations (Refer to TP 4 in annex):

- Considering the RLC SDU or the RLC SDU segment for which NACK was received for retransmission

- Considering to select the RLC SDU for retransmission, when both transmission buffer and the retransmission buffer are empty or no new RLC SDU or SDU segment can be transmitted

### 2.3.2 Impacts on receiver RLC

RAN2 agreed to have a local timer at receiver RLC for determining the abandoned SDU; however, it is not yet decided whether this local timer is defined per SDU or per RLC entity. Though both the approaches are feasible, however, there is greater complexity and spec impact with per SDU approach with multiple timers to manage individually. Having one local timer per RLC would be more effective and is aligned with the existing mechanism of t-Reassembly operation. That is, specification description can be largely inherited from the legacy, and consequently, it makes the specification effort quite manageable and risk-free. In summary, local timer (e.g. termed as “t-Outdated”) for determination of abandoned SDUs should be defined per RLC entity.

Proposal 6: At the receiver RLC, define a new timer “t-Outdated” per RLC entity for determining abandoned SDUs.

Operationaly, the local timer “t-outdated” is started upon determining a gap (i.e., at least one missing byte segment of the SDU). Upon expiry of timer “t-Outdated”, receiver RLC advances the receiving window (RX_Next), discards all received byte-segments of SDU with SN < RX_Next and determine the SDU as abandoned. Procedural description is provided in TP5 in the annex.

Proposal 7: At the expiry of timer “t-Outdated”, receiver RLC advances the receiving window, discards all received byte-segments of SDU with SN < RX_Next and determine the SDU as outdated and abandoned. (Refer to TP5 in annex)

# 3 Conclusion

Based on the discussion above, RAN2 is requested to consider the following proposals:

Proposal 1: UE triggers an autononous retransmission when the remaining time of a RLC PDU becomes less than a configured threshold. (Refer to TP1 in annex)

Proposal 2: Do not adopt the following as triggers for autonomous retransmission of RLC PDU:

- Number of HARQ retransmisions exceeding a configured threshold

- The availability of spare PUSCH resources for transmission

Proposal 3: Dedicated polling thresholds and the associated counters are introduced for delay-critical RLC SDUs. (Refer to TP2 in annex)

Proposal 4: Transmitter RLC abandons the transmission of the AMD PDU containing RLC SDU or a segment thereof when discard for RLC SDU is indicated from PDCP. (Refer to TP3 in annex)

Proposal 5: Transmitter RLC excludes the RLC SDU, for which transmisison is abandoned, for the following considerations (Refer to TP 4 in annex):

- Considering the RLC SDU or the RLC SDU segment for which NACK was received for retransmission

- Considering to select the RLC SDU for retransmission, when both transmission buffer and the retransmission buffer are empty or no new RLC SDU or SDU segment can be transmitted

Proposal 6: At the receiver RLC, define a new timer “t-Outdated” per RLC entity for determining abandoned SDUs.

Proposal 7: At the expiry of timer “t-Outdated”, receiver RLC advances the receiving window, discards all received byte-segments of SDU with SN < RX_Next and determine the SDU as outdated abandoned. (Refer to TP5 in annex)

# 4 Annex

## 4.1 	TP1 (Autonomous Retransmission)

### 5.3.2 Retransmission The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following: - STATUS PDU from its peer AM RLC entity. When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity or when the remaining time for an RLC SDU becomes less than a threshold [THRESHOLD_AUTO_RETX], if configured, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission. - consider the RLC SDU for which remaining time is less than [THRESHOLD_AUTO_RETX] for retransmission. When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached. When retransmitting an RLC SDU or an RLC SDU segment, the transmitting side of an AM RLC entity shall: - if needed, segment the RLC SDU or the RLC SDU segment; - form a new AMD PDU which will fit within the total size of AMD PDU(s) indicated by lower layer at the particular transmission opportunity; - submit the new AMD PDU to lower layer. When forming a new AMD PDU, the transmitting side of an AM RLC entity shall: - only map the original RLC SDU or RLC SDU segment to the Data field of the new AMD PDU; - modify the header of the new AMD PDU in accordance with the description in clause 6.2.2.4; - set the P field according to clause 5.3.3.

## 4.2 	TP2 (Polling Enhancements)

#### 5.3.3.2 Transmission of a AMD PDU Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - if the RLC PDU contains a delay-critical RLC SDU or a delay-critical RLC SDU segment and when configured for [Polling_Enhancement]: - increment DC_PDU_WITHOUT_POLL by one; - increment DC_BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if DC_PDU_WITHOUT_POLL >= DC_pollPDU; or - if DC_BYTE_WITHOUT_POLL >= DC_pollByte: - include a poll in the AMD PDU as described below. - else: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below. NOTE: Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation. To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall: - set the P field of the AMD PDU to "1"; - set PDU_WITHOUT_POLL and DC_PDU_WITHOUT_POLL to 0; - set BYTE_WITHOUT_POLL and DC_BYTE_WITHOUT_POLL to 0. Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall: - set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; - if t-PollRetransmit is not running: - start t-PollRetransmit. - else: - restart t-PollRetransmit.

## 4.3 TP3 & TP4 (Unnecessary Retransmission Avoidance – Tx side)

TP3:

#### 5.2.3.1 Transmit operations ##### 5.2.3.1.1 General The transmitting side of an AM RLC entity shall prioritize transmission of RLC control PDUs over AMD PDUs. The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of AMD PDUs containing not previously transmitted RLC SDUs or RLC SDU segments. When configured so, the transmitting side of an AM RLC entity shall abandon the transmission of the AMD PDUs containing RLC SDUs or segments thereof which has been indicated for discard from upper layer. The transmitting side of an AM RLC entity shall maintain a transmitting window according to the state variable TX_Next_Ack as follows: - a SN falls within the transmitting window if TX_Next_Ack <= SN < TX_Next_Ack + AM_Window_Size; - a SN falls outside of the transmitting window otherwise. The transmitting side of an AM RLC entity shall not submit to lower layer any AMD PDU whose SN falls outside of the transmitting window. For each RLC SDU received from the upper layer, the AM RLC entity shall: - associate a SN with the RLC SDU equal to TX_Next and construct an AMD PDU by setting the SN of the AMD PDU to TX_Next; - increment TX_Next by one. When submitting an AMD PDU that contains a segment of an RLC SDU, to lower layer, the transmitting side of an AM RLC entity shall: - set the SN of the AMD PDU to the SN of the corresponding RLC SDU. The transmitting side of an AM RLC entity can receive a positive acknowledgement (confirmation of successful reception by its peer AM RLC entity) for an RLC SDU by the following: - STATUS PDU from its peer AM RLC entity. When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall: - send an indication to the upper layers of successful delivery of the RLC SDU; - set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet.

TP4:

### 5.3.2 Retransmission The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following: - STATUS PDU from its peer AM RLC entity. When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer and if it is not considered previously for abandoning of transmission: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission. When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached. When retransmitting an RLC SDU or an RLC SDU segment, the transmitting side of an AM RLC entity shall: - if needed, segment the RLC SDU or the RLC SDU segment; - form a new AMD PDU which will fit within the total size of AMD PDU(s) indicated by lower layer at the particular transmission opportunity; - submit the new AMD PDU to lower layer. When forming a new AMD PDU, the transmitting side of an AM RLC entity shall: - only map the original RLC SDU or RLC SDU segment to the Data field of the new AMD PDU; - modify the header of the new AMD PDU in accordance with the description in clause 6.2.2.4; - set the P field according to clause 5.3.3. ……… #### 5.3.3.2 Transmission of a AMD PDU Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements and excluding RLC SDU or RLC SDU segment for which transmission is abandoned) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below. NOTE: Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation. To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall: - set the P field of the AMD PDU to "1"; - set PDU_WITHOUT_POLL to 0; - set BYTE_WITHOUT_POLL to 0. Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall: - set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; - if t-PollRetransmit is not running: - start t-PollRetransmit. - else: - restart t-PollRetransmit. #### ….. #### 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements and excluding RLC SDU or RLC SDU segment for which transmission is abandoned); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer and which has not been abandoned for transmission, for retransmission; or - consider any RLC SDU which has not been positively acknowledged and which has not been abandoned for transmission, for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

## 4.4 	TP5 (Unnecessary Retransmission Avoidance – Rx side)

#### 5.2.3.2 Receive operations ##### 5.2.3.2.1 General The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows: - a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size; - a SN falls outside of the receiving window otherwise. When receiving an AMD PDU from lower layer, the receiving side of an AM RLC entity shall: - either discard the received AMD PDU or place it in the reception buffer (see clause 5.2.3.2.2); - if the received AMD PDU was placed in the reception buffer: - update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see clause 5.2.3.2.3). When t-Reassembly expires, the receiving side of an AM RLC entity shall: - update state variables and start t-Reassembly as needed (see clause 5.2.3.2.4). ##### 5.2.3.2.2 Actions when an AMD PDU is received from lower layer When an AMD PDU is received from lower layer, where the AMD PDU contains byte segment numbers y to z of an RLC SDU with SN = x, the receiving side of an AM RLC entity shall: - if x falls outside of the receiving window; or - if byte segment numbers y to z of the RLC SDU with SN = x have been received before: - discard the received AMD PDU. - else: - place the received AMD PDU in the reception buffer; - if some byte segments of the RLC SDU contained in the AMD PDU have been received before: - discard the duplicate byte segments. ##### 5.2.3.2.3 Actions when an AMD PDU is placed in the reception buffer When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall: - if x >= RX_Next_Highest: - update RX_Next_Highest to x+ 1. - if all bytes of the RLC SDU with SN = x are received: - reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer; - if x = RX_Highest_Status: - update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received and which is not determined as outdated. - if x = RX_Next: - update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received and which is not determined as outdated. - if t-Reassembly is running: - if RX_Next_Status_Trigger = RX_Next; or - if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU and SN = RX_Next is determined as outdated; or - if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size: - stop and reset t-Reassembly. - if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above): - if RX_Next_Highest> RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU and SN = RX_Next is not determined as outdated: - start t-Reassembly; - set RX_Next_Status_Trigger to RX_Next_Highest. - if t-Outdated is running: - if RX_Outdated_Trigger = RX_Next; or - if RX_Outdated_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU or SN=RX_Next is determined as outdated; or - if RX_Outdated_Trigger falls outside of the receiving window and RX_Outdated_Trigger is not equal to RX_Next + AM_Window_Size: - stop and reset t-Outdated. - if t-Outdated is not running (includes the case t-Outdated is stopped due to actions above): - if RX_Next_Highest> RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU and SN = RX_Next is not determined as outdated; or - start t-Outdated; - set RX_Outdated_Trigger to RX_Next_Highest. ##### 5.2.3.2.4 Actions when t-Reassembly expires When t-Reassembly expires, the receiving side of an AM RLC entity shall: - update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Status_Trigger for which not all bytes have been received; - if RX_Next_Highest> RX_Highest_Status +1: or - if RX_Next_Highest = RX_Highest_Status + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Highest_Status before the last byte of all received segments of this SDU and SN = RX_Next is not determined as outdated: - start t-Reassembly; - set RX_Next_Status_Trigger to RX_Next_Highest. ##### 5.2.3.2.5 Actions when t-Outdated expires When t-Outdated expires, the receiving side of an AM RLC entity shall: - update RX_Next to the SN of the first RLC SDU with SN >= RX_Outdated_Trigger for which not all bytes have been received; - discard all received byte-segments, if any, of SN < RX_Next and consider SN as outdated and abandoned; - if RX_Next_Highest> RX_Next +1: or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU and SN = RX_Next is not determined as outdated: - start t-Outdated; - set RX_Outdated_Trigger to RX_Next_Highest.