3GPP TSG-RAN WG2 Meeting #129	R2-2500701

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	 8.13.3

Source:	 Huawei, HiSilicon

Title:	 Control Plane Procedures for Multi-hop Relay

Document for:  Discussion and Decision

# 1	Introduction

During the RAN2#127 and RAN2#128 meeting we had discussed the control plane procedure for the multi-hop relay and had made the following agreements with FFSs (highlighted with yellow).

RAN2#127 agreements [1]:

- The following connections are assumed as a baseline to be needed: - From last Relay UE perspective: - A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and - A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED. - From intermediate relay UE perspective (including first relay UE): - A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and - An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED. - FFS what RRC states are supported for the intermediate relay UE - From U2N Remote UE perspective: - A direct (non-relayed) PC5 connection with Intermediate Relay UE, and - An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED. - The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18) - The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

RAN2#128 agreements [2]:

Agreements for baseline CP procedure: - In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED. Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED. FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE. FFS if SRB0 would be forwarded before connection establishment of each intermediate relay UE. - The figure and description under P1 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted. - In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in any RRC state when the U2N remote UE is in RRC_CONNECTED. - In approach 2, any intermediate relay UE which happens to be in RRC_CONNECTED towards the last relay UE’s serving gNB and is operating as a remote UE is assumed to obtain its relaying configuration (RLC channel configuration, SRAP configuration, etc.) in dedicated signalling from the last relay UE’s serving gNB via the U2N connection. - The figure and description under P4 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 2 (relays other than the Last Relay may/may not remain in RRC_IDLE/RRC_INACTIVE when the remote UE is in RRC_CONNECTED) is adopted. - Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED. - 1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell. - 2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment. All agreements in this WI apply to both these cases unless otherwise specified. Agreements for QoS split: - For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation. This does not preclude the possibility of assistance information from the UE(s). - If enhancements allow some intermediate relay UEs to remain in idle/inactive, the PDB split between the Uu hop and all remaining hops is performed by the network. FFS how to split the PDB over each of the individual remaining hops. Agreements for SI forwarding: - In multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s). FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE). Agreements for SRAP design: - Maintain the principle that the SRAP header enables mapping from the e2e bearer ID to the egress PC5 RLC channel. This does not preclude changes/rewrites/addition of the SRAP header by the intermediate UEs. - Remote UE ID and BEARER ID are included in Rel-19 SRAP header for multi-hop L2 U2N relay.

During the email discussion after RAN2#127 meeting under [POST127][402][Relay], we had discussed and compared the two approach/architecture below more thoroughly [3].

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC.

These two approaches now are still on the table with more companies supporting the approach 1. In this contribution, we discuss the details of the key issues for the two approaches and provide comparison of the two approaches that would be useful in deciding down selection one of the two approaches during this meeting.

# 2	Discussion

## 2.1	CP procedures for Approach 1

### Connection establishment

The figure and description under P1 in [3] - report for post #127 meeting email discussion has been agreed to serve as a baseline connection establishment procedure for Approach 1 for further discussion which still contains some FFSs. In the following, we will discuss these FFSs identified in the email discussion and the agreements for this procedure.

- The U2N Remote UE, First Relay UE, Intermediate Relay UE, and Last Relay UE perform discovery procedure, and establish a PC5-RRC connection between each adjacent UE (U2N Remote UE<->First Relay UE, First Relay UE <-> Intermediate Relay UE, Intermediate Relay UE <-> Last Relay UE) using the NR sidelink PC5 unicast link establishment procedure. [FFS whether to support PC5-RRC connection establishment between some adjacent UEs after transmission of the first RRC message in step 2.] - The L2 U2N Remote UE sends the first RRC message (i.e., RRCSetupRequest) for its connection establishment with gNB via the First Relay UE, using a specified PC5 Relay RLC channel configuration. The first Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the First Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment via the Intermediate Relay UE (using similar actions as a U2N Remote UE) upon reception of a message from U2N Remote UE on the specified PC5 Relay RLC channel. The Intermediate Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the Intermediate Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment via the Last Relay UE (using similar actions as a U2N Remote UE) upon reception of a message from the First Relay UE on the specified PC5 Relay RLC channel. The Last Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the Last Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment upon reception of a message from the Intermediate Relay UE on the specified PC5 Relay RLC channel. In each of the previous sub-steps, if a given relay UE and its parent relay UE both need to enter RRC_CONNECTED, the given relay UE cannot do so until the parent relay UE has completed its own RRC connection establishment. The Last Relay UE receives SRB0 relaying Uu Relay RLC channel configuration for the Intermediate Relay UE from gNB. The Intermediate Relay UE receives SRB0 relaying Relay RLC channel configuration for the First Relay UE from gNB. The gNB configures SRB0 (for U2N Remote UE) relaying RLC channel to the first Relay UE. The gNB responds with an RRCSetup message to U2N Remote UE. The RRCSetup message is sent to the U2N Remote UE using SRB0 relaying Last Relay RLC channel over Uu and the specified PC5 Relay RLC channels over each of the PC5 links. [FFS whether the Last Relay UE can send SUI on behalf of all other relay UEs.] - The gNB, Last Relay UE, Intermediate Relay UE and First Relay UE perform relaying channel setup procedure over Uu. According to the configuration from the gNB, the First Relay/U2N Remote UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the U2N Remote UE/First Relay UE over PC5, the Intermediate Relay/First Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the First Relay UE/Intermediate Relay UE over PC5 and the Last Relay UE/ Intermediate Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the Intermediate Relay UE/Last Relay UE over PC5. [FFS if each relay UE can establish RLC channel for relaying of SRB1 at the same time as its connection establishment in step 2]. - The RRCSetupComplete message is sent by the U2N Remote UE to the gNB via the First Relay UE, Intermediate Relay UE and the Last Relay UE using SRB1 relaying channels over PC5 and SRB1 relaying channel configured to the Last Relay UE over Uu. Then the U2N Remote UE is RRC_CONNECTED with the gNB. - The L2 U2N Remote UE and gNB establish security following the Uu security mode procedure and the security messages are forwarded through the First Relay UE, Intermediate Relay UE, and Last Relay UE. - The gNB sends an RRCReconfiguration message to the U2N Remote UE via the Last Relay UE, Intermediate Relay UE, and First Relay UE to setup the end-to-end SRB2/DRBs of the U2N Remote UE. The U2N Remote UE sends an RRCReconfigurationComplete message to the gNB via the First Relay UE, Intermediate Relay UE, and Last Relay UE as a response. In addition, the gNB may configure additional Uu Relay RLC channels between the gNB and Last Relay UE, and PC5 Relay RLC channels between each of the Intermediate Relay UE, First Relay UE, and U2N Remote UE for the relaying traffic. Based on the above procedure, for gNB to control each relay UE by RRC, each relay UE needs to be in RRC connected. As a result, for connection establishment of the remote UE, each relay UE should trigger its own connection establishment. For the last relay UE, Uu connection establishment is performed. However, for the other relay UEs, upon reception of a message on SL-SLB0, they perform connection establishment as though they are acting as a remote UE.

#### 2.1.1.1 Latency for Connection Establishment Procedure

A couple of companies have indicated during the email discussion that main drawback of Approach 1 is that latency of remote UE connection establishment increases as the number of hops increases, mainly in the case that all the relay UEs participating in the multi-hop connection establishment are in RRC IDLE/INACTIVE state. Figure 1 shows the procedure when applying the legacy mechanism to trigger the RRC IDLE/INACTIVE relay UE, in which relay UE triggers the RRC connection establishment when receiving the SRB0 message from its adjacent remote UE. We can see that the remote UE needs to wait for the finish of the connection establishment procedures of all the relay UEs which is done in a recursively way before it can receive the configuration for the SRB1. The connection establishment behaviours of the intermediate relay UE are the same as a remote UE in Rel-17/Rel-18 (answer to the FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE).

Proposal 1: The connection establishment of an Intermediate Relay UE should be captured in specification as connection establishment of a remote UE with the trigger of receiving the SRB0 message from its downstream child UE in the multi-hop chain.

Figure 1. Connection establishment procedure with IDLE/INACTIVE relay UEs by applying Rel-17 mechanism.

An important point to note is that the scenario where the Remote UE triggers connection establishment (Figure 1) while all the relay UEs are RRC IDLE/INACTIVE is unlikely to occur frequently in practice. A more common scenario is that the last and intermediate relay UEs are already in connected state and have established the connection to the network for their own traffic and then they start serving the remote UE.

Observation 1: The scenario where all the relay UEs involved in the multi hop connection establishment are in the RRC IDLE/INACTIVE state when the remote UE triggers the connection establishment is unlikely to occur frequently in practice. A more common scenario in is that the last and intermediate relay UEs (including the first relay UE) are already in connected state and have established the connection to the network for their own traffic before they start serving the remote UEs.

Furthermore, the latency problem in Approach 1 can be addressed with few simple solutions. For instance, the remote UE can prioritize selecting an RRC CONNCETED relay UE if the service requirements of the remote UE are stringent. Alternatively, we can impose a limit on the maximum hop number between the relay UEs and the network when RRC IDLE/INACTIVE relay UEs perform the multi-hop relay operations. Another approach is to trigger the connection establishment procedures of the relay UEs in parallel, as shown in Figure 2. In step 3, the SRB0 message can be delivered to the gNB via the default PC5 relay RLC channel over the PC5 link between two UEs. This, in turn, can trigger the SRB0 message delivery of all the RRC IDLE/INACTIVE relay UEs in step 4a, 4b, and 4c in parallel. As a result, the connection establishment of the remote UE can be accelerated.

Figure 2. Parallel connection establishment mechanism for IDLE/INACTIVE relay UEs.

In the above procedure, the SRB0 messages form the remote UE can be forwarded before connection establishment of each intermediate relay UE (which corresponding to the FFS if SRB0 would be forwarded before connection establishment of each intermediate relay UE).

Besides, we believe latency increase is not specific to Approach 1 and increase in latency is also an issue for Approach 2 where Intermediate Relay UEs are both in RRC CONNECTED and IDLE/INACTIVE states. The data transmission from the remote UE needs to wait after the UE ID allocation/QoS split/bearer configuration for both the approaches. The difference is that there will be an increased Uu signalling in Approach 1, that is controlled by the network, while there will be increased SL signalling in Approach 2 among the relay UEs due to local ID assignment and QoS split. The details can be found in our analysis for approach 2 in clause 2.2.

Observation 2: Latency increase is not specific to Approach 1 only. The data transmission from the remote UE needs to wait after the UE ID allocation/QoS split/bearer configuration is done for both the approaches. There will be an increased Uu signalling in Approach 1, that is controlled by the network. For approach 2, there will be increased SL signalling among the relay UEs due to local ID assignment and QoS split.

Proposal 2: Latency increase is not specific to Approach 1 and shall not be used as a show stopper for adopting Approach 1 for the multi hop relay in Rel 19. The latency can be improved for Approach 1 using the following solutions.

- Remote UE can prioritize selecting an RRC CONNCETED relay UE if the service requirements of remote UE are stringent.

- Restriction/limit on the maximum hop number can be applied between the relay UEs and the network when the RRC IDLE/INACTIVE relay UEs perform the multi-hop relay

- SRB0 messages of remote UE can be forwarded before connection establishment of each intermediate relay UE via the default PC5 relay RLC channels between UEs, which can trigger the parallel connection establishment of the RRC IDLE/INACTIVE relay UE

#### 2.1.1.2 Need for DRB Establishment for Relay UEs

Some companies have indicated that an intermediate relay UE needs to be configured with Uu SRAP configuration at least for the default DRB/DRB and SRB.

We don’t think we need any DRBs or default DRB for the relay UEs to be in RRC connected state when the remote UE triggers the connection establishment procedure while all the relay UEs are in RRC IDLE state. These relay UEs can be moved to RRC CONNECTED state by the network just to serve the remote UE without having their own data to be transferred to the network.

Alternatively, in other scenarios, intermediate relay UE may first get connected as a remote UE to transfer their own data via the last relay UE and go through the DRB establishment procedure and then start serving any remote or other intermediate relay UEs in the vicinity for relaying their data.

We don’t see any issues with these two scenarios

Proposal 3: No DRBs or default DRB needs to be established for the last or intermediate relay UEs to move them from RRC IDLE to RRC Connected state before serving the remote UE when the remote UE triggers the connection establishment procedure.

#### 2.1.1.3 Failures during connection establishment

Some companies have indicated that Remote UE or an intermediate relay UE may fail to set up its RRC_CONNECTION, due to its parent relay UE (s) taking too long time to setup their RRC_CONNECTIONS. We believe the failures can be resolved with extended timers for multi hop. T300 timer could be increased for multi-hop U2N relay operation in accordance with the hop count.

Proposal 4: Extend the T300 timer for multi-hop U2N relay operation in accordance with the hop count to avoid any failures during connection establishment procedure.

#### 2.1.1.4 Other FFSs

In the following tabular, we list the remaining  FFSs related to the connection establishment procedure in the report [3] and provide our insights and opinions.

| FFSs | Our opinions |
| --- | --- |
| - FFS whether to support PC5-RRC connection establishment between some adjacent UEs after transmission of the first RRC message in step 2. | No. We need to clarify the meaning of “after”. Does it mean that the PC5-RRC connection establishment is triggered by the first RRC message, or the PC5-RRC connection establishment happens later than the first RRC message? We think the PC5-RRC connection establishment should be performed in step 1, triggered after the remote UE selects the relaying path and establishes the PC5 connection with the first relay UE. The PC5-RRC connection establishment of the relay UEs and the sending of the first RRC message can happen in parallel since they are implemented by different UEs (remote UE and the relay UEs respectively) in different procedures. Overall, we do not think the PC5-RRC connection establishment can be contained in step 2 in the stage-2 procedure. |
| - FFS whether the Last Relay UE can send SUI on behalf of all other relay UEs. | No. In current spec, the SUI uses the L2 ID to indicate the connected remote UE. Without enhancement, the last relay UE cannot obtain the L2 ID of all the relay UEs. We believe that reusing the legacy mechanism is sufficient where each relay UE reports the SUI and indicates the L2 ID of its adjacent child UE. |
| - FFS if each relay UE can establish RLC channel for relaying of SRB1 at the same time as its connection establishment in step 2 | Open to discussion. By applying the rel-17 mechanism, the RLC channel configurations for relaying of SRB1 at relay UEs are configured sequentially, as shown in Figure 1. With default PC5 relay RLC channels between the UEs, the gNB can configure the relaying of SRB1 for the relay UEs simultaneously, similar to the parallel SRB0 message forwarding in Figure 2. This approach reduces the latency of the remote UE's connection establishment procedure and allows for quicker data transmission start-up. |

To summarize, we have the following proposal for the above FFSs.

Proposal 5a: The PC5-RRC connection establishment between adjacent relay UEs after transmission of the first RRC message in step 2 is not supported.

Proposal 5b. The Last Relay UE cannot send the SUI on behalf of all other relay UEs. Each relay UE shall report the SUI and indicate the L2 ID of its adjacent child UE to the gNB.

Proposal 5c. RAN2 can discuss the necessity to enable the parallel RLC channel establishment for relaying of SRB1 at each relay UE.

### Protocol stack

Following the agreements, the Control Plane and the User Plane protocol stacks of approach 1 for multi-hop relay with one intermediate Relay UE can be shown in Figure 3. These protocol stack can be taken as the basis for the R19 Multi-hop relay including two or more hops. If there are more than two hops, i.e., additional intermediate relay UE between the last relay and first relay UE, the intermediate relay UE will have hop- by-hop PC5 SRAP/RLC/MAC/PHY layers with its adjacent relay UEs

(a) Control plan

(b) User plan

Figure 3. Protocol stack for multi-hop L2 U2N relay

Proposal 6: Adopt the Control Plane and User Plane protocol stack for Multi-hop L2 U2N relay as shown in Figure 3. SRAP layer is reused for bearer mapping and remote UE identification purpose.

### 2.1.3 Local ID allocation

The single-hop relay can be a special case of the multi-hop relay. For backward compatibility, in approach 1, the legacy local ID allocation mechanism in Rel-17 can be reused, then the procedure for multi-hop relay can be:

- Step 0: the remote UE reports its L2 ID to the gNB

- Step 1: the parent relay UE reports the L2 ID of the remote UE to request the local ID allocation.

- Step 2: gNB allocates the local ID and configures the local ID for the remote UE identified by the L2 ID. The local ID will be configured to the remote UE and all the relay UE along the multi-hop relaying path for the remote UE.

Same as legacy, the local ID for remote UE in multi-hop relay can be 8 bits.

Proposal 7: Reusing the single-hop relay mechanism to support the Local ID allocation for multi-hop relay:

- Parent relay UE of the remote UE reports the L2 ID to the gNB to request the local ID allocation, the uniqueness of the local ID can be guaranteed by the gNB by implementation.

- The remote UE local ID is 8 bits.

### 2.1.4 QoS split

Based on approach 1, the gNB determines the configurations of all the remote UEs and relay UEs, so the QoS can be guaranteed by the gNB by implementation.

Proposal 8. The QoS split is determined by gNB and guaranteed by gNB by implementation.

### SRAP PDU format and SRAP configuration

Same as  Rel-17, the SRAP layer is reused for bearer mapping and remote UE identification purpose. The remote UE ID and bearer ID can be included in the SRAP PDU header for data forwarding. For a relay UE, it will receive the SRB/DRB data from the ingress relay RLC channel of the ingress link, and forward the data to the egress relay RLC channel of the egress link.

In the UL, when the relay UE receives the data from its child UE (a relay UE or a remote UE), it will forward the data to its parent relay UE according to the SRAP PDU header and the bearer mapping configuration. The bearer mapping configuration need to contain the mapping between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel. Since SRB and DRB can share same index, ingress PC5 relay RLC channel between the relay UE and its child UE can be used to identify the SRB index and the DRB index.

In the DL, the relay UE can also forward the data based on the mapping between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel of the egress link, and ingress PC5 relay RLC channel between the relay UE and its parent UE can be used to identify the SRB index and the DRB index..

For above, we can have the following proposals for the SRAP PDU format and SRAP configuration design for the multi-hop relay.

Proposal 9: The legacy SRAP PDU format can be reused for multi-hop relay, i.e., remote UE ID and bearer ID are included in the SRAP PDU header for data forwarding.

Proposal 10: The SRAP configuration contains the mapping configuration between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel.

- For the first relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the egress PC5 relay RLC channel between the first relay UE and remote UE, the egress PC5 relay RLC channel between the first relay UE and its parent relay UE.

- For intermediate relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the identity (L2 ID) of its child relay UE, the egress PC5 relay RLC channel between the intermediate relay UE and its child relay UE, the egress PC5 relay RLC channel between the intermediate relay UE and its parent relay UE.

- For the last relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the identity (L2 ID) of its child relay UE, the egress Uu relay RLC channel on Uu, the egress PC5 relay RLC channel between the last relay UE and its child relay UE.

Besides, since the relay UE can connect to several child UEs if it is not the first relay UE, the path information corresponding to the egress link is needed. For handling the path information, there are two options that can be considered.

Derivation of implicit path information by the relay UEs - The last or the intermediate relay UE may implicitly derive the path information by mapping local ID of the remote UE (carried in the SRAP header) to the L2 ID of the child relay UE during the uplink transmission, e.g., uplink transmission of RRCSetupRequest message. This mapping can then be used to route the downlink packets for the remote UE via the child relay UE.

Explicit signaling of the path information to the relay UEs - The other option is to explicitly provide the path information through the identity (L2 ID) of the child relay UE in the SRAP configuration for the intermediate and last relay UE.

Proposal 11: For handling the Multi hop relay path information there are two options that can be considered

Option 1 - The path information can be implicitly delivered by last or the intermediate relay UE by mapping local ID of the remote UE (carried in the SRAP header) to the L2 ID of the child relay UE during the uplink transmission of RRCSetupRequest message

Option 2 – The path information can be explicitly provided through the identity (L2 ID) of the child relay UE in the SRAP configuration for the intermediate and last relay UE.

### 2.1.6 System information/ paging forwarding

In Rel-17, the RRC CONNECTED remote UE can use the on-demand SIB framework to request the SIB(s). The RRC IDLE/INACTIVE remote UE can indicate the requested SIB type(s) to its relay UE, then relay UE can trigger the on-demand SIB acquisition procedures on the Uu and forward the acquired SIB(s) to the remote UE via PC5.

In approach 1 for multi-hop relay, the Rel-17 on-demand SIB framework can be reused but with some enhancements for the RRC IDLE/INACTIVE remote UEs. First, the RRC IDLE/INACTIVE remote UE indicates the required SIB(s) to the intermediate Relay UE. If the intermediate relay UE is also in RRC IDLE/INACTIVE state and has no required SIB(s), it needs to send the request to its parent relay UE, e.g., via the PC5 message. It means that the PC5 message form the intermediate Relay UE to its parent relay UE should indicate all the required SIB(s) requested from all the connected remote UE(s) (which may be also a relay UE), and the intermediate relay UE itself.

Proposal 12: The intermediate Relay UE should indicate all the required SIB(s) requested from all the connected remote UE(s) and the intermediate Relay UE itself to its parent relay UE.

In ReL-17, the RRC IDLE/INACTIVE remote UE provides the remote UE ID (5G-S-TMSI or I-RNTI) to the relay UE, then the U2N relay UE will forwards the paging to the remote UE if the received paging message contains the remote UE ID.

In Rel-19 multi-hop relay, the last relay UE needs to help forward the paging for all the connected RRC IDLE/INACTIVE UEs including those act as remote UE only or also act as a relay UE (e.g., the intermediate relay UE and the first relay UE). Therefore, in addition to the 5G-S-TMSI or I-RNTI of the intermediate Relay UE itself, the intermediate Relay UE needs to forward the UE IDs of the connected RRC IDLE/NACTIVE UEs (which may be also a relay UE) to its parent relay UE.

Proposal 13: The intermediate Relay UE needs to indicate all the UE ID of the connected RRC IDLE/NACTIVE UEs to its parent relay UE for paging delivery.

### 2.1.7 Radio link failure handling

In approach 1, we can follow the Rel-17 principle for radio link management, i.e., the U2N Remote UE suspends the Uu radio link management, since the remote UE and the intermediate relay UE connects to network via indirect link and there is no Uu link anymore. The PC5/Uu RLF may happen for the multi-hop scenario in the following cases as depicted in Figure 4.

(a) PC5 RLF of the remote UE

(b) PC5 RLF of the first relay UE

(c) PC5 RLF of the intermediate relay UE

(d) Uu RLF of the last relay UE

Figure 4.	RLF declaration cases in multi-hop L2 U2N relay

In case (a) of Figure 5, when the remote UE detects the PC5 RLF over the PC5 link between the remote UE and the first Relay UE, it should trigger the RRC connection re-establishment or relay reselection, which is similar to the Rel-17 handling.

Proposal 14: Similar to Rel-17 SL relay, when the remote UE in multi-hop relay detects the PC5 RLF, it can trigger the RRC connection re-establishment.

In case (b) of Figure 5, when the first relay UE detects the PC5 RLF over the PC5 link between the first relay UE and the intermediate relay UE, it can send a notification message with cause value “PC5 RLF” to the remote UE (assume the link between the remote UE and the first relay UE is good) and trigger the RRC connection re-establishment or relay reselection. Upon receiving the notification message, the remote UE can determine whether to release the PC5 link with the first relay UE and triggers the RRC connection re-establishment or relay reselection. If the remote UE releases the PC5 link, the remote UE can perform the relay reselection and the cell selection, and further trigger the RRC connection re-establishment via the new relay or via the Uu link (when in RRC CONNECTED state).

Proposal 15: When the first relay UE detects the PC5 RLF, it can send a notification message with cause value “PC5 RLF” to the remote UE and trigger the RRC connection re-establishment.

Proposal 16: Upon receiving the notification message with cause value “PC5 RLF”, the remote UE can determine whether to release the PC5 link with the first relay UE and triggers the RRC connection re-establishment.

In case (c) of Figure 5, when the intermediate relay UE detects the PC5 RLF, it can send a notification message with cause value “PC5 RLF” to the first relay UE and trigger the RRC connection re-establishment or relay reselection. Upon receiving the notification message with cause value “PC5 RLF”, the first relay UE triggers RRC connection re-establishment as legacy in Rel-17. Furthermore, the first relay UE needs to notify the U2N Remote UE, but what cause value should be contained in the notification message should be discussed. The following options can be considered.

- Option 1: introduce a new cause value which indicates PC5 RLF of the intermediate relay UE, with the identity of the intermediate relay UE

- Option 2: introduce a new cause value which indicates RLF has happened on the indirect link, without the identity of the intermediate relay UE.

The remote UE behaviour when receiving the notification message from the Intermediate Relay UE can be similar as before.

In case (d) of Figure 5, the situation is similar to case (c), the only difference is the RLF happens on the Uu link of the last relay UE. In this case, the intermediate relay UE would receive the RLF indication from its parent relay UE, need to forward the RLF indication to its child UE, and also triggers the RRC connection re-establishment or relay reselection. In this case, the cause value in the notification message can be “Uu RLF”.

Proposal 17: Upon receiving the notification message with cause value “Uu/PC5 RLF” from the parent relay UE, the intermediate relay UE needs to send a notification message to its child UE. The following options can be considered as the cause value.

- Option 1: introduce a new cause value which indicates Uu/PC5 RLF of the relay UE, with the identity of the relay UE

- Option 2: introduce a new cause value which indicates RLF has happened on the indirect link, without the identity of the relay UE that detects the RLF.

### 2.1.8 Forward compatibility

In the updated WID[4], the forward compatibility was considered and plan to be concluded in RAN#107.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

The above solutions can be naturally extended to two and more additional hops.

Proposal 18. Based on approach 1, we can naturally support multi-hop relay with two and more additional hops.

## 2.2	CP procedures for Approach 2

### 2.2.1	Connection establishment

The figure and description under P4 of [3] was agreed to serve as a baseline connection establishment procedure for Approach 2 for further discussion. In the following, we will discuss the FFSs identified in the email discussion and the agreements for this procedure.

- The U2N Remote UE, First Relay UE, Intermediate Relay UE, and Last Relay UE perform discovery procedure, and establish a PC5-RRC connection between each adjacent UE (U2N Remote UE<->First Relay UE, First Relay UE <-> Intermediate Relay UE, Intermediate Relay UE <-> Last Relay UE) using the NR sidelink PC5 unicast link establishment procedure. - The L2 U2N Remote UE sends the first RRC message (i.e., RRCSetupRequest) for its connection establishment with gNB via the First Relay UE, using a specified PC5 Relay RLC channel configuration. If the First Relay UE is in RRC_CONNECTED, it sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. Otherwise, it obtains the configuration from SIB or preconfiguration, or from the remote UE’s serving gNB. If the Intermediate Relay UE is in RRC_CONNECTED, it sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. Otherwise, it obtains the configuration from SIB or preconfiguration. The Last Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the relay operation for the U2N Remote UE. If the Last Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment upon reception of a message on the specified PC5 Relay RLC channel. After the Last Relay UE's RRC connection establishment procedure and sending the SidelinkUEInformationNR message, gNB configures SRB0 relaying Uu Relay RLC channel to the Last Relay UE. The gNB responds with an RRCSetup message to U2N Remote UE. The RRCSetup message is sent to the U2N Remote UE using SRB0 relaying Last Relay RLC channel over Uu and the specified/preconfigured PC5 Relay RLC channels over each of the PC5 links. FFS which option to obtain the relaying configuration (SIB/preconfiguration or the remote UE’s serving gNB) is used by relay UEs which remain in IDLE/INACTIVE while the remote UE is in RRC_CONNECTED. - According to (pre)configuration, the First Relay/U2N Remote UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the U2N Remote UE/First Relay UE over PC5, the Intermediate Relay/First Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the First Relay UE/Intermediate Relay UE over PC5 and the Last Relay UE/ Intermediate Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the Intermediate Relay UE/Last Relay UE over PC5. - The RRCSetupComplete message is sent by the U2N Remote UE to the gNB via the First Relay UE, Intermediate Relay UE and the Last Relay UE using SRB1 relaying channels over PC5 and SRB1 relaying channel configured to the Last Relay UE over Uu. Then the U2N Remote UE is RRC_CONNECTED with the gNB. - The L2 U2N Remote UE and gNB establish security following the Uu security mode procedure and the security messages are forwarded through the First Relay UE, Intermediate Relay UE, and Last Relay UE. - The gNB sends an RRCReconfiguration message to the U2N Remote UE via the Last Relay UE, Intermediate Relay UE, and First Relay UE to setup the end-to-end SRB2/DRBs of the U2N Remote UE. The U2N Remote UE sends an RRCReconfigurationComplete message to the gNB via the First Relay UE, Intermediate Relay UE, and Last Relay UE as a response. In addition, the gNB may configure additional Uu Relay RLC channels between the gNB and Last Relay UE, and PC5 Relay RLC channels between each of the Intermediate Relay UE, First Relay UE, and U2N Remote UE for the relaying traffic.

Although the above signalling flow for Approach 2 may seem simpler compared to Approach 1, some important issues are still not clear on how to support the above steps, particularly the step 2. There is one FFS related to step 2 that states whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment. For this FFS, we can have the following options.

Option 1: all the intermediate relay UEs need to move to RRC_CONNECTED state when the remote UE triggers connection establishment, an intermediate relay UE connects to remote UE’s serving gNB.

Option 2: all the intermediate relay UEs needs to move to RRC_CONNECTED state when the remote UE triggers connection establishment, an intermediate relay UE can connect to a gNB different from the remote UE’s serving gNB.

Option 3: the intermediate relay UEs does not need to be in RRC_CONNECTED state when the remote UE triggers connection establishment, the relaying configuration is determined based on the SIB or preconfiguration.

Option 1 is the same as the scenario in approach 1 that all the UE are connected to the remote UE’s serving gNB, which is also the serving gNB of the last relay UE. Therefore, the Rel-17 U2N relay mechanisms can be extended to support this case since the gNB can guarantee the E2E performance of the remote UE, in this case  approach 2 is the same as approach 1.

However, we think most proponents for approach 2 propose the Option 2 and Option 3, which are based on the Rel-18 U2U relay design and assumes intermediate relay UE are out of the control of the remote UE’s serving gNB. In this case, the intermediate relay UE needs to get information over the PC5 link to determine the relaying configuration for the E2E data transfer for the remote UE, including:

- The local ID allocation to identify the remote UE

- The split QoS for each hop to determine the PC5 relay RLC channel configuration

- The bearer mapping information in the previous hop used to determine the bearer mapping on the current hop

The above issues also are the key issue for the support of multi-hop U2U relay, and we will discuss these issues in the following sections.

Observation 3: Most proponents for approach 2 supports the case that the intermediate relay UE can be in a different serving gNB or in different RRC state as the remote UE when the remote UE triggers connection establishment.

Proposal 19: For approach 2 , before considering its adoption for multi-hop relay, we need to address the following issue when the intermediate relay UE can be in a different serving gNB or in different RRC state as that of the remote UE.

- The local ID allocation to identify the remote UE

- Split QoS determination for each hop to determine the PC5 relay RLC channel configuration

- Bearer mapping configuration determination on each hop.

### 2.2.2 Protocol stack

According to the agreement, the UP and CP protocol stacks can be the same as that in approach 1, as shown in Figure 3. In addition, there may be E2E PC5 connection between the remote UE and the last relay UE based on the multi-hop U2U relay mechanism, then all the intermediate can be in different serving cell with remote UE and last relay UE. Therefore, for approach 2, we may need the protocol stack (at least the CP protocol stack) of multi-hop U2U relay to support the multi-hop U2N relay in addition to the E2E protocol stack in Figure 3.

Observation 4: The UP and CP protocol stacks for approach 2 are the same as that in approach 1. In addition, we may need the CP protocol stack of multi-hop U2U relay between the remote UE and the last relay UE.

### 2.2.3 Local ID allocation

According to the discussion in clause 2.2.1, we can consider   the same options on the intermediate relay UE’s RRC state and it’s the serving cell.

In option 1, all the UE should be in connected and connects to the serving cell of the last relay UE. In this case, the last relay UE can report all the UE context along the path to the network, and the gNB can allocate the local ID, or we can just apply the mechanism in approach 1. In the former case, it is unclear how the last relay UE will get all the other UEs information (e.g., L2 ID) and report to network especially considering there is no E2E PC5 RRC connection between non-adjacent UEs.

In option 2 and option 3, the intermediate relay UE can be in the different serving cell with the remote UE, or in RRC IDLE/INACTIVE state when the remote UE is in RRC CONNECTED state. In this case, the risk for collision of the allocated UE ID exists.

- If we rely on remote UE’s serving gNB to allocate UE ID, since the intermediate relay UE may provide the relay service between different remote UE with different last relay UE, it will get the same local ID for different remote UEs from these remote UE’s serving cell.

- If we rely on the intermediate relay UE’s serving cell to allocate UE ID, its serving cell can guarantee that the UE IDs will not conflict inside the intermediate relay UE. However, if the serving cell of another intermediate relay UE along the path allocates the same UE ID for a different remote UE and we want to use the UE ID along the path, collision can happen.

- If we rely on relay UE to allocate UE ID, the situation is the same. Each relay UE cannot guarantee the uniqueness of the UE ID for a remote UE at other hops.

To avoid the local ID collision, some coordination mechanism may be needed between the relay UEs, which  could  incur large latency.

Observation 5: Allocation of local ID could be extremely complex with Approach 2, since there is no centralized node to guarantee the uniqueness of the remote UE local ID along the path.

### 2.2.4 QoS split

For approach 2, since the intermediate relay UE can be in IDLE/INACTIVE or can be in a cell of different gNB i.e. not the one controlling the last relay UE/ remote UE, there is no central node to split QoS for each hop. One possible solution can be the last relay UE’s gNB will first split the QoS into Uu hop part and the part between the last relay UE and the remote UE, then all the intermediate relay UEs need to participate in the QoS splitting procedure to determine the split QoS on each hop between the adjacent UEs. In this approach, the gNB may need to know some more information to determine the split between Uu and PC5, e.g., the PC5 hop number.

This approach is complex on the split QoS coordination on each hop, which can in turn introduce a large delay.

Observation 6: Significant complex new PC5 signalling procedures have to be designed for coordinating the Split QoS on each hop for Approach 2.

### SRAP PDU format and SRAP configuration

In approach 2, we still need the remote UE local ID and bearer ID information in the SRAP header to support the data routing. Whether reusing the legacy SRAP PDU format, and which SRAP PDU format (SRAP PDU format for U2N relay or SRAP PDU format for U2U relay) can be reused depends on the local ID allocation design.

Observation 7: the SRAP PDU format used for approach 2 depends on the local ID allocation mechanism.

How to configure the RLC channel configuration and SRAP configuration for a relay UE that is in RRC_IDLE/RRC_INACTIVE for approach 2 is also not unclear. Different RRC state and coverage states needs to be discussed/captured separately. Additionally, whether the remote/relay UEs are in same or in different cell needs to be discussed/captured separately.

These solutions will be extremely complex and are unnecessary for R19.

Observation 8: How to configure the RLC channel for a relay UE that is in RRC_IDLE/RRC_INACTIVE for approach 2 is unclear. Different RRC state and coverage states needs to be discussed/captured separately. Additionally, whether the remote/relay UEs are in the same or  different cell needs to be discussed/captured separately.

### 2.2.6 System information/ paging forwarding

If there is E2E PC5 connection between the remote UE and the last relay UE, the legacy mechanism in Rel-17 can be reused directly. If not, the mechanism for approach 1 can be used.

Observation 9: The legacy Rel-17 U2N relay mechanism or the mechanism for approach 1 can be used for approach to support the System information/ paging forwarding, which depends on whether there is E2E PC5 connection between the remote UE and the last relay UE.

### 2.2.7 Radio link failure handling

In approach 2, we will also have the RLF declaration cases in figure 4.

In case (a) of Figure 5, when the remote UE detects the PC5 RLF, it should trigger the RRC connection re-establishment or relay reselection, which is similar to the Rel-17 handling.

In case (b) and (c) of Figure 5, when the relay UEs between the remote UE and the last relay UE detect the PC5 RLF, the relay UE needs to notify the remote UE to change the relaying path, therefore notification message is also needed on the PC5 link. If there is no E2E PC5 connection between non-adjacent UEs, the hop-by-hop notification messages are needed.

In case (d) of Figure 5, if there is E2E PC5 connection between the remote UE and the last relay UE, the last relay UE can send a notification message to the remote UE by just using the Rel-17 mechanism. If there is not, hop-by-hop notification messages are needed. The remote UE can decide whether to keep to release the connection with the last relay UE, and notify the intermediate relay UE to keep to release the relaying configurations.

Above all, the RLF handling can be similar to that in approach 1, with some minor difference, we can discuss the detailed solutions after the approach 2 mechanism is determined.

Observation 10: The RLF handling in approach 2 can be similar to that in approach 1, with some minor difference.

### 2.2.8 Limitation on the service continuity

The WI objective states that [4]

- Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

A serious limitation of approach 2 is that Service Continuity Scenario C and D cannot be supported with approach 2 as relay UEs can be in different cell/ different gNB and bringing them to connected state will be extremely complex requiring inter-gNB signalling.

Path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell cannot be achieved by Approach 2 as the gNB has no control over the relay UE which might be connected to different gNB

Observation 11: A serious limitation of approach 2 is that Service Continuity Scenarios C and D cannot be supported with approach 2 as relay UEs can be in different cells/ different gNBs and bringing them to connected state will be extremely complex.

### 2.1.9 Forward compatibility

In approach 2, since the intermediate relay UE can be in RRC IDLE/INACTIVE, or under the control of a different network node, the remote UE connects to the last relay UE using the mechanism similar to the U2U relay. If there is only one relay UE between the remote UE and the last relay UE, the U2U relay mechanism can be reused. However, if we want to support two and more addition hops, RAN2 will need to discuss the multi-hop U2U relay.

Proposal 20. For Approach 2, RAN2 need to discuss the multi-hop U2U relay to extend the Rel-18 U2U relay, to support the multi-hop relay with two and more additional hops.

## Comparison of approach 1 and approach 2

Based on above discussion, we can summary a table to compare approach 1 and approach 2 as below.

Table 1: Comparison of Approach 1 and Approach 2

|  | Approach 1 | Approach 2 |
| --- | --- | --- |
| Connection establishment | Legacy Rel-17 U2N relay mechanism can be reused. | Some issues are still not clear, like the local ID allocation, the bearer mapping determination, |
| Protocol stack | Extend the Rel-17 U2N relay to consider the additional hops | The same with approach 1. In addition, the CP protocol stack of multi-hop U2U relay may be needed between the remote UE and the last relay UE. |
| Local ID allocation | Reuse legacy Rel-17 U2N relay mechanism and gNB performs the local ID allocation | Complex. Local ID confliction can happen on the intermediate relay UE. |
| QoS split | Reuse Rel-17 mechanism that gNB guarantees the QoS of the remote UE | Complex. How to perform the split QoS on the PC5 hops is not clear. |
| SRAP configuration | Legacy SRAP configuration for Rel-17 U2N relay can be reused with limited enhancements. | Complex. |
| SRAP PDU format | Reuse legacy SRAP PDU format for Rel-17 U2N relay | There can be different SRAP PDU formats on the Uu hop and on the PC5 hop. |
| System information/ paging forwarding | Reuse Rel-17 mechanism with limited enhancements | Reuse Rel-17 mechanism or can use the mechanism for approach 1. |
| RLF handling | Reuse Rel-17 mechanism with limited enhancements | Reuse Rel-17 mechanism with limited enhancements |
| Latency problem | The latency problem exists, happened in corner cases, and can be solved with some simple solution. | The latency problem exists, but unavoidable since the large latency is from the baseline procedures, like local ID allocation, QoS split, and bearer mapping configuration determination. |
| Support of service continuity cases | Can support all the service continuity cases, i.e., cases A, B, C, and D | Cannot support the service continuity cases C and D |
| Extendable to more hops | Yes, extendable and no additional spec impact to support two and more hops. | Yes, extendable. May need to support multi-hop U2U relay feature to support two and more hops. |

Based on above comparison, we can have the following observation and proposals.

Observation 12. Based on the comparison of approach 1 and approach 2 in Table 1, we can have the following conclusion for approach 1.

- The local ID allocation, QoS spilt, SRAP configuration mechanisms for approach 1 is straightforward and does not require any complex procedures. Approach 1 is the natural extension of R17 U2N Relay and can support all of these procedures without any complexity.

- The latency problem only happens in corner cases where the remote UE triggers the connection establishment when all the relay UE are in idle/ inactive state, and can be solved with some simple solution

Observation 13. Based on the comparison of approach 1 and approach 2 in Table 1, we can have the following conclusion for approach 2.

- The local ID allocation, QoS spilt, SRAP configuration mechanisms for approach 2 is complex and the complexity increases with the  hops  on PC5 link since there is not central node.

- The latency problem is unavoidable since the latency is inherited from the baseline procedures.

- Cannot support the service continuity cases C and D

- May need to support multi-hop U2U relay feature to support two and more hops.

Proposal 21: Considering the complexity and limitations in approach 2, RAN 2 agrees not to adopt Approach 2 for Rel-19 multi-hop relay.

Proposal 22 : RAN2 can agree to adopt approach 1 to support the Rel-19 multi-hop relay, which would naturally ensure forward compatible for future extensions with additional relays.

# 3	Conclusion

## CP procedures for Approach 1

Connection establishment

Latency for Connection Establishment Procedure

Observation 1: The scenario where all the relay UEs involved in the multi hop connection establishment are in the RRC IDLE/INACTIVE state when the remote UE triggers the connection establishment is unlikely to occur frequently in practice. A more common  scenario in is that the last and intermediate relay UEs (including the first relay UE) are already in connected state and have established the connection to the network for their own traffic before they start serving the remote UEs.

Observation 2: Latency increase is not specific to Approach 1 only. The data transmission from the remote UE needs to wait after the UE ID allocation/QoS split/bearer configuration is done for both the approaches. There will be an increased Uu signalling in Approach 1, that is controlled by the network. For approach 2, there will be increased SL signalling among the relay UEs due to local ID assignment and QoS split.

Proposal 2: Latency increase is not specific to Approach 1 and shall not be used as a show stopper for adopting Approach 1 for the multi hop relay in Rel 19. The latency can be improved for Approach 1 using the following solutions.

- Remote UE can prioritize selecting  an RRC CONNCETED relay UE if the service requirements of remote UE are stringent.

- Restriction/limit on the maximum hop number can be applied between the relay UEs and the network when the RRC IDLE/INACTIVE relay UEs perform the multi-hop relay

- SRB0 messages of remote UE can be forwarded before connection establishment of each intermediate relay UE via the default PC5 relay RLC channels between UEs, which can trigger the parallel connection establishment of the RRC IDLE/INACTIVE relay UE

Need for DRB Establishment for Relay UEs

Proposal 3: No DRBs or default DRB needs to be established for the last or intermediate relay UEs to move them from RRC IDLE to RRC Connected state before serving the remote UE when the remote UE triggers the connection establishment procedure.

Failures during connection establishment

Proposal 4: Extend the T300 timer for multi-hop U2N relay operation in accordance with the hop count to avoid any failures during connection establishment procedure.

Other FFSs

Proposal 5a: The PC5-RRC connection establishment between adjacent relay UEs after transmission of the first RRC message in step 2 is not supported.

Proposal 5b. The Last Relay UE cannot send the SUI on behalf of all other relay UEs. Each relay UE shall report the SUI and indicate the L2 ID of its adjacent child UE to the gNB.

Proposal 5c. RAN2 can discuss the necessity to enable the parallel RLC channel establishment for relaying of SRB1 at each relay UE.

Protocol stack

Proposal 6: Adopt the Control Plane and User Plane protocol stack for Multi-hop L2 U2N relay as shown in Figure 3. SRAP layer is reused for bearer mapping and remote UE identification purpose.

Local ID allocation

Proposal 7: Reusing the single-hop relay mechanism to support the Local ID allocation for multi-hop relay:

- Parent relay UE of the remote UE reports the L2 ID to the gNB to request the local ID allocation, the uniqueness of the local ID can be guaranteed by the gNB by implementation.

- The remote UE local ID is 8 bits.

QoS split

Proposal 8. The QoS split is determined by gNB and guaranteed by gNB by implementation

SRAP PDU format and SRAP configuration

Proposal 9: The legacy SRAP PDU format can be reused for multi-hop relay, i.e., remote UE ID and bearer ID are included in the SRAP PDU header for data forwarding.

Proposal 10: The SRAP configuration contains the mapping configuration between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel.

- For the first relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the egress PC5 relay RLC channel between the first relay UE and remote UE, the egress PC5 relay RLC channel between the first relay UE and its parent relay UE.

- For intermediate relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the identity (L2 ID) of its child relay UE, the egress PC5 relay RLC channel between the intermediate relay UE and its child relay UE, the egress PC5 relay RLC channel between the intermediate relay UE and its parent relay UE.

- For the last relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the identity (L2 ID) of its child relay UE, the egress Uu relay RLC channel on Uu, the egress PC5 relay RLC channel between the last relay UE and its child relay UE

Proposal 11: For handling the Multi hop relay path information there are two options that can be considered

Option 1 - The path information can be implicitly delivered by last or the intermediate relay UE by mapping local ID of the remote UE (carried in the SRAP header) to the L2 ID of the child relay UE during the uplink transmission of RRCSetupRequest message

Option 2 – The path information can be explicitly provided through the identity (L2 ID) of the child relay UE in the SRAP configuration for the intermediate and last relay UE.

System information/ paging forwarding

Proposal 12: The intermediate Relay UE should indicate all the required SIB(s) requested from all the connected remote UE(s) and the intermediate Relay UE itself to its parent relay UE.

Proposal 13: The intermediate Relay UE needs to indicate all the UE ID of the connected RRC IDLE/NACTIVE UEs to its parent relay UE for paging delivery.

Radio Link failure handling

Proposal 14: Similar to Rel-17 SL relay, when the remote UE in multi-hop relay detects the PC5 RLF, it can trigger the RRC connection re-establishment.

Proposal 15: When the first relay UE detects the PC5 RLF, it can send a notification message with cause value “PC5 RLF” to the remote UE and trigger the RRC connection re-establishment.

Proposal 16: Upon receiving the notification message with cause value “PC5 RLF”, the remote UE can determine whether to release the PC5 link with the first relay UE and triggers the RRC connection re-establishment.

Proposal 17: Upon receiving the notification message with cause value “Uu/PC5 RLF” from the parent relay UE, the intermediate relay UE needs to send a notification message to its child UE. The following options can be considered as the cause value.

- Option 1: introduce a new cause value which indicates Uu/PC5 RLF of the relay UE, with the identity of the relay UE

- Option 2: introduce a new cause value which indicates RLF has happened on the indirect link, without the identity of the relay UE that detects the RLF.

Forward compatibility

Proposal 18. Based on approach 1, we can naturally support multi-hop relay with two and more additional hops.

## CP procedures for Approach 2

Connection establishment

Observation 3: Most proponents for approach 2 supports the case that the intermediate relay UE can be in a different serving gNB or in different RRC state as the remote UE when the remote UE triggers connection establishment.

Proposal 19: For approach 2 , before considering its adoption for multi-hop relay, we need to address the following issue when the intermediate relay UE can be in a different serving gNB or in different RRC state as that of the remote UE.

- The local ID allocation to identify the remote UE

- Split QoS determination for each hop to determine the PC5 relay RLC channel configuration

- Bearer mapping configuration determination on each hop.

Protocol stack

Observation 4: The UP and CP protocol stacks for approach 2 are the same as that in approach 1. In addition, we may need the CP protocol stack of multi-hop U2U relay between the remote UE and the last relay UE.

Local ID allocation

Observation 5: Allocation of local ID could be  extremely complex with Approach 2, since there is no centralized node to guarantee the uniqueness of the remote UE local ID along the path.

QoS split

Observation 6: Significant complex new PC5 signalling procedures  have to be designed for coordinating the Split QoS on each hop for Approach 2.

SRAP PDU format and SRAP configuration

Observation 7: the SRAP PDU format used for approach 2 depends on the local ID allocation mechanism.

Observation 8: How to configure the RLC channel for a relay UE that is in RRC_IDLE/RRC_INACTIVE for approach 2 is unclear. Different RRC state and coverage states needs to be discussed/captured separately. Additionally, whether the remote/relay UEs are in the same or  different cell needs to be discussed/captured separately.

System information/ paging forwarding

Observation 9: The legacy Rel-17 U2N relay mechanism or the mechanism for approach 1 can be used for approach to support the System information/ paging forwarding, which depends on whether there is E2E PC5 connection between the remote UE and the last relay UE

Radio Link failure handling

Observation 10: The RLF handling in approach 2 can be similar to that in approach 1, with some minor difference.

Limitation on the service continuity

Observation 11: A serious limitation of approach 2 is that Service Continuity Scenarios C and D cannot be supported with approach 2 as relay UEs can be in different cells/ different gNBs and bringing them to connected state will be extremely complex.

Forward compatibility

Proposal 20. For Approach 2, RAN2 need to discuss the multi-hop U2U relay to extend the Rel-18 U2U relay, to support the multi-hop relay with two and more additional hops

## Comparison of approach 1 and approach 2

Observation 12. Based on the comparison of approach 1 and approach 2 in Table 1, we can have the following conclusion for approach 1.

- The local ID allocation, QoS spilt, SRAP configuration mechanisms for approach 1 is straightforward and does not require any complex procedures. Approach 1 is the natural extension of R17 U2N Relay and can support all of these procedures without any complexity.

- The latency problem only happens in corner cases where the remote UE triggers the connection establishment when all the relay UE are in idle/ inactive state, and can be solved with some simple solution

Observation 13. Based on the comparison of approach 1 and approach 2 in Table 1, we can have the following conclusion for approach 2.

- The local ID allocation, QoS spilt, SRAP configuration mechanisms for approach 2 is complex and the complexity increases with the  hops  on PC5 link since there is not central node.

- The latency problem is unavoidable since the latency is inherited from the baseline procedures.

- Cannot support the service continuity cases C and D

- May need to support multi-hop U2U relay feature to support two and more hops.

Proposal 21: Considering the complexity and limitations in approach 2, RAN 2 agrees not to adopt Approach 2 for Rel-19 multi-hop relay.

Proposal 22 : RAN2 can agree to adopt approach 1 to support the Rel-19 multi-hop relay, which would naturally ensure forward compatible for future extensions with additional relays.

# 4	Reference

- Session Chair Notes on Relay and Positioning RAN2 #127;

- Session Chair Notes on Relay and Positioning RAN2 #128;

- R2-2410006, Report of [POST127][402][Relay] Multi-hop relay control plane.

- RP-242349, New WID on NR sidelink multi-hop relay.