3GPP TSG-RAN WG2 Meeting #129	R2-2500702

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.13.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on service continuity for Multi-hop Relay

Document for:	Discussion and Decision

# Introduction

In the WID for NR sidelink multi-hop relay in [1], we have the following description for service continuity scenarios for multi-hop U2N relay.

Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In RAN2 128 meeting, the two scenarios Scenario A and B in the first priority were discussed and it was agreed that the Figure 1 and Figure 2 of R2-2410589 [3] can be used as a baseline for intra-gNB multi-hop indirect to direct path switching procedure to be captured in TS 38.300. In this paper, we discuss the two scenarios in the second priority and provide the signalling flows for these two scenarios. Additionally, we also provide the text proposal for the scenario A and B in the annex which can be used as a baseline for TS 38.300.

# Discussion

## 2.1 Service continuity procedures

In the following subsection, the stage-2 procedures are described for:

- Intra-gNB direct to multi-hop indirect path switching, and

- Intra-gNB single-hop indirect to multi-hop indirect path switching,

## 2.1.1 Intra-gNB direct to multi-hop indirect path switching

Figure 1 shows the procedure for the intra-gNB direct to multi-hop indirect path switching case which is extended from single hop intra-gNB  direct to indirect path switching procedure as defined in clause 16.12.6.2 in TS 38.300.

Figure 1.  Intra-gNB direct to multi-hop indirect path switching procedure

Step 1.	The UE (i.e., L2 U2N Remote UE in next steps)  reports one or multiple candidate L2 U2N Relay UE(s), after it measures/discovers the candidate L2 U2N Relay UE(s). The triggering Event Y1 and Event Y2 can be used in the measurement configuration.

-	Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2)

-	Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold)

Therefore, we have the following proposals

Proposal 1: The triggering Event Y1/Y2 can be adopted and reused in the measurement configuration for the multi-hop U2N relay.

Proposal 2: The measurement reporting includes at least the L2 ID of the First Relay UE, the First Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the First Relay UE).

Step 2.	The gNB decides to switch the L2 U2N Remote UE onto multi-hop indirect path.

Step 3.     The gNB sends the RRCReconfiguration message to the targe L2 U2N multi-hop relay (last relay, intermediate relay and the first relay UE) to reconfigure the connection for Remote UE .

Step 4.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the first relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end Uu radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the direct path after reception of the RRCReconfiguration message from the gNB.

Step 5.	The L2 U2N Remote UE establishes PC5-RRC connection with the first relay UE.

Step 6.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the L2 U2N Relay UEs (last relay, intermediate relay and the first relay UE).

Step 7.	The data path is switched from direct to multi-hop indirect path between the L2 U2N Remote UE and the gNB. The PDCP re-establishment or PDCP data recovery in uplink is performed by the L2 U2N Remote UE for lossless delivery during path switch if gNB configures it .

To summarize,  we have the following proposal

Proposal 3: Agree the above procedure in Figure 1 as the baseline for intra-gNB direct to multi-hop indirect path switching procedure to be captured in TS 38.300.

## 2.1.2 Intra-gNB single-hop indirect to multi-hop indirect path switching

Figure 2 shows the intra-gNB single-hop indirect to multi-hop indirect path switching procedure  which is extended from single hop intra-gNB  indirect to indirect path switching procedure as defined in clause 16.12.6.3 in TS 38.300.

Figure 2.  Intra-gNB single-hop indirect to multi-hop indirect path switching procedure

Step 1.	 The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s), when the sidelink measurement quantity shows deteriorated link quality. The triggering Event Z1 and Event Z2 can be used in the measurement configuration.

- Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2

- Event Z2 (if agreed): Candidate L2 U2N Relay UE becomes an offset better than the serving L2 U2N Relay UE

Therefore, we have the following proposals

Proposal 4: The triggering Event Z1 and Event Z2 (if agreed) can be adopted and reused in the measurement configuration for the multi-hop U2N relay.

Proposal 5: The measurement reporting includes at least the L2 ID of the First Relay UE, the First Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the First Relay UE).

Step 2.	The gNB decides to switch the L2 U2N Remote UE onto multi-hop indirect path.

Step 3.    The gNB sends the RRCReconfiguration message to the target L2 U2N multi-hop relay (last relay, intermediate relay and the first relay UE) to reconfigure the connection for Remote UE .

Step 4.   The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the target L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic, and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops UP and CP transmission over the single-hop indirect path via the source L2 U2N relay UE after the reception of the RRCReconfiguration message from the gNB.

Step 5.     The L2 U2N Remote UE establishes PC5-RRC connection with the target First Relay UE.

Step 6.	The L2 U2N Remote UE sends the RRCReconfigurationComplete message to the gNB via the targe L2 U2N multi-hop relay (last relay, intermediate relay and the first relay UE), using the configuration provided in the RRCReconfiguration message. From this step, the L2 U2N Remote UE uses the RRC connection via the targe L2 U2N multi-hop relay to the gNB .

Step 7.  The gNB sends the RRCReconfiguration message to the source L2 U2N relay to reconfigure the connection between the source L2 U2N relay UEs and the gNB. The RRCReconfiguration message to the source L2 U2N relay UE can be sent any time after step 4 based on gNB implementation (e.g., to release Uu or PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

Step 8.	Either source L2 U2N Relay UE's AS layer or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation after step 4 or step7.

Step 9.	The data path is switched from the source L2 U2N Relay UE to the target L2 U2N multi-hop Relay UE between the L2 U2N Remote UE and the gNB. This step can be any time after step 7.

To summarize,  we have the following proposal

Proposal 6: Agree the above procedure in Figure 2 as the baseline for intra-gNB single-hop indirect to multi-hop indirect path switching to be captured in TS 38.300.

## 2.1.3 Multi-hop target relaying path selection

Unlike the traditional direct-to-indirect path switching, the gNB needs to select a multi-hop relaying path based on the measurement reporting, not just the target relay UE for the remote UE to do path switching. The following information  can help gNB to choose a better target path.

- Relay UE’s serving cell ID, and the sidelink measurement quantity between the remote UE and the first relay UE.

- The sidelink measurement quantity between the adjacent relay UEs on the candidate multi-hop relaying path.

- The hop number of the relaying path

Proposal 7: the gNB can determine the target multi-hop relaying path based on the following information:

- First Relay UE’s serving cell ID, and the sidelink measurement quantity between the remote UE and the first relay UE.

- The sidelink measurement quantity between the adjacent UEs on the candidate multi-hop relaying path.

- The hop number of the relaying path

The information in the first item could  already be contained in the measurement reporting by reusing the Rel-17 mechanism.

The information in the second item is important, since the sidelink measurement quantity on each hop can impact the data forwarding of the remote UE. Based on legacy mechanism, remote UE can only measure and report the sidelink measurement quantity of the first relay UE. Therefore, enhancements are needed to take the sidelink measurement quantity between adjacent relay UEs along the candidate relaying path into consideration when selecting the target relaying path.

One possible solution is the relay UEs can indicates the sidelink measurement quantity and of its adjacent relay UE (in upstream direction) to the its adjacent relay UE in the downstream direction.  With this method, the first relay UE can get all the sidelink measurement quantity  of each hop of the Relay UEs along its relaying path, then it can forward the sidelink measurement quantity to the remote UE in the discovery message. Remote UE can use the per-hop sidelink measurement quantity to filter proper candidate relaying path, or just report this information to gNB for target path determination.

Proposal 8: The remote UE can obtain the sidelink measurement quantity information between the adjacent relay UEs  along the candidate multi-hop relaying path to help determining a better target path. This information can be provided by the first relay UE in the discovery message.

Based on current RAN2 discussion, we have two candidate approaches to support the multi-hop relay.

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC.

For approach 1, since we assume “The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell”, the gNB can know the hop number of the relaying path since all the relay UE are in RRC_Connected state.

As for approach 2, it cannot support scenario C and D since the relay UEs except for the last relay UE can be connected to a different gNB from the one serving  remote UE/last relay UE. Therefore, we do not consider approach 2 when discussing the scenario C and D.

Observation 1: For Approach 1 the gNB can know the hop number of the relaying path since all the relay UE are in RRC_Connected state. However, this is not possible for Approach 2 as relay UEs, except for the last relay UE,  can be connected to a different gNB

For Approach 1, we propose reusing the Rel-17 direct discovery procedures, where the intermediate relay UE first searches for a parent relay UE, followed by the remote UEs searching for the first relay UE in the multi-hop scenario [3]. Once the first relay UE connects to a parent relay UE, it will then perform the discovery process as a relay UE. If the first relay UE provides its RRC state information in the discovery message and is in the RRC_CONNECTED state, the remote UE can infer that all upstream relays in the multi-hop chain are also in the RRC_CONNECTED state. This enables the remote UE to select an appropriate path where all relay UEs are in the RRC_CONNECTED state, ensuring service continuity for scenarios C and D

Proposal 9: The remote UE can obtain the RRC state information of the first relay UE from the discovery message, enabling it to select an appropriate path where all relay UEs are in the RRC_CONNECTED state.

# Conclusion

In this paper, we discuss the two scenarios in the second priority and provide the signalling flows for these two scenarios. We have the following observation and proposals.  The text proposal of the two scenarios in the first priority (Priority A and Priority B)  is also provided in the annex.

Service Continuity Procedures

Intra-gNB direct to multi-hop indirect path switching

Proposal 1: The triggering Event Y1/Y2 can be adopted and reused in the measurement configuration for the multi-hop U2N relay.

Proposal 2: The measurement reporting includes at least the L2 ID of the First Relay UE, the First Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the First Relay UE).

Proposal 3: Agree the above procedure in Figure 1 as the baseline for intra-gNB direct to multi-hop indirect path switching procedure to be captured in TS 38.300.

Intra-gNB single-hop indirect to multi-hop indirect path switching

Proposal 4: The triggering Event Z1 and Event Z2 (if agreed) can be adopted and reused in the measurement configuration for the multi-hop U2N relay.

Proposal 5: The measurement reporting includes at least the L2 ID of the First Relay UE, the First Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the First Relay UE).

Proposal 6: Agree the above procedure in Figure 2 as the baseline for intra-gNB single-hop indirect to multi-hop indirect path switching to be captured in TS 38.300.

Multi-hop target relaying path selection

Proposal 7: the gNB can determine the target multi-hop relaying path based on the following information:

- First Relay UE’s serving cell ID, and the sidelink measurement quantity between the remote UE and the first relay UE.

- The sidelink measurement quantity between the adjacent UEs on the candidate multi-hop relaying path.

- The hop number of the relaying path

Proposal 8: The remote UE can get the sidelink measurement quantity information between the adjacent relay UEs  and the RRC State information along the candidate multi-hop relaying path to help determining a better target path.

Observation 1: For Approach 1 the gNB can know the hop number of the relaying path since all the relay UE are in RRC_Connected state. However, this is not possible for Approach 2 as relay UEs, except for the last relay UE,  can be connected to a different gNB

Proposal 9: The remote UE can obtain the RRC state information of the first relay UE from the discovery message, enabling it to select an appropriate path where all relay UEs are in the RRC_CONNECTED state.

# References

[1] RP-242349 Revised WID SL_relay_multihop

[2] 3GPP TS 38.300, V18.3.0, NR; NR; NR and NG-RAN Overall Description

[3] R2-2500700	Relay discovery and (re)selection for multi-hop Relay

# Annex: Text Proposal

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Start Changes>>>>>>>>>>>>>>>>>>>>>>>>>>

### 16.12.X	Service Continuity for multi-hop U2N relay

#### 16.12.X.0	General

The service continuity procedure is applicable for the mobility cases of path switch from multi-hop indirect to single-hop indirect or direct path and from single-hop indirect or direct to multi-hop indirect path when the L2 U2N Remote UE and L2 U2N Relay UEs belong to the same gNB. For intra-gNB path switching, the gNB decides to trigger path switching and the path switch type.

#### 16.12.X.1		Intra-gNB multi-hop indirect to direct path switching

For service continuity of multi-hop U2N relay, the following procedure is used, in case of multi-hop U2N Remote UE switching from multi-hop indirect to direct path under the same gNB:

Figure 16.12.X.1-1.  Intra-gNB multi-hop indirect to direct path switching procedure

- The Uu measurement configuration and measurement report signalling procedures are performed to evaluate both multi hop relay link measurement and Uu link measurement. The measurement results from L2 U2N Remote UE are reported when configured measurement reporting criteria are met. The sidelink relay measurement report shall include every L2 U2N Relay UE's source L2 ID on the mult hop, serving cell ID (i.e., NCGI/NCI), and sidelink measurement quantity result. The sidelink measurement quantity can be SL-RSRP of the serving L2 U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used.

2.	The gNB decides to switch the L2 U2N Remote UE onto direct Uu path.

3.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The L2 U2N Remote UE stops User Plane and Control Plane transmission via the L2 U2N Relay UE after reception of the RRCReconfiguration message with the path switch configuration.

4.	The L2 U2N Remote UE synchronizes with the gNB and performs Random Access.

5.	The UE (i.e., L2 U2N Remote UE in previous steps) sends the RRCReconfigurationComplete message to the gNB via the direct path, using the configuration provided in the RRCReconfiguration message. From this step, the UE (i.e., L2 U2N Remote UE in previous steps) uses the RRC connection via the direct path to the gNB.

6.	The gNB sends the RRCReconfiguration message to the L2 U2N Relay UE to reconfigure the connection between the L2 U2N Relay UE and the gNB. The RRCReconfiguration message to the L2 U2N Relay UE can be sent any time after step 3 based on gNB implementation (e.g., to release Uu Relay RLC Channel and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

7.	Either L2 U2N Relay UE or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation.

8.	The data path is switched from indirect path to direct path between the UE (i.e., previous L2 U2N Remote UE) and the gNB. The PDCP re-establishment or PDCP data recovery in uplink is performed by the UE (i.e., previous L2 U2N Remote UE) for lossless delivery during path switch if gNB configures it.

NOTE 1:	Step 8 can be executed any time after step 4. Step 8 is independent of step 6 and step 7.

#### 16.12.X.2		Intra-gNB multi-hop indirect to single-hop indirect path switching

The gNB can select a L2 U2N Relay UE in any RRC state i.e., RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED, as a target L2 U2N Relay UE for multi-hop indirect to single-hop indirect path switch.

For service continuity of multi-hop U2N relay, the following procedure is used, in case of multi-hop U2N Remote UE switching from multi-hop indirect to single-hop indirect path under the same gNB:

Figure 16.12.X.2-1.  Intra-gNB multi-hop indirect to single-hop indirect path switching procedure

1.	The L2 U2N Remote UE reports one or multiple candidate Relay UE(s) and sidelink measurement between First Relay UE and L2 U2N Remote UEUE to the gNB, after it measures/discovers the candidate L2 U2N Relay UE(s)

-	The L2 U2N Remote UE filters the appropriate L2 U2N Relay UE(s) according to relay selection criteria before reporting. The L2 U2N Remote UE shall report only the L2 U2N Relay UE candidate(s) that fulfil the higher layer criteria;

- The reporting includes at least the L2 U2N Relay UE ID, a L2 U2N Relay UE’s serving cell ID, and sidelink measurement quantity information. SD-RSRP is used as sidelink measurement quantity..

2.	The gNB decides to switch the L2 U2N Remote UE to a target L2 U2N Relay UE under the same gNB.

3.   The gNB sends an RRCReconfiguration message to the target L2 U2N Relay UE, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

4.   The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the target L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic, and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops UP and CP transmission over the multi-hop indirect path via the source L2 U2N multi-hop relay after the reception of the RRCReconfiguration message from the gNB.

5.  The L2 U2N Remote UE establishes PC5-RRC connection with the target L2 U2N Relay UE.

6.	The L2 U2N Remote UE sends the RRCReconfigurationComplete message to the gNB via the target L2 U2N Relay UE, using the configuration provided in the RRCReconfiguration message. From this step, the L2 U2N Remote UE uses the RRC connection via the target L2 U2N Relay UE to the gNB .

7.  The gNB sends the RRCReconfiguration message to the source L2 U2N multi-hop relay to reconfigure the connection between the source L2 U2N multi-hop relay UEs and the gNB. The RRCReconfiguration message to the source L2 U2N multi-hop relay UEs can be sent any time after step 4 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

8.	Either source First L2 U2N Relay UE's AS layer or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation after step 4 or step7.

9.	The data path is switched from the source L2 U2N Relay UE to the target L2 U2N Relay UE between the L2 U2N Remote UE and the gNB. This step can be any time after step 7.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<End Changes>>>>>>>>>>>>>>>>>>>>>>>>>