3GPP TSG-RAN WG2 Meeting #129	R2-2500703

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	 8.8.1

Source: 	Huawei, HiSilicon, Turkcell

Title: 	Discussion on simultaneous operation between GNSS and NR NTN

Document for:	Discussion and Decision

# Introduction

In LS on simultaneous operation between GNSS and NR NTN[1], RAN4 reach consensus that simultaneous GNSS and NR-NTN operation for (e)Redcap UE and non-(e)Redcap UE is required, however it cannot be guaranteed in some occasion for some implementations for some NTN bands.

- Actions To RAN WG2: ACTION: - RAN4 respectfully ask to introduce improvements, if necessary, to existing IDC solution(s) and related IDC capabilities to address this in-device coexistence issue between GNSS reception and NR NTN transmission.

In this paper, we discuss the details of the key issues for reliable in-device co-existence solution.

# Discussion

When configured by the gNB, the UE reports IDC problem along with the related FDM and TDM assistance information to the network. The network can use either TDM or FDM mechanism to resolve IDC interference upon receiving the IDC assistance information from the UE.

In addition to the FDM and TDM based solution , gNB can configure the UE to perform autonomous denial to address IDC problem. Once configured by the network, the UE can autonomously deny UL transmission in a particular UL slot as long as it is within the maximum number of the UL slots for which the UE is allowed to deny any UL transmission.

Using FDM solution to resolve IDC problem by moving the UE to a different frequency may not be feasible in the NTN due to limited number of available frequency bands within the NTN deployment. Therefore, FDM solution may not be well suited for NTN.

Regarding the autonomous denial, UE’s action for autonomous denial may occur at randomly and the NTN network is not aware of it. Additionally,  the autonomous denial may not create a sufficiently long uplink gap for the UE to receive GNSS signal.

In contrast,  the TDM solution utilizes the discontinuous reception (DRX) mechanism which is capable of creating sufficient gaps.

For NTN UE configured with DRX the active duration/time and inactive duration/time during the DRX cycle is characterised as follows:

- Active duration/time: The UE performs NR-NTN transmission during this period.

- Sleep/Inactive duration/time: The UE performs GNSS reception during this period.

Given these factors, we believe the TDM solution is the most suitable approach to address the in-device coexistence issue between GNSS reception and NR-NTN transmission. This solution is further explored in detail.

## 2.1 Conflict between GNSS reception duration and DRX active time

The enhanced R18 IDC solution was designed primarily for terrestrial networks and appears to work well in that context. However, for the NTN networks the DRX cycle must meet the following conditions: Sleep/Inactive time must be greater than the GNSS reception time. This ensures that the UE can use the updated GNSS information when the next active time begins.

According to the TS36.331, the processing time for the UE to receive GNSS in LTE is specified as follows:

#### – GNSS-PositionFixDuration The IE GNSS-PositionFixDuration indicates the time duration required for the UE to acquire a GNSS position. Value s1 corresponds to 1 second, s2 corresponds to 2 seconds and so on. GNSS-PositionFixDuration information element -- ASN1START GNSS-PositionFixDuration-r18 ::= ENUMERATED{ s1, s2, s3, s4, s5, s6, s7, s13, s19, s25, s31} -- ASN1STOP

According to TS 38.331, The DRX cycle and active time length are specified as follows:

IDC-TDM-Assistance-r18 ::= SEQUENCE { cycleLength-r18 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32, ms35, ms40, ms60, ms64, ms70, ms80, ms96, ms100, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240}, startOffset-r18 INTEGER (0..10239), slotOffset-r18 INTEGER (0..31), activeDuration-r18 CHOICE { subMilliSeconds INTEGER (1..31), milliSeconds ENUMERATED { ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200, ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } }, ... }

For UEs that  require a GNSS reception time longer than 10 seconds, the GNSS reception cannot be completed during the sleep/inactive time, even when the maximum DRX cycle (10240 ms) and minimum active time (1 ms) are configured. This creates a conflict between the GNSS reception duration and the current DRX active time due to the limitations of the UE's ability to receive GNSS.

If the UE’s location information cannot be updated from the GNSS, NR-NTN transmission may not function properly, particularly in scenarios where the UE location changes rapidly.

Observation 1: For UEs that require a GNSS reception time longer than 10s, the GNSS reception cannot be completed during the sleep/inactive time.

Proposal 1: It  is important  to discuss whether the GNSS reception time specified in LTE can be directly reused in NR-NTN.

Due to the current DRX cycle length, some GNSS reception times cannot be completed in the sleep/inactive time, creating a conflict with the DRX  active time. Therefore, a solution is needed to address this problem. One possible solution is to extend the DRX cycle length for IDC-TDM assistance, for example, up to 64 seconds while also extending the active duration. This would allow for more inactive/sleep time for UEs that need over 10 seconds of GNSS reception.

Proposal 2: If Proposal 1 is accepted, a simple solution to resolve the conflict between GNSS reception duration and DRX active time would be to extend the DRX cycle length for IDC-TDM assistance, for example, up to 64 seconds, while also extending the active duration. This would allow for more inactive/sleep time for UEs that need over 10 seconds of GNSS reception.

## 2.2 IDC TDD Solution enhancement for improved NR-NTN scheduling efficiency

If we further analyse IDC issue caused by NR-NTN transmissions we can observe that only the uplink (UL) power leakage of NR-NTN transmissions interferes with GNSS reception while  NR-NTN downlink (DL) reception does not affect the GNSS reception. However, the current IDC TDM solution utilizing DRX mechanism stops uplink and downlink transmissions during the sleep/inactive time, meaning unaffected downlink transmissions/receptions cannot be performed. This will result  in low NR-NTN scheduling efficiency when the enhanced TDM IDC solution with extended DRX cycle is used for NR-NTN.

Observation 2: Only the uplink (UL) power leakage of NR-NTN transmissions interferes with GNSS reception while  NR-NTN downlink (DL) reception does not affect the GNSS reception

Observation 3: The current IDC TDM solution utilizing DRX mechanism stops uplink and downlink transmissions during the sleep/inactive time, meaning unaffected downlink transmissions cannot be performed, which will result  in low NR-NTN scheduling efficiency.

One possible way to address low scheduling efficiencies is to use the enhanced TDM IDC solution only for uplink for NTN while the DL transmissions can continue without any restrictions even during the inactive/sleep period.

It is important to note that if DL transmission is allowed while UL transmission is not, additional measures may be necessary to prevent uncontrolled UL transmission. For instance, UL feedback (ACK/NACK) from the UE could be blocked by disabling HARQ in NTN.

Regarding CSI reporting, even with the existing DRX mechanism, the UE will not report CSI on PUCCH during its sleep/inactive time. The issue with CSI reporting on PUSCH is not specific to the enhanced DRX solution and it is a common issue with the traditional DRX solution. Therefore, CSI reporting can indeed reuse existing operations, as it may only affect GNSS acquisition at certain times, without significantly impacting the overall GNSS acquisition process

Hence we propose

Proposal 3: Use the enhanced TDM IDC solution only for uplink transmissions for NTN while the DL transmissions can continue without any restrictions even during the inactive/sleep period.

Proposal 4: UL feedback (ACK/NACK) from the UE could be blocked by disabling HARQ when using enhanced TDM IDC solution for uplink.

# Conclusion

In this paper, we discuss how to resolve this in-device coexistence issue between GNSS reception and NR NTN transmission, using existing IDC solution(s) and related capabilities. We present the following observation and proposals.

Observation 1: For UEs that require a GNSS reception time longer than 10s, the GNSS reception cannot be completed during the sleep/inactive time.

Proposal 1: It  is important  to discuss whether the GNSS reception time specified in LTE can be directly reused in NR-NTN.

Proposal 2: If Proposal 1 is accepted, a simple solution to resolve the conflict between GNSS reception duration and DRX active time would be to extend the DRX cycle length for IDC-TDM assistance, for example, up to 64 seconds, while also extending the active duration. This would allow for more inactive/sleep time for UEs that need over 10 seconds of GNSS reception.

Observation 2: Only the uplink (UL) power leakage of NR-NTN transmissions interferes with GNSS reception while  NR-NTN downlink (DL) reception does not affect the GNSS reception

Observation 3: The current IDC TDM solution utilizing DRX mechanism stops uplink and downlink transmissions during the sleep/inactive time, meaning unaffected downlink transmissions cannot be performed, which will result  in low NR-NTN scheduling efficiency.

Proposal 3: Use the enhanced TDM IDC solution only for uplink transmissions for NTN while the DL transmissions can continue without any restrictions even during the inactive/sleep period.

Proposal 4: UL feedback (ACK/NACK) from the UE could be blocked by disabling HARQ when using enhanced TDM IDC solution for uplink.

# 4	Reference

- R4-2419898, LS on simultaneous operation between GNSS and NR NTN, TSG-RAN WG4 Meeting # 113.