3GPP TSG-RAN WG2 Meeting #129	R2-2500704

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	8.9.2

Source:	Huawei, HiSilicon

Title:	Further consideration on Store and Forward

Document for: Discussion and Decision

# 1	Introduction

In the last meeting, the following agreements were made regarding the Store & Forward (S&F) satellite operation [1]:

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

In this contribution, we provide some further considerations on the Store and Forward satellite operation.

# 2	Discussion

## 2.1 Remaining time of current operating mode

### 2.1.1 Switching from normal mode to S&F mode

In the last meeting, it was agreed that the remaining time for the satellite mode switch from S&F to normal would be broadcasted, while the switch from normal mode to S&F mode remains an FFS.

If the serving cell switches from normal mode to S&F mode, it indicates that the feeder link will become unavailable at certain point in time.

In the legacy mechanism, the t-service in SIB19 indicates when the current serving cell will stop providing service in the area it is currently covering. If the cell supports both normal mode and S&F mode, it may become unavailable to legacy UEs when switching to S&F mode. However, at this point, the t-service time has not yet been reached, creating uncertainty how NW and legacy UEs should behave in this situation.

Even if the NW releases the legacy UEs to RRC_IDLE, legacy UEs may not perform neighbouring cell measurement before t-Service time. One possible network implementation is to set the t-Service to the switching time from the normal mode to S&F mode. In this case, t-service serves as the indication of the remaining time of current operating mode. For the S&F UEs, a new t-service time (e.g., t-serviceSF) would be required to indicate the actual stop time for the cell’s service.

In that case, the UEs that do not support S&F will begin measurements and transition to normal cells by the time of the mode switch (i.e., t-service).  Meanwhile UEs that support S&F will continue to be served by the satellite until the new t-service time (e.g., t-serviceSF) is reached.

Proposal 1: The legacy t-Service is reused as the transition timing from normal mode to “S&F operation” mode while a new t-ServiceSF is introduced to indicate the stop-serving time of the current cell.

### 2.1.2 Switching from S&F mode to normal mode

Regarding mode switching from S&F to normal, the following were noted as FFS:

1) FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode;

2) FFS which SIB is used (SIB1 or SIB31);

3) FFS if absolute or relative time is used.

For the first bullet, we believe the UE behaviour in spec should be clear as per the agreement. Specifically, UEs supporting S&F may choose not to initiate certain NAS procedures (e.g., ATTACH, TAU) immediately, but instead wait until the feeder link becomes available and then initiate the procedure. For the UEs that do not supporting S&F mode, they should wait until the cell switches to normal mode before camping on the cell.

Proposal 2: UEs supporting S&F may delay certain NAS procedures until the feeder link becomes available, while UEs not supporting S&F should wait until the feeder link is available before camping on the cell.

For the second bullet, we don’t see the necessity to include the timing indication in SIB1. SIB31, which provides satellite assistance information for the serving cell, would be more suitable for this indication.

For the third bullet, similar to t-Service, using absolute time is straightforward and simple. If remaining time is used, UE would need to periodically check the remaining time in SIB or maintain a timer to determine the exact switching time is, which would add  complexity for both UE and network.

Proposal 3: A new t-ServiceSF is introduced as the transition timing from “S&F operation” mode to normal mode, while the legacy t-Service remains as the stop-serving time of the current cell.

In summary, for the transition between S&F mode and normal mode, the legacy t-Service refers to the stop of normal mode and the new t-ServiceSF refers to the stop of S&F mode

Proposal 4: For the transition between S&F mode and normal mode, the legacy t-Service refers to the stop of normal mode and the new t-ServiceSF refers to the stop of S&F mode.

## 2.2	Paging in S&F satellite operation

In the legacy paging procedure, the arrival of MT data triggers the network to page UE. In the S&F scenario, when feeder link is available, the CN sends MT data to satellite, which receives and stores it. When service link is available, the satellite pages UE or the UE initiates RRC connection, at which point the network sends MT data to UE. However, if the satellite does not receive any MT data when feeder link is available, there is no need for the satellite to page UE when service link is available. In this case, the UE doesn’t need to monitor paging or initiate RRC connection.

Therefore, if UE is informed when the NW does not have any MT data buffered for an area, the UE does not need to monitor the paging or initiate RRC connection, which helps save UE power.

Proposal 5: If the NW doesn’t have any MT data buffered for an area, it informs the UE so that the UE doesn’t need to monitor paging or initiate RRC connection.

## 2.3	Access control

For both S&F architectures, the UE’s DL NAS data can be stored at the MME on board the satellite serving the UE at next pass. In this case, if the UE is unable to access the intended satellite, the stored data will be lost. Given that the current access control mechanism may lead to rejection of the UE’s RRC connection request due to the load status, it is beneficial for the eNB to prioritize the access of the UE, if it is subsequently re-attempting the access to the NW, to avoid potential data loss. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure

Proposal 6: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

# 3	Conclusion

This contribution discusses the store and forward operation for IoT NTN and we have the following proposals:

Remaining time of current operating mode

Proposal 1: The legacy t-Service is reused as the transition timing from normal mode to “S&F operation” mode while a new t-ServiceSF is introduced to indicate the stop-serving time of the current cell.

Proposal 2: UEs supporting S&F may delay certain NAS procedures until the feeder link becomes available, while UEs not supporting S&F should wait until the feeder link is available before camping on the cell.

Proposal 3: A new t-ServiceSF is introduced as the transition timing from “S&F operation” mode to normal mode, while the legacy t-Service remains as the stop-serving time of the current cell.

Proposal 4: For the transition between S&F mode and normal mode, the legacy t-Service refers to the stop of normal mode and the new t-ServiceSF refers to the stop of S&F mode.

Paging in S&F satellite operation

Proposal 5: If the NW doesn’t have any MT data buffered for an area, it informs the UE so that the UE doesn’t need to monitor paging or initiate RRC connection.

Access control

Proposal 6: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

# 4	References

- R2-2410913, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 128.