3GPP TSG-RAN WG2 Meeting #129	R2-2500706

Athens, GR, 17th – 21st Feb, 2025

Agenda Item:	8.10.2.1

Source: 	China Unicom

Title:  	Discussion on MRO enhancement for LTM

Document for:	Agreement

Introduction

In the prior meeting, RAN2#127bis and RAN2#128 reached consensus on several enhancements for MRO related to LTM as referenced [1] [2].

- RAN2#127bis agreements - Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). - RAN2#128 agreements - No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues.

This document offers further analysis of potential impacts of MRO enhancement for LTM.

Discussion

- 2.1	MRO enhancement for LTM

Implicit RA-type indicator

According to the agreements reached in the RAN2 meeting regarding Rel-19 SON/MDT, it remains unclear how to implicitly indicate the RA type of an LTM cell switch. Companies have proposed several candidate indicators for the RA type, such as RA information + lastHO-type, and information on whether and how the UE obtained the TA. According to the current TS 38.331 specification, the UE will set RA-InformationCommon when a RA problem occurs, or when beam failure recovery fails, or when the connectionFailureType is handover failure (hof). Even if the network knows that an LTM cell switch has occurred based on the lastHO-type, if the RA information may be logged in the RLF report due to other failure cases, the network cannot distinguish between a RACH- less failure and a RACH-based failure.

According to the agreements of the RAN2#127b meeting, the UE logs and reports whether and how it obtained the TA value used for a failed LTM switch (indicated by the gNB or determined by the UE). LTM failure cases can then be classified as follows:

- If the UE reports, via RLF reports, that it acquired the TA indicated by the network and/or determined by the UE, then the network will know that the UE obtained the TA, and a RACH-less LTM cell switch failure has occurred.

- If the UE does not report any indication of how it obtained the TA value, then the network will know that the UE did not obtain a valid TA, and a RACH-based LTM cell switch failure has occurred.

Additionally, with the early TA acquisition indications (either gNB-indicated or UE-determined), the network can determine whether the gNB-indicated configuration or the UE-determined configuration is effective. Thus, the network can deactivate the ineffective configuration that does not contribute to early TA acquisition, which is clearly beneficial for reducing the signalling overhead of both the network and the UE. Therefore, with a two-bit indication, the RA type of the LTM cell switch can be implicitly indicated.

Proposal 1: Whether and how the UE obtained the TA value used for a failed LTM switch can implicitly indicate the RA type of the LTM cell switch.

For UE-determined early TA acquisition, some companies have proposed that the TA of the target cell measured by the UE may change from valid to invalid after receiving the LTM switch command. This may cause a RACH-less LTM cell switch failure when PDCCH order-based early TA acquisition is not configured for this UE. In such a case, the network may want to know the TA value measured by the UE and compare it with the TA calculated by the network to investigate whether the TA value is invalid. In the case where both UE-determined and network-configured early TA acquisition are implemented, the current specification stipulates that the network can ensure the validity of the TA based on its implementation, regardless of whether the TA measured by the UE is valid or not. Therefore, TA value logging is not required in this case.

Proposal 2: In the case where only UE-based early TA acquisition is configured and PDCCH order-based early TA acquisition is not, the TA value of the target cell should be logged in the RLF report.

Sub-optimal LTM cell switch

The network's aim in configuring UE-based TA acquisition or transmitting PDCCH order-based TA via MAC CE is to reduce the LTM cell switch time and service interruption. If, after sending the early TA acquisition configuration(s) to the UE, the network discovers that a RACH-based LTM, rather than a RACH-less LTM, is triggered because the UE did not obtain a valid TA, this indicates a sub-optimal LTM cell switch procedure, resulting in the waste of radio and signalling resources.

Regarding sub-optimal LTM cell switching, RAN2 has agreed to include RACH-less and RACH-based indicators in the SHR. However, there are still numerous cases that need to be clarified for optimization. For example, the UE may not obtain a valid TA even if it is configured to acquire it. Therefore, it is necessary to log the detailed reasons for whether and how the UE acquires a valid TA in the SHR. The detailed analysis includes the following successful LTM cell switch scenarios:

- Scenario 1: The RACH-less indicator is included in the SHR, and the UE successfully acquires a valid TA, either through gNB-indicated, UE-determined, or both methods.

- Scenario 2: The RACH-based indicator is included in the SHR, and the UE fails to acquire a valid TA with either gNB-indicated, UE-determined, or both configured methods.

- Scenario 3: The RACH-based indicator is included in the SHR, and the UE is not configured with any early TA acquisition configurations.

- Scenario 4: The RACH-less indicator is included in the SHR. The UE acquires two TAs, both through gNB-indicated and UE-determined methods, but the UE-determined TA is invalid.

In both Scenario 2 and Scenario 3, the target gNB will be indicated as having a RACH-based indicator. The gNB cannot determine which scenario occurred and led to the RACH-based LTM-cell switch. For example, if Scenario 2 occurred, the gNB will never know that the UE-based TA acquisition or PDCCH-order-based TA acquisition configurations were not executed. It simply assumes that it has configured the correct configurations and obtained the corresponding results, which leads to the waste of signalling resources. Therefore, the gNB should identify this sub-optimal scenario, and the UE needs to log the TA information in the SHR. Thus, it is necessary to determine whether and how the UE obtained the TA value used for a successful LTM switch, and this information should be logged into the SHR.

Proposal 3: UE should log whether and how the UE got the TA value used for a successful LTM switch (gNB indicated or UE determined) in SHR.

For Scenario 4, additional UE-determined configurations are wasted because they do not provide a valid TA after the UE receives the LTM cell switch command. The UE can compare whether the UE-determined and NW-configured TA values are the same and then log this information into the SHR report to highlight this sub-optimal scenario. The network can then deactivate the UE-determined early TA acquisition configuration since it is not functioning effectively

Proposal 4: Log a flag indicating whether the UE-determined and NW-configured TA values are the same.

Conclusion

RAN2 is kindly asked to discuss the following proposals:

Proposal 1: Whether and how the UE obtained the TA value used for a failed LTM switch can implicitly indicate the RA type of the LTM cell switch.

Proposal 2: In the case where only UE-based early TA acquisition is configured and PDCCH order-based early TA acquisition is not, the TA value of the target cell should be logged in the RLF report.

Proposal 3: UE should log whether and how the UE got the TA value used for a successful LTM switch (gNB indicated or UE determined) in SHR.

Proposal 4: Log a flag indicating whether the UE-determined and NW-configured TA values are the same.

References

- R2_127b_Maint_SONMDT_Mattias_20241017_1300

- R2_128_Maint_SONMDT_Mattias_20241121_final