3GPP TSG-RAN WG2 #129	R2-2500709

Athens, Greece, 17-21 February

Agenda Item: 7.0.2.13

Source:	Ericsson

Title:	Remaining details on TDD UL/DL Configuration for Two TA

Document for:	Discussion, Decision

# Addition of a parameter for TDD-UL-DL configuration.

RAN2 received LS in [1] which is asking for an RRC parameter for TDD-UL-DL-ConfigCommon configuration for each cell for inter-cell mTRP operation.

For Rel-18 Two TA operation, RAN1 has identified that the RRC parameter TDD-UL-DL-ConfigCommon for the neighbour cell is missing. RAN1 made the following agreement.

Agreement Send an LS to RAN2 to ask RAN2 to add missing RRC parameter TDD-UL-DL-ConfigCommon for each cell for inter-cell mTRP operation.

# 2	Actions

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to add the RRC parameter TDD-UL-DL-ConfigCommon for each cell with additionalPCI configured for two TA operation.

In RAN2#128, RAN2 discussed the addition of TDD UL/DL configuration for the additionalPCIs. RAN2 agreed there is no need to clarify the related capability field multiDCI-InterCellMultiTRP-TwoTA-r18 to also support the TDD UL/DL configuration.

The discussion on how to implement the parameter addition in TS 38.331 did not conclude. There was no consensus whether the TDD UL/DL configuration is to be added to each additional PCI or not. If the configuration is to be added to each additionalPCI, then each cell should be a TDD cell. If these are not added to each additional PCI, the views varied between assuming some of the cells being FDD cells or still assuming all of the cells TDD cells. Further, for those TDD cells that did not get own TDD config, one company assumed the serving cell TDD UL/DL configuration for these additionaPCIs. Our view, and therefore the proposal, is to add the TDD configuration to each additionalPCIs assuming also PRACH configuration is added to these additional PCIs. We assume the serving cell and the additionalPCIs are all TDD cells but it is also ok to have condition on this.

- Agree to add configuration for TDD-UL-DL-ConfigCommon for each additionalPCI.

The email discussion [POST128][008][MIMOevo] RRC CR on TDD UL/DL Configuration for Two TA (Ericsson) concluded that the parameter is added in IE ServingCellConfig as a list for TDD-UL-DL-ConfigCommon configuration for the additional PCIs. There was consensus to do this by extending the IE MIMOParam.

- Agree to add the configuration for TDD-UL-DL-ConfigCommon in IE ServingCellConfig by extending the IE MIMOParam.

Our proposal was to have a new to a AddModList additionalTDDConfig-perPCI-ToAddModList-r18. While there was also some other proposals this would still be our view. In below, there is an attempt to accommodate some of the comments received during the email discussion:

MIMOParam-v18xy ::= SEQUENCE {

additionalTDDConfig-perPCI-ToAddModList-r18   SEQUENCE (SIZE (1.. maxNrofAdditionalTDDConfigs-r18)) OF  AdditionalTDDConfig-perPCI-ToAddMod-r18

OPTIONAL, -- Cond 2TA-Only

additionalTDDConfig-perPCI-ToReleaseList-r18  SEQUENCE (SIZE (1.. maxNrofAdditionalTDDConfigs-r18)) OF AdditionalPCIIndex-r17                                  OPTIONAL  -- Need N

}

AdditionalTDDConfig-perPCI-ToAddMod-r18 ::=          SEQUENCE {

additionalTDDConfig-Index-r18  AdditionalPCIIndex-r17,

tdd-UL-DL-ConfigurationCommon-r18       TDD-UL-DL-ConfigCommon                                              OPTIONAL -- Cond TDD

}

- Agree to use a new AddModList additionalTDDConfig-perPCI-ToAddModList-r18 for the configuration for TDD-UL-DL-ConfigCommon.

The details how to implement the field descriptions and cond descriptions would depend on the outcome of the discussion related to Proposal1. We have a suggestion as baseline in[3]. As in above, the CR implements the 2TA-Only condition inside the mimoParam-r18 but it could as well be the condition to configuring mimoParam. The CR, as above also implements the TDD config for each additional PCI in the element AdditionalTDDConfig-perPCI-ToAddMod-r18. If RAN2 concludes that the configuration is given to all additionalPCIs, then there could only be one condition covering both 2TA-Only and TDD.

- Agree CR in R2-2500710 as baseline for the discussion.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Agree to add configuration for TDD-UL-DL-ConfigCommon for each additionalPCI.

Proposal 2	Agree to add the configuration for TDD-UL-DL-ConfigCommon in IE ServingCellConfig by extending the IE MIMOParam.

Proposal 3	Agree to use a new AddModList additionalTDDConfig-perPCI-ToAddModList-r18 for the configuration for TDD-UL-DL-ConfigCommon.

Proposal 4	Agree CR in R2-2500710 as baseline for the discussion.

# References

R1-2409179, LS on TDD UL/DL Configuration for Two TA, RAN1(Ericsson).

R1-2407447, LS on missing RRC parameters for early UL sync PRACH transmission for LTM

R2-2500710, Correction on TDD UL/DL Configuration for Two TA, Ericsson, RAN2#129, February 2025, Athens