3GPP TSG-RAN WG2 Meeting #129	R2-2500714

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.2.3

Source:	Nokia

Title:	AIoT Random Access Aspects

WID/SID:	Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution focus on details of contention-based and contention-free access, including re-access for failure handling, msg content/format, discussion on use of unified solution (i.e. solution 3)

# 2	Discussion

As mentioned in our contribution on AIoT paging R2-2500716, we see three containers of data being a part of the triggering procedures for any AIoT data communication. This discussion paper relates to the resource and access occasion control containers, which contains fields for access related configuration.

Control header (session identifier, container type, …)

Paging control container

Resource control container

Access occasion control container

## 2.1	AIoT access round determination

During the SID, two different views on the mode of operation between reader and devices being messages bundled by either devices, or by communication step. Note that the figures below only indicate a single frequency resource but multiple may be present based on RAN1 agreement. This does not affect the device operation concept though as it will just be devices operating in parallel. Figure 1 illustrates a scenario where a device selects or is assigned a resource for msg1 (access occasion) in time and frequency and subsequent messages follows in a request-response manner until the reader and device has exchanged the information needed for the paging ‘session’. Figure 2 illustrates a different way of resource assignment, where the devices and reader resolves the contention based access, or part of it, before further transmitting data. In Figure 2, it is illustrated by the reader and device exchanging msg1 and msg2, but this could also be msg3 and msg4 (if required).

Figure 1 device bundled procedure

Figure 2 Stepwise bundled procedure

In the end, since the device will always follow the reader commands, which procedure is available could be up to the reader, as it mainly depends on the number of resources assigned for and in between msg1. However, it also depends on whether the resources are preconfigured within the device, or can be dynamically signalled by the reader. With suitable resource configuration, it is even possible to handle an “inventory followed by command” procedure i.e. a procedure where the reader, hence device, is not aware of whether there will be a command until later in the procedure.

Observation 1: Since the device follows the reader instructions, both procedure in Figures 1 and 2 are practically usable.

Observation 2: The case “inventory followed by command” is a combination of the procedures in Figure 1 and Figure 2.

The advantage of the device bundle is, that the device can be (pre)configured to determine end of any communication as soon as the next set of resources for msg1 starts. This allows the device to potentially only be active within the timeframe from its own msg1 till the next msg1 resource and there is no explicit indication needed from the gNB to go to sleep or similar. On the drawback side, there may be wasted resources in between the devices access if a certain amount of resources are allocated only. Similarly, there may be less flexibility for multiple retransmissions.

The advantages of the stepwise bundled procedure is, that this is extremely flexible seen from the reader side. The device will continue to monitor for messages until it runs out of energy, or a new paging session begins. This means that the reader can request retransmissions, allow for later commands, and much more. However, it also may require some enhancements such as a go-to-sleep/end of transmission signal, or a timer based solution to allow the device to sleep.

Observation 3: The procedure in Figure 1 consumes the least power as a device would go to a sleep mode as soon as it notices a new paging/access occasion trigger.

In any case, since the solutions are interchangeable, one could argue that for a inventory only set of devices, the reader could select a power efficient device bundled procedure, and otherwise the stepwise. We believe that either of the designs also follow RAN1 agreements such as the one below from RAN1#118, which mentions that a set of resources can be assigned for msg1.

Agreement RAN1 studies following: - A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s).

The decision between procedures could also be tied up to the service type indication, allowing for the device to know whether it should assume a new access occasion resource to be end of communication, or other measures. However, since this will be up to the reader/deployment configuration, we don’t believe there is a need to over-optimize for the power consumption in the stepwise bundling, as we assume devices supporting command, and other advanced procedures may have more energy available.

Observation 4: In case the reader chooses the procedure in Figure 2, is should be aware of its impact on the device battery life.

Proposal 1: The paging could indicate whether a command may come after the inventory i.e. whether the device is expected to go to sleep after the next access round trigger.

Proposal 2: RAN2 should mainly focus on simple go-to-sleep signalling/procedures and not overengineer requirements for ACK/NACKS etc.

## 2.2	AIoT signalling procedure for AIoT access control

### 2.2.1	Paging session identification

We do believe that for either of the two messages described here, RAN2 should agree on a header which refers to the current paging (session) i.e. a paging identifier and potentially a reader identifier.

Proposal 3: Reader messages should all carry a header enabling the device to identify the given paging session i.e. paging ID, reader ID, etc.

### 2.2.2	Round information message

The response phase comprises several access occasions to allow access for multiple devices. When/if the reader is not aware of the exact IDs and/or number of IDs of the devices in its proximity, and try to collect ‘blind’ information/inventory some flexibility is needed as to the configured number of access occasions. Likewise, since the reader is not aware of whether all devices can reply without failure in a single response phase, we believe that multiple CBRA phases may be beneficial, also known as access rounds.

Observation 5: Multiple access rounds are beneficial to both blind detection of devices and re-access.

The round information message is the one controlling the access to the round i.e. the specific resource configuration. It should be sent once per set of resources available for devices to transmit msg1 in. This allows for the reader to configure the access related parameters for each round within a session, and to have multiple rounds with a larger/smaller set of resources e.g. Q factor.

Proposal 4: The round information message is the one controlling the access to the round i.e. the specific resource configuration.

#### 2.2.2.1	Time/frequency resources

We believe that the access resources should be allowed to be transmitted separately of the part of the paging message carrying the IDs and other general configuration. This may be viewed as what RAN2 discuss as “subsequent pagings” which can in turn be of smaller size than the paging since it does not need to carry the IDs.

The access resources can be either a simple number like a Q-factor, or a specific set of/mask of resources to be used at least for msg1. Based on this information, the device can then identify and select the correct time/frequency resource to transmit msg1 in.

Proposal 5: Time/frequency resources can be indicated in a message not being the paging message i.e. to control the access of the current round.

#### 2.2.2.2	Resource types

In some cases, the reader may choose to allow a certain amount of the assigned resources to be used for reaccess, but it may also choose not to. Thus, the resource configuration could also contain information on whether devices are allowed to use said resources for reaccess or not.

### 2.2.3 	Access occasion message

## In case security is enabled, a security context has to been established such data can be protected. As agreed in SA3 KI#4 and KI#5, the system shall provide the means to authenticate, either one way or mutual, confidentiality, integrity and replay protect and protect the device ID, this doesn’t imply that all security primitives shall be enabled, but the design shall provide the means to do it when required by the deployment.

## A security procedure discussed in SA3 to support all KI’s includes the transmission of a derived Nonce to the device from the core related entity i.e. reader in case of AIoT. The device responds to the Nonce with its own generated Nonce which enables authentication. In the subsequent communications for the same device, if the device is already authenticated, the authentication process can be omitted if storage is enabled.

Figure 3 Simple security procedure

Observation 6: Authentication needs at least two steps to complete before upper layer data should be transferred.

Device specific parts of a message comes with the security and NAS header as defined by SA3 and SA2 respectively. These are to be seen as unicast messages, and thus should not be collated with messages controlling the access for multiple devices. Therefore, we see the potential need for a message preceding any msg1 at least in case security is needed.

Proposal 6: A unicast message can be designed to allow for at least security and NAS header.

#### 2.2.3.1	Access occasion identification

Assuming the devices are minimalistic, they may not want to keep count of the number of access occasions i.e. resources for msg1’s that has passed. It may also be that a single one of these are not identified correctly due to clock drift, or bad channel quality. Hence, we propose that an access occasion message should carry the access occasion number to help the device select the correct resource for a msg1.

Proposal 7: Based on RAN2 CBRA procedure, the access occasion trigger should carry also the access occasion number

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Since the device follows the reader instructions, both procedure in Figures 1 and 2 are practically usable.

Observation 2: The case “inventory followed by command” is a combination of the procedures in Figure 1 and Figure 2.

Observation 3: The procedure in Figure 1 consumes the least power as a device would go to a sleep mode as soon as it notices a new paging/access occasion trigger.

Observation 4: In case the reader chooses the procedure in Figure 2, is should be aware of its impact on the device battery life.

Proposal 1: The paging could indicate whether a command may come after the inventory i.e. whether the device is expected to go to sleep after the next access round trigger.

Proposal 2: RAN2 should mainly focus on simple go-to-sleep signalling/procedures and not overengineer requirements for ACK/NACKS etc.

Proposal 3: Reader messages should all carry a header enabling the device to identify the given paging session i.e. paging ID, reader ID, etc.

Observation 5: Multiple access rounds are beneficial to both blind detection of devices and re-access.

Proposal 4: The round information message is the one controlling the access to the round i.e. the specific resource configuration.

Proposal 5: Time/frequency resources can be indicated in a message not being the paging message i.e. to control the access of the current round.

Observation 6: Authentication needs at least two steps to complete before upper layer data should be transferred.

Proposal 6: A unicast message can be designed to allow for at least security and NAS header.

Proposal 7: Based on RAN2 CBRA procedure, the access occasion trigger should carry also the access occasion number