3GPP TSG-RAN WG2 Meeting #129	R2-2500716

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.2.2

Source:	Nokia

Title:	Paging design for Ambient IoT

WID/SID:	Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#106 the WID for AIoT was agreed, with the following scope for RAN2.

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

This Tdoc discuss a baseline paging structure for the AIoT MAC.

# 2	Discussion

## 2.1	Paging message design

The paging message should be robust and easy to decode, such the device can quickly determine whether it should continue to be in an on-state or not. Although the WID does not preclude segmentation in R2D, at least for the paging message this should be true.

Observation 1: Segmentation of paging messages causes overhead and additional power usage.

Proposal 1: Paging messages shall not support segmentation.

In the study phase, RAN1 provided the upper limit of the TBS to be 1000 bits for all device types. We believe it is safe to assume that the different types of devices are selected to be future proof, and the size of the paging should consider 1000 as a maximum value.

Observation 2: RAN1’s reply on TBS was for all device types, thus the 1000 bits is a hard limit for the paging size, even for future releases.

During the study, it was discussed whether to deliver AIoT information by using a set of invariants but mutually different paging messages, or whether to allow dynamic content with a universal paging message, carrying generally only a subset of all possible information.

Our proposal is to construct control message dynamically from one or more containers with static content. Three types of container are considered :

Control header (session identifier, container type, …)

Paging control container

Resource control container

Access occasion control container

- Control header

  - Identification of the session i.e. paging ID, or reader IDpaging

- Paging control container

  - identify session and trigger specific device(s) for communication during the response phase

- resource control container

  - controlling/setting the number of resources available for the round to the device, etc

- access occasion control container

  - controlling the current access i.e. carrying one or more D2R resources for the device i.e. for segmentation, indicate the access occasion number to avoid the device to keep track of elapsed occasions, and other

Figure 1 Signalling structure for R2D communication

Multiple messages are illustrated in Figure 1, where the two control sub-messages are delivered within the encapsulating paging message in order to avoid multiple stand-alone initial messages. In case the radio conditions are adverse, or the payload is large, either sub-message could be transmitted sequentially in time, instead of in the same time instance, bypassing the segmentation issue at the cost of latency for the initiation of the procedure. Indicating the content within a header would easily allow the device to determine which message(s) is received.

Observation 3: The segmentation can be omitted for bad radio conditions by defining three types of messages which can essentially also be combined.

Proposal 2: An R2D control message consists of one or more optional containers with pre-defined (static) content, each container specifying one category of control information related (eg, paging IDs, resource grants, access config).

### 2.1.1	Security related aspects

As mentioned in R2-2500713 security related parameters could be carried within an access occasion indicator, since this is anyway not intended for broadcasted data. Hereby, we can circumvent to consider the payload size in the paging, in case the messages are split. However, in case the reader only wants to target a single device, it may be best to recall the required size which would be of around 256 bits.

Observation 4: Security related information may take up to 256 bits of information, but can be omitted from the paging if carried in another device specific message.

## 2.2	Paging control container

As mentioned above, a paging can be seen as a compilation of multiple messages. This section relates to the paging control container initial information needed for initial access.

During the study, the term AIoT paging was adopted to identify the initial message which would trigger the AIoT devices targeted by the ID(s) within to reply to the reader. During the study, it was identified that subsequent pagings with an unique identifier is also be needed to trigger additional procedures for the same response phase. Contrary to NR and other 3GPP technologies, the paging carries also more data to control the access of the AIoT interface, since the AIoT device is considered connectionless. Thus, the term paging may be misleading and “trigger message” may be more suitable for the actual specification, as the message triggers certain operation(s).

Proposal 3: RAN2 to call the first AIoT paging of a device response phase “initial trigger message”, and the subsequent messages as “trigger message”.

The AIoT work will be defining the paging message contents and purpose, in order to determine what to trigger by the initial paging, and whether other messages are needed to control the operation of the devices.

According to the RAN2 agreements in the study, the very initial message should at least contain;

- ID of the device(s),

  - According to the WID, this may be addressing a single device, or a broadcasted

- Paging identifier,

  - to allow the device to determine whether the current message is related to the paging containing a specific configuration and addressed to the device using an ID

- Number of resources

  - at least for CBRA, the number of resources must be known in order for the device to select a random resource

Other types of message contents are discussed below.

### 2.2.1	Device IDs

As of the WID, AIoT devices can be addressed using a single ID or a broadcast manner (all devices). Furthermore, it is mentioned that RAN2 is encouraged to design the device ID field(s) such that multiple devices can be addressed.

Addressing a single device is easy, as it needs a single ID field to work. Similarly, communicating to a masked ID set may also be trivial. However, for a list of devices it may be more difficult due to the limited size of the data. Assuming the static information takes the first 100 bits, there will only be room for 9 device IDs of 96 bits in case of perfect radio conditions.

Based on SA2 design seen in Figure 2, the ID may be separated into two parts being; a static network identifying payload, and a payload dynamic in both size and content. Since at least the static payload is to be considered identical for the devices, communicating to a set of devices can be compressed at least by separating part 1 and part 2 and then transmitting only part 1 a single time, and part 2 for each of the devices. Assuming the part 1 is at least half the ID size, the compression can allow for almost the double amount of device to be addressed.

Figure 2 SA2 design of the device ID

Observation 5: WID states that the work should have forward compatibility in mind w.r.t communicating to a fixed group i.e. list of devices when designing the AIoT paging message.

Proposal 4: RAN2 considers compression of device IDs involving at least common data like network/operator identification. FFS other portions and further details.

### 2.2.2	On service type indication

RAN2 discussed during the study whether a service type indication is needed within the paging message being 1) inventory, inventory & command, or command only. The intention of such indication would be to allow the device to shut down and save energy. As of S2-2501249, command only was excluded by SA2, thus raising the question whether there is need for service type indication.

Observation 6: The reader may issue a command only after the inventory step.

Service type indication will be needed primarily to indicate whether the device is to expect a command after the inventory, essentially allowing the device to stop listening until a new paging is detected. However, there may be a case where the reader/AIoT-F does not know whether the intended devices are addressed and thus whether any command is to be issued. Thus, a device should at least be able to differentiate between inventory only and inventory and command.

Proposal 5: The reader differentiates service type "inventory only" and "inventory and command" to indicate to the device that a command may be delivered to it.

In case the device is indicated to expect a command but the command is only provided a longer timeframe after the initial access procedure, it may be that the AIoT-F will not issue said command, or the command has been lost. Hence, it may be reasonable to allow the device to go to sleep after a (pre)configured amount of time.

Proposal 6: A device expects a command expected to be delivered within a predefined time after initial paging and/or inventory response. FFS timer details.

Furthermore, there is the question of whether the device is expected to reply to a given command. In some cases, like updating important information, it may be important for the reader to know whether the device has received the command, whereas in others it may not. One case where it is indifferent would be a read command, since the reply may be the payload requested. In yet another case, if the command takes longer to prepare it may be important for the reader to know that the command is received and that additional resources should be provided later.

Observation 7: Device benefits from knowing whether the reader expects a command response.

Proposal 7: The reader expects a command response from a device within the validity of specified response resources and subject to max response timer. FFS timer details.

Proposal 8: RAN2 to decide whether the reader indicates to the device the expectation of a command response explicitly (eg via a response request flag), or simply permits an on-demand response within the validity of response resources and/or max timer.

Proposal 9: RAN2 to discuss how to correctly associate the random access and command in a given device, including the repetition of information from random access (eg, device ID, paging ID, 16 bit RA ID) for the command.

### 2.2.3	On access type indication

The WID defines that for contention based random access only type 1 is supported, unless the hybrid one is enabled through signaling. This leaves only the differentiation between CBRA and CFA to be decided, and here we believe that it can be inferred by the ID type.

Proposal 10: To simplify signaling, RAN2 to agrees that

- an AIoT paging of a single-device implies “Contention-free access” i.e. the gNB assigns the resources for the single device,

- an AIoT paging of a device group or all devices implies “Contention-based random access”, since this may be an unknown set of devices, and

- an AIoT paging of multiple device IDs (if supported) implies “Contention-free access”

For contention free access, we propose to assume the UE may use the 3-steps as baseline i.e.

- paging, by the reader

- random ID transmission, by the device

- echoing of random ID, by the reader

- full ID transmission, full ID by the device

This is to ensure that security procedure is still in place even for contention free access. In case less steps are needed, RAN2 should look at signaling optimization i.e. the unified solution. One may argue that the random ID is wasteful transmission since it is not used for contention resolution, but it could later be used for identifying the device for a command, or similar data. Hence, for the sake of forward compatibility, and simple device implementation, we believe this is good enough.

### 2.2.4	Reader identification

During the study, it was discussed whether there is a need for reader identification i.e. in case multiple readers may issue parallel requests. Arguments has been made that parallel requests is not how the AIoT feature it will be implemented i.e. only a single reader will be providing a request at a time. We see that adding a field for the case where additional readers are operating in parallel is not processing heavy nor is it requiring a lot of bits. This could be useful for many cases i.e. when a warehouse is using mobile devices, or has redundancy coverage to ensure line of sight in even the complex indoor scenarios.

Proposal 11: A reader identifier is included in the paging/message header.

### 2.2.5	Optimized signaling procedure

The WID defines that it is possible to use the unified solution (solution 3) for the random access procedure. This would be an advantage for not only CBRA but also CFA in terms of resource usage. Our understanding of the unified procedure is, that it can be achieved by a single bit indicating whether the UE is allowed to transmit the full ID together with the random ID and skip the third step i.e. echoing of the random ID by the reader.

Observation 8: To align all procedures CFRA, the random ID transmission in msg1 shall be mandatory irrespective of subsequent procedures.

RAN2 can discuss endless various optimization, but 3-step will always be the most reliable solution compared to less steps. Thus, we believe that as a baseline msg2 and msg3 should not be included in the procedure in case the device is allowed to transmit the full ID within msg1, but can be seen as a further enhancement, since msg2 will carry no instructions.

Observation 9: In case the reader wants reliability and/or to enable security, it will use 3-step CBRA.

Proposal 12: A reader may request the device to transmit its full ID together with the random ID in msg1 in which case the device will not expect, nor reply to msg2 from the reader. FFS whether an extra bit can be added to indicate acknowledgement of through msg2.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Segmentation of paging messages causes overhead and additional power usage.

Proposal 1: Paging messages shall not support segmentation.

Observation 2: RAN1’s reply on TBS was for all device types, thus the 1000 bits is a hard limit for the paging size, even for future releases.

Observation 3: The segmentation can be omitted for bad radio conditions by defining three types of messages which can essentially also be combined.

Proposal 2: An R2D control message consists of one or more optional containers with pre-defined (static) content, each container specifying one category of control information related (eg, paging IDs, resource grants, access config).

Observation 4: Security related information may take up to 256 bits of information, but can be omitted from the paging if carried in another device specific message.

Proposal 3: RAN2 to call the first AIoT paging of a device response phase “initial trigger message”, and the subsequent messages as “trigger message”.

Observation 5: WID states that the work should have forward compatibility in mind w.r.t communicating to a fixed group i.e. list of devices when designing the AIoT paging message.

Proposal 4: RAN2 considers compression of device IDs involving at least common data like network/operator identification. FFS other portions and further details.

Observation 6: The reader may issue a command only after the inventory step.

Proposal 5: The reader differentiates service type "inventory only" and "inventory and command" to indicate to the device that a command may be delivered to it.

Proposal 6: A device expects a command expected to be delivered within a predefined time after initial paging and/or inventory response. FFS timer details.

Observation 7: Device benefits from knowing whether the reader expects a command response.

Proposal 7: The reader expects a command response from a device within the validity of specified response resources and subject to max response timer. FFS timer details.

Proposal 8: RAN2 to decide whether the reader indicates to the device the expectation of a command response explicitly (eg via a response request flag), or simply permits an on-demand response within the validity of response resources and/or max timer.

Proposal 9: RAN2 to discuss how to correctly associate the random access and command in a given device, including the repetition of information from random access (eg, device ID, paging ID, 16 bit RA ID) for the command.

Proposal 10: To simplify signaling, RAN2 to agrees that

- an AIoT paging of a single-device implies “Contention-free access” i.e. the gNB assigns the resources for the single device,

- an AIoT paging of a device group or all devices implies “Contention-based random access”, since this may be an unknown set of devices, and

- an AIoT paging of multiple device IDs (if supported) implies “Contention-free access”

Proposal 11: A reader identifier is included in the paging/message header.

Observation 8: To align all procedures CFRA, the random ID transmission in msg1 shall be mandatory

Observation 9: In case the reader wants reliability and/or to enable security, it will use 3-step CBRA.

Proposal 12: A reader may request the device to transmit its full ID together with the random ID in msg1 in which case the device will not expect, nor reply to msg2 from the reader. FFS whether an extra bit can be added to indicate acknowledgement of through msg2.