3GPP TSG-RAN WG2 Meeting #129	R2-2500719
Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.7.5

Source:	InterDigital

Title:	Discussion on RLC enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion

# 1. Introduction

The WID for R19 XR has the following objective for RLC enhancements [1]:

- Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

In RAN2 #128 meeting, we made several agreements for RLC enhancement [2], which are provided the appendix. In this contribution we further discuss the RLC enhancement topics. Specifically, we further discuss enhancement to avoid unnecessary retransmission and timely RLC retransmission.

# Discussion

  - 2.1 Timely RLC retransmissions

In the last meeting, RAN2 made an agreement to support both autonomous retransmission and polling enhancement, and the network can configure either or both. In the following, we further discuss the detail procedure of each enhancement.

Autonomous retransmission:

To enhance the RLC re-transmission for an RLC SDU with small PDB, the delay gap between the initial transmission and the re-transmission of the RLC SDU can be reduced. One possibility is to allow the UE to retransmit an RLC SDU without waiting for the RLC status reporting from the network. For example, the UE can retransmit the RLC SDU if the remaining time of the SDU is smaller than a configured threshold. Such retransmission can help improve the successful reception of the RLC SDU with small delay budget. Alternatively, the UE may fail to transmit a resulting RLC PDU due to an improper scheduling of transmission parameters such as the resource size, MCS, beam, etc. Therefore, with multiple HARQ retransmissions, the UE may still fail to successfully transmit the RLC PDU. In such cases, it can be beneficial for the UE to trigger a retransmission of the RLC SDU using another set of transmission parameters. Specifically, the UE may retransmit the RLC SDU, if the number of HARQ retransmissions for the RLC PDU exceeds a threshold.

For autonomous RLC retransmission based on the remaining time of a SDU, the network can properly configure a delay threshold to minimize the capacity loss while still guarantee the QoS (e.g., reliability) of the service. Regarding retransmission based on the number of HARQ retransmissions, capacity loss is minimal as the UE may still need to retransmit the SDU as it has likely not been received successfully.

Proposal 1:	The UE retransmits a RLC SDU if the remaining time of the SDU is smaller than a configured threshold or the number of HARQ retransmissions is greater than a configured threshold.

Polling enhancement:

Regarding the polling procedure in NR, the UE is allowed to poll the RLC reception status of the gNB by setting the polling bit in the RLC header to one in the following cases:

    - The amount of data without polling is larger than a threshold (i.e., PDU_WITHOUT_POLL >= pollPDU; or BYTE_WITHOUT_POLL >= pollByte).

    - Its buffer (initial and retransmission buffer) will be empty after the current RLC PDU.

    - Its window is stalling (e.g., the gap between the current SN and the ACKed SN is greater than the window size).

In XR applications, the receiver decodes the received data per PDU set. Therefore, the reception status of each PDU set is more important than the reception status of each PDU in a PDU set. Moreover, in an XR application, the PDU sets may arrive at the UE periodically with a certain jitter. It is also expected that the amount of data per PDU set may vary from one PDU set to another. Specifically, the number of PDUs and the number of bytes per PDU set can be different among PDU sets.

If the current polling procedure is used, in which the UE polls the gNB after a configured number of PDUs or bytes, it is most likely that the UE may poll the RLC status of the gNB in the middle of a PDU set. As a result, the reception status of each PDU set may not be reported. To reduce the delay in determining the reception status of a PDU set, the UE can be configured with a smaller values of pollByte and/or pollPDU. However, this configuration may result in high signaling overhead as the UE may need to receive many more RLC status reports when the values of pollByte and/or pollPDU is small. Therefore, it may be beneficial to enhance the polling procedure, which may help the UE to receive the RLC status of one or more PDU sets. For example, the polling bit can be set based on the transmission of a PDU set (e.g., the last PDU of a PDU set) or it can be set periodically based on the PDU set traffic pattern.

Observation 1: 	Polling the gNB after a configured number of PDUs or bytes does not inform the UE of the RLC reception status of each PDU set.

Proposal 2:	The UE enables the polling in the last transmission of a PDU set to request the status report after transmissions of each uplink PDU set.

For delay-critical RLC SDU, upon its transmission, it is beneficial to be aware of the reception status of the RLC SDU as soon as possible. It is because the Tx RLC entity may then trigger retransmission of the RLC SDU to guarantee that the RLC SDU can reach the Rx entity before the PSDB. However, in general, the Rx RLC entity transmits the status report based on the polling indication from the Tx entity. Therefore, it is beneficial to allow the Tx RLC to set the polling indication for delay-critical RLC SDU.

Proposal 3:	The Tx RLC entity includes a polling indication when the remaining time of a RLC SDU being smaller than a configured threshold.

When both autonomous retransmission and polling enhancement are configured, it is beneficial for the UE to poll the network first. In this case, the UE may receive the status of the PDU before autonomous retransmission is triggered. If polling happens after autonomous transmission, the UE may already make the autonomous retransmission before receiving the status report, which may result in waste of resource. Therefore, a sensible configuration configures the remaining time threshold for polling enhancement to be bigger than for autonomous retransmission to enable the UE to poll the network before autonomous retransmission is triggered.

Observation 2:	The remaining time threshold for polling enhancement should generally be configured bigger than for autonomous retransmission.

  - 2.2 Avoiding unnecessary retransmissions

In the last meeting, it was agreed that a new RLC timer at the Rx is introduced to determine the obsolete RLC SDUs at the Tx RLC, and the timer starts when the gap is detected at RLC layer. Upon the expiry of the newly introduced timer, an RLC status reporting should be triggered. It is also agreed that the discarded RLC SDUs are reported as ACK in the RLC status report. This is to minimize useless re-transmissions at the Tx side when the Rx has already moved its receiving window such that those RLC SDUs would get discarded.

Proposal 4:	At the Rx RLC, an RLC status report is triggered upon expiry of the timer.

# Conclusion

In this contribution, the following observations are made:

Observation 1: 	Polling the gNB after a configured number of PDUs or bytes does not inform the UE of the RLC reception status of each PDU set.

Observation 2:	The remaining time threshold for polling enhancement should generally be configured bigger than for autonomous retransmission.

Based on this, the following proposals are made:

Proposal 1:	The UE retransmits a RLC SDU if the remaining time of the SDU is smaller than a configured threshold or the number of HARQ retransmissions is greater than a configured threshold.

Proposal 2:	The UE enables the polling in the last transmission of a PDU set to request the status report after transmissions of each uplink PDU set.

Proposal 3:	The Tx RLC entity includes a polling indication when the remaining time of a RLC SDU being smaller than a configured threshold.

Proposal 4:	At the Rx RLC, an RLC status report is triggered upon expiry of the timer.

# References

- RP-241771, “Revised WID on XR (eXtended Reality) for NR Phase 3”, Nokia, 2024-09.

- RAN2#128 chair notes.

- 3GPP TS 38.322, Radio Link Control (RLC) protocol specification, v.18.0.0, 2023-12.

# Appendix: RAN2 #128 on RLC enhancement for XR

- Agreements on AL-FEC (related to LS from SA2) - There is no consensus in RAN2 that AL-FEC ratio information is useful for the gNB for both RLC AM and RLC UM. - RAN2 understanding is that in case this information would be provided to the gNB, it is up to gNB how/whether to consider it, i.e. no impact on RAN2 specifications Agreements on Timely RLC retransmissions Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement, and NW can configure either or both of them. Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced Special handling to avoid PDCP control PDU discard is not needed. A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.