3GPP TSG-RAN WG2 Meeting #129	R2-2500725

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.13.2

Source:	Nokia

Title:	Relay discovery and (re)selection

WID/SID:	NR_SL_relay_multihop - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#128, it was agreed:

Agreements:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

The following cases are supported for L2 multihop relay:

- One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE).

- Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

FFS if the last relay UE can use the same L2ID for both of the connections in either case.

Cases with two indirect paths to the gNB for the same remote UE are not supported.

An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs.

If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

This Tdoc discusses the FFS for relay discovery and (re)selection.

# 2	Discussion

## 2.1	FFS from RAN2#128

Issue 1: FFS if operation as an intermediate relay UE towards a different cell would be supported.

In RAN2#128, it was argued that a UE operating as the last relay UE in a cell might have higher Uu RSRP when the UE acts as an intermediate relay UE towards a different cell. This scenario would be beneficial when there are limited number of relay capable UEs since one relay UE can serve different paths towards different cells simultaneously. However, it might require more complicated handling of such relay UE considering different paths quality towards different cells. Therefore, to simplify the overall procedure and complete the work in stable, we propose to focus on the same cell scenario in Rel-19 multi-hop.

Proposal 1. In Rel-19 multi-hop relay, RAN2 prioritizes the same cell scenario, and the last relay UE should not operate as an intermediate relay UE.

Issue 2: FFS if there would be additional restrictions in the case of no upper bound.

The purpose of upper bound is to prevent a UE that is too close to the cell center from acting as an intermediate relay UEs. Therefore, in practice, it’s unlikely that the network wouldn’t configure the upper bound for the intermediate relay UE. Even if this bound is not configured, we fail to see a scenario or motivation that would restrict the UE from becoming an intermediate relay through other means. Therefore, we propose not to introduce any additional restriction.

Proposal 2. RAN2 does not impose any additional restrictions on intermediate relay operations, even for the case when the upper bound is not configured.

Issue 3: FFS if the last relay UE can use the same L2ID for both of the connections in either case.

RAN2 agreed to support the following scenarios:

Even when a single UE is involved in different paths, we see no practical motivation to use different L2 IDs. It would also be simpler from UE’s perspective to manage a single L2 ID, unless a specific issue is identified that requires the use of different L2 IDs.

Proposal 3. The last relay UE can use the same L2 ID for both of the connections when the last relay UE has two connections with an intermediate relay UE and a remote UE, or with two different remote UEs.

## 2.2	Relay Discovery

The Rel-17 U2N relay solution might not be optimized for MH relay scenarios, e.g., certain RRC procedures would not be function properly with multi-hop transmission, since RRC timer values may not be insufficient to cover the increased latency by multi-hop transmission. This implies that Rel-17/18 remote UE may not be capable of doing multi-hop transmission, which needs to be considered in discovery procedure.

For instance, a Rel-19 UE could still send Rel-17/18 RD messages if it is located close to the gNB, such as when the UE serves as the last relay UE. This would allow Rel-17/18 UEs to select the Rel-19 UE for single-hop transmission. On the other hand, if the Rel-19 UE is farther from the gNB, it should send a Rel-19 RD message to ensure that only the UEs capable of handling multi-hop transmission select such Rel-19 UE.

Proposal 4: RAN2 discuss if Rel-19 UE can transmit the legacy RD message, and if so, under what conditions the Rel-19 UE can send the legacy RD message.

Proposal 5: RAN2 to send an LS to SA2 to align whether/how to send the legacy RD message as in R2-2500723.

# 3	Conclusion

This document has made the following proposals:

Proposal 1. In Rel-19 multi-hop relay, RAN2 prioritizes the same cell scenario, and the last relay UE should not operate as an intermediate relay UE.

Proposal 2. RAN2 does not impose any additional restrictions on intermediate relay operations, even for the case when the upper bound is not configured.

Proposal 3. The last relay UE can use the same L2 ID for both of the connections when the last relay UE has two connections with an intermediate relay UE and a remote UE, or with two different remote UEs.

Proposal 4: RAN2 discuss if Rel-19 UE can transmit the legacy RD message, and if so, under what conditions the Rel-19 UE can send the legacy RD message.

Proposal 5: RAN2 to send an LS to SA2 to align whether/how to send the legacy RD message as in R2-2500723.