3GPP TSG-RAN WG2 Meeting #129	R2-2500727

Athens, Greece, 17th – 21st of February 2025	R2-2410213

Agenda item:	8.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on NTN downlink coverage enhancement

WID/SID:	NR_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#106 the NR_NTN_Ph3 WID was updated in RP-243300 as follows:

The objectives of the work item are the following: - Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

This paper discusses the topics of downlink coverage enhancement.

# 2	Discussion

## 2.1	SSB periodicity enhancements

In RAN plenary, the NR_NTN_Ph3 WID has been updated to include the SSB extension to up to 160 ms. This section discusses and proposes a way forward based on the issue.

### 2.1.1	MIB impact

For the SSB extension we believe it is important to note that in 38.331 the following is specified;

5.2.1 Introduction System Information (SI) is divided into the MIB and a number of SIBs and posSIBs where: - the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms (TS 38.212 [17], clause 7.1) and it includes parameters that are needed to acquire SIB1 from the cell. The first transmission of the MIB is scheduled in subframes as defined in TS 38.213 [13], clause 4.1 and repetitions are scheduled according to the period of SSB; NOTE 1: If the period of SSB is larger than 80 ms, the MIB is transmitted with the same periodicity as that of SSB. - the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

This means, that with the SSB period of up to 160 ms, there may be some impact on MIB transmission as well. A similar situation occurs with the transmission of SIB1. According to the above specification, in case of multiplexing pattern 1, SIB1 period is hard coded into 20 ms. This means that in case of SSB periodicities longer than 20 ms, the SIB1 periodicity may not follow the SSB transmissions. In our view, the specification should prevent the network from using an SSB extended periodicity (e.g., 160 ms) together with this pattern configuration. For that reason, we propose the following:

Proposal 1: RAN2 to include in the study the impact on MIB transmission and reception due to the SSB periodicity extension e.g. allowing MIB periodicity to follow SSB even for periods lower than 160 ms, and to prevent a joint configuration of CORESET multiplexing pattern 1 together with SSB periods longer than 20 ms.

### 2.2.2	Neighbor cell measurement configuration

At RAN2#128bis it was agreed to investigate UE consumption and impact of the SSB extension, the need for separate signalling for DL CE cells SMTCs, and continue to discuss how to enable a UE to measure the correct neighbor cells while considering the energy consumption aspect.

R2-2410804 Downlink coverage enhancement SMTC impacts Sequans Communications discussion Rel-19 NR_NTN_Ph3-Core R2-2408970 Proposal 1: RAN2 assumes that the number of configured SMTCs may increase due to beam-hopping / larger SSB periodicity (depending on RAN1/4 solution) - LG thinks the discussion on additional SMTC is up to other groups and then thinks we don’t need to take this assumption for now - Google is not sure we need to extend the number of STMCs even if larger periodicity will be used - Nokia supports this proposal and thinks this is also consistent with the options being considered by RAN1 - Sequans thinks there could be the need for multiple SMTCs per frequency layer and this is the baseline scenario that we would have to consider - QC is not sure why we need to increase the number of SMTCs Proposal 2: RAN2 investigates related additional power consumption impact (including legacy UEs) As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs) Proposal 3: RAN2 considers signaling DL CE cells SMTCs separately (e.g. new SMTC5 list) - Sequans thinks that if we don’t update the signalling then we only have one periodicity RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells Observation 1: The UE should ideally consider only SMTCs corresponding to the closest neighbor cells Proposal 4: Consider enhancements to allow location-based UE SMTC selection (broadcast of neighbor cells reference location / SSB indexes associated with SMTCs) - Nokia agrees but thinks we could work on a UE-centric solution - QC thinks there is no need to capture anything for this at this stage. LG, CATT, Samsung agree Can come back in the next meeting

Considering neighbour cell measurements, the serving cell is providing SMTC windows and measurement gaps (MG) to UEs from the assumption that the neighbouring cell is transmitting the SSB with a certain periodicity. In 38.306, the following is specified regarding number of SMTC windows.

parallelSMTC-r17 Indicates whether the UE supports NTN SSB based RRM measurements on target cells belonging to 4 SMTC-s on a single frequency carrier. If a UE does not include this field but includes nonTerrestrialNetwork-r17, the UE supports NTN SSB based RRM measurements on target cells belonging to 2 SMTC-s on a single frequency carrier.

As of this, configuring the SMTC windows may not be trivial due to 1) the sparce transmission pattern of the SSBs if chosen around 160 ms and 2) some UEs are limited to 2 SMTC windows, whereas others may have 4.

Looking in a common handover case in Figure 1, where a UE is served by cell #0 and should start measuring on cell #1 and #2, three scenarios for neighbor cell SSB offset scenarios:

- SSB transmission pattern for cells #1-#6 are evenly spaced e.g. with the same SSB offset

- SSB transmission pattern for cells #1-#6 are close to each other in time e.g. start of the first SSB transmission is close to the start of the last SSB transmission within a group

- SSB transmission pattern for cells #1-#6 are placed without regular inter-cell SSB spacing e.g. there is no pattern between the individual spacing of the different SSB transmissions

In case of 1, a single SMTC configuration may be sufficient to cover all possible neighbor cell measurements of cells #1-#6, but such mutual configuration will only be possible in a very few corner cases. For instance, if we assume the cells #1-#6 are configured to have a evenly spaced SSB offset, the neighbor cells to any of the cells within the picture will not be able to have similar lean configuration. Thus, in any case, we will end up needing an extension in the number of SMTC windows.

Observation 1: SMTC configuration using the existing number of SMTC windows is not possible.

Figure 1 Example figure of UE position in NR NTN cells

Similar issue is present with the case 2, where a longer SMTC window is configured to cover a group of neighbor cell measurements. This is only a valid solution in case the SSB offsets are very close to each other, as otherwise the UE would measure for longer periods of time than needed and such scenarios can be assumed to be rare.

Observation 2: Configuring SMTC windows covering group of SSB measurements will be very energy inefficient.

Case 3 covers, in our view, a larger subset of all possible configurations that the UE will experience across the area covered by a single satellite, thus this is what should be focused.

Proposal 2: RAN2 to assume that there is no distinct pattern to be configured e.g. for SMTC windows and at least the number of SMTC windows must be extended.

Looking at Figure 1, we also see that the number of cells that in principle, when approaching the cell edge, the UE should only measure a few cells continuously, as the other cells are not candidate cells. The effect of the increased number of SMTC windows, will be that the UE will be mandated to measure within all SMTC windows continuously, which may be energy inefficient.

Observation 3: SMTC window configuration of all possible SSB transmissions of neighbouring cells is energy inefficient.

One potential solution is, that the SMTC window is location dependent, but this brings the issue that the NTN UE is required to be aware of the neighbour cells SSB transmission areas within the serving cell and correlate this with the UEs own location relatively to the serving cell and neighbour cells location at all times. The network could provide area specific SMTC windows, but this would increase the overhead significantly, as the format in SIB2,3,4 would for instance go from

smtc                                SSB-MTC                                         OPTIONAL,       -- Need S

to

smtc_area1                         SSB-MTC                                         OPTIONAL,       -- Need S

area1

….

smtc_areax                         SSB-MTC                                         OPTIONAL,       -- Need S

areax

where the areax is the area where smtc_areax is to be applied. Depending on the area format the overhead can be considerable. Furthermore, for some measurements like inter satellite measurements in active mode measurement gaps are needed, which would mean they also should be area dependent. An further additional drawback is that idle UEs would need to update their understanding of their own location through GNSS update and SIB reading in order to get the SMTC information belonging to their location.

Observation 4: Location based SMTC configuration may be a solution to optimising the energy consumption of an NR NTN UE performing neighbour cell measurements on a subset of the configured SMTC windows.

Observation 5: Location based SMTC configuration provide additional signalling overhead and requires a UE to be aware of the relative location between not only the gNB but also all neighbouring cells.

Another solution is that the UE could detect which neighboring cells are relevant by scanning over one or a few SSB periods. After these scanning periods the UE can limit the measurements and corresponding measurement occasions to the relevant neighboring cells.

Furthermore, SSB transmission windows may not be fixed. The above example only considers the fact that a single cell transmits a single SSB periodically. However, if this is the case there may be no real data transmission since all the gNBs will do is transmit half frames including SSBs. This could result in the SMTCs being dynamic, i.e. changes occasionally, and in such cases the serving cell may have to reconfigure the SMTC windows and measurement gaps through RRC signalling may on top of that lead to extra signalling overhead.

Observation 6: Optimal SMTC window configuration may be dynamic for a given UE if we account not only for SSB transmission but also data transmission.

One way to avoid this overhead can be to instead provide the UE with information about the neighboring cell SSB configuration, such the UE can adjust its behavior, like skipping a measurement gap when it knows the neighboring cells have gone to a lower SSB periodicity. Likewise, the cell could provide information on for how long the neighbor cells are in a certain state (common signal only, or active with UL/DL traffic), allowing a UE to make reasonable decisions about when to read the SSBs considering the UEs own state.

Cell specific measurements of the SSBs are most needed at times of initial access and during mobility events. During mobility the measurements are used to find the candidate cell and to synchronize to the new cell. Having the SSBs less frequent available during the mobility may degrade the performance. In case of Earth Fixed Cells (EFC) the mobility events are well defined and deterministic. To optimize the mobility performance, the SSB periodicity of the candidate cell may during the mobility event set to a lower value in order to maximize the mobility performance.

Proposal 3: RAN2 to study at least four aspects of downlink coverage enhancements

- Increasing the number and duration of SMTC windows;

- Enabling a UE to scan for neighboring cells over a period and autonomous limit the measurements afterwards to the relevant cells based on measurements

- Setting the SSB period to a lower value during mobility events in case of Earth Fixed Cells.

## 2.2	Cell barring

RAN2 agreed in RAN #127bis to introduce a barring mechanism to avoid rel-17 and Rel-18 UE to access a Rel-19 cell with extended SSB periodicity:

Agreements:

1.	If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE)

2.	If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

3.	(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

Furthermore, RAN2 agreed in RAN2 #128 that:

1. If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

There is still a need to discuss how this new barring bit would interact with the existing barring bits. There are two existing barring bits that are relevant are: cellBarred and cellBarred-NTN, which can be “barred” or “not barred”. From 38.304, we have that:

- cellBarred (IE type: "barred" or "not barred") Indicated in MIB message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is ignored by UEs supporting NTN for NTN access, or by UEs supporting ATG for ATG access. - cellBarredNTN (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is ignored if the UE does not support NTN access.

One concern w.r.t barring is, that since NTN and NR can coexist enabling DL-CE specific barring would mean that the network should now consider 3 types of barring (cell, NTN and NTN with CE-for coverage enhancements), and 3 types of UEs (non-NTN, NTN, NTN with CE).

The agreements state that the barring bit is only applicable if there is a need, which initially was due to the fact that it was uncertain whether the SSB extension solution was adapted within RAN1. This is already understood in RAN1 that with a 160 ms SSB extension a UE should able to camp on a cell with SS/PBCH periodicity of 160 ms as the FL Summary states only larger than 160 ms is an issue.

From Thursday session Observation Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan. Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

RAN1 agreement here is slightly ambiguous since it states that backwards compatibility is not guaranteed, but that a UE is only not able to camp on a cell with periodicity longer than 160 ms. One issue for backwards compatibility is the MIB, as we raise in section 2.1.1, but we are of the firm believe that if a good UE implementation is able to perform a successful initial scan it should be allowed camp best effort.

Observation 7: A legacy UE may be able to camp on a cell with SS/PBCH blocks periodicity at 160 ms or below.

Observation 8: There should be no reason not to allow a UE to camp on a cell it can see.

Proposal 4: RAN2 not to instroduce barring bits for DL CE.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: RAN2 to include in the study the impact on MIB transmission and reception due to the SSB periodicity extension e.g. allowing MIB periodicity to follow SSB even for periods lower than 160 ms, and to prevent a joint configuration of CORESET multiplexing pattern 1 together with SSB periods longer than 20 ms.

Observation 1: SMTC configuration using the existing number of SMTC windows is not possible.

Observation 2: Configuring SMTC windows covering group of SSB measurements will be very energy inefficient.

Proposal 2: RAN2 to assume that there is no distinct pattern to be configured e.g. for SMTC windows and at least the number of SMTC windows must be extended.

Observation 3: SMTC window configuration of all possible SSB transmissions of neighbouring cells is energy inefficient.

Observation 4: Location based SMTC configuration may be a solution to optimising the energy consumption of an NR NTN UE performing neighbour cell measurements on a subset of the configured SMTC windows.

Observation 5: Location based SMTC configuration provide additional signalling overhead and requires a UE to be aware of the relative location between not only the gNB but also all neighbouring cells.

Observation 6: Optimal SMTC window configuration may be dynamic for a given UE if we account not only for SSB transmission but also data transmission.

Proposal 3: RAN2 to study at least four aspects of downlink coverage enhancements

- Increasing the number and duration of SMTC windows;

- Enabling a UE to scan for neighboring cells over a period and autonomous limit the measurements afterwards to the relevant cells based on measurements

- Setting the SSB period to a lower value during mobility events in case of Earth Fixed Cells.

Observation 7: A legacy UE may be able to camp on a cell with SS/PBCH blocks periodicity at 160 ms or below.

Observation 8: There should be no reason not to allow a UE to camp on a cell it can see.

Proposal 4: RAN2 not to introduce barring bits for DL CE.