3GPP TSG-RAN WG2 #129	R2-2500729

Athens, Greece, Feb 17th– 21st, 2025

Agenda Item:	8.5.3

Source:	Ericsson

Title:	Discussion on on-demand SIB1 for NES

Document for:	Discussion, Decision

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised [4]. The core part objective relevant to OD-SIB1 based on the revised WID is:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify the corresponding core requirements, for the above features [RAN4].

During the RAN WG2 Meeting #128, the following agreements were reached for the on-demand SIB1:

Agreements on OD-SIB1

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.

In the Appendix we also provide the complete list of all RAN2 agreements reached for on-demand SIB1.

As agreed in the latest WID RAN#105 [4], RAN2 should:

- Specify procedures and signaling method(s) for Case 2 [RAN1/2]:

- Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell.

Furthermore, during RAN2#127bis meeting the following general agreement concerning the WI phase was made:

RAN WG2 Meeting #127bis

- We will inherit all agreements made during SI phase to WI phase.

Taking this into consideration, in this paper, we discuss the remaining open questions concerning following aspects:

- The UL WUS configuration,

- MSG1 based on-demand SIB1 acquisition procedure,

- Impact on Random Access report,

- On-demand SIB1 and Automatic neighbour relation.

# 2	UL WUS configuration

Different aspects related to the UL WUS configuration have been discussed in previous RAN2 meetings. However, there are several open questions, which we will discuss in this section.

The first question concerns the agreement from the RAN2#127bis meeting according to which a UE should check if SIB1 is currently broadcasted by the NES cell before requesting it:

RAN WG2 Meeting #127bis

Broadcast status for OD-SIB1

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

It is important to observe that in certain scenarios checking if SIB1 is being broadcasted or provided on-demand before requesting SIB1 may lead to unnecessary energy consumption on the UE side because a UE may wait unnecessarily long before concluding that it needs to request SIB1. Since the broadcasting is periodical, this issue can be addressed by informing a UE about the timing of the next tentative SIB1 occasion. This information can be provided in the UL WUS configuration, and the UE can use this information to determine during which period it needs to check for SIB1 before requesting on-demand SIB1, i.e., before transmitting the UL WUS. The advantage of this solution is twofold. First, it enables the UE to know when to monitor for a potential SIB1 transmission, which is beneficial in terms of UE energy savings since it creates an opportunity for the UE to sleep during periods when potential SIB1 transmissions are not expected. Second, it ensures that the UE only monitors for as long as necessary to comply with the RAN2#127bis agreement on the “broadcast status for OD-SIB1,” thereby minimizing on-demand SIB1 acquisition delay.

The UL WUS configuration includes the information on SIB1 timing according to which SIB1 is tentatively transmitted.

The second question concerns the validity of the UL WUS configuration. In what follows, we address this question in different cases, i.e., when the UE is in Cell A and when the UE is NES Cell.

Validity of the UL WUS configuration within Cell A:

In RAN2#127bis meeting it was agreed that the UL WUS configuration is provided in a new SIBxx:

RAN WG2 Meeting #127bis

NES Cell’s WUS configuration via existing SIB or new SIB

⦁	NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

Our proposal is to rely on the legacy SI modification procedures while the UE is camping on Cell A that operates in legacy mode (i.e., provides SIB1 according to legacy, allows legacy UEs to camp, and does not provide UL-WUS configuration for itself), which means that UE relies on the legacy SI modification procedures to maintain a valid version of the UL WUS configuration, i.e., a UE needs to ensure that it maintains a valid version of UL WUS configuration by maintaining a valid version of Cell A’s SIBxx.

When the UE camps in Cell A(i.e., provides SIB1 according to legacy, allows legacy UEs to camp, and does not provide UL-WUS configuration for itself), a UE needs to ensure that it maintains a valid version of UL WUS configuration by maintaining a valid version of SIBxx.

Validity of the UL WUS configuration within NES Cell:

The following agreement that is relevant to the question of the UL WUS configuration validity within the NES Cell was reached during the RAN WG2 Meeting #128:

RAN WG2 Meeting #128

Validity of UL WUS configuration

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

Regarding the above agreement, we observe that “directly succeeding cell reselection” does not have a temporal meaning, i.e., it does not have any meaning for how long the UL WUS configuration is valid. Rather, “directly succeeding cell reselection” means that the UL WUS configuration that was obtained in Cell A is valid within a NES cell only if that NES cell is the cell that was reselected right after Cell A. Furthermore, the above agreement implies that if a UE leaves Cell A and later returns to it, the UE cannot take for granted that the stored UL WUS configuration is valid. Instead, upon the return to Cell A, the UE needs to ensure (e.g., via legacy SI procedures) that it has a valid UL WUS configuration before attempting to reselect a NES Cell.

The agreement from RAN2#128 meeting on the “validity of UL WUS configuration” implies that the UL WUS configuration that was obtained in Cell A is valid within the NES cell only if that NES Cell is the cell that was reselected right after Cell A.

The agreement from RAN2#128 meeting on the “validity of UL WUS configuration” implies that if a UE leaves Cell A and later returns to it, the UE needs to ensure (e.g., via legacy SI procedures) upon the return that it has a valid UL WUS configuration before attempting to reselect a NES Cell.

To answer the question of for how long the UE should consider the UL WUS configuration valid within the NES Cell, we consider the following aspects.

First, in RAN2#127 meeting it was agreed that once the UE camps on the NES cell, the NES Cell itself can provide an UL WUS configuration update (if any), e.g., via legacy SI modification procedure:

RAN WG2 Meeting #127

WUS configuration of NES cell from NES cell:

⦁	Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

It is important to observe that since the typical UL WUS configuration is rather static, if there is no need to update the UL WUS configuration content, the NES Cell should not be forced to transmit SIBxx, as this would not be aligned with the objective to minimize the energy consumption in the NES Cell.

The typical UL WUS Configuration is rather static, and hence the frequent updates of the UL WUS configuration are not expected.

Unnecessary SIBxx transmissions in the NES Cell do not comply with the objective to minimize the energy consumption in the NES Cell.

If the UE does not receive an updated UL WUS configuration from the NES Cell, the UE can consider that its stored UL WUS configuration is still valid within the NES Cell.

The NES Cell is not obliged to transmit SIBxx (i.e., even upon the SIBxx validity expiration) unless there is an update to the UL WUS configuration content.

Second, according to the following agreement from the RAN2#127bis, RAN2 prefers not to start the discussion on the question of how to ensure the synchronization of the UL WUS configuration updates between Cell A and NES Cell:

RAN WG2 Meeting #127bis

WUS Configuration update between cell A and NES cell:

⦁	RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

Therefore, according to the above agreement,  if the validity of the UL WUS configuration obtained from Cell A is tied to the Cell A’s SIBxx validity while the UE is within the NES Cell that was reselected right after Cell A, then the UE would need upon the expiration of the SIBxx validity timer to reselect Cell A just to obtain an updated version of SIBxx from Cell A, and then reselect the NES Cell again. Clearly, this would lead to unnecessary cell reselections, and hence to unnecessary signaling, and energy consumption both on the UE and the network side.

Coupling the validity of the UL WUS configuration of a NES Cell obtained from Cell A to the Cell A’s SIBxx validity while the UE is camping in that NES Cell, introduces unnecessary cell reselections, and hence unnecessary signalling and energy consumption both on the UE and the network side.

Considering the existing RAN2 agreements and the above observations, we think that the validity of the UL WUS configuration obtained from Cell A should be decoupled from the Cell A’s SIBxx validity while the UE is within the NES Cell that was reselected right after Cell A.

The UL WUS configuration is valid within the NES Cell until the NES Cell itself provides an UL WUS configuration update, i.e., irrespective of the validity of the SIBxx that contains the UL WUS configuration.

SI validity area and the UL WUS Configuration validity:

We continue with addressing the FFS from the RAN WG2 Meeting #128 on SI validity area. In that regard, let us recall that according to the TS 38.331, the IE SI-SchedulingInfo contains a parameter systemInformationAreaID, which is a bitstring of 24 bits:

-- ASN1START

-- TAG-SI-SCHEDULINGINFO-START

SI-SchedulingInfo ::=               SEQUENCE {

schedulingInfoList                  SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo,

si-WindowLength                     ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280, s2560-v1710, s5120-v1710 },

si-RequestConfig                    SI-RequestConfig                                                OPTIONAL,  -- Cond MSG-1

si-RequestConfigSUL                 SI-RequestConfig                                                OPTIONAL,  -- Cond SUL-MSG-1

systemInformationAreaID             BIT STRING (SIZE (24))                                          OPTIONAL,  -- Need R

...

}

...

-- TAG-SI-SCHEDULINGINFO-STOP

-- ASN1STOP

systemInformationAreaID Indicates the system information area that the cell belongs to, if any. Any SIB with areaScope within the SI is considered to belong to this systemInformationAreaID. The systemInformationAreaID is unique within a PLMN/SNPN.

In addition to the systemInformationAreaID bitstring, there a binary field for each scheduled SIB whether the given SIB follows this parameter or not. If not enabled by the binary field, a SIBxy is cell specific and if enabled, then the same SIBxy is valid in all other cells that have the same bitstring as the field systemInformationAreaID in IE SI-SchedulingInfo in the respective SIB1. When new SIBs are added the question is whether they would support the binary field areaScope:

SIB-TypeInfo ::=                    SEQUENCE {

type                                ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9,

sibType10-v1610, sibType11-v1610, sibType12-v1610, sibType13-v1610,

sibType14-v1610, spare3, spare2, spare1,... },

valueTag                            INTEGER (0..31)                                                 OPTIONAL, -- Cond SIB-TYPE

areaScope                           ENUMERATED {true}                                               OPTIONAL -- Need S

}

areaScope Indicates that a SIB is area specific. If the field is absent, the SIB is cell specific.

In our view there is no reason not to enable this for SIBxx, which carries the UL WUS configuration. It would anyway be up to network whether to enable the area specific functionality for SIBxx or not. Furthermore, as systemInformationAreaID is part of SIB1, there is no need to distinguish or discuss which cell type (Cell A, NES cell, legacy cell not providing UL WUS config) would belong to the area for SIBxx as it would be up to network implementation.

Enable use of field areaScope for SIBxx that contains UL WUS configuration.

Furthermore, since legacy SIB1 does not support the validity area concept, we see no reason to discuss this for OD-SIB1 either. SIB1 has many cell-specific parameters related to cell reselection and cell access, and we hope it can just be observed that OD-SIB1 would not follow the validity area concept.

UL WUS configuration in the case of multiple NES Cells:

In what follows we discuss the aspects related to the UL WUS configuration in the case of multiple NES Cells. The versions of Scenario 1a with multiple Cells A and NES Cells were discussed during the WG2 Meeting #126, and the following agreements were made:

RAN WG2 Meeting #126

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with NES Cell.

As highlighted in the agreement on “Further clarification”, the same Cell A may assist multiple NES Cells.  One of the remaining questions that needs to be addressed is whether the same UL WUS configuration can be used for multiple NES Cells. The benefit of using the same UL WUS configuration for multiple NES Cells is that it allows for minimizing the size of the message that carries the UL WUS configuration(s). However, using different UL WUS configurations for different NES Cells may be needed if Cell A assists multiple NES Cells that are in different bands (e.g., NES Cells in FR1 and FR2) or have different deployment characteristics such as different cell sizes. The benefit of using different UL WUS configurations is that it allows for the flexibility to provide different UL WUS configuration parameters for different NES Cells. Furthermore, in order to avoid waking up multiple NES cells by the same UL WUS configuration/transmission, distinguished UL WUS configurations for various NES cells are favourable. Therefore, in order to achieve the balance between the message size, unnecessary invocations in multi-NES-Cell deployments, and configuration flexibility, the NW should be able to decide whether to use the same or different UL WUS configurations for multiple NES Cells.

Using the same UL WUS configuration for multiple NES Cells allows for minimizing the size of the message that carries the UL WUS configuration(s).

Using different UL WUS configurations for different NES Cells avoids unnecessary/false NES Cell wakeups and allows for the flexibility to provide different UL WUS configuration parameters for different NES Cells when needed (e.g., if Cell A assists both NES cells in FR1 and FR2, different cell sizes, etc.).

It should be possible for the network to indicate a set of common UL WUS configuration parameters for multiple NES Cells and/or separate UL WUS configurations.

# 3	On-demand SIB1 acquisition

The aspects related to the acquisition of on-demand SIB1 such as MSG1 and MSG3 based on-demand acquisition procedures and RAR monitoring were discussed during previous RAN2 meetings. The discussions concluded with the following agreements:

RAN WG2 Meeting #125bis

Agreements on on-demand SIB1:

- Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

RAN WG2 Meeting #126

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

RAN WG2 Meeting #127

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

According to the agreements from the previous meetings, RAN2 agreed to support only MSG1 based on-demand SIB1 request procedure. Furthermore, RAN2 agreed that as a baseline the UE is expected to receive the RAR as a part of MSG1 based SIB1 on-demand request procedure. The details concerning the RAR for MSG1 based on-demand SIB1 request procedure require, however, further discussion.

Similarly, as the UE can use MSG1 to request on-demand SIB1, the NES Cell can use RAR as a feedback message to instruct the UE how to behave upon its reception. For example, the RAR can indicate the information when the UE can expect on-demand SIB1 transmissions relative to the RAR window (e.g., an offset value to on-demand SIB1 transmission and/or a time window during which the UE can expect on-demand SIB1 transmission can be introduced for this purpose),  and whether on-demand SIB1 is transmitted in all directions or only in some SSBs. The RAR can contain the explicit parameter values or a pointer to the values preconfigured in the UL WUS configuration.

RAR message can be used for indicating the UE behaviour concerning when to expect on-demand SIB1 transmission, and in which directions the transmission of on-demand SIB1 can be expected.

The aspects related to the RAR have been also discussed by RAN1 and the following agreements were made:

RAN1#118

Agreement

RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Agreement

At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options:

-Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission

-Option 2: starting time and duration are indicated in the UL WUS configuration

Agreement

At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options:

- Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission

- FFS: Definition of RAR reception time

- Option 2: The reference time point is defined based on the UL-WUS transmission time

- Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission

RAN1#118bis

Agreement

RAN1 to discuss the contents of RAR in response to UL WUS using the legacy RAR for OSI as a starting point.

RAN1#119

Agreement

The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration.

- Unit of the duration is in slot

- FFS: Starting time

Agreement

The reference time point to determine the window starting time for on-demand SIB1 is based on the RAR window for UL WUS wherein UE successfully received a RAR.

- FFS: Use starting or ending slot of the RAR window as reference time

As highlighted in the list of RAN1 agreements, RAN1 agreed that the network provides the starting time and duration of the time window during which on-demand SIB1 will be provided. Furthermore, it is clear from the above list of RAN1 agreements that RAN1 will continue the discussion on the details such as where and how these parameters can be provided, e.g., how to define the reference time point that indicates the beginning of the on-demand SIB1 window. While RAN1 is finalizing these details, RAN2 can discuss the aspects related to the UE behaviour concerning the monitoring of on-demand SIB1. According to our understanding, the purpose of the starting point and on-demand SIB1 window agreed in RAN1 is to inform the UE from what time instance and for how long after the specified time instance on-demand SIB1 will be provided. Therefore, the UE should keep monitoring for on-demand SIB1 through the on-demand SIB1 window defined in RAN1 until it receives on-demand SIB1.

After the UL WUS transmission, the UE keeps monitoring for on-demand SIB1 during the on-demand SIB1 window defined in RAN1.

Related to the above discussion on the acquisition procedure for the on-demand SIB1, RAN2 made the following agreement during the the WG2 Meeting #128:

RAN WG2 Meeting #128

RACH Failure for OD-SIB1

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

The above agreement is about the scenario in which a UE receives the RAR (i.e., MSG2) in response to the UL WUS transmission (i.e., MSG1), but the UE fails to receive SIB1. In our view, it is not clear what “UE fails to receive SIB1” means, i.e., at what point the UE considers that the on-demand SIB1 acquisition procedure failed. Therefore, we propose the following.

Clarify that “UE fails to receive SIB1” implies that the UE fails to receive an on-demand SIB1 by the end of the on-demand SIB1 window introduced by RAN1.

# 4	Impact on Random Access report

Another important feature that may be affected by on-demand SIB1 is Random Access (RA) report, which comprises vital information for RACH optimization. Since on-demand SIB1 procedure uses RA procedure, this case should be supported. One of the mandatory IEs of RA report is RA purpose, e.g., “access related”. One issue is that none of the existing values for RA purpose covers the case where RA procedure was used for request for SIB1. Note that the existing value “requestForOtherSI” specifies that RA procedure was used to request other SI, i.e., SIB2, SIB3, etc. SIB1, however, is a part of the so-called minimum SI (the MIB in the SSB is the other). Hence the value does not qualify for being reused.

None of the existing values for RA purpose in RA report covers the case where RA procedure was used for request for SIB1.

With this issue, the introduction of on-demand SIB1 leaves RA report ill-defined, and the UE behaviour unclear. Hence, the issue must be resolved in Rel-19.

The introduction of on-demand SIB1 has an impact on RA report that must be addressed in Rel-19.

Fortunately, there are many spare values for RA purpose. A simple solution could be to add a new value, e.g., “requestForSIB1” (the name is FFS). We propose RAN2 to discuss the impact of on-demand SIB1 on RA report and consider the addition of a new value for RA purpose to accommodate for the case where RA procedure was used for request for SIB1.

RAN2 to discuss the impact of on-demand SIB1 on RA report.

# 5 On-demand SIB1 and Automatic Neighbour Relation

One of the important features that may be affected by on-demand SIB1 is Automatic Neighbour Relation (ANR). Introduction of on-demand SIB1 affects the ANR in a way that NES Cell with on-demand SIB1 cannot be discovered by the ANR. According to the current specification (c.f., Section 15.3.3 of 38.300 [3]), if a cell is not broadcasting SIB1, the UE aborts the Cell Global Identity (CGI) reporting procedure and reports "noSIB1":

“In case the detected NR cell does not broadcast SIB1, the UE may report noSIB1 indication as specified in TS 38.331 [2].”

Therefore, if a UE relies on legacy ANR means according to which the UE indicates “noSIB1” only, it may be challenging to distinguish the case of a faulty cell not having SIB1 from the case of a NES Cell that does not provide SIB1 at certain times intentionally. Furthermore, if the coexistence of on-demand SIB1 and ANR is not addressed, operators may need to manually provision and manage NES cells with on-demand SIB1 in the neighbour relation tables. Since this is not a practical and desirable solution, there is a need for new means that would allow for a functional coexistence of on-demand SIB1 and ANR.

If a UE relies on legacy ANR means according to which the UE indicates “noSIB1” only, it may be challenging to distinguish the case of a faulty cell not having SIB1 from the case of a NES Cell that does not provide SIB1 at certain times intentionally.

Discuss how to support a functional coexistence of on-demand SIB1 and ANR.

One solution to support the coexistence of on-demand SIB1 and ANR could be that the NW configures the UE whether CGI-Info is to be captured for NES Cells with on-demand SIB1. Another solution could be that CGI-Info (or parts of it) of the NES cell is included in the UL WUS configuration such that the UE already knows or only needs to capture the UL WUS configuration and hence avoids asking for SIB1 for the sake of ANR. The later may be possible if the UE can obtain the UL WUS configuration. In the following, we discuss how to address the problem if the UE finds a NES Cell, but it cannot obtain the UL WUS configuration for the NES Cell. We refer to this problem as the problem of a missing UL WUS.

The problem of a missing UL WUS configuration is illustrated in Figure 1, and it occurs in a scenario where a part of the coverage area of the NES Cell is not covered by a Cell A, but it is covered by another cell, here denoted Cell X. The scenario depicted in Figure 1 can occur in reality, for various reasons. For example, if the coverage area of a cell has a very irregular shape which is typical e.g., in cities, due to various obstructions or if the coverage area of a cell changes over time, e.g., due to Coverage and Capacity Optimization (CCO). In these cases, the UE can find the NES Cell, but it cannot find (any) Cell A (i.e., the UE cannot camp on the NES Cell). Therefore, the problem of a missing UL WUS configuration may lead to additional issues in the context of the coexistence of on-demand SIB1 and ANR.

Figure 1: An example of the scenario in which the missing UL WUS configuration problem occurs.

The coverage area of a Cell A (or NES Cell) can have an irregular shape (e.g., in cities due to various obstructions) and/or can change over time (e.g., due to performing CCO), and thus the coverage overlap of Cell A and a NES Cell can also vary.

Additional issues in the context of the coexistence of on-demand SIB1 and ANR can occur if the UE finds a NES Cell, but it cannot obtain the UL WUS configuration for the NES Cell.

The above observations imply that there is a need to support the detection of problems with missing UL WUS configuration and other similar problems with on-demand SIB1 provisioning. On the one hand, there is a need to support the means that would allow the NES Cell to identify itself as a NES Cell without having to rely on a Cell A. For example, the means that would allow a UE to detect that a cell is a NES Cell (i.e., that is using on-demand SIB1 transmission) from the SSB/MIB of the cell itself. On the other hand, there is a need for means that would allow a UE to inform the NW about problems with provisioning the on-demand SIB1 and the UL WUS configuration.

A NES Cell needs to be able to identify itself as a NES cell without relying on a Cell A so that UEs can efficiently detect missing UL WUS configuration.

There is a need for means that would allow a UE to inform the NW about problems with the provisioning of the on-demand SIB1 and the UL WUS configuration.

Related to this aspect, RAN1 agreed the following during the RAN1#118bis meeting:

RAN1#118bis

Agreement

UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A

According to the above agreement from RAN1, a UE can use at least the UL WUS configuration from Cell A to determine a cell with on-demand SIB1. It is important to observe that if a UE relies only on Cell A when determining a cell supporting on-demand SIB1 (as captured in the RAN1 agreement) a UE may not be able to detect faulty configuration if a Cell A has not been visited (as illustrated in Figure 1). Since the RAN1 agreement does not exclude the other means that would allow a UE to determine a cell supporting on-demand SIB1 (i.e., the agreement states that “at least UL WUS configuration from Cell A” is used), we propose the following.

RAN2 to support NES Cell identifying itself as a NES cell based on SSB content without having to rely on a Cell A.

Discuss the means that would allow a UE to inform the NW about problems with on-demand SIB1 and UL WUS configuration provisioning.

# 6	Conclusion

In this contribution we discuss the second objective highlighted in the WID. Based on the discussion in the previous sections we made the following observations:

Observation 1	The agreement from RAN2#128 meeting on the “validity of UL WUS configuration” implies that the UL WUS configuration that was obtained in Cell A is valid within the NES cell only if that NES Cell is the cell that was reselected right after Cell A.

Observation 2	The agreement from RAN2#128 meeting on the “validity of UL WUS configuration” implies that if a UE leaves Cell A and later returns to it, the UE needs to ensure (e.g., via legacy SI procedures) upon the return that it has a valid UL WUS configuration before attempting to reselect a NES Cell.

Observation 3	The typical UL WUS Configuration is rather static, and hence the frequent updates of the UL WUS configuration are not expected.

Observation 4	Unnecessary SIBxx transmissions in the NES Cell do not comply with the objective to minimize the energy consumption in the NES Cell.

Observation 5	If the UE does not receive an updated UL WUS configuration from the NES Cell, the UE can consider that its stored UL WUS configuration is still valid within the NES Cell.

Observation 6	Coupling the validity of the UL WUS configuration of a NES Cell obtained from Cell A to the Cell A’s SIBxx validity while the UE is camping in that NES Cell, introduces unnecessary cell reselections, and hence unnecessary signalling and energy consumption both on the UE and the network side.

Observation 7	Using the same UL WUS configuration for multiple NES Cells allows for minimizing the size of the message that carries the UL WUS configuration(s).

Observation 8	Using different UL WUS configurations for different NES Cells avoids unnecessary/false NES Cell wakeups and allows for the flexibility to provide different UL WUS configuration parameters for different NES Cells when needed (e.g., if Cell A assists both NES cells in FR1 and FR2, different cell sizes, etc.).

Observation 9	RAR message can be used for indicating the UE behaviour concerning when to expect on-demand SIB1 transmission, and in which directions the transmission of on-demand SIB1 can be expected.

Observation 10	None of the existing values for RA purpose in RA report covers the case where RA procedure was used for request for SIB1.

Observation 11	The introduction of on-demand SIB1 has an impact on RA report that must be addressed in Rel-19.

Observation 12	If a UE relies on legacy ANR means according to which the UE indicates “noSIB1” only, it may be challenging to distinguish the case of a faulty cell not having SIB1 from the case of a NES Cell that does not provide SIB1 at certain times intentionally.

Observation 13	The coverage area of a Cell A (or NES Cell) can have an irregular shape (e.g., in cities due to various obstructions) and/or can change over time (e.g., due to performing CCO), and thus the coverage overlap of Cell A and a NES Cell can also vary.

Observation 14	Additional issues in the context of the coexistence of on-demand SIB1 and ANR can occur if the UE finds a NES Cell, but it cannot obtain the UL WUS configuration for the NES Cell.

Observation 15	A NES Cell needs to be able to identify itself as a NES cell without relying on a Cell A so that UEs can efficiently detect missing UL WUS configuration.

Observation 16	There is a need for means that would allow a UE to inform the NW about problems with the provisioning of the on-demand SIB1 and the UL WUS configuration.

Based on the discussion in the previous section, we propose the following:

Proposal 1	The UL WUS configuration includes the information on SIB1 timing according to which SIB1 is tentatively transmitted.

Proposal 2	When the UE camps in Cell A(i.e., provides SIB1 according to legacy, allows legacy UEs to camp, and does not provide UL-WUS configuration for itself), a UE needs to ensure that it maintains a valid version of UL WUS configuration by maintaining a valid version of SIBxx.

Proposal 3	The NES Cell is not obliged to transmit SIBxx (i.e., even upon the SIBxx validity expiration) unless there is an update to the UL WUS configuration content.

Proposal 4	The UL WUS configuration is valid within the NES Cell until the NES Cell itself provides an UL WUS configuration update, i.e., irrespective of the validity of the SIBxx that contains the UL WUS configuration.

Proposal 5	Enable use of field areaScope for SIBxx that contains UL WUS configuration.

Proposal 6	It should be possible for the network to indicate a set of common UL WUS configuration parameters for multiple NES Cells and/or separate UL WUS configurations.

Proposal 7	After the UL WUS transmission, the UE keeps monitoring for on-demand SIB1 during the on-demand SIB1 window defined in RAN1.

Proposal 8	Clarify that “UE fails to receive SIB1” implies that the UE fails to receive an on-demand SIB1 by the end of the on-demand SIB1 window introduced by RAN1.

Proposal 9	RAN2 to discuss the impact of on-demand SIB1 on RA report.

Proposal 10	Discuss how to support a functional coexistence of on-demand SIB1 and ANR.

Proposal 11	RAN2 to support NES Cell identifying itself as a NES cell based on SSB content without having to rely on a Cell A.

Proposal 12	Discuss the means that would allow a UE to inform the NW about problems with on-demand SIB1 and UL WUS configuration provisioning.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TS 38.331 V18.1.0, Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification, Release 18.

3GPP TS 38.300 V18.1.0, Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description, Release 18.

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN105, Melbourne, Australia, September 2024.

# Appendix A

### RAN WG2 Meeting #125bis

1.	At least RAN2 starts scenario 1a. Other scenarios are not excluded.

a.	Scenario 1a: Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell. with below potential RAN2 impacts:

i.	Add WUS configuration in SIB of cell A.

ii.	Cell reselection from cell A to NES cell, including trigger condition and cell barring changes.

iii.	Whether allow camping, paging and SIB update in NES cell.

iv.	Cell reselection from NES cell to cell A or normal cell.

2.	RAN2 assumes that RACH procedure is reused for UE to request on-demand SIB1.

3.	UL WUS configuration includes at least below information:

a.	RACH configuration

4.	A UE needs to know a UL WUS configuration to request SIB1 of which cell.

5.	Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

6.	The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

### RAN WG2 Meeting #126

1.	Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

2.	More than one Cell A may provide configuration for the same NES cell.

3.	The same Cell A may assist more than one NES Cells.

4.	RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

5.	Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

6.	For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

7.	Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

8.	If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

9.	After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

10.	RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

11.	NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

12.	How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

13.	If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

14.	After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

15.	RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

16.	As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

17.	Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

18.	RAN2 to wait for RAN1’s progress whether to support scenario 3.

### RAN WG2 Meeting #126

1.	No consensus for RAN1 case 3 in RAN2.

2.	We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

3.	Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

4.	Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

5.	Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

6.	Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

7.	Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

8.	Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

9.	Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

10.	RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

11.	RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

### RAN WG2 Meeting #127bis

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

### RAN WG2 Meeting #128

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.