3GPP TSG-RAN WG2 #129	Tdoc R2-2500731

Athens, Greece, Feb 17th– 21st, 2025

Agenda Item:	8.5.2

Source:	Ericsson

Title:	Discussion on on-demand SSB for NES

Document for:	Discussion, Decision

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised [1]. The first objective concerns the on-demand SSB SCell operation, as shown below.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, SCell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The list of RAN1 agreements is available in the Appendix.

# 2	Configuration and signalling aspects

This contribution addresses the aspects concerning the activation/deactivation of on-demand SSB transmissions on SCells, configuration, signaling, L3 measurements, and SCell activation procedure.

## 2.1	MAC CE for on-demand SSB

RAN2 has discussed the MAC CE design in RAN2#127 and RAN2#127bis meetings and reached the following agreements:

RAN2#127 Agreements

OD-SSB transmission indication:

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

RAN2#127bis Agreements

OD-SSB transmission indication:

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

It is important to observe that in RAN1, the functional details on the MAC CE activation of the on-demand SSB are still open. Based on RAN1#118 agreements (see the complete list of the agreements in the Appendix), it appears that RAN1 is considering separate handling for each value of the on-demand SSB configuration and has agreed that at least the periodicity is one such parameter. This type of design could potentially mean that there is a separate field in the MAC CE for each RRC parameter of the on-demand SSB configuration. However, since the only parameter indicated so far is periodicity, it is difficult to assess whether this type of design which has many fields associated to one SCell is feasible or not. Another type of design would be that there are N on-demand SSB configurations, configured via RRC signalling, with different parameter values and the MAC CE points out one of the configurations to be activated/deactivated. Yet another option is to have M “base configurations” configured by RRC and a few separate parameters to be selected/indicated directly by the MAC CE. Furthermore, the MAC CE may activate on-demand SSB, and/or indicate specific parameters for it, per SCell, or for a list of SCells. Which way to design the MAC CE depends on the complexity needed for each activated on-demand SSB.

Furthermore, in RAN1#118bis the following relevant agreements were reached:

Agreement

- For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options.

  - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

  - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - Option 3: Configuration/indication of the duration of on-demand SSB transmission window

  - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell

  - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

  - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed

  - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI

  - FFS: Each option is applicable to which Cases or Scenarios

  - FFS: Details related to each of the above options

Agreement

- For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1.

  - Sub-carrier spacing of the on-demand SSB

    - FFS if this can be absent

  - Physical Cell ID of the on-demand SSB

  - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index

  - Downlink transmit power of on-demand SSB

  - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

In RAN1#119 the following relevant (to this contribution) agreements were reached:

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

While additional parameters for on-demand SSB configuration by RRC were agreed in RAN1#118bis meeting and in RAN1#119, functional details needed for the MAC CE design are not finalized. In order to proceed with the MAC-CE design, RAN2 would need to know what exactly is to be controlled by MAC CE for each SCell.

There are many open questions in RAN1 that impact the MAC CE design for on-demand SSB.

Wait for RAN1 to progress with on-demand SSB design before discussing the MAC CE details further.

## 2.2	RRC configuration for on-demand SSB

Concerning the configuration aspects, the discussion in RAN1#117 meeting concluded with the following agreements:

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

As highlighted in the agreement above, one of the possible solutions is that the network provides the UE with different options for the on-demand SSB transmission patterns by an RRC configuration. These options for the SSB transmission patterns could be given outside of the SCell configuration since we think that the SSB patterns can be commonly configured for several SCells. However, to support certain level of flexibility, the configurations could be different for FR1 and FR2. This way, configurations for FR1 and FR2 would be shared among serving cells operating on FR1 and FR2, respectively. It is important to note that while the configuration options for the SSB transmission patterns would be per FR, the activation of a certain SSB transmission pattern could be different for different Scells within the same FR.

In what follows we discuss the advantages of configuring on-demand SSB per FR over both, configuring on-demand SSB per SCell, and configuring the same on-demand SSB for FR1 and FR2. First, we observe that configuring on-demand SSB per FR requires fewer bits than configuring on-demand SSB per SCell. Second, we observe that configuring on-demand SSB per FR allows more flexibility than configuring the same on-demand SSB for both FR1 and FR2. This is important because FR1 and FR2 have different characteristics (e.g., number of potential beams employed in each FR, SCS) that may require different on-demand SSB configurations. For example, as the maximum number of beams is different in FR1 and FR2, the potential number of deployed on-demand SSB beams would vary, which could lead to different periodicities, and hence different starting points/offsets for the on-demand SSBs in FR1 and FR2. Therefore, configuring on-demand SSB per FR can be seen as a compact solution that provides needed flexibility.

Define OnDemand SSB configuration per FR which is applicable for all SCells in that FR.

The IE CellGroupConfig configures the SCells using the IE SCellConfig

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                        OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity                  OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                     OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                                 OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                            OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                       OPTIONAL,   -- Need N

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Need S

]],

[[

preConfGapStatus-r17             BIT STRING (SIZE (maxNrofGapId-r17))                           OPTIONAL,   -- Cond PreConfigMG

goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17                                  OPTIONAL,   -- Need R

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }                                OPTIONAL    -- Need M

]],

[[

plmn-IdentityInfoList-r17       SetupRelease {PLMN-IdentityInfoList}                            OPTIONAL,   -- Cond SCellSIB20-Opt

npn-IdentityInfoList-r17        SetupRelease {NPN-IdentityInfoList-r16}                         OPTIONAL    -- Cond SCellSIB20-Opt

]]

}

smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent and absoluteFrequencySSB is included, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. If the SCell is an SSB-less SCell (i.e., the IE absoluteFrequencySSB in ServingCellConfigCommon is absent), this field is absent.

The UE uses the smtc configured here in the CellGroupConfig when configured, and if not configured, UE uses the smtc1 in the corresponding MO. Regardless of whether the OD-SSB configuration is SCell specific or not it seems UE can use the timing information of the OD-SSB configured in CellGroupConfig. This configuration, for which RAN2 should get parameters from RAN1, would be present any time UE is supposed to operate with OD-SSB since it is needed for the MAC CE functionality. Therefore, it is unclear if additional smtc(s) which include the same timing information for the various OD-SSB patterns are needed in the corresponding SCellConfig and/or MOs.

RAN2 to discuss the need to configure additional SMTCs(to RAN1 parameters) in the MO and/or SCellConfig for various OD-SSB patterns for configured SCells.

# 3	L3 measurements

Another aspect that was discussed during RAN2 #127 meeting concerns the measurements on on-demand SSBs, and the conclusions are summarized in the following list of agreements:

RAN2#127 Agreements

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

R2-2407414: Proposal 3: RAN2 WG to discuss the issue of false measurement report       		 triggering due to no SSB transmission when on-demand SSB is deactivated.

- RAN2 does not handle this issue in OD-SSB

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Via its serving cell configuration, the UE has the configuration for on-demand SSB and receives the MAC CE for activation of the on-demand SSB reception. Since the UE has this information, it is not evident that any enhancement is needed for the L3 RRM configuration. In what follows, we summarize the status of RAN1 and RAN4 discussions that are relevant from the perspective of the measurements. RAN1 reached the following agreements on the always-on and on-demand SSB:

RAN1 #116bis Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not.

  - For on-demand SSB on the cell, downselect between the following alternatives

    - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not.

    - Alt-2: On-demand SSB is limited to non-cell-defining SSB.

      - FFS: Further limitations to on-demand SSB

RAN1 #118 Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following is supported

  - On-demand SSB on the cell is not located on synchronization raster.

  - On-demand SSB on the cell is non-cell-defining SSB

- FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

RAN1 #118bis Agreement

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases.

  - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB

  - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

Furthermore, RAN4 sent LS to RAN1(RAN2 cc) in R4-2416913 and RAN1 has responded as follows:

RAN1 would like to thank RAN4 for the LS on SSB relation in On-demand SSB and SSB adaptation on Scell and would like to provide the following replies.

Obj 1

Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?):

- The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms.

- The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB.

- RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB.

Reply to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?):

- RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst.

- RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB.

Reply to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?):

- The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?):

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Obj 3

Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?):

- RAN1 agreed that at least SSB burst periodicity is adapted.

- There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options.

Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?):

- The frequency location is same before and after SSB adaptation.

Reply to Q3(What is the spatial relation before and after SSB adaptation?):

- There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation.

- At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

2	Actions

To RAN4:

ACTION: 	RAN1 respectfully asks RAN4 to take the above into account in their future work.

From the measurement perspective, one question that needs to be addressed is how the UE makes serving cell quality derivation when on-demand SSB is used. In the RAN2#127bis meeting, RAN2 attempted to conclude whether a UE combines the always-on and on-demand SSBs or not in different cases. However, as RAN1/RAN4 had not concluded their discussions, RAN2 could not conclude the discussion either. We are currently proposing in our RAN4 companion paper that UE would be able to combine always-on SSB and OD-SSB to perform deactivated Scell measurements[2]. In our view the serving cell measurements for activated Scell should follow the same principle.

RAN2 to agree that from RAN2 perspective it would be preferable that UE combines SSB and OD-SSB measurements in the L3 filter especially if they are on same channel raster point.

Furthermore, based on RAN1 and RAN4 discussions, a case where on-demand SSB can be on another channel raster point than the serving cell CD-SSB is still open. The Question 3 about the frequency location is as follows:

- The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

In case there is a scenario where OD-SSB can be on a different frequency location from the "absoluteFrequencySSB" configured in IE ServingCellConfigCommon, (by the fields "physCellId" and "downlinkConfigCommon  frequencyInfoDL  absoluteFrequencySSB") or in IE ServingCellConfigCommonSIB, there is a need to investigate how the serving cell measurement object (MO) is handled, as this is needed for the serving cell quality derivations. For RedCap, which may configure and use an SSB of another frequency, the SSB configuration and servingcellMO links are added in the BWP configuration as follows:

#### –	BWP-DownlinkDedicated

The IE BWP-DownlinkDedicated is used to configure the dedicated (UE specific) parameters of a downlink BWP.

BWP-DownlinkDedicated information element

-- ASN1START

-- TAG-BWP-DOWNLINKDEDICATED-START

BWP-DownlinkDedicated ::=           SEQUENCE {

…

]],

[[

…

nonCellDefiningSSB-r17              NonCellDefiningSSB-r17                                            OPTIONAL,   -- Need R

servingCellMO-r17                   MeasObjectId                                                  OPTIONAL -- Cond MeasObject-NCD-SSB

]],

…

}

-- TAG-BWP-DOWNLINKDEDICATED-STOP

-- ASN1STOP

Similar to RedCap, RAN2 could conclude that a serving cell MO for the on-demand SSB of another frequency is simply linked to the serving cell configuration rather than adding an additional frequency to the original serving cell MO.

RAN2 to agree that in the case where the OD-SSB is of another frequency than the frequency of a serving cell, the MO for the OD-SSB is linked to the serving cell configuration, for OD-SSB measurement purposes.

# 4	SCell activation procedure

SCell activation procedure has been discussed in RAN4 and the following agreements were reached during the RAN4#112bis meeting:

RAN4 #112bis Agreement

Issue 1-4-5: Which type of SSB is used during SCell activation

Agreement:

- In Case 1, UE is required to always use OD-SSB for Scell activation.

- In Case 2, when both OD-SSB and always-on SSB are transmitted,

  - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline

  - The agreement can be revisited if different with other WG

On this topic, RAN2 can discuss whether to confirm the RAN4 assumption that only on-demand SSB is used for SCell activation procedure when both always-on and on-demand SSBs are transmitted. In our opinion, there appear to be no RAN2 issues for UEs to combine always-on SSB and OD-SSB or to use only on-demand SSB for SCell activation procedures when the network transmits both always-on SSB and on-demand SSB. Since RAN4 had the disclaimer that their agreement is pending on the other WGs, we think that it would be beneficial to conclude in RAN2 whether the RAN4 assumption can be confirmed.

RAN2 to confirm that when network transmits both always-on SSB and on-demand SSB there appears to be no RAN2 issues for UEs to combine always-on SSB and OD-SSB or to use only on-demand SSB for SCell activation procedure.

# 5	Conclusion

In this contribution we discuss the second objective highlighted in the WID. Based on the discussion in the previous sections we made the following observations:

Observation 1	There are many open questions in RAN1 that impact the MAC CE design for on-demand SSB.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Wait for RAN1 to progress with on-demand SSB design before discussing the MAC CE details further.

Proposal 2	Define OnDemand SSB configuration per FR which is applicable for all SCells in that FR.

Proposal 3	RAN2 to discuss the need to configure additional SMTCs in the MO and/or SCellConfig for various OD-SSB patterns for configured SCells.

Proposal 4	RAN2 to agree that from RAN2 perspective it would be preferable that UE combined in the L3 filter the SSB and OD-SSB measurements especially if they are on same channel raster point.

Proposal 5	RAN2 to agree that in the case where the OD-SSB is of another frequency than the frequency of a serving cell, the MO for the OD-SSB is linked to the serving cell configuration, for OD-SSB measurement purposes.

Proposal 6	RAN2 to confirm that when network transmits both always-on SSB and on-demand SSB there appear to be no RAN2 issues for UEs to combine always-on SSB and OD-SSB or to use only on-demand SSB for SCell activation procedure.

# References

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN#105, October 2024.

R4-xxxxx, “Discussion on OD-SSB in Rel-19 NES”, RAN4#114 Athens, Ericsson

# Appendix

### RAN1#116 Agreements

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

### RAN1#116bis Agreements

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not.

  - For on-demand SSB on the cell, downselect between the following alternatives

    - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not.

    - Alt-2: On-demand SSB is limited to non-cell-defining SSB.

      - FFS: Further limitations to on-demand SSB

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

    - FFS further details on L1 and/or L3 measurement

Agreement

The following agreement from RAN1#116 is modified (in red)

- For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

Agreement

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

### RAN1#117 Agreements

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

  - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

    - This DCI signaling does not provide SCell activation/deactivation.

    - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

  - FFS: Scenarios where the above signalings are applicable

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

Agreement

- At least support L1 measurement based on on-demand SSB

  - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

    - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Agreement

For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

  - The SSB time domain positions of on-demand SSB burst are configured by gNB.

- FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

- Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

- FFS: Whether the value of T is predefined or indicated/configured by gNB

- FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE.

  - Sub-carrier spacing of the on-demand SSB

  - Physical Cell ID of the on-demand SSB

  - Location of on-demand SSB burst

  - Downlink transmit power of on-demand SSB

  - FFS: Other parameters

  - FFS: Whether each of above parameters is configured/indicated explicitly or not

### RAN1#118 Agreements

Agreement

- Update the previous RAN1 agreement as follows.

  - At least support L1 measurement based on on-demand SSB

    - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

      - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

      - The support of LTM is a separate discussion point

Agreement

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Agreement

For a cell supporting on-demand SSB SCell operation, at least the following is supported

- On-demand SSB on the cell is not located on synchronization raster.

- On-demand SSB on the cell is non-cell-defining SSB

FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

Agreement

Support L3 measurement based on on-demand SSB

- Further work on L3 measurement is up to RAN2/RAN4

Agreement

LS to RAN2 for on-demand SSB SCell operation is agreed. Final LS in R1-2407438.

Agreement

The previous RAN1 agreement made in RAN1#117 is revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

  - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - FFS: Whether the value of T is predefined or indicated/configured by gNB

  - (Working assumption) T=T_min

  - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

LS on timeline for On-demand SSB operation on SCell is agreed in R1-2407565.

### RAN1#118bis Agreements

Agreement

- For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options.

  - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

  - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - Option 3: Configuration/indication of the duration of on-demand SSB transmission window

  - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell

  - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

  - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed

  - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI

  - FFS: Each option is applicable to which Cases or Scenarios

  - FFS: Details related to each of the above options

Agreement

- For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1.

  - Sub-carrier spacing of the on-demand SSB

    - FFS if this can be absent

  - Physical Cell ID of the on-demand SSB

  - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index

  - Downlink transmit power of on-demand SSB

  - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

Agreement

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB.

  - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB

  - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB

  - FFS: Whether OD-SSB and always on SSB have same beam or not

Conclusion

No consensus on the support of on-demand SSB SCell operation triggered by UE.

Agreement

The previous RAN1 agreement is partly confirmed and further revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

      - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - (Working assumption) T=T_min

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases.

- Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB

- Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain