3GPP TSG-RAN WG2 Meeting #129							          R2-2500732

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.4

Source: 	NEC

Title:	Ambient-IoT Data Transmission

Document for:	Discussion/Decision

- Introduction

The Rel-19 WID for Ambient IoT (also referred to as A-IoT) includes following RAN2-led objectives [1].

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

This contribution discusses the data transmission aspects based on the RAN2-led objectives and the latest RAN1 and RAN2 agreements.

- Discussion

  - D2R Data transmission

    - Segmentation

AS per TR38.769 [2], the following candidate solutions for segmentation are considered:

The segmentation study focuses on D2R direction (it can be further considered on R2D direction). For the candidate solution for segmentation:
   -	 The sequence number, the segment number and the number of segments are not supported;
   -	 An indication is used to indicate to the reader on whether the data is segmented and whether the MAC PDU is      the last segment. It needs to be further discussed on the size of this indication (one or two bits) and the corresponding further details;
   -	 It is beneficial for the reader to be able to trigger a re-transmission of a segment;
   -	 It is assumed that the A-IoT device will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all segment(s) are stored in upper layer(s).

Indication of segmentation

One-bit indication of the segmentation can be interpreted as:

- 0, segments other than the last one;

- 1, the last segment or there is only one segment.

Based one-bit indication, the Reader can determine whether to schedule another D2R transmission. In addition, the combination of segments can be carried out in the higher layer or application layer. Additionally, considering the variable data amount at the A-IoT device, the message size information can be provided by the device. Therefore, there is no need to provide extra information for the combination or message size via the indication of segmentation. So we propose:

RAN2 to adopt One-bit indication of segmentation: 
- 0, segments other than the last one;
- 1, the last segment or there is only one segment.

Transmission and Retransmission of a segment

It is assumed that the A-IoT device will not support AS layer (MAC layer) buffering for A-IoT segmentation functionalities, i.e., all segment(s) are stored in upper layer(s).

For transmission of each data segment, device only needs to know the starting and ending position for current transmission requested by reader. This can be implemented by following steps:

- Step 1: Upon D2R transmission of segment#0, the device stores a “start position” (starting position of segment#0) and a “next start position” (starting position of the next data segment to be transmitted);

- Step 2: In scheduling for D2R segment#1, reader provide a starting information and a size information to device. If segment#0 is successfully decoded, the starting information may indicate D2R segment#1 to start from the “next start position”; If segment#0 is not successfully decoded, the starting information may indicate D2R segment#1 to start from the “start position”;

- Step 3: Based on the starting information, the device considers the starting position of segment#1 to be the “start position” or the “next start position”. Then device construct segment#1 for transmission with the starting position and the size information.

Some examples can be seen in following figures:

Example of segment#0

Fig.1 Example of segment#1 (ACK for segment#0)

Fig.2 Example of segment#1: NACK for segment#0, size of segment#1 < size of segment#0)

Upon each segment transmission, the device stores a start position (starting position of the data segment for current transmission) and a next start position (starting position of the next data segment to be transmitted).

Upon scheduling for next segment, reader needs to provide starting information. Based on the starting information, the device considers the starting position of a data segment for transmission to be the start position or the next start position.

The starting information provided by the reader can be an explicit indication (either the start position or the next start position), or implicit indicated by ACK or NACK for last segment.

Upon scheduling for next segment, reader needs to provide a size information.

Upon receiving the D2R transmission scheduling information, device inform upper layer to construct the requested segment with the starting position and the size information.

After detecting the failure of D2R transmission, the Reader can schedule another D2R transmission. The scheduled traffic block size (TBS) may be the same as or different from the previous one (Fig.2 shows an example that the TBS is smaller than the previous one).  Upon receiving the D2R transmission scheduling information, the A-IoT device just retrieves the data stored in upper layer (the data may include the previous segment) to form the MAC PDU according to the D2R transmission scheduling information. Thus, there is no need to design a method for the retransmission of a specific segment at A-IoT MAC layer.

- There is no need to design a method for the retransmission of a specific segment at A-IoT MAC layer.

    - Message size indication

In RAN2#127bis [4] meeting, there is an agreement on message size reporting shown as below:

- From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size.  Two options can be captured: 1) from the CN and 2) from the device (simple message size indication).    Capture the advantages/disadvantages.

As we agreed it is beneficial for the reader to know an estimate of expected D2R message size, the main difference between two options from our perspective would be on different efficiency. Device based option is more dynamic reported than CN based option, therefore more real and accurate message size can be provided in each D2R transmission. With this real-time message size indication, reader can do the best scheduling to guarantee high efficiency of D2R message transmission. Apparently, the disadvantage of device based message size indication is more transmission overhead in PRDCH and obviously additional spec impact on indicator signalling design. However, for AIoT system it is worthy to have such real-time/frequent report since higher scheduling efficiency is of much importance for AIoT device. Lower efficiency D2R scheduling may result in additional D2R transmission with more power consumption. If device run out of power, energy harvesting and delay transmission would be envisioned which needs another round of random access procedure and large delay for this round of inventory/command task. From perspectives of both latency and energy consumption of AIoT devices, more efficient scheduling with relatively higher transmission overhead is preferred.

- D2R message size reported by device can provide more efficient scheduling of D2R transmissions.

- D2R message size reported by device can potentially provide D2R transmission with low latency and low power consumption.

- D2R message size reported by device has larger spec impact and signaling overhead on extra design of report format.

In terms of device based D2R message size report, simple message size indication with less spec impact and simple signalling design as well as overhead is preferred. Targeting this, a ratio based message size reporting mechanism is proposed, where a ratio defined as remaining message size over previous scheduled size at device side is designed. By introducing this metric, efficient data scheduling for A-IoT system can be supported as shown in Fig. 1.

Fig. 1 Ratio based message size reporting mechanism for A-IoT communication

To be specific,

Step 1. A first PRDCH with TBS1 indication of a subsequent PDRCH is transmitted from reader to device.

Step 2. After device receives the PRDCH and obtains TBS1, a ratio r = remaining message size / TBS1 = (total D2R message size - TBS1) / TBS1 will be calculated by device. Next, device reports this ratio together with the scheduled D2R transmission in corresponding PDRCH.

Step 3. When reader receives PDRCH successfully, reader calculates the remaining D2R message size by multiply the reported r with TBS1. Based on the calculated remaining message size, reader could indicate TBS2 in another PRDCH transmission for another PDRCH scheduling. This procedure will repeat until the completion of all D2R message transmission.

- Ratio based message size report is a simple and efficient mechanism for A-IoT message scheduling.

Ratio based message size reporting can be supported for efficient scheduling in A-IoT.

    - Padding

For the D2R transmission, it may happen that the grant is larger than the upper/higher layer data available to transmit. In NR, padding is used to solve this problem by reaching the exact grant size in UL. Since there is no need to keep byte alignment for A-IoT, the device can terminate the transmission early according to the exact data size. How to solve it is also related to the following agreement of RAN1#116bis [3] meeting on acquiring the end of PDRCH transmission:

RAN1#116bis Agreements: For the reader to acquire the end of PDRCH transmission, study at least following options: • Option 1: D2R postamble immediately follows the PDRCH • Option 2: Based on control information

Either one of the above options can be used for the indication of the early PDRCH termination. So we suggest not to adopt padding function at MAC layer and the exact method for the indication of the early PDRCH termination is up to RAN1.

RAN2 doesn’t support padding function. And the method for the indication of the early PDRCH termination is up to RAN1.

- 2.1.4.	“AS ID” for purpose of D2R scheduling and R2D reception

AS per TR38.769 [2], the followings are captured on “AS ID” for purpose of D2R scheduling and R2D reception:

From higher layer perspective, it is assumed that this "AS ID" should be a short AS layer ID, rather than the full upper layer device ID. It needs to be further discussed if this "AS ID" can be based on partial upper layer device ID. It needs to be further discussed on the length of this "AS ID". From higher layer perspective, following options are possible for this "AS ID" (it is aimed to define one common design for all access procedures in clause 6.3.4, if technically possible):

-	Option 1: a random ID (if used in first D2R message) can be reused;

-	Option 2: the reader assigns this "AS ID" to the device. It needs to be further discussed via which R2D message.

-	Option 3:	It is up to the reader whether to reuse the random ID (if used in first D2R message) as the "AS ID" or to assign a new "AS ID". It needs to be further discussed via which R2D message.

NOTE 1:	The further down-selection for "AS ID" needs to consider the involved message and whether the reader needs to handle the collision.

For Option 1, potential random ID collision is a potential issue. In our understanding, the potential random ID collision risk would be quite low since RAN2#126 agreed that “RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose”. If the design target is to realize an almost collision-free “AS ID”, one potential solution is to combine Option 1 and 2, i.e., defining AS ID with the random ID used in Msg.1 as the baseline, additionally only when the random ID collision occurs, the reader assigns an “AS ID” which is collision free from random IDs the reader has received/echoed so far. In such a case, the reader assigned “AS ID” has the same length as random ID, i.e., 16 bits.

Another issue is how to define “AS ID” for contention free access since contention free access is agreed to skip the contention resolution in Step 2, i.e., only device ID will be sent from device to the reader as the response to the A-IoT paging message which indicates the contention-free access. We think the above combination of Option 1 and 2 also works well by allowing the reader to assign an “AS ID” for contention free access devices.

In our understanding, Option 3 captured in the TR is a kind of combination of Option 1 and 2, we propose to support Option 3 for defining “AS ID”.

Regrading “AS ID” for purpose of D2R scheduling and R2D reception, support to use the random ID (if used in the first D2R message) as the baseline. Additionally, the reader is allowed to assign a (new) 16-bit “AS ID” for the contention free access or for the case in which the random ID collision is detected.

For the reader assigned “AS ID”, we still have one FFS, i.e., via which R2D message to inform devices the reader assigned “AS ID”. Based on the overall AS procedures between A-IoT device and reader, there are three alternatives:

Alt.1: reader provides the assigned “AS ID” via R2D message in Step B (i.e., Msg.2)

Alt.2 reader provides the assigned “AS ID” via R2D message in Step C (e.g., R2D data transmission in C1)

Note: Considering the base line for “AS ID” is to use the random ID (if used in the first D2R message), reader provides the assigned “AS ID” via A-IoT paging message is not listed as an alternative.

Fig.1 Overall AS procedures between A-IoT device and reader

Both alternatives are workable. With Alt.1, D2R scheduling and R2D reception via “AS ID” can be supported from the beginning of Step C, we think Alt.1 is more flexible than Alt.2.

Reader provides the assigned “AS ID” via R2D message in Step B (i.e., Msg.2).

- 2.1.5.	Handling of D2R transmission failure

The handling of D2R data transmission failure has been studied during SI phase. It is understood that the subsequent R2D data transmission after the D2R data transmission does not need to be always sent. In case of D2R data transmission failure, the A-IoT device follows the reader subsequent R2D instruction, if any.

For the handling of D2R transmission failure during Step C: data transmission, the reader can schedule a subsequent D2R transmission when the failure is detected. Upon receiving the D2R transmission scheduling info, the A-IoT device multiplexes data stored in upper layer (the data may include the previous segment) and D2R AS-layer control information, if any, according to the TBS, i.e., the A-IoT device just follows the received R2D to transmit following D2R regardless of whether it is for handling failure or not.

For the handling of D2R transmission failure during the Step B: D2R data transmission, the reader can re-send A-IoT Msg2 to specific device(s) to echo the random ID(s), in case of failure reception of corresponding D2R data transmission (i.e. "Msg3") after the initial A-IoT Msg2, together with the corresponding D2R resource scheduling. This can trigger the A-IoT device to re-send the same D2R data transmission (i.e. "Msg3"). Therefore, we think RAN2 doesn’t need to further discuss handling of D2R transmission failure.

RAN2 doesn’t need to further discuss handling of D2R transmission failure.

  - R2D Data transmission

    - Repetition

Transmission via the A-IoT air interface between the reader and the A-IoT device may encounter signal fading or blockage, just like traditional Uu interface. This may lead to D2R data transmission failure. In RAN2#127bis [4] meeting, there is an agreement on repetition of R2D transmission (e.g., upper layer command) by the reader to trigger re-sending the same D2R response shown as below:

1 In case of D2R data transmission failure, device follows the reader instruction:

	it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

	reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

That is, the reader may send inventory/command several times. To control the number of repetitions, the following methods can be considered:

- The specific number of repetition is visible to the reader from CN to control the repetition.

- The survival time of a single inventory/command request is visible to the reader. And the specific repetition times depend on the reader implementation.

The control information for repetition (e.g. specific number of repetitions or survival time of a single inventory/command request) is useful to be visible to the reader from CN.

  - MAC PDU

    - Multiplexing

The transmission of A-IoT NAS signaling and the transmission of upper layer data are usually separate. Thus, there is no need to consider the multiplexing of multiple layer2 (e.g., logical) channels. For A-IoT, only one logical channel is agreed for A-IoT transmission. Therefore, there is no need to consider the LCP of multiple layer2 channels.

However, in RAN2#127bis [4] meeting, there is an agreement on energy bits shown as below:

Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can.  1 bit indication can be captured in the TR.    We will capture the use case in the TR.   FFS whether it is reader controlled.

The energy status indication can be reported to the Reader via MAC layer. Also the message size can be reported to the Reader via MAC layer. These indicators, also referred to as AS layer control information, are highly relevant for data transmission. Upon receiving the D2R transmission scheduling info, the A-IoT device just multiplexes data stored in upper layer and D2R AS layer control information, if any, according to the TBS.  So we propose:

RAN2 to considerate the multiplexing of the upper layer data and the control information of MAC layer.

    - PDU format

Both the control information and the upper layer data can be transmitted in one MAC PDU, so the sub-MAC PDU can be introduced for the upper layer data and the MAC control element (CE) can be introduced for the control information. In addition, for control information such as the energy status indication, MAC CE of fixed length can be introduced to simplify the receiving operation of the Device. So we propose:

RAN2 to introduce sub-MAC PDU for higher layer data and MAC CE for the control information of MAC layer.

The MAC CE for the control information of MAC layer is of fixed length.

  - Assistance information on device energy status report

During the SI phase, the use of the assistance information on device energy status report was studied. Followings are captured in the TR:

-	The energy status report from the A-IoT device to the reader, which indicates that the A-IoT device, if the A-IoT device can determine this, does not have sufficient energy to perform the remaining/follow-up procedure.

-	The A-IoT device may report a 1-bit energy status indication to the reader in a D2R message. It needs to be further discussed if and how the reader controls this indication of such report.

-	The reader may take this indication into account for the remaining/follow-up procedure (up to the reader implementation), e.g., the reader may choose not to transmit the subsequent messages for some time, or the reader may choose to not do anything, etc.

In our understanding, energy status report is beneficial from the following perspectives:

-	improve communication availability: By indicating whether a device has sufficient energy to remain available for communication, energy status report helps maintain reliable communication links between the device and the reader. It ensures that devices can communicate successfully without being interrupted due to lack of energy.

-	Resource Optimization in A-IoT AS-layer procedures: energy status report allows for optimized use of communication resources by ensuring that the devices participating in CBRA and CFRA procedures are capable of completing necessary message exchanges, thereby avoiding resource wastage.

Support energy status report from A-IoT devices to reader.

- Conclusion

Based on the discussion in this document, we have the following observations and proposals:

Observation 1:	There is no need to design a method for the retransmission of a specific segment at A-IoT MAC layer.

Observation 2:	D2R message size reported by device can provide more efficient scheduling of D2R transmissions.

Observation 3:	D2R message size reported by device can potentially provide D2R transmission with low latency and low power consumption.

Observation 4:	D2R message size reported by device has larger spec impact and signaling overhead on extra design of report format.

Observation 5:	Ratio based message size report is a simple and efficient mechanism for A-IoT message scheduling.

Proposal 1:	RAN2 to adopt One-bit indication of segmentation:  
- 0, segments other than the last one; 
- 1, the last segment or there is only one segment.

Proposal 2:	Upon each segment transmission, the device stores a start position (starting position of the data segment for current transmission) and a next start position (starting position of the next data segment to be transmitted).

Proposal 3:	Upon scheduling for next segment, reader needs to provide starting information. Based on the starting information, the device considers the starting position of a data segment for transmission to be the start position or the next start position.

Proposal 4:	The starting information provided by the reader can be an explicit indication (either the start position or the next start position), or implicit indicated by ACK or NACK for last segment.

Proposal 5:	Upon scheduling for next segment, reader needs to provide a size information.

Proposal 6:	Upon receiving the D2R transmission scheduling information, device inform upper layer to construct the requested segment with the starting position and the size information.

Proposal 7:	Ratio based message size reporting can be supported for efficient scheduling in A-IoT.

Proposal 8:	RAN2 doesn’t support padding function. And the method for the indication of the early PDRCH termination is up to RAN1.

Proposal 9:	Regrading “AS ID” for purpose of D2R scheduling and R2D reception, support to use the random ID (if used in the first D2R message) as the baseline. Additionally, the reader is allowed to assign a (new) 16-bit “AS ID” for the contention free access or for the case in which the random ID collision is detected.

Proposal 10:	Reader provides the assigned “AS ID” via R2D message in Step B (i.e., Msg.2).

Proposal 11:	RAN2 doesn’t need to further discuss handling of D2R transmission failure.

Proposal 12:	The control information for repetition (e.g. specific number of repetitions or survival time of a single inventory/command request) is useful to be visible to the reader from CN.

Proposal 13:	RAN2 to considerate the multiplexing of the upper layer data and the control information of MAC layer.

Proposal 14:	RAN2 to introduce sub-MAC PDU for higher layer data and MAC CE for the control information of MAC layer.

Proposal 15:	The MAC CE for the control information of MAC layer is of fixed length.

Proposal 16:	Support energy status report from A-IoT devices to reader.

- Reference

- RP-243326, Rel-19 WID of Ambient IoT.

- TR38.769-j00.

- Chairman Notes, RAN1 #116bis meeting.

- Chairman Notes, RAN2 #127bis meeting.