3GPP TSG-RAN WG2 Meeting #129                        R2-2500760

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.3

Source: 	Transsion Holdings

Title:  	Discussion on L1 event triggered measurement reporting

Document for:	Discussion and Decision

- Introduction

In the last RAN2#128 meeting, the L1 event triggered measurement reporting was discussed and the following agreements were made[1]:

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

In this contribution, we continue to discuss the measurement enhancement for LTM.

- Discussion

- 2.1 Measurement event configuration for LTM

In previous meeting it has been agreed that any beam in candidate RS configuration can be used for LTM event evaluation to reduce latency, which means that if any beam among the beams configured by LTM RS configuraiton meets the event, it triggerd MR. However due to the lots of configured LTM RS and the limited measurement capability, the beam triggers the L1 MR maybe not the best beam which will result the reduced performance. To avoid this, some meaurement priority for candidace cell or beam can be defined. For example, for some cells the UE is connected to, the UE is more likely to switch to the intra-DU candidate cells and for some other cells the UE is more likely to switch to the inter-DU/inter-CU candidate cells. To make this, the measurement priority of the candidate cell or beam can be defined or signaled with the measurement configuraiton.

Proposal 1:  The measurement priority for candidace cell or beam can be defined, UE priority meaures the candidate cell or beam with high priority.

2.2 Event triggered L1 measurement purpose for LTM

As agreed in previous meeting, the event triggered L1 measurement shoud be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

The R18 LTM supports UE-based TA measurement for early synchronization and the exact time the UE performs TA measurement is up to UE implementation, thus the network doesn’t  know whether the UE has performs TA measurement or not. If the event triggered L1 MR is used for trigger early synchronization, it may cause repeated triggering early synchronization if the UE has already get the TA of candidates cell by UE-based TA measurement. To avoid this there can be two approaches: One is the purpose of the event triggered L1 measurement can be included in the measurement configuration. If the purpose is to select candidate beam/cell to trigger early synchronization and the UE has already get the TA of candidate cells, the UE can consider these candidate cells unavailable or doesn’t perform L1 measurement on these candidate cells or doesn’t trigger L1 measurement report if event condition is met to avoid repeated triggering early synchronization. The other one is the TA available information can be included in the L1 measurement report. If the network receives L1 measurement report with TA available information for a candidate cell, then the network will not send PDCCH order for this candidate cell to trigger early synchronization as the UE has already get the TA. The TA available information can be just one bit to indiate the network that the UE has already get the TA of the candidate cell.

Proposal 2: The purpose of event triggered L1 measurement can be included in the measurement configuration to avoid repeated triggering early synchronization.

Proposal 3: The TA available information can be included in the measurement report to avoid repeated triggering early synchronization.

- 3 Conclusions

In this contribution we discussed issues related to measurement enhancement for LTM and made the following proposals:

Proposal 1:  The measurement priority for candidace cell or beam can be defined, UE priority meaures the candidate cell or beam with high priority.

Proposal 2: The purpose of event triggered L1 measurement can be included in the measurement configuration to avoid repeated triggering early synchronization.

Proposal 3: The TA available information can be included in the measurement report to avoid repeated triggering early synchronization.

- 4 References

- “Draft_RAN2_128_Meeting_Report_v1”