3GPP TSG-RAN WG2 Meeting #129	R2-2500764

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.7.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On RRM measurement gaps/restrictions related enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements on RRM measurement gaps/restrictions related enhancements were made in RAN2 #127bis and RAN2 #128:

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

Agreements on RRM measurement gaps impacts - No MG-specific enhancements are needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped. - No MG-specific enhancements is needed for DRX operation.

In this contribution, we focus on the RAN2 impacts from solutions considered by RAN1/RAN4 and provide our view on the need for semi-static solutions.

# 2	MAC impacts

As RAN2 has agreed in RAN2#128 that no MG-specific enhancements are needed for DSR and DRX operation, impact on MAC specifications is expected to be minimal. Essentially, the only change in MAC specification due to RRM measurement gaps/restrictions is in clause 5.14 (Handling of measurement gaps), where it needs to be specified the clause that describes the behaviour of the MAC entity during a MG which has not been indicated as skipped by DCI signalling.

Observation 1: There is very small impact on MAC specifications due to RRM measurement gaps/restrictions. RAN2 only needs to specify that clause 5.14 (Handling of measurement gaps) only describes the behaviour of the MAC entity during an activated MG which has not been indicated as skipped by DCI signalling.

# 3	On the need of semi-static solutions

The need for semi-static solutions for RRM measurement gap/scheduling restrictions skipping has been already heavily discussed in RAN1 and RAN4. In RAN1#118 (August 2024), RAN1 first limited the solutions for RRM measurement gap/scheduling restrictions skipping to two alternatives: dynamic and semi-static. Dynamic solution (i.e., Alt 1-1 - Explicit DCI indication) was agreed to be the RAN1 working assumption. Therefore, dynamic solution is RAN1 working assumption since RAN1#118. During RAN1#118bis (November 2024), RAN4 sent an LS to RAN1, confirming the feasibility of the dynamic solution.

Observation 2: RAN1 already made the down selection between DCI and RRC schemes.

Semi-static schemes were proven to provide inferior performance as compared to the DCI-based scheme in both RAN1 and RAN4, see e.g. the results in R4-2418890.  As shown in Figure 4 and 5 of R4-2418890, the capacity gain of the semi static scheme (Alt-3) is very minor in comparison to the dynamic scheme (DCI-based). This is mainly because semi-static solutions are highly depending on the network being able to accurately predict when measurement gaps/scheduling restriction are overlapping with XR data bursts, while XR traffic is dynamic as characterized by jitter and cell load can introduce additional scheduling delay.

Observation 3: Simulations in RAN1 and RAN4 showed that dynamic schemes (DCI-based) can provide a better trade-off between XR performance and MG/scheduling restrictions skipping ratio as compared to semi-static schemes. E.g., dynamic schemes can achieve higher XR performance for the same skipping ratio, or similar XR performance with a lower skipping ratio (higher skipping ratio implies higher impact on mobility performance).

Another problem of semi-static solutions relying on traffic pattern prediction, is that the network needs a training phase to determine the XR traffic pattern. Only after that, the network can determine the skipping pattern to be configured to the UE that matches the XR traffic. Therefore, during such training phase, the UE would experience reduced performance.

Observation 4: Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start and delay the start of using skipping for XR traffic.

Also, semi-static schemes may require an RRC reconfiguration every time the UE needs to increase or decrease the number of measurement occasions based on e.g. its relative location within a cell.

Observation 5: Semi-static schemes are also dependent on an accurate estimate of the number of measurement occasions needed by a UE, which may depend on the UE mobility state, possibly resulting in a high number of reconfigurations.

Proposal 1: Follow the RAN1 working assumption and consider DCI-based schemes only, i.e. semi-static schemes are not further discussed.

# 3	Conclusion

This document has made the following observations:

Observation 1: There is very small impact on MAC specifications due to RRM measurement gaps/restrictions. RAN2 only needs to specify that clause 5.14 (Handling of measurement gaps) only describes the behaviour of the MAC entity during an activated MG which has not been indicated as skipped by DCI signalling.

Observation 2: RAN1 already made down selection between DCI and RRC schemes.

Observation 3: Simulations in RAN1 and RAN4 showed that dynamic schemes (DCI-based) can provide a better trade-off between XR performance and MG/scheduling restrictions skipping ratio as compared to semi-static schemes. E.g., dynamic schemes can achieve higher XR performance for the same skipping ratio, or similar XR performance with a lower skipping ratio (higher skipping ratio implies higher impact on mobility performance).

Observation 4: Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start and delay the start of using skipping for XR traffic.

Observation 5: Semi-static schemes are also dependent on an accurate estimate of the number of measurement occasions needed by a UE, which may depend on the UE mobility state, possibly resulting in a high number of reconfigurations.

And proposed the following:

Proposal 1: Follow the RAN1 working assumption and consider DCI-based schemes only, i.e. semi-static schemes are not further discussed.