3GPP TSG-RAN WG2 Meeting #129	R2-2500765

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.7.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	LCP enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#128:

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

# 2	Discussion

## 2.1	LCH priority adjustments

Point 2 of the RAN2#128 agreements states that, as an optional capability, the UE can also support to fallback to default priority in the 2nd round of the LCP procedure. What is not clear from the agreement, is whether the capability is with or without signalling and whether it will be enabled based on NW configuration.

The RAN2 guidelines for UE capability definitions in R2-2002378 include indication to avoid defining any functionality that has no RRC configuration but is dependent on capability bits.

Observation 1: based on the guidance, RAN2 should avoid defining any functionality that has no RRC configuration but is dependent on capability bits.

So, in case of capability with signalling, the capability should enable a specific RRC configuration. The only RRC configuration that such capability could enabled is the NW configuring a UE indicating capability to support fallback to default priority in the 2nd round of the LCP procedure, whether it shall fallback to default priority in case all the delay-critical data has been allocated in the 1st round.

On the other hand, the LCP prioritization is done after the NW allocates resources to the UE. The NW makes use of all the available information to make the best decision such that delay-critical data can be transmitted within the delay budget. Used information for that purpose are coming from UE reports such as BSR, and enhanced BSR in Rel-18, DSR, and currently under discussion enhanced DSR with multiple remaining time thresholds. Each of those provide a certain level of granularity for LCGs buffer size and buffer size together with remaining time.

Observation 2: NW decides on the resource allocation based on the available information it has from UE reports such as BSR and DSR.

Observation 3: It is not clear based on what information or specific condition the NW would decide to configure a UE, which indicates support to fallback to default priority in the 2nd round of LCP procedure, not to do so.

In case of capability without signaling, this essentially corresponds to leave for UE implementation whether to fallback to default priority in the 2nd round of the LCP procedure.

Observation 4: Capability without signaling corresponds to leave for UE implementation whether to fallback to default priority in the 2nd round of LCP procedure.

Therefore, we suggest RAN2 revisit the agreement from RAN2#128 based on one of the following options:

- Option #1: No capability

  - It is agreed that the additional LCH priority is always applied to both the first round and the second round of the LCP procedure, or that all UEs support fallback to default priority in the 2nd round of the LCP procedure

- Option #2: Capability without signaling

  - It is left for UE implementation whether to fallback to default priority in the 2nd round of LCP procedure

- Option #3: Capability with signaling

  - In which case the NW is enabled to configure whether the UE shall fallback to default priority in the 2nd round of the LCP procedure

Proposal 1: RAN2 to choose from one of the following options:

- Option #1: No capability

  - Option #1a: the additional LCH priority is always applied to both the 1st and 2nd round of the LCP procedure,

  - Option #1b: all UEs support fallback to default priority in the 2nd round of the LCP procedure

- Option #2: capability without signaling

  - It is left for UE implementation whether to fallback to default priority in the 2nd round of LCP procedure

- Option #3: capability with signaling

  - The NW is enabled to configure whether the UE shall fallback to default priority in the 2nd round of the LCP procedure

Our preference, based on what discussed above, is option #1 for simplicity.

## 2.2	Other LCP impact

RAN#105 included UL congestion signalling in the Rel-19 XR Phase 3 objectives. Currently, XR rate control per DRB/per QoS flow is being discussed in RAN2. When the bit rate changes, the GBR for the DRB would also change, which means the PBR for the LCH should be reflected as well. If the PBR is not updated based on bit rate set by rate control, this may lead to a starvation problem. For example, a DRB/LCH is configured with a given PBR.  Later, rate control decreases the bit rate for the corresponding DRB/LCH. If the PBR is not correspondingly adjusted, MAC may allocate a relatively higher number of resources to the corresponding LCH as compared to lower priority LCHs. Then, the starvation becomes more critical as the network gets congested and less resources are available. RAN2 should discuss the impact of bit rate adjustment on PBR setting.

Proposal 2: RAN2 to discuss the impact of bit rate adjustment on PBR setting.

# 3	Conclusion

Remaining issues on LCP are discussed in this contribution with the following proposals proposed:

Proposal 1: RAN2 to choose from one of the following options:

- Option #1: No capability

  - Option #1a: the additional LCH priority is always applied to both the 1st and 2nd round of the LCP procedure,

  - Option #1b: all UEs support fallback to default priority in the 2nd round of the LCP procedure

- Option #2: capability without signaling

  - It is left for UE implementation whether to fallback to default priority in the 2nd round of LCP procedure

- Option #3: capability with signaling

  - The NW is enabled to configure whether the UE shall fallback to default priority in the 2nd round of the LCP procedure

Proposal 2: RAN2 to discuss the impact of bit rate adjustment on PBR setting.