3GPP TSG RAN WG2 #129	R2-2500775

Athens, GR, Feb 17th–21st, 2025

Agenda Item:	7.0.2.15

Source:	ZTE Corporation, Sanechips

Title:	Reply LS on QMC Coordination for RRC Segmentation in NR-DC

Document for:	Discussion and Decision

Introduction

RAN3 LS have been received regarding the RRC segmentation indication in NR-DC, see [1], with following questions:

With respect to the above, RAN3 kindly asks RAN2 the following: - Question 1: Can the UE receive an AppLayerMeasConfig-r17 IE for MCG configuration containing rrc-SegAllowedSRB5-r18 parameter? - Question 2: Can the UE receive an AppLayerMeasConfig-r17 IE for SCG configuration containing rrc-SegAllowedSRB4-r17 parameter? - Question 3: If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter, how does the UE treat its RRC segmentation state for SRB5 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB5 or does the UE consider that the RRC segmentation function for SRB5 shall be disabled? - Question 4: If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, how does the UE treat its RRC segmentation state for SRB4 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB4 or does the UE consider that the RRC segmentation function for SRB4 shall be disabled?

The contribution intends to discuss RAN3’s questions in the LS and provide the corresponding reply LS.

Discussion

- Discussion for Questions 1 and 2

-------------------------------------------------------------------38.331------------------------------------------------------------------

–	AppLayerMeasConfig

The IE AppLayerMeasConfig indicates configuration of application layer measurements.

AppLayerMeasConfig information element

-- ASN1START

-- TAG-APPLAYERMEASCONFIG-START

AppLayerMeasConfig-r17 ::=           SEQUENCE {

measConfigAppLayerToAddModList-r17   SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayer-r17     OPTIONAL, -- Need N

measConfigAppLayerToReleaseList-r17  SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayerId-r17   OPTIONAL, -- Need N

rrc-SegAllowedSRB4-r17               ENUMERATED {enabled}              OPTIONAL, -- Need R

...,

[[

rrc-SegAllowedSRB5-r18               ENUMERATED {enabled}              OPTIONAL, -- Need R

idleInactiveReportAllowed-r18        ENUMERATED {enabled}              OPTIONAL  -- Need R

]]

}

[...]

-- TAG-APPLAYERMEASCONFIG-STOP

-- ASN1STOP

AppLayerMeasConfig field descriptions

rrc-SegAllowedSRB4 This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB4. The field is only configured for an MCG. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

rrc-SegAllowedSRB5 This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB5. The field is only configured for an SCG. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

-------------------------------------------------------------------38.331------------------------------------------------------------------

According to current specs, rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 is configured to UE via AppLayerMeasConfig-r17, which are separately used for MCG and SCG transmission measurement reports. And the intention are captured as the yellow highlighted text in the field description. It is further noticed that rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 indications are defined per SRB with Need R, which means that when the UE receives a configuration lacking these fields, the UE should release the corresponding field, i.e., consider the segmentation in corresponding SRB is disabled.

Observation 1: rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 is configured to UE via AppLayerMeasConfig-r17, which are separately used for MCG and SCG transmission measurement reports.

Observation 2: rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 indications are defined per SRB with Need R, which means that when the IE is absent, the UE will consider the segmentation in corresponding SRB is disabled.

It is possible that both segmentation in MN and SN is enabled, in such case the received AppLayerMeasConfig-r17 IE for MCG configuration or SCG configuration shall contain both rrc-SegAllowedSRB5-r18 and rrc-SegAllowedSRB4-r17. Otherwise, UE will release the field that is not present, and consider the corresponding segmentation is disabled.

Here is an example. When the UE supports RRC segmentation, MCG first configures QoE measurement and enables the SRB4 segmentation function. Further, when QoE measurement also needs to be configured for SCG, if the configuration information sent by SCG does not include the indication to enable the SRB4 segmentation function (i.e., rrc-SegAllowedSRB4), then UE will release rrc-SegAllowedSRB4 field based on the Need R type, therefore  disabling the SRB4 segmentation function. Such behavior is not intended,  therefore in order for NW to provide the rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 properly, coordination is needed between MN and SN, which can be discussed by RAN3.

Observation 3: When QoE report segmentation is enabled for both MN and SN, the UE received  AppLayerMeasConfig-r17 IE for MCG configuration or SCG configuration shall contain both rrc-SegAllowedSRB5-r18 and rrc-SegAllowedSRB4-r17 to allow segmentation in both leg.

Based on above analysis, it can be confirmed that UE can receive an AppLayerMeasConfig-r17 IE for MCG configuration containing rrc-SegAllowedSRB5-r18 parameter and UE can receive an AppLayerMeasConfig-r17 IE for SCG configuration containing rrc-SegAllowedSRB4-r17 parameter. Therefore below proposal is made:

Proposal 1: Reply to Questions 1/2 that UE can receive an AppLayerMeasConfig-r17 IE for MCG configuration containing rrc-SegAllowedSRB5-r18 parameter and UE can receive an AppLayerMeasConfig-r17 IE for SCG configuration containing rrc-SegAllowedSRB4-r17.

- Discussion for Questions 3 and 4

------------------------------------------------------------- TS 38.331 ------------------------------------------------------------------

#### 5.7.16.2	Initiation

[]

3>	if the encoded RRC message is larger than the maximum supported size of one PDCP SDU specified in TS 38.323 [5]:

4>	if the RRC message segmentation is enabled based on the field rrc-SegAllowedSRB4 received in appLayerMeasConfig and the reportingSRB is SRB4, or

4>	if RRC message segmentation is enabled based on the field rrc-SegAllowedSRB5 received in appLayerMeasConfig and the reportingSRB is SRB5:

5>	initiate the UL message segment transfer procedure as specified in clause 5.7.7 for transmission via the SRB indicated in the field reportingSRB in MeasConfigAppLayer;

4>	else:

5>	discard the RRC message;

-------------------------------------------------------------- TS 38.331 ----------------------------------------------------------------

Analysis on Question 3/4 was covered by previous discussion. UE behavior is also clearly captured in the procedure part as cited above, which indicates that UE determine to enable the segmentation based on the received rrc-SegAllowedSRB4-r17/rrc-SegAllowedSRB5-r18 field is present.

Therefore, below proposal is made:

Proposal 2: For Questions 3/4 of LS, reply to RAN3 that if the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18/rrc-SegAllowedSRB4-r17 parameter, the UE considers that the RRC segmentation function for SRB5/SRB4 is disabled respectively.

Based on the above analysis, we provide a draft LS as Annex A to this paper.

Proposal 3: If P1/2 is agreed, RAN2 agrees on the content of draft LS in the Annex .

Conclusion

In the above sections, the following observations and proposals were made:

Observation 1: rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 is configured to UE via AppLayerMeasConfig-r17, which are separately used for MCG and SCG transmission measurement reports.

Observation 2: rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 indications are defined per SRB with Need R, which means that when the IE is absent, the UE will consider the segmentation in corresponding SRB is disabled.

Observation 3: When QoE report segmentation is enabled for both MN and SN, the UE received  AppLayerMeasConfig-r17 IE for MCG configuration or SCG configuration shall contain both rrc-SegAllowedSRB5-r18 and rrc-SegAllowedSRB4-r17 to allow segmentation in both leg.

Proposal 1: Reply to Questions 1/2 that UE can receive an AppLayerMeasConfig-r17 IE for MCG configuration containing rrc-SegAllowedSRB5-r18 parameter and UE can receive an AppLayerMeasConfig-r17 IE for SCG configuration containing rrc-SegAllowedSRB4-r17.

Proposal 2: For Questions 3/4 of LS, reply to RAN3 that if the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18/rrc-SegAllowedSRB4-r17 parameter, the UE considers that the RRC segmentation function for SRB5/SRB4 is disabled respectively.

# References

- R3-247888 LS on QMC Coordination for RRC Segmentation in NR-DC

- TS 38.331

- Annex A– Draft Reply LS to RAN3

3GPP TSG-RAN WG2 Meeting #129

Athens, GR, Feb 17th–21st, 2025

Title:	Reply LS on QMC Coordination for RRC Segmentation in NR-DC

Response to:	R3-247888

Release:	Release 18

Work Item:	NR_QoE-Core

Source:	RAN2

To:	RAN3

CC:

Contact person:	Zhihong Qiu

qiu.zhihong@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall description

RAN2 thanks RAN3 for the LS. After discussion, below are RAN2 responses to Questions 1-4:

- Question 1: Can the UE receive an AppLayerMeasConfig-r17 IE for MCG configuration containing rrc-SegAllowedSRB5-r18 parameter?

- RAN2 response:Yes. UE can receive an AppLayerMeasConfig-r17 IE for MCG configuration containing rrc-SegAllowedSRB5-r18 parameter.

- Question 2: Can the UE receive an AppLayerMeasConfig-r17 IE for SCG configuration containing rrc-SegAllowedSRB4-r17 parameter?

- RAN2 response:Yes. UE can receive an AppLayerMeasConfig-r17 IE for SCG configuration containing rrc-SegAllowedSRB4-r17 parameter.

- Question 3: If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter, how does the UE treat its RRC segmentation state for SRB5 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB5 or does the UE consider that the RRC segmentation function for SRB5 shall be disabled?

- RAN2 response: If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter,UE considers the RRC segmentation function for SRB5 is disabled.

- Question 4: If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, how does the UE treat its RRC segmentation state for SRB4 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB4 or does the UE consider that the RRC segmentation function for SRB4 shall be disabled?

- RAN2 response: If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, UE considers the RRC segmentation function for SRB4 is disabled.

It is kindly asked that RAN3 takes above responses into account in their future work and check if any specs updates is needed.

- Actions

To RAN3:

ACTION: 	RAN2 kindly asks RAN3 to take the above into account in their future work, and check if any specs updates is needed.

- Dates of next RAN2 meetings

RAN2#129-bis	07 Apr – 11 Apr 2025	China

RAN2#130	19 May– 23 May 2025	Malta