3GPP TSG-RAN WG2 Meeting #129                                                                  R2-2500777

Athens, Greece, 17th – 21st February, 2025

Source: 		  ZTE Corporation, Sanechips

Title:                  Paging procedure for A-IoT

Agenda item:    8.2.2

Document for:  Discussion and Decision

- Introduction

During RAN2#125bis~RAN2#128, RAN2 has completed the working group study for supporting Ambient IoT in NR and achieved much progress on the aspects of protocol stack, functionalities, random access-like and paging-like procedures etc. The study result has been captured in the TR 38.769 [1].

In RAN#106, WID for A-IoT has been agreed in [2] and the scope and RAN2 objectives are as below:

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. WGs begin their discussions from the decisions already made in TR 38.769, with the following refinements for the scope: The following objectives are set, within the General Scope: <skip unrelated parts> - RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included <skip unrelated parts>

In this paper, we will give initial discussion on the specification work in RAN2 for supporting A-IoT Paging function.

- Discussion

2.1 Main functionalities of A-IoT Paging message

The following general procedure in A-IoT air interface [1] is cited in below as reference for following discussion:

(TR 38.769 V2.0.1) Figure 6.3.1-1 Overall AS procedures between A-IoT device and reader

- 2.1.1 To indicate the targeted devices

For a basic scenario that service request from CN/AF indicates to operate (e.g., inventory) a certain number of devices, the reader triggers an initial A-IoT Paging message. The A-IoT Paging message can contain one identifier or no identifier to trigger the selected/indicated devices to access network and give response (e.g., to send back its real device ID) to the CN/AF. Here both “one identifier” and “no identifier” can correspond to device ID information. Per our understanding, “one identifier” corresponds to that a single device, or a group of devices, or a set of devices are selected and “no identifier” corresponds to that all the relevant devices are selected. The WID scope also requires the forward compatibility for Paging message design, such that multiple identifiers can be contained in one paging message.

It can be seen that the device ID information is the most important part in the A-IoT Paging message. The device needs to compare its stored device ID information with the provided device ID information and determine whether it needs to respond the A-IoT operation. From the procedure perspective, the device’s MAC entity needs to firstly address this information to determine whether it is the targeted device before proceeding with subsequent processes. If this information were placed inside a container that is not visible to the device’s MAC layer and needs to be handed over to higher layer (e.g., A-IoT NAS layer) for processing, the MAC procedure for handling Paging message will be interrupted and cannot resume until the result of checking device ID information is returned from the higher layer to MAC layer. So from RAN2 perspective, there should be device ID information included in the Paging message that is visible to the MAC layer.

Observation 1a: From the procedure perspective, the device’s MAC entity needs to firstly address the device ID information in A-IoT Paging message to determine whether it is the targeted device before proceeding with subsequent processes.

SA2 [3] has agreed following principles for Identifier and Identification Management for A-IoT device:

      - An Ambient IoT Device is configured with a permanent Ambient IoT Device Identifier which can be assigned by an operator or by a third party. The Identifier is used to identify Ambient IoT Device and locate the corresponding authentication server.

And according to SA2 conclusion, at least permanent Ambient IoT Device Identifier can be supported which generally includes part1 and part2 information. Two types of permanent Ambient IoT Device Identifier are given as below examples:

(TR 23.700-13 v1.3.0) Figure 8.2.1-1: The example of Operator allocated ID

(TR 23.700-13 v1.3.0) Figure 8.2.1-2: The example third party allocated ID

It can be seen that the mentioned options for A-IoT device ID are generally kind of structured design. The part1 information is generally for identifying the country and/or network/operator/third party holder. The Part2 information (e.g., EPC) can be for identifying the company or the owner or the service group to which the device belongs, and also for identifying each individual device within the scope defined by the preceding parts.

Such structured design for A-IoT device ID will facilitate selecting devices that belong to a specific network, company, or service category, or just simply choosing a subset of devices, by applying simple mask/filter rules. For example, the “mask/filter” may have a certain binary pattern for a range of bits [m, n] on the sub-part, e.g., “company/owner/group” part, of the part2 of structured device ID. A device can perform bitwise AND operation between its device ID and the mask to check whether the bits [m, n] in its own ID are aligned with the binary pattern in the “mask/filter”, if yes, the device can consider being selected for the operation on the group which is represented by the mask pattern.

Observation 1b: It can be seen from SA2 study that the A-IoT device ID would have kind of structured design. Such structured design for A-IoT device ID will facilitate selecting a subset of devices by applying kind of mask/filter rules.

It is easy to understand that in more or usual cases, A-IoT service would involve operating on a group or a set of devices. But considering the low storage capabilities of devices, it is very unlikely to assign explicit group IDs to the devices in advance. Therefore, a more common way for the CN/AF to implicitly indicate a group of devices is to provide information of structured device ID and the corresponding “mask/filter”. Furthermore, according to Observation 1a, this information should be explicitly carried in A-IoT Paging message in A-IoT air interface, making it visible to the MAC layer of the device.

Observation 1c: In more or usual cases, A-IoT service would involve operating on a group or a set of devices. But due to low storage capabilities, it is very unlikely to assign explicit group IDs to the devices in advance. Therefore, a more common way for the CN/AF to implicitly indicate a group of devices is to provide information of structured device ID and the corresponding “mask/filter”.

During SI stage, there were some discussion regarding the need of end-to-end security protection on device ID information and also service data. However, till now, SA3 hasn’t achieved clear agreements on this aspect. Meanwhile, from RAN2 perspective, we see in many scenarios, the device ID information can be achieved by applying “mask/filter” on structured device ID. If the structure of the device ID is defined in 3GPP specification and can be visible in the A-IoT air interface, we think the security risk of directly including the “mask/filter” rules in the A-IoT air interface would be very negligible.

Observation 1d: SA3 hasn’t achieved clear agreements on whether device ID information needs end-to-end security protection. From RAN2 perspective, we see in many cases, the device ID information can be achieved by applying “mask/filter” on structured device ID. If the structure of the device ID is defined in 3GPP spec and can be visible in the A-IoT air interface, the security risk of directly including the “mask/filter” rules in the A-IoT air interface would be very negligible.

Based on the above observations, we give the following proposal:

Proposal 1a: It’s suggested to explicitly include the device ID information in A-IoT Paging message, with intention to make it visible to the device’s MAC entity.

Proposal 1b: In R19, it’s suggested to mainly support providing information of structured device ID and the corresponding “mask/filter” as the device ID information.

Generally, if the device ID information can be explicitly/visibly provided to the reader via a service request from CN/AF, the reader can just “copy” it into the A-IoT Paging message in the A-IoT air interface. However, According to the use cases discussed in SA1, it’s possible for the reader to receive a service request from CN/AF which aims to a large number of devices. Please note, when the CN/AF triggers service request for A-IoT operation, it’s generally according to a business requirement (e.g., to filter the devices belonging to a certain company, owner or business type etc.) while doesn’t need to consider the resources, capacity or signaling efficiency etc. factors in A-IoT air interface.

According to our analysis in previous meetings, such A-IoT service request aiming to a large number of devices may reach the scale of hundreds of thousands of devices for one time inventory. There may also be stringent time restriction for the entire inventory operation, e.g., to complete within hour level or even shorter timer period. It’s easy to understand such service request may cause heavy load and possible heavy collision during service procedure at the A-IoT air interface.

Observation 2: Although kind of upper layer mask/filter scheme may be supported, it’s still possible for BS reader to receive a service request, e.g., inventory, aiming to a very large number of devices. Such service request may cause heavy load and possible heavy collision for the following service procedure at the A-IoT air interface.

Therefore, we think to apply additional “mask/filter” scheme in the AS layer is needed with intention to reasonably split a CN/AF service request into several smaller service requests. Each smaller service request will trigger an A-IoT Paging procedure aiming to a smaller device group with a fewer number of devices. By this way, it can be helpful to reduce the collision probability within each Paging round and improves the overall efficiency at A-IoT air interface.

Proposal 1c: It’s suggested to support the application of additional AS layer “mask/filter” scheme on the device ID information from a CN/AF service request, with intention to reasonably split such CN/AF service request into several A-IoT Paging messages, and to have each of the A-IoT Paging address a fewer number of devices.

- 2.1.2 To trigger the random access (Q-selection algorithm)

According to the study progress during SID, there has been common understanding that slotted-ALOHA and Q-selection algorithm similar to that used in legacy RFID will be adopted as the basic schemes for the device to select access occasions and also for avoiding collision among devices during the random access process.

Q-selection algorithm is a general anti-collision way through time domain resources distribution. Shortly to say, even a device is filtered out via matching with the provided device ID information, it doesn’t means the device can response to the reader immediately. Instead, upon receiving a A-IoT Paging message which includes an initial Q value, the device can generate a random number, e.g., Qd, between [0~Qmax) , as its response time slot and loads this random slot number into a slot counter in its side. Here Qmax corresponds to maximum occasion value which is derived from the initial Q value provided in A-IoT Paging. Furthermore, only when the Qd equals to 0, the device is truly allowed to initiate access to the reader. For the device with slot counter (Qd) not equal to 0, it needs to wait for other R2D trigger from the reader to decrease its slot counter until it equals 0.

By doing above, it not only can resolve the issue of devices being unable to exactly determine access occasions due to lack of stable clock [5], but also can help to achieve the goal of reducing conflicts by distributing the time points when devices initiate random access through random Qd.

Proposal 2a: For facilitating the device to determine access occasions when having no stable clock and also for reducing conflicts by distributing the time points when devices initiate random access, kind of anti-collision scheme similar as Q-algorithm in RFID needs to be supported. Correspondingly, an initial Q value needs to be provided in A-IoT Paging message.

Moreover, the reader further needs to send some subsequent R2D (DL) trigger messages to reduce the slot counter inside the devices or even adjust the Q value. Different from the initial A-IoT Paging message, the R2D trigger message is mainly for reducing the slot counter in the devices which has been selected by the initial A-IoT Paging message. So these subsequent R2D trigger messages still can be seen within the current paging round and generally it has no need to provide device ID information in the R2D trigger message. The end of current paging round is generally when all devices in the group are successfully inventoried or the maximum time duration for inventory is reached (the devices which give no response can be marked as failed).

Furthermore, if the reader detects some collisions where multiple devices are responding at the same time, it can adjust the initial Q value, e.g., to provide a new Q value. Upon receiving this new Q value, all devices that haven’t responded will regenerate a new random slot number Qd. Usually, in the collision case, a larger Q value can be provided so that the device can regenerate Qd within a larger value range. On the contrary, it’s also possible that at some point the reader doesn't receive any response from the device. That means some time slots are unnecessary idle, the reader can provide a new Q value with a smaller value, to trigger the devices regenerate a new Qd that are more tightly distributed. It’s easy to understand that such new Q value can be provided in the R2D trigger message.

Proposal 2b: If Proposal 2a is agreed, R2D trigger message from the reader needs to be introduced for reducing the slot counter in device side or further providing an adjusted Q value.

Furthermore, considering that it’s possible some targeted devices may possibly miss the initial A-IoT Paging message as they may occasionally stop monitoring DL R2D channel due to low battery power, or they may fail to demodulate A-IoT Paging message due to bad channel conditions or long distance from the reader, Simply sending an initial A-IoT Paging message only once may not achieve the purpose of reaching all targeted devices. Therefore, in order to increase the probability of devices receiving the paging message, it can be considered to enhanced Paging transmission scheme, e.g., within a paging round, to insert a small number of additional A-IoT Paging messages carrying device ID information, or alternatively, the device ID information also can be put into some of the R2D trigger messages.

The following Figure 1 and 2 respectively illustrate the basic and enhanced Paging transmission schemes. It can be seen from the figure that, if additional A-IoT Paging message is inserted, it can just replace the R2D trigger on the corresponding location with the consideration that this Paging message is also allowed to reduce the Qd in the devices.

Figure 1. Basic scheme for Initial A-IoT Paging and R2D trigger transmission

Figure 2. Enhanced scheme for Initial A-IoT Paging and R2D trigger transmission

Proposal 2c: In order to increase the probability of devices receiving the paging message, it can be considered to transmit multiple A-IoT paging messages during a paging round, or alternatively, the device ID information also can be provided in some of R2D trigger messages.

Proposal 2d: If multiple A-IoT paging messages can be transmitted during a paging round, RAN2 can further discuss whether an indicator needs to be provided in the Paging message to indicate whether it is the initial A-IoT paging message of this paging round.

- 2.1.3 To provide D2R transmission resources

During SI stage, there was much discussion regarding the need for the reader to provide the TDMA and/or FDMA resources to the devices in the A-IoT Paging message. The device may need to choose a resource occasion from the configured TDMA and/or FDMA resources for performing D2R transmission in random access procedure.

According to the latest progress in RAN1 discussion, for the time domain resources, we understand it means X occasions after each R2D trigger (e.g., explicit R2D trigger message or A-IoT Paging message). In the following figures, examples are given for different X values:

Figure 1. Time-domain resources for X=1                                          Figure 2. Time-domain resources for X

According to further analysis from physical layer perspective in [4], company has observation that TDMA of Msg1 transmissions with X>1 cannot provide obvious gain in access efficiency compared to X=1. Therefore, it’s preferred not to support TDMA for Msg1 transmission, e.g., not support X>1 case.

Based on this analysis, from RAN2 perspective, we understand the device doesn’t needs to do explicit TDMA resources selection. The device mainly needs to determine the exact slot point for initiating the random access according to the Q-selection algorithm which is discussed in above section 2.1.2.

Proposal 3a: It’s no need to provide TDMA resources in A-IoT Paging message.

For FDMA resource selection, we understand RAN1 already agree to support that a device can select a frequency domain resource from a frequency domain resource set. And RAN1 also agree that the available frequency domain resource set should be specified by RAN1, including the number of frequency domain resources and their locations. Meanwhile, the indicator for the frequency domain resource set needs to be transmitted in a R2D transmission, e.g., the A-IoT Paging message.

Proposal 3b: The indication for frequency domain resource set needs to be provided in A-IoT Paging message.

- 2.1.4 To indicate delta paging for a same service request

During SI stage, RAN2 has agreed a basic case that reader is able to trigger multiple/subsequent A-IoT paging messages that are associated with the same request from CN/AF (Case-1). In this case, each A-IoT Paging message can correspond to a paging round. The main reason lies in that if there are a large number of target devices, due to some reasons, e.g., resource conflicts or occasional unreachability of some devices, it may not be possible to ensure that all devices have the opportunity to respond in a single paging round. Therefore, additional one or more paging rounds can be triggered for the same service request and same set of target devices, allowing those that did not have the chance to respond previously to complete their responses in subsequent paging rounds. Meanwhile, RAN2 also agree to study the scheme to avoid duplicated response from devices.

In a certain paging round, a targeted device may be able to find chance to trigger random access procedure, and it can determine whether it has accessed with successfully contention resolution via exchanging Msg1 and Msg2 with the reader. The device also can determine whether its Msg3 transmission has been successfully received. The device that has received ACK for Msg3 transmission can internally mark its status as “has been inventoried”, meanwhile, the device that determines the failure of Msg3 transmission according to the NACK feedback can internally mark its status as “has not been inventoried”.

Observation 3: In a certain paging round, a device can determine whether it has accessed with successfully contention resolution via exchanging Msg1 and Msg2 with the reader. The device can also determine whether its Msg3 transmission has been successfully received based on the explicit R2D failure/success feedback indication for Msg3 (if provided).

When another A-IoT Paging message is received, if the UE with status of “has been inventoried” can identify by kind of way that the newly received A-IoT Paging message is associated with the same service request as a previously responded Paging message (e.g., to identify a delta paging relative to the first paging), the device can skip this newly received Paging message, e.g., would not give duplicated response to a same service request. Meanwhile, for the device with status of “has not been inventoried”, it still needs to response to this newly received Paging message. Including an NDI (new data indication) indicator in the A-IoT Paging message(s) in the first paging round associated with a service request, or including a delta paging indicator in the A-IoT Paging message(s) in the subsequent paging rounds associated with the same service request, can assist the device in identify this delta paging case.

Proposal 4: For supporting multiple paging rounds triggered by a certain service request from CN/AF, the following alternatives can be discussed for avoiding duplicated response from devices:

- Alt 1: To include an NDI (new data indication) indicator in the A-IoT Paging message(s) in the first paging round associated with a service request

- Alt2: To include a delta paging indicator in the A-IoT Paging message(s) in the subsequent paging rounds associated with the same service request

- 2.1.5 The main contents in the DL MAC PDU for Paging procedure

In above section 2.1.1~2.1.4, we discuss the essential DL messages and also their contents in A-IoT Paging procedure. Also with reference to the discussion on MAC layer design in [6], we will give a brief discussion on the possible format and also the necessary information elements for the MAC PDU types related to A-IoT Paging procedure.

DL MAC PDU (Paging/MSG0)

The Paging MAC PDU needs to include at least the Message type field and the device ID information field which can be used by the device to determine whether it’s the targeted device. From RAN2 perspective, we assume this information needs to be addressed by the MAC entity, so that the MAC entity in device can know whether to process the following procedure.

Furthermore, the Paging MAC PDU may need to provide the Q-selection related parameters, e.g., the initial Q value and at least the FDMA resources for D2R transmission.

Based on the above considerations, the example design for Paging MAC PDU can be as following:

DL MAC PDU (R2D trigger)

The R2D trigger MAC PDU is used to trigger the device counting down its slot counter. By this way, the device can determine the exact occasion when it is allowed to transmit the MSG1. This also helps reduce conflicts between device accesses by implementing a Q-selection algorithm.

As this MAC signaling itself can be a trigger for counting down the slot counter, there may be no other necessary information to be provided to the device. So all other fields can be omitted.

According to the discussion in section 2.1.2, the Q-selection algorithm can be further enhanced. Therefore it may need to provide adjusted Q value. Such parameters can be optionally provide within the MAC PDU payload.

Based on the above discussion, the example design for R2D trigger MAC PDU can be as following:

Proposal 5: RAN2 discuss the example format for DL MAC PDUs for A-IoT Paging procedure.

2.2 Other contents in A-IoT Paging message

- 2.2.1 Necessity of session ID in Paging

In previous RAN2 meetings, some companies think a case needs to be discussed where the reader needs to concurrently handle multiple different service requests from CN/AF (Case-2). As a result, the reader can concurrently trigger several A-IoT Paging corresponding to different service requests.

As mentioned above, for Case-1, if a device confirms its Msg3 transmission has been received, it does not need to response the subsequent paging messages. However, if another paging message triggered by different service request from CN/AF can also reach to this device at the same time, the device may become confused on whether it needs to respond or it may just incorrectly ignore the “new” paging message. So in order that the device can differentiate these two paging messages, it’s proposed in some previous contributions that reader can also provide an ID (e.g., session ID) in the paging message to associate it with a certain CN service request. Shortly to say, even for a device with status of “has been inventoried”, if it receives another paging message and the session ID in this paging is different from the one indicated in a previous paging messages, the device would understand these two paging messages are associated with different requests from the CN and still needs to response to the latter paging message. But if the session ID is same as the one in previous paging messages, the device can still skip the latter paging message.

However, with the following considerations, we see not much necessity for supporting Case-2 in A-IoT air interface:

- Considering that the typical A-IoT operation is to inventory a device group or all devices within a certain area, we understand that the CN does not need to trigger multiple service requests in succession for the same device group or the same area within a short time period. Even if this occurs, it may be in abnormal scenario and just be a rare case. A more reasonable way for CN would be to wait for the results of the previous inventory request before deciding to initiate another inventory request. So we assume Case-2 is rare case and the reader will rarely receive multiple different CN service requests targeting the same batch of devices in a short time period.

Observation 4a: Considering that the typical A-IoT operation is to inventory a device group or all devices within a certain area, the CN does not need to trigger multiple service requests in succession for the same device group or the devices in the same area within a short time period.

- Even if the reader receives another service request from CN/AF before it finishes handling a previous one, it’s possible for the reader to handle this by its implementation, e.g., to hold the new CN service request until the previous request is completely processed. This can be seen as a serial processing method. There may be comments that serial processing on two CN requests might introduce additional delay for the devices that are targeted by the second request. However, if we consider from the resources perspective, in the serial processing case, the A-IoT paging/RA procedures for each CN service request can be allocated more resources and potentially requires less time to finish. When multiple CN services are processed in parallel, the A-IoT paging/RA procedures for each CN request may be allocated fewer resources, increasing the time required for completing the procedures for each CN request. Therefore, the total time for processing multiple CN service requests serially may not necessarily be significantly longer than the time required for processing multiple services in parallel.

Observation 4b: From reader perspective, if only one CN service request is processed at a time, more resources can be allocated to the A-IoT procedures of this CN service request. Therefore, the total time for processing multiple CN service requests serially may not necessarily be significantly longer than the time required for processing multiple CN service requests in parallel.

We have also observed the following complexities for the devices for supporting Case-2:

- If session ID is introduced in paging message, the device needs to store at most [Maximum number of concurrent CN service requests supported −1] session IDs, then the device can determine whether the newly received session ID is different from all previously received session IDs. This imposes additional storage overhead on the device, and the longer the session ID, the greater the overhead.

- It needs to further consider the value range of session ID and the number of bits required in paging message. Some companies thought that the CN may also allocate a session ID for each CN service request, and it can be directly used in the paging message in A-IoT air interface. However, considering that the CN may keep triggering services, the session ID assigned by the CN might continue to grow until it becomes very large, which would occupy too many bits if directly placed in the paging message. So this way is infeasible. If agreed to introduce, the session ID in the paging message should be (re)assigned by the reader, with consideration that only a limited number of different CN service requests need to be supported in A-IoT air interface in parallel. But even so, considering it’s possible that new CN service requests exceeding the maximum supported request number may temporarily arrive, if the length of session ID is set too short, there may occur session ID wrap-around issue, which may cause the device to incorrectly ignore the paging messages for new CN service request.

Observation 4c: If a session ID is to be introduced in paging to facilitate the device to identify a paging triggered by a different CN/AF service request, it will impose additional storage overhead on the device (multiple times of the total length of a session ID), and the longer the session ID, the greater the overhead. Moreover, if a session ID is too short, there may still be session ID wrap-around issue.

Based on the above observations, we give the following proposal:

Proposal 6: There is no need to introduce session ID (e.g., the ID corresponds to different service requests from CN/AF) in A-IoT Paging message.

- 2.2.2 Necessity of reader ID in paging

In previous meeting, there was some discussion on whether A-IoT devices need to differentiate multiple paging messages triggered by different readers. A corresponding case has been mentioned that “one CN A-IoT service request may be sent to multiple A-IoT readers”. Some companies suggested to introduce session ID and reader ID in the paging message to address this case. If a device receives two paging message with the same session ID but different reader IDs, the device can choose to respond to only one of the paging messages and ignore the other.

Firstly, RAN4 has discussed the suitable number of simultaneously active BS readers. It was observed that activating only a single reader can significantly improve the baseline SINR. And it’ required that the number of simultaneously active readers shall ensure the baseline SINR meets the target value. So RAN4 has achieved the following agreements:

Agreement in RAN4#112: - Minimum distance between active readers: 60m as baseline, other values can be reported by other companies. - 2 readers are activated in one drop as baseline

According to RAN4 agreement, it can be seen that in a certain area, at most two readers can perform A-IoT operations at the same time and they also need to keep a sufficiently large distance between them. Considering the very limited demodulation capability of the devices, even for devices located at the boundary between two readers, from the RAN2 perspective, it is very unlikely the device could simultaneously demodulate two paging messages. Therefore, it’s useless to let two BS readers to send the same paging message to the same device simultaneously.

Observation 5a: According to RAN4 agreement, at most two readers in a certain area can perform A-IoT operations at the same time and they need to keep a sufficiently large distance between them. Considering the very limited demodulation capability of the devices, it is very unlikely the device could demodulate two paging messages at the same time. Therefore, it’s useless to let two BS readers to send the same paging message to the same device simultaneously.

Secondly, based on the understanding of the A-IoT paging procedure, upon reception a CN service request, the reader will allocate A-IoT resources according to the scale of the targeted devices. It can also trigger multiple paging rounds and adjust the scheduling parameters to control the access of the devices based on the conflicts situation. So even if it can be assumed that the device can receive paging messages from two BS readers, and if a reader (e.g., reader-B) has triggered inventory for the device but the device can deliberately choose not to respond it (device may choose to respond another reader, reader-A), it will lead to a waste of this reader's resources, and the multiple paging rounds would be performed in vain. Moreover, the reader-B will also waste some time waiting for the device to respond which may result in a delay in some other services.

Observation 5b: Having two BS readers simultaneously perform the same A-IoT service for one device, but allowing the device to respond to only one of the readers, would result in resource wastage and service delay for the other reader.

Finally, we see no clear justification for CN to trigger multiple readers to perform a same service request at the same time as it will also cause additional signalling overhead in the interfaces between CN and reader. So we assume CN needs to avoid such case based on correct association among [CN node, reader, devices] which may be established in previous inventory procedure.

Based on above analysis, we give the following proposals:

Proposal 7a: RAN2 assume the case that a device receives multiple A-IoT Paging messages from different readers can be avoided and no need to address it.

However, in the following two scenarios, we see the necessity of introducing reader ID in paging message to address the potential issues:

- Scenario 1 (reader ID is needed in the paging to help device identify a new reader): According to the previous discussion on multiple paging rounds, we assume the device which have internally marked its status as “has been inventoried” could skip a paging message with a delta paging indication. However, for a device that has previously performed an inventory in another reader (old reader), when it moves to a new reader, as there are no legacy SIB messages to help it identify the new cell, it might continue to maintain its status and incorrectly ignore a paging message received in the new reader. In order to avoid this issue, it’s suggested to introduce a short and simple reader ID in the paging message, the device needs to record the reader ID of the reader where it was successfully inventoried in last time. Then if the device later receives a new paging with different reader ID, the device can clear the status and respond to this paging message as being inventoried for the first time.

Proposal 7b: It’s suggested to introduce a reader ID in the A-IoT Paging message.

- Scenario 2 (reader ID is needed in D2R message to help reader identify messages that do not aim to it): Also for the case that a device locates at the boundary between two readers, although we assume it’s very likely the device cannot simultaneously demodulate two paging messages from different readers due to the limited device capability, it still needs to consider the case that the response (just triggered by one paging from one reader, e.g., reader-A) sent by the device can be demodulated by both of the readers as they have higher demodulation capabilities. To address this case, it’s suggested that the device can include reader ID of the reader (e.g., reader-A) in the UL D2R messages and the reader-A is the one from which the device receives the paging messages. If the reader-B receives the Msg3, e.g., the UL D2R message containing device ID information, but identifies the reader ID included in Msg3 does not match its own reader ID, reader-B can choose to discard the received Msg3.

Proposal 7c: RAN2 discuss whether it’s needed to introduce a reader ID in the D2R messages, e.g., Msg3.

- Conclusion

According to above discussion for paging procedure in air interface, the following proposals are given:

Proposal 1a: It’s suggested to explicitly include the device ID information in A-IoT Paging message, with intention to make it visible to the device’s MAC entity.

Proposal 1b: In R19, it’s suggested to mainly support providing information of structured device ID and the corresponding “mask/filter” as the device ID information.

Proposal 1c: It’s suggested to support the application of additional AS layer “mask/filter” scheme on the device ID information from a CN/AF service request, with intention to reasonably split such CN/AF service request into several A-IoT Paging messages, and to have each of the A-IoT Paging address a fewer number of devices.

Proposal 2a: For facilitating the device to determine access occasions when having no stable clock and also for reducing conflicts by distributing the time points when devices initiate random access, kind of anti-collision scheme similar as Q-algorithm in RFID needs to be supported. Correspondingly, an initial Q value needs to be provided in A-IoT Paging message.

Proposal 2b: If Proposal 2a is agreed, R2D trigger message from the reader needs to be introduced for reducing the slot counter in device side or further providing an adjusted Q value.

Proposal 2c: In order to increase the probability of devices receiving the paging message, it can be considered to transmit multiple A-IoT paging messages during a paging round, or alternatively, the device ID information also can be provided in some of R2D trigger messages.

Proposal 2d: If multiple A-IoT paging messages can be transmitted during a paging round, RAN2 can further discuss whether an indicator needs to be provided in the Paging message to indicate whether it is the initial A-IoT paging message of this paging round.

Proposal 3a: It’s no need to provide TDMA resources in A-IoT Paging message.

Proposal 3b: The indication for frequency domain resource set needs to be provided in A-IoT Paging message.

Proposal 4: For supporting multiple paging rounds triggered by a certain service request from CN/AF, the following alternatives can be discussed for avoiding duplicated response from devices:

- Alt 1: To include an NDI (new data indication) indicator in the A-IoT Paging message(s) in the first paging round associated with a service request

- Alt2: To include a delta paging indicator in the A-IoT Paging message(s) in the subsequent paging rounds associated with the same service request

Proposal 5: RAN2 discuss the example format for DL MAC PDUs for A-IoT Paging procedure.

Proposal 6: There is no need to introduce session ID (e.g., the ID corresponds to different service requests from CN/AF) in A-IoT Paging message.

Proposal 7a: RAN2 assume the case that a device receives multiple A-IoT Paging messages from different readers can be avoided and no need to address it.

Proposal 7b: It’s suggested to introduce a reader ID in the A-IoT Paging message.

Proposal 7c: RAN2 discuss whether it’s needed to introduce a reader ID in the D2R messages, e.g., Msg3.

- References

- 3GPP TR 38.769 V2.0.1, Study on solutions for ambient IoT (Internet of Things) (Release 19), December, 2024

- 3GPP RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN Plenary#106, December, 2024

- TR 23.700-13 v1.3.0 (2025-01), Study on Architecture support of Ambient power-enabled Internet of Things (Release 19), January. 2025

- R1-2500127, Discussion on Ambient IoT multiple access and timing, ZTE Corporation, Sanechips, RAN1#120, February, 2025

- R2-2500778, Random access procedure for A-IoT, ZTE Corporation, Sanechips, RAN2#129, February, 2025

- R2-2500779, MAC layer design and other aspects for A-IoT, ZTE Corporation, Sanechips, RAN2#129, February, 2025