3GPP TSG-RAN WG2 Meeting #129							     R2-2500779

Athens, Greece, 17th – 21st February, 2025

Source: 	ZTE Corporation, Sanechips

Title: 	MAC layer design and other aspects for A-IoT

Agenda item:	8.2.4

Document for: 	Discussion and Decision

- Introduction

In this contribution we discuss the details of MAC procedures and PDU formats for A-IoT data transmission.

- General design for A-IoT MAC layer

  - Introduction

During SID study, RAN2 has made assumption that, on A-IoT interface between the device and the reader, there is A-IoT MAC layer only and no other L2 AS layer. Furthermore, there is no CP/UP protocol stack differentiation. In the following sections, we will discuss the main procedures supported by A-IoT MAC layer, and the associated MAC Protocol Data Units, formats and parameters.

  - MAC Entities

Given the requirement for simplicity of A-IoT device, RAN2 can assume that only one MAC entity is supported by the device which would not be configured by the reader and instead, the MAC entity can be initialized by the device itself upon power-on.

Proposal 1a: Only one MAC entity is supported by the A-IoT device which will not be configured by the reader but be initialized by the device itself upon power-up.

Based on the study progress from physical layer, it’s straightforward to have the following further proposal:

Proposal 1b: The MAC entity of the A-IoT device handles the following transport channels:

-	RDCH, the R2D Shared channel for transmission from reader to device;

-	DRCH, the D2R Shared channel for transmission from device to reader.

  - MAC services

For A-IoT device, the MAC layer is between the upper layer and the physical layer, so the MAC layer will provide services to upper layer and expect the services from the physical layer.

Since no other L2 AS layer would be supported, here the upper layer is NAS-like layer in RAN2 assumption. According to the SID study, we assume the data segmentation and ACK-NACK feedback for the transmission (in D2R direction) would be supported by the MAC layer itself, therefore no ACK-NACK feedback is expected from the physical layer.

The initial consideration on the services that the MAC layer can provide and obtain are as below.

Proposal 2:

The MAC layer provides the following services to upper layers:

- Data transfer services;

- Radio resource allocation

The MAC layer expects the following services from the physical layer:

- Data transfer services;

  - MAC functions

Since no other AS protocol layers, such as RRC, PDCP, and RLC is supported, all A-IoT functionality agreed during the study should be directly supported by MAC layer. In the following proposal we propose the MAC layer functionality based on the agreements reached during the study item.

Proposal 3: The MAC protocol supports at least the following functions:

- Connection control:

- Paging;

- Random access;

- Radio resource selection for D2R transmission;

- Data transmission from/to transport channels;

- Data segmentation, at least for D2R;

- Transmission error correction through ACK-NACK feedback.

The relevance of MAC functions for uplink, downlink is indicated in following Table 1

Table 1: The link direction association of MAC functions.

| MAC function | R2D | D2R |
| --- | --- | --- |
| Connection control-Paging | X |  |
| Connection control-Random access | X | X |
| Radio resource selection |  | X |
| Data transmission from/to transport channels | X | X |
| Data packet segmentation |  | X |
| Error correction through ACK-NACK scheme |  | X |

  - MAC procedures

    - Device state

It was agreed during the study item that from RAN2 perspective, the device is always monitoring PRDCH (as long as there is enough energy). However, the device should be receiving different types of R2D messages depending on various conditions. The following aspects need to be considered:

- Before paging: Before receiving a paging indication, the device should monitor PRDCH for paging messages. This is the only type of message that the device should be monitoring and responding to in this state.

- After paging and before MSG1 transmission: After the device is paged, the device should be monitoring PRDCH for broadcast messages that trigger transmission and retransmission of MSG1.

- After MSG1 transmission: Once MSG1 is transmitted, the device should monitor PRDCH for unicast messages addressed to the RN16 (i.e. messages including the RN16). The device should monitor the PRDCH using the ASID until the current paging round is completed. This may be until receiving an ACK for MSG3/MSGA in case of inventory procedure or until the command procedure is completed. Since there may be more than one R2D messages in case of command procedure after MSG4, an explicit indication is needed from the reader to indicate the completion of command procedure.

So, based the above, at any given time the device will be monitoring PRDCH for one of the following:

- Broadcast messages for paging

- Broadcast messages form MSG1 trigger/retrigger

- Unicast messages addressed to device’s ASID (RN16)

Proposal 4a: When the device has sufficient energy, the device shall monitor PRDCH to receive:

- broadcast messages before being paged

- broadcast messages for MSG1 triggering after receiving paging and before MSG1 transmission

- unicast messages including the ASID of the device once MSG1 is transmitted until the current paging round is completed

Proposal 4b: For inventory use case, the paging round is considered completed upon reception of an ACK for the MSG3/MSGA.

Proposal 4c: For command use case, the reader includes and explicit indication in R2D message to indicate the completion of the paging round.

    - Paging procedure

As mentioned in [2], the A-IoT paging functionality is to use A-IoT paging message to indicate device(s) that need to respond. Also as mentioned by the scope of WID, a paging message can contain one identifier or contain no identifier which generally means that the paging message targets all the devices. Furthermore, the design of paging message format should allow that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

The paging procedure is triggered by a DL Paging MAC signaling received on the PRDCH channel. The following two flow chats can roughly describe the device behaviour during the Paging procedure, with corresponding status transition in the device.

- The procedure starts when the device has powered on and stand by in the status "Before Paging"

- When the device receives a DL R2D MAC PDU, the device checks whether the received MAC PDU is a Paging MAC PDU:

    - If No: The process loops back to the initial status, waiting for another DL R2D MAC PDU.

    - If Yes: The process continues to the next step.

- The device checks whether it is the targeted device for the paging message according to the device identifier information provided in the Paging MAC PDU:

    - If No: The process loops back to the initial status, waiting for another DL R2D MAC PDU.

    - If Yes: The process continues to the next step.

- If the device is the targeted device, it generates a Qd value between 0 and Qmax or based on the TDMA resources. This Qd is used as the length of a slot counter with the device.

- The device checks whether Qd is equal to 0:

    - If No: the device stores the Qd value and uses it as the length of a slot counter with the device. And then the device transits to the status of “After Paging”.

    - If Yes: The process continues to the next step.

- The device performs FDMA resource selection (if needed) and send MSG1.

- The device transits to the status of “After MSG1”.

If the device cannot initiate the random access procedure upon receiving the Paging MAC PDU, the device needs to wait for the R2D trigger MAC PDU for counting down its slot counter. The following procedure can be performed:

- The procedure starts when the device stand by in the status "After Paging"

- When the device receives a DL R2D MAC PDU, the device checks whether the received MAC PDU is a R2D trigger MAC PDU:

    - If No: The process loops back to the initial status, waiting for another DL R2D MAC PDU.

    - If Yes: The process continues to the next step.

- If the received MAC PDU is a R2D trigger MAC PDU, the device decrements the Qd slot counter by 1 to achieve a new Qd value, e.g., Qd’.

- The device checks whether Qd’ is equal to 0:

    - If No: The process loops back to the initial status, waiting for another DL R2D MAC PDU.

    - If Yes: The process continues to the next step.

- The device performs FDMA resource selection (if needed) and send MSG1.

- The device transits to the status of “After MSG1”.

    - Random Access procedure

Upon sending MSG1, the device initiates the random access procedure. The following procedure can be performed:

- The procedure starts when the device stand by in the status "After MSG1"

- When the device receives a DL R2D MAC PDU, the device checks whether the received MAC PDU is a MSG2 MAC PDU:

    - If No: The process loops back to the initial status, waiting for another DL R2D MAC PDU.

    - If Yes: The process continues to the next step.

- The device checks whether the RN16 contained in MSG2 is equal to the RN16 stored after it sends MSG1:

    - If No: The process loops back to the initial status, waiting for another DL R2D MAC PDU.

    - If Yes: The process continues to the next step.

- The device sends MSG3 on the D2R resource indicated in MSG2.

- The device checks whether there will be subsequent DL/UL data according to some ways:

    - If No: The device transits to the status of “Before Paging”

    - If Yes: The device transits to the status of “After MSG3” to handle further DL/UL data.

  - MAC PDU formats and parameters

    - Background in 5G NR MAC spec

Before giving analysis on MAC PDU format and parameter for A-IoT, we have the following observations for 5G NR MAC spec:

- Observation 1a for MAC PDU:

  - To match the key functionalities of the MAC layer of 5G NR, several types of MAC PDUs are defined. The main types include two: one is the general MAC PDU that carry data, as well as DL control information and UL UE assistance information over different logical channels in DL-SCH and UL-SCH. The other is the MAC PDU with specific functions for transmitting MSG2/MSGB.

- Observation 1b MAC subheader:

  - The MAC subheader is a very important design in the existing MAC PDU for 5G NR. Multiple subheaders can be included in MAC PDU as guide for the MAC payloads that follow (e.g., MAC SDUs or MAC CEs for general MAC PDU, or RAR for RAR MAC PDU, or fallbackRAR/successRAR for MSGB MAC PDU). Since different MAC payloads may need to be carried during each transmission, and their lengths vary, it’s beneficial to provide MAC subheaders in advance to indicate the necessary format information of the following MAC payloads to the peer node. Such a design also offers flexibility and scalability. However, the disadvantage of having such MAC subheader is that it increases complexity and also that the subheader may contain unused redundant bits (to achieve byte aligned in length), leading to unnecessary signaling overhead.

  - Moreover, this design can also achieve the effect of multiplexing MAC subPDUs. For example, in DL, the RARs for several different UEs can be multiplexed within a RAR MAC PDU. Or in UL, the assistant information reported by UE (e.g., PHR MAC CE) and also the service SDU from this UE can be multiplexed within a UL-SCH MAC PDU.

- Observation 1c for MAC CE:

  - There are many various MAC CEs defined in MAC spec. By these MAC CE, certain control information from the network or status/assistant information reported from the UE can be transmitted in a signaling efficient way (e.g., compared with transmitting information via RRC). Another advantage of this design is its flexibility, meaning that, when it’s needed, the same MAC CE can be included in different MAC PDUs along with different MAC SDUs.

  - However, the cons of this flexibility is that it necessitates the specific type IDs, e.g., LCID or eLCID, leading to additional signaling overhead. As more MAC CEs are defined, the value of LCID/eLCID for the new MAC CE is increased, and the bit space required for these type IDs may also gradually expand.

    - General principles for MAC layer design for A-IoT

Based on the above discussion on the main MAC procedures and also the reference from 5G NR MAC spec, we need to discuss some general design principles for MAC PDU and its format for A-IoT.

In legacy 5G NR, a MAC PDU is a bit string that is byte aligned in length. For A-IoT, due to backscattering transmission in UL, the message size cannot be large. Therefore, we need to avoid unimportant or unnecessary overhead as much as possible for A-IoT MAC PDU. Generally, it’s preferred that the MAC PDU for A-IoT no longer needs to be byte aligned and can be variable.

Proposal 5: RAN2 discuss a newly designed MAC PDU for A-IoT. Such MAC PDU for A-IoT no longer needs to be byte aligned and can be with variable length.

According to the discussion for MAC functions in section 2.4 and the corresponding procedures in section 2.5, several DL or UL signaling will be needed for different functionalities. For example, we may need Paging and R2D trigger signaling for the Paging function, MSG1, MSG2, MSG3 for the random access function and further UL or DL data signaling for data transmission function.

Furthermore, based on the agreement that only MAC layer and physical layer will be defined in AS layer protocol stack, all the assumed signaling needs to be defined in MAC layer. This may be a difference from that in 5G NR. In 5G NR, only part of functionalities related to connection control/data transmission need to be supported in MAC layer, while for A-IoT, all the functionalities need to be supported in MAC layer. For example, we need to define the MAC signaling for Paging, which is implemented in RRC layer in 5G NR.

Taking the MAC layer design in 5G NR as reference and also considering the Observation 1a, we suggest to also define different types of MAC PDU for different functionalities. Therefore, the following high-level proposal is given:

Proposal 6a: For A-IoT, it’s suggested to support different types of MAC PDU for different MAC functions, at least the following MAC PDUs can be considered:

- DL MAC PDU (Paging/MSG0)

- DL MAC PDU (R2D trigger)

- UL MAC PDU (MSG1)

- DL MAC PDU (MSG2)

- UL MAC PDU (MSG3)

- DL MAC PDU (DL data)

- UL MAC PDU (UL data)

There may be several alternatives to implement proposal 6a. The following proposals are given.

Proposal 6b: RAN2 discuss the following alternatives for supporting different types of MAC PDU:

- Alt1: To define a common MAC PDU format for both DL(PRDCH) and UL(PDRCH) transmission with a Message type indication to differentiate the MAC PDUs for different functionalities;

- Alt2: To define a DL MAC PDU format and a UL MAC PDU format, and for each of them, to further use a Message type indication to differentiate the MAC PDUs for different functionalities;

- Alt3: To define separate MAC PDU formats for different functionalities where the MAC PDU itself can be an implicit Message type indication.

No matter which alternative is chosen, it’s straightforward to define an explicit or implicit Message type indication for differentiating the MAC PDUs for different functionalities.

Proposal 6c: An explicit or implicit Message type indication can be defined in MAC PDU for differentiating the MAC PDUs for different functionalities.

From the perspective of procedure, it’s clear that the devices in different status would expect different MAC signaling. For example, for a device that is just power-on, it would expect the broadcast Paging MAC signaling. Meanwhile, for a device that has already been in RA procedure, it would expect the unicast MAC signaling. So another indication about cast type may give further help to the device, enabling it to stop processing an unexpected MAC signaling as early as possible. Note that this only allows the device to exit the message processing procedure earlier but does not save the power consumption associated with receiving the entire MAC signaling. This is because the device anyway needs to receive the entire bit stream of the MAC signaling and perform CRC check before parsing or processing it.

Proposal 6d: RAN2 can further discuss whether a cast type indication is needed with intention to enable the device to stop processing an unexpected MAC signaling as early as possible.

Another related discussion is whether transmitter identification and receiver identification of the MAC PDU are needed. We think such requirement may come from other protocol layer, e.g., physical layer. So this can be discussed later based on the progress of general design of MAC PDU and the input requirement from other working groups.

Proposal 6e: RAN2 can discuss later whether the transmitter identification and receiver identification of the MAC PDU are needed based on the progress of general design of MAC PDU and the input requirement from other working groups.

Another aspect to consider is whether to have the similar design of subPDUs within each MAC PDU as that in 5G NR MAC layer, where each MAC subPDU can consist of a MAC subheader and a MAC payload. As mentioned in Observation 1b, the design of MAC subheader can achieve the flexibility and scalability, and also the possibility of multiplexing several MAC payloads. Meanwhile, it also has its cons, e.g., complexity and overhead issue. RAN2 can make such design as a candidate option (Alt1) for further discussion. General examples for Alt1 can be as following:

(a)

(b)

Figure 1: Alt1 for MAC PDU format

Given the very low processing capability of the A-IoT device when relying on environmental power, the challenges of receiving or transmitting long MAC PDUs is obvious. Therefore, the feasibility and benefits of multiplexing multiple subPDUs within a single MAC PDU are doubtful. Further, there is minimal demand for combining MAC signaling for multiple devices in the downlink. In the uplink, it is recommended to transmit individual signaling messages rather than aggregating multiple signals. Therefore, another simple Alt2 as in the following Figure 2 can be considered, that is, to define a MAC PDU format that consists of a MAC header and MAC content/MAC payload. The MAC header would include the mandatory information to facilitate the device to address this MAC PDU and/or the necessary format indication of the following MAC PDU payload.

Figure 2: Alt2 for MAC PDU format

It’s easy to understand that the format in Alt2 would have less flexibility than that of Alt1. For example, the “length” field in the MAC PDU header of a DL MAC PDU can be used to inform the device how long or how many fields following this “length” field need to be sequentially parsed. By this way and also with an implicit assumption that the fields following the “length” field need to have fixed length, the device can correctly parse specific fields in the MAC PDU header one by one.

Proposal 7: RAN2 discuss the following alternatives for MAC PDU format:

- Alt1: to define a MAC PDU format that consists of one or more MAC subPDUs. And each MAC subPDU consists of a MAC subheader and a MAC payload.

- Alt2: to define a MAC PDU format that consists of a MAC header and MAC content/MAC payload.

- Other detailed aspects

  - ASID

During the study item, a number of options were studied for the ASID.

- RN16

- Reader assigned ID

In case of inventory procedure there is no real need for a separate ASID being assigned by the reader since the procedure anyway terminates after MSG4/MSGB. Even if a new ASID is assigned, the MSG2 anyway needs to include RN16 (to acknowledge MSG1). Thus, reader assigned ASID doesn’t really suit of benefit the inventory procedure.

In order to minimise device complexity, it is better to have a unified procedure for both inventory and command use cases. Given the above, we think there is really no real need to have a reader assigned ASID and reusing RN16 as ASID is simple and works well for both inventory and command use cases.

Observation 2a: reader assigned ASID doesn’t benefit inventory procedure

Observation 2b: having a reader assigned ASID just for optimising the command use case is not essential and results in unnecessary complexity for the device

Based on the above, we propose the following.

Proposal 8: RN16 included in MSG1 is used as the device ID during the current paging round

  - Segmentation and retransmissions

    - For D2R transmissions

It was agreed during the study item that segmentation is beneficial in D2R direction and when segmentation is supported, it should also be possible for the reader to schedule retransmission of segments.

In general, for acknowledged data transfer, a window size is maintained at the transmitter and receiver and the transmitter waits for ack/nack indication for the data transmitted within this window size. The simplest protocol that would implement acknowledged data transfer would use a window size of (i.e. stop and wait protocol). The advantage of this is that the number of bits needed for ack/nack are minimised and the device complexity is also low. Based on this, we propose a stop and wait protocol of window size 1 for D2R transmissions. This is depicted in Figure 3 below.

Figure 3: UP protocol for D2R transmissions

Based on the above, we propose the following:

Proposal 9a: Feedback for D2R data (ACK/NACK//NDI- details TBD) is supported and the feedback can be included in R2D message (from MSG2 onwards) along with the grant (TB size).

Proposal 9b: If ACK is received, the device transmits the next segment of data from upper layers (size of the segment is the TB size in the latest grant).

Proposal 9c: If NACK is received, the device retransmits the upper layer bits starting from the beginning of the previous segment (again size of the segment is the TB size in the latest grant).

    - For R2D transmissions

As far as segmentation is concerned, D2R and R2D are different because the R2D message may be broadcast (e.g. paging message) or unicast (MSG2 onwards) unlike a D2R message which is always unicast (i.e. from a device to the reader). The question is then whether it is feasible to segment all these messages and if so, what mechanisms are used to do so.

Segmenting broadcast/multicast messages comes with additional complexity from the protocol perspective since the segments need to be addressed to a group of devices and all the devices need to store all these segments even if the data is not necessarily for that device. This is really needed only if for instance a large number of individual device IDs are included in the paging message or if the paging message contains one-to-one mapping of paging IDs to MSG1 resources (e.g. CFRA) for a large number of devices. However, both these cases are optional and can be up to the implementation of the reader. A reader can potentially split the paging procedure into multiple rounds instead and include a subset of paging IDs/CFRA resources in individual paging rounds. So, segmentation of the paging message (i.e. MSG0) is not needed.

Observation 3: Paging message is broadcast and supporting segmentation for broadcast messages is more complex and is not needed as the reader can split the device IDs into multiple paging rounds and/or minimise the CFRA resources if the paging message exceeds the maximum R2D TB size

Proposal 10a: Segmentation is not supported for paging message (i.e. MSG0).

Then, the question is whether segmentation is needed for other R2D messages (e.g. command). This depends on the upper layer payload size. If the upper layer payload size in R2D direction can exceed the maximum R2D payload size, then segmentation may be needed. So, this can be left until SA2 responds to our earlier LS.

Proposal 10b: Wait for SA2 input before deciding whether unicast R2D messages (i.e. MSG2 onwards) can be segmented.

Unlike the D2R direction, supporting similar functionality for ACK/NACK in R2D direction is a lot more complex. The issue is that ACK/NACK for R2D messages need to be generated by the device and generating this and transmitting ACK/NACK for every R2D message/segment may be complex and would need additional functionality at the device side, which may not be practical in simple tags/devices.

Proposal 10c: ACK/NACK is not supported for R2D data.

- Conclusion and proposals

Based on the above analysis, the following proposals are given for MAC layer design and some other aspects for A-IoT.

Proposal 1a: Only one MAC entity is supported by the A-IoT device which will not be configured by the reader but be initialized by the device itself upon power-up.

Proposal 1b: The MAC entity of the A-IoT device handles the following transport channels:

-	RDCH, the R2D Shared channel for transmission from reader to device;

-	DRCH, the D2R Shared channel for transmission from device to reader.

Proposal 2:

The MAC layer provides the following services to upper layers:

- Data transfer services;

- Radio resource allocation

The MAC layer expects the following services from the physical layer:

- Data transfer services;

Proposal 3: The MAC protocol supports at least the following functions:

- Connection control:

- Paging;

- Random access;

- Radio resource selection for D2R transmission;

- Data transmission from/to transport channels;

- Data segmentation, at least for D2R;

- Transmission error correction through ACK-NACK feedback.

Proposal 4a: When the device has sufficient energy, the device shall monitor PRDCH to receive:

- broadcast messages before being paged

- broadcast messages for MSG1 triggering after receiving paging and before MSG1 transmission

- unicast messages including the ASID of the device once MSG1 is transmitted until the current paging round is completed

Proposal 4b: For inventory use case, the paging round is considered completed upon reception of an ACK for the MSG3/MSGA.

Proposal 4c: For command use case, the reader includes and explicit indication in R2D message to indicate the completion of the paging round.

Proposal 5: RAN2 discuss a newly designed MAC PDU for A-IoT. Such MAC PDU for A-IoT no longer needs to be byte aligned and can be with variable length.

Proposal 6a: For A-IoT, it’s suggested to support different types of MAC PDU for different MAC functions, at least the following MAC PDUs can be considered:

- DL MAC PDU (Paging/MSG0)

- DL MAC PDU (R2D trigger)

- UL MAC PDU (MSG1)

- DL MAC PDU (MSG2)

- UL MAC PDU (MSG3)

- DL MAC PDU (DL data)

- UL MAC PDU (UL data)

Proposal 6b: RAN2 discuss the following alternatives for supporting different types of MAC PDU:

- Alt1: To define a common MAC PDU format for both DL(PRDCH) and UL(PDRCH) transmission with a Message type indication to differentiate the MAC PDUs for different functionalities;

- Alt2: To define a DL MAC PDU format and a UL MAC PDU format, and for each of them, to further use a Message type indication to differentiate the MAC PDUs for different functionalities;

- Alt3: To define separate MAC PDU formats for different functionalities where the MAC PDU itself can be an implicit Message type indication.

Proposal 6c: An explicit or implicit Message type indication can be defined in MAC PDU for differentiating the MAC PDUs for different functionalities.

Proposal 6d: RAN2 can further discuss whether a cast type indication is needed with intention to enable the device to stop processing an unexpected MAC signaling as early as possible.

Proposal 6e: RAN2 can discuss later whether the transmitter identification and receiver identification of the MAC PDU are needed based on the progress of general design of MAC PDU and the input requirement from other working groups.

Proposal 7: RAN2 discuss the following alternatives for MAC PDU format:

- Alt1: to define a MAC PDU format that consists of one or more MAC subPDUs. And each MAC subPDU consists of a MAC subheader and a MAC payload.

- Alt2: to define a MAC PDU format that consists of a MAC header and MAC content/MAC payload.

Proposal 8: RN16 included in MSG1 is used as the device ID during the current paging round

Proposal 9a: Feedback for D2R data (ACK/NACK//NDI- details TBD) is supported and the feedback can be included in R2D message (from MSG2 onwards) along with the grant (TB size).

Proposal 9b: If ACK is received, the device transmits the next segment of data from upper layers (size of the segment is the TB size in the latest grant).

Proposal 9c: If NACK is received, the device retransmits the upper layer bits starting from the beginning of the previous segment (again size of the segment is the TB size in the latest grant).

Proposal 10a: Segmentation is not supported for paging message (i.e. MSG0).

Proposal 10b: Wait for SA2 input before deciding whether unicast R2D messages (i.e. MSG2 onwards) can be segmented.

Proposal 10c: ACK/NACK is not supported for R2D data.

- References

[1] 3GPP TR 38.769 V2.0.1, Study on solutions for ambient IoT (Internet of Things) (Release 19), December, 2024

[2] 3GPP RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN Plenary#106, December, 2024