3GPP TSG-RAN2#129									          R2-2500782

Athens, Greece, Feb. 17th – 21st, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	LCP enhancements for XR

Agenda item:	8.7.4.1

Document for: 	Discussion and Decision

# Introduction

Rel-19 XR WI[1] has an objective for specify additional Logical Channel priority handling using delay/deadline information of packets to enable high XR capacity while meeting delay requirements/avoiding too late PDUs.

In RAN2#126 meeting, the following related agreements has been made [2]:

- LCP enhancements – LCH prioritization - Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH - We try to avoid RAN1 impacts. - RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. - For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial - The solutions should not disallow non-delay critical data from using an UL grant. - LCP prioritization within a logical channel will not be considered in RAN2 discussions - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

In RAN2#127 meeting, the following related agreements has been made [3]:

- LCP Enhancement - RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. - FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3).

In RAN2#127bis meeting, the following related agreements has been made [4]:

- As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold. For now, we will use “LCH priority-adjusted data”.

In RAN2#128 meeting, the following agreements have been made[5]:

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

In this contribution we will further investigate the possible scheduling enhancements per this objective.

# Discussion

It has been agreed to introduce an independent per-LCH remaining time threshold for applying delay-critical priority in RAN2, and LCP prioritization within a logical channel will not be considered in RAN2 discussions, based on the threshold, “LCH priority-adjusted data” can be determined per LCH, e.g. if there is data in one logical channel with remaining time less than or equal to the threshold, all the data in the logical channel will be prioritized for transmission, and the data in the logical channel will be called “LCH priority-adjusted data”.

Proposal 1: RAN2 confirms that “LCH priority-adjusted data” is all data in a logical channel having data with remaining time less than or equal to the remaining time threshold configured for LCP.

When studying modifications to LCP, it is important to ensure that this doesn’t result in unforeseen consequences to other services. Specifically, it should be noted that LCP is used also to ensure that SRBs are prioritised over DRBs and certain MAC CEs are sent with higher priority than data. It should be kept in mind that during poor radio conditions there may be RAN level signalling data and/or MAC CEs which may be triggered (e.g. measurements/handover signalling etc). Allowing this signalling to go through unimpeded is important because this can resolve the poor radio conditions which actually cause the problems in the first place. Any delay to such signalling will prolong the adverse conditions at radio level and will only have overall negative consequences. In order to avoid impacts to signalling connection between UE and network it is important to ensure that any enhancements to LCP do not impact the priorities of SRBs over DRBs

Observation 1: During temporary deterioration of radio conditions, additional RAN level signalling may be generated (e.g. measurements/HO signalling) and this signalling is essential to quickly alleviate the problems in the first place.

In RAN2#127 meeting, it has been agreed that:

- RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. - FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3).

To improve the transmission for LCH priority-adjusted data, enhancements can be considered for the resource allocation stage of LCP: e.g. the priority of the logical channel having LCH priority-adjusted data is treated to be higher than the priority of the logical channel without LCH priority-adjusted data, and then allocate the resource based on LCH priority.

Proposal 2: At the stage for allocation of resources in LCP, treat the priority LCH having “LCH priority-adjusted data” (e.g. the data with remaining time below the remaining time threshold) is higher than the priority of the logical channel without “LCH priority-adjusted data”; and then allocate the resource based on LCH priority.

# Conclusion and proposals

The following proposals are made:

Proposal 1: RAN2 confirms that “LCH priority-adjusted data” is all data in a logical channel having data with remaining time less than or equal to the remaining time threshold.

Proposal 2: At the stage for allocation of resources in LCP, treat the priority of LCH having “LCH priority-adjusted data” (e.g. the data with the remaining time below the remaining time threshold) is higher than the priority of the logical channel without “LCH priority-adjusted data”; and then allocate the resource based on LCH priority.

# References

- RP-243318, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#106

- Draft_R2_126_Meeting_Report_v2

- Draft_RAN2_127_Meeting_Report_v1

- Draft_RAN2_127b_Meeting_Report_v1

- Draft_RAN2_128_Meeting_Report_v1

- 3GPP TS 38.321, V18.4.0 (2024-12), NR;Medium Access Control (MAC) protocol specification.