3GPP TSG-RAN2#129	R2-2500794

Athens, Greece, Feb 17-21, 2025

Agenda item:		8.7.6

Source:		Lenovo

Title:		 XR Rate Control

Document for:		Discussion and Decision

- Introduction

For XR traffic, it would be beneficial to enable the application to adapt the codec rate based on the network conditions to avoid dropping of packets over the air interface. Such discussion has been ongoing in the RAN Plenary with the following being agreed as the way forward in the last meeting, RAN #105 [1].

- Specify in MAC layer XR rate control signaling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

Additionally, the following agreements were made at the RAN2 #127-bis and RAN2 #128 meetings [2], as follows:

Agreements from RAN2 #127-bis 1. FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. 2. RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - Extend the Bit Rate field - Define a new bit rate table to provide sufficient granularity for XR traffic - Introduce new values for the bitRateMultiplier 3. Send LS to SA4 asking about range/granularity which is required. Agreements from RAN2 #128 1. RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. 2. gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN. 3. Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. 4. RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. 5. RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE. 6. FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

This contribution looks at how the XR rate control information may be signaled in the downlink to support source rate adaption to uplink congestion.

- Discussion

Currently, there already exists a mechanism in NR for bitrate control on a per LCH basis. The Recommended Bitrate (RBR) MAC CE can be used by the gNB to indicate a new bitrate to the UE to combat for uplink congestion or other changes in network conditions that may impact the data transmission. This MAC CE is a 2-byte one with 6-bits for Logical Channel ID (LCID) and 6-bits for the indication of the recommended bitrate. The bitrate value is indicated by means of an index and a bitRateMultiplier component [3]. The index points to a bitrate value ranging from 0 to 8000kbps and the bitRateMultiplier can take values 1, 40, 70, 100 and 200 making the total bitrate value ranging from 0 to 1.6Gbps.

- The Recommended Bitrate MAC CE can indicate bitrate values ranging from 0 to 1.6Gbps.

For XR applications that have very strict requirements in terms of data rate and latencies, it is beneficial for the application to be able to adapt the source rates based on the congestion and network conditions on the uplink. During the plenary meetings, many companies have been discussing whether the Recommended Bitrate MAC CE can be reused for XR applications. In our view, the existing Recommended Bitrate MAC CE may pose certain inefficiencies with respect to XR:

- Granularity:

Currently the Recommended Bitrate MAC CE is signalled on a per LCH basis, i.e., on a per DRB basis. In the case of XR, multiple QoS flows may be mapped to the same DRB. Upon receiving a RBR MAC CE, the UE cannot differentiate the bitrate for each QoS flow which can impact the upper layer throughput.

- Bitrate Range:

Although the RBR MAC CE can cover up to 1.6Gbps, the granularity of the values may not be sufficient for XR applications. With only 56 indices and a single multiplier, the recommended bitrate value may not accurately match the bitrate required by the application which can degrade the user-experience.

- The UE cannot differentiate the bitrate for each QoS flow of a DRB from the RBR MAC CE.

- The granularity of bitrate values of the legacy RBR MAC CE may not be sufficient for XR applications.

In the previous meeting, RAN2 had concluded that a UL rate indication from gNB to UE on a per QoS flow basis is supported. RAN2 had further proposed two options – whether to indicate the UL rate recommendation in an enhanced MAC CE at QoS flow level, or to reuse the existing RBR MAC CE format but with a RRC configuration to indicate the adaptable QoS flows. In our view, the exact benefit of providing bitrate recommendations at a per QoS flow level still remains unclear. Currently, the Access Network Bitrate Recommendation (ANBR) feature is an optional one wherein the access network can inform a MTSI client about a change in the recommended bitrate by means of an ANBR message. It is also understood that a single access bearer can carry multiple RTP streams, in which case ANBR applies to the sum of the individual RTP stream bitrates on that bearer [4]. Since the application is best capable of choosing the right flow to adapt, we think that the existing RBR MAC CE format on a per LCH basis is sufficient for handling UL rate indication. Since the discussion is still on going in RAN2 and since feedback has not yet been provided by the other working groups regarding the conclusions made in RAN2, we think it is best to postpone the discussion for how the rate indication is to be signalled.

RAN2 to postpone the discussion on how UL rate indication is signalled until a response is received from SA4/ RAN3.

Additionally, SA4 has provided a response regarding the range and granularity of UL bitrates that need to be supported. The range varies from 100kbps to 40Mbps whereas a granularity of 1% is preferred from the application perspective although a coarser granularity of 10% is also sufficient. According to the MAC specs, the current table for bitrate recommendation values provides sufficient range for XR applications but does not provide sufficient granularity.

In the previous RAN2 meetings, three approaches were concluded for provided UL bitrate recommendations:

- Extend the Bitrate Field:

With this approach, the current bitrate table can be extended to include more code points. Since SA4 has confirmed that the range of appropriate bitrate values for XR applications is already covered by the existing table, this approach seems redundant.

- Define a new Bitrate Table:

Designing a new base table with appropriate step sizes catered to XR applications can provide the network with full flexibity while enabling UL bitrate recommendations to the UE. The new table can be based on the range and granularity recommendation provided by SA4 to be well suited for XR codec rates. Refer Section 5.

- Introduce new values for bitRateMultiplier:

In order to provide bitrate recommendation values with sufficient range and granularity, the bitRateMultiplier may need to be frequently reconfigured. Such frequent reconfigurations of the bitRateMultiplier can lead to a lot of signalling overhead which may aggravate the UL congestion rather than alleviate it.

RAN2 to define a new bitrate table to provide sufficient granularity for XR traffic.

- Conclusion

- Observation 1: The Recommended Bitrate MAC CE can indicate bitrate values ranging from 0 to 1.6Gbps.

- Observation 2: The UE cannot differentiate the bitrate for each QoS flow of a DRB from the RBR MAC CE.

- Observation 3: The granularity of bitrate values of the legacy RBR MAC CE may not be sufficient for XR applications.

- Proposal 1: RAN2 to postpone the discussion on how UL rate indication is signalled until a response is received from SA4/ RAN3.

- Proposal 2: RAN2 to define a new bitrate table to provide sufficient granularity for XR traffic.

- References

[1] RP-242390: Way Forward on WID Scope Revision, Nokia

[2] RP-242517: Status Report for XR Work Item

[3] 3GPP TS 38.321: Medium Access Control (MAC) protocol specification.

[4] 3GPP TS 26.114: Group Services and System Aspects, IMS, Multimedia and Telephony, Media handling and interaction

- Annex

In our view, the new table needs to have a range of 100kbps to 40Mbps and a step size of at least 10%. Based on the following conclusions, we have formulated the new bitrate table as follows:

| Index | NR Recommended Bitrate value (kbps) | Index | NR Recommended Bitrate value (kbps) | Index | NR Recommended Bitrate value (kbps) |
| --- | --- | --- | --- | --- | --- |
| 0 | 100 | 23 | 895.4302433≈895 | 46 | 8017.953205≈8018 |
| 1 | 110 | 24 | 984.9732676≈985 | 47 | 8819.748526≈8820 |
| 2 | 121 | 25 | 1083.470594≈1083 | 48 | 9701.723378≈9702 |
| 3 | 133.1≈133 | 26 | 1191.817654≈1192 | 49 | 10671.89572≈10672 |
| 4 | 146.41≈146 | 27 | 1310.999419≈1311 | 50 | 11739.08529≈11739 |
| 5 | 161.051≈161 | 28 | 1442.099361≈1442 | 51 | 12912.99382≈12913 |
| 6 | 177.1561≈177 | 29 | 1586.309297≈1586 | 52 | 14204.2932≈14204 |
| 7 | 194.87171≈195 | 30 | 1744.940227≈1745 | 53 | 15624.72252≈15625 |
| 8 | 214.358881≈214 | 31 | 1919.43425≈1919 | 54 | 17187.19477≈17187 |
| 9 | 235.7947691≈236 | 32 | 2111.377675≈2111 | 55 | 18905.91425≈18906 |
| 10 | 259.374246≈259 | 33 | 2322.515442≈2323 | 56 | 20796.50567≈20797 |
| 11 | 285.3116706≈285 | 34 | 2554.766986≈2555 | 57 | 22876.15624≈22876 |
| 12 | 313.8428377≈314 | 35 | 2810.243685≈2810 | 58 | 25163.77186≈25164 |
| 13 | 345.2271214≈345 | 36 | 3091.268053≈3091 | 59 | 27680.14905≈27680 |
| 14 | 379.7498336≈380 | 37 | 3400.394859≈3400 | 60 | 30448.16395≈30448 |
| 15 | 417.7248169≈418 | 38 | 3740.434344≈3740 | 61 | 33492.98035≈33493 |
| 16 | 459.4972986≈459 | 39 | 4114.477779≈4114 | 62 | 36842.27838≈36842 |
| 17 | 505.4470285≈505 | 40 | 4525.925557≈4526 | 63 | 40526.50622≈40527 |
| 18 | 555.9917313≈556 | 41 | 4978.518112≈4979 |  |  |
| 19 | 611.5909045≈612 | 42 | 5476.369924≈5476 |  |  |
| 20 | 672.7499949≈673 | 43 | 6024.006916≈6024 |  |  |
| 21 | 740.0249944≈740 | 44 | 6626.407608≈6626 |  |  |
| 22 | 814.0274939≈814 | 45 | 7289.048369≈7289 |  |  |