3GPP TSG-RAN WG2 Meeting #129	R2-2500798

Athens, Greece, 17 - 21, Feb, 2025

Agenda Item:	8.1.3

Source:	Xiaomi

Title:	Discussion on NW-side data collection

Document for:	Discussion

Introduction

For NW side model, data collection is essential to collect data to train AI model. In this contribution, we discuss open issues of NW-side data collection.

Discussion

Common procedure for BM and Pos

If the data is collected and reported by UE, data collection procedure can be divided to two steps. Upon receiving network configuration on training data collection, UE first starts to collect and log data. Then, UE can report the data to NW based on report trigger/request. In the following sections, we would discuss data collection initiation and data report respectively.

Data collection control

One important aspect is how to initiate UE to perform data collection. The simplest way is NW can send explicit indication to start or stop data collection. However, it is not clear that whether multiple data collection configurations can be provided to UE simultaneously or not.

As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.

We understand the data would be use cases specific, e.g. BM and CSI, or even sub use case specific. For example, one data sample of spatial BM prediction consists measurement results of set A and B beam at the same time. While, another data sample of temporal BM prediction requires historic measurement results of set B beam and one measurement result of set A beam. Therefore, NW should be allowed to provide multiple data collection configurations for different (sub) use cases.

Proposal 1: Multiple NW-side training data collection configurations can be configured to UE.

A subsequent question is whether dynamic activation/deactivation of data collection is supported. We understand the real question is whether NW-side training data collection needs to be controlled by network with a shorter latency or not. It’s always possible for NW to setup/release the data collection configuration by RRC. According to LS from RAN1 [1], it is observed that the latency requirement of training data collection is rather relaxed. There is no need to introduce L1/L2 dynamic control from latency point of view.

Proposal 2: L1/L2 signalling based dynamic activation/deactivation of NW-side training data collection is not supported.

It’s agreed the radio condition-based event triggered logging is supported.

Event-triggered data logging will be supported. At least radio condition based event triggered logging will be supported.

It is possible that NW may only need UE to collect data under certain radio channel conditions, considering model may be different under different radio channel conditions. Radio channel condition may change due to UE movement and NW may need to frequently send indication to start or stop NW-side training data collection or updating training data collection configuration, where RSRP/RSRQ/SINR can be considered. Furthermore, in this release, the functionalities are considered as cell-specific. Therefore, the radio condition should be based on serving cell’s radio condition. In legacy measurement report event, A1 and A2 are supported. We understand A1 and A2 alike event can also be supported for data logging initiation. It’s up to NW implementation to decide which event is useful.

Proposal 3: Serving cell’s radio condition (e.g. RSRP, RSRQ, SINR) is supported in data logging event. A1 and A2 alike event are supported.

The straightforward way to support event is based on threshold. Similar as legacy MR event, the event fulfilment can be based on radio condition, e.g. RSRP, above or lower than the threshold. One open issue is whether TTT like timer is needed. In legacy, TTT like timer is introduced to avoid frequent measurement report. However, the data logging event only triggers UE to perform data logging, but it doesn’t trigger collected data reporting. Furthermore, collecting more data would be helpful for training a generic model. Therefore, there’s no need to support TTT like timer for data logging event.

Proposal 4: For data logging event, TTT like timer is not supported.

Even if UE supports the data logging, it’s possible that different UE chipset/vendor has different implementation. The supported storage size may be different. As indicated in [1], different types of training data may require different storage sizes. Also, the data reporting configuration may be associated with the storage size, e.g. longer report period would result in UE storing more data. UE can report the supported data logging storage size to NW. NW can decide which data is to be logged by UE and the appropriate configuration, e.g. report period. The reported logging storage size can be used across all use cases.

Proposal 5: UE can support different data logging storage size across all use cases, which can be indicated by UE capability.

Data report

Availability Indication

It’s agreed during RAN2 #127bis and #128 meeting, UE can indicate the availability of logged data and also report the reason for triggering of indication (e.g. low power state, low memory, etc) for the status.

The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication. and FFS how data availability indication is sent to the network. The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

NW can choose whether request UE to report the data based on the reason. If the report is triggered by low power state, NW can choose not to request UE to report logged data and wait for UE recovery from low power state. But if the report is triggered by full buffer, UE has stopped data logging. NW may want to request UE to report logged data and resume the data logging. We don’t see motivation to further introduce other event to trigger UE report. RAN2 can confirm only these two reasons are needed.

Proposal 6: RAN2 confirms only low power state and low memory are considered as reason for triggering of indication.

It’s not clear whether the reason is reported as part of availability indication or separate indication. Recalling that it’s agreed NW can configure radio condition-based logging event, which means UE will only collect data if radio condition event is fulfilled. When UE reports low power state to network as the reason for triggering the indication, it is possible that radio condition event has not been fulfilled at UE side before, which lead to no data logging/stored at UE side. But NW is not aware whether UE has any logged data. NW will send another request to check whether UE has stored data or not. UE further responses with an availability indication, where NW can request logged data reporting if there’s available data. Alternatively, to avoid the additional signalling, UE can report low power state indication together with an availability indication. With that, NW can decide whether to request UE to report stored data or not immediately. To reduce signaling overhead and exchange between NW and UE, UE can include the reason as part of availability indication.

Proposal 7: UE includes the reason for triggering of indication (e.g. low power state, low memory) as part of data availability indication.

It’s not clear which RRC message is used to report this availability indication. According to the above discussion, the trigger can occur at any time. UE should be able to initiate the RRC message anytime. Potential message can be UAI or MR or new message. We understand the intention of this indication is similar to assistance information, where UAI is more appropriate.

Proposal 8: UAI is used to report the data availability indication and reason of report.

Releasing of logged data

We understand new UE variables will be introduced to store the logged data. Therefore, we shall discuss how to release the logged data. Similar handling as VarMeasReportList can be reused.

After data reporting, UE can release the logged collected data. UE doesn’t need to log the old data anymore. It can be released, so that new data can be collected.

Proposal 9: UE releases the logged data after report.

If NW has collected enough data, NW may release the data collection configuration UE shall also release the logged data. But if NW modifies training data collection configuration, e.g. modify set A, it’s not clear whether UE shall release or keep the logged data. We understand UE shall still keep the logged data, since NW may still need the old data to train a generic model. If NW doesn’t need the old data, NW modify the configuration by release and setup procedure. So, NW has the flexibility to control whether UE can keep the data implicitly.

Proposal 10: If NW release the data collection configuration, UE releases the logged data. If NW modify the data collection configuration, UE keeps the logged data.

The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after).

Another open is issue is when UE trigger the report before or after buffer is full. We understand this can be up to UE implementation. However, UE doesn’t need to further indicate the buffer is already or going to be full to NW. UE only indicates the reason of low memory to NW.

Proposal 11: It’s up to UE to decide whether buffer is full, i.e. before or after buffer is full.

Mobility handling

RAN1 agreed the NW side additional condition can be associated with a ID. This ID is unique within a cell. Therefore, same functionality in different cells may be associated with different IDs. Furthermore, it’s difficult to ensure the ID is global unique. Therefore, it’s possible the same ID means different NW side additional condition in two cells. When UE moves from one cell to another, UE may log data collected from multiple cells. These cells may be associated with different NW side additional condition. Therefore, when UE reports the data, UE shall indicate the cell and NW-side additional condition that the data is collected. The associated NW information can be cell ID and associated ID.

Proposal 12: If logged data are collected from multiple cells, UE reports the logged data together with the corresponding cell ID and associated ID that it is collected.

NW-side data collection for Pos

(2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

(1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

(1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

For AI positioning, there are there use cases with the AI model deployed at Network side, however, we only consider the first priority use cases in this section.

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

Note:	For gNB- and OAM-centric data collection, there may be a need to consult with RAN3 and SA5 whether/how OAM is to be involved.

Note:	For possible impacts due to positioning use cases, there may be a need to consult with RAN3 whether/how NRPPa is to be involved.

According to the TR 38.843 [2], we had made some conclusions on NW-sided data collection and Model training for positioning as below.

Model Training:

o	For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.

	Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress.

o	For LMF-side model, the LMF is the termination point for training data.

For Case 3a, if the termination point for training data is the gNB, there may be no specification impact on data collection. However, if the termination point is the LMF, the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training. Since we haven’t agreed that the LMF is the termination point for training data in Case 3a, we won’t consider the LMF as the termination point for now.

For Case 3b, the LMF is responsible for data collection, and the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training.

Proposal 13: For Case 3a, if the termination point for training data is the gNB, there may be no specification impact on data collection.

Proposal 14: For Case 3b, the LMF is responsible for data collection, and the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training.

For the network side data collection for AI positioning, SA2 had made some progresses as below [3]:

Figure 5.18.1-1: Data collection by LMF to train the AI/ML based positioning model using UE measurements

The LMF needs to obtain input data for AI/ML based positioning model training and may also request the UE to provide the UE location. The AI/ML based positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

1.	The LMF starts data collection for the purpose to train a AI/ML Model for UE Positioning. The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

2.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest").

3.	The LMF checks the user consent for data collection for model training.

4.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

According to SA2 conclusion, the LME will request the UE in an area of interest to perform data collection. Which means that the UE should stop data collection if the UE is not in the area of the intertest. Therefore, we think the LMF should send the area of interest to the UE for UE performing data collection.

Proposal 15: For Case 3b, LMF sends area of interest of data collection to UE when requesting it to perform the data collection.

For non-AI/ML positioning methods, the UE measures PRS based on PRS configuration provided by LMF, which is common for all UEs. For AI/ML based training data collection, the same PRS configuration can also be used by the UE to collect data for NW-side model. On the other hand, considering offline training is used, LMF may configure UE with a different measurement configuration compared to non-AI/ML positioning method. Therefore, LMF may also indicate a different PRS configuration  to the UE for NW-side data collection.

Proposal 16: For Case 3b, LMF indicates the PRS configuration for UE performing NW-side data collection, which can be the same or different PRS configuration for non-AI/ML PRS configuration.

Conclusion

Based on the discussion in section 2, we have following proposals:

For both BM and Pos:

Data collection control

Proposal 1: Multiple NW-side training data collection configurations can be configured to UE.

Proposal 2: L1/L2 signalling based dynamic activation/deactivation of NW-side training data collection is not supported.

Proposal 3: Serving cell’s radio condition (e.g. RSRP, RSRQ, SINR) is supported in data logging

event. A1 and A2 alike event are supported.

Proposal 4: For data logging event, TTT like timer is not supported.

Proposal 5: UE can support different data logging storage size across all use cases, which can be indicated by UE capability.

Data report

Proposal 6: RAN2 confirms only low power state and low memory are considered as reason for triggering of indication.

Proposal 7: UE includes the reason for triggering of indication (e.g. low power state, low memory) as part of data availability indication.

Proposal 8: UAI is used to report the data availability indication and reason of report.

Proposal 9: UE releases the logged data after report.

Proposal 10: If NW release the data collection configuration, UE releases the logged data. If NW modify the data collection configuration, UE keeps the logged data.

Proposal 11: It’s up to UE to decide whether buffer is full, i.e. before or after buffer is full.

Mobility handling

Proposal 12: If logged data are collected from multiple cells, UE reports the logged data together with the corresponding cell ID and associated ID that it is collected.

For Pos:

Proposal 13: For Case 3a, if the termination point for training data is the gNB, there may be no specification impact on data collection.

Proposal 14: For Case 3b, the LMF is responsible for data collection, and the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training.

Proposal 15: For Case 3b, LMF sends area of interest of data collection to UE when requesting it to perform the data collection.

Proposal 16: For Case 3b, LMF indicates the PRS configuration for UE performing NW-side data collection, which can be the same or different PRS configuration for non-AI/ML PRS configuration.

Reference

[1] R2-2311720	Reply LS on Data Collection Requirements and Assumptions

[2] TR 38.843 v18.0.0 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[3] TS 23.273-j00, Technical Specification Group Services and System Aspects; 5G System (5GS) Location Services (LCS)