3GPP TSG-RAN WG2 Meeting #129	R2-2500799

Athens, Greece, 17 - 21, Feb, 2025

Agenda Item:	8.1.4

Source:	Xiaomi

Title:	Discussion on UE-side data collection

Document for:	Discussion

Introduction

For UE side model, data collection is essential to collect data to train AI model. In [1], following work is expected for RAN2,

For RAN2 - During Rel-19 RAN2 continues studies on AIML UE sided data collection (excluding 1b) - Study RAN aspects related to data transfer over UP - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion). - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. - Discuss on the scalability aspects of CP - RAN2 can Liaise with other WGs if/when needed

In this contribution, we discuss open issues of UE-side data collection.

Discussion

NG-RAN involvement in the control and configuration

During RAN2 #128 meeting, following agreements training data collection configuration for UE-side model:

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains. - The network can provide the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection. - FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

For UE side model, the data collection is triggered by UE or UE side server. The desired data collection for UE-side model may require NW to provide a specific configuration, e.g. set A or set B. If the desired configuration has been provided by NW, UE can collect the data without additional indication to NW. In this method, there is no impact to NW. However, if the desired data can’t be collected in current network configuration, UE can send request NW to provide the desired configuration and perform UE-side data collection upon receiving network configuration.

Proposal 1: For UE side data collection, if current configuration is not aligned with the desired configuration, UE requests NW to provide desired configuration for data collection. If NW has provided the desired configuration, UE can start data collection directly without additional indication.

As replied by RAN1 that associated ID be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. During UE data collection, UE may not perform any AI inference, i.e. AI functionality is no activated. It’s not clear whether NW would provide the associated ID to a UE without AI functionality activation. If NW doesn’t provide the associated ID, UE can include desired associated ID, when requesting UE-side model training data collection configuration.

Proposal 2: In UE-side model training data collection request, UE can include associated ID(s) of the requested functionalities. (UAI)

For UE-side training data collection, different from RRM measurement and NW-side training data collection, it is better to allow UE have more controllability of its own model’s training data collection procedure

First of all, during UE-side training data collection, it is possible that UE may want to terminate its own training data collection based on its own implementation status for many reasons. For example:

- When UE considers itself has enough data collected

- When UE is overheated/low power state and wants to temporarily stop training data collection for UE-side model

- When UE want to prioritize other services compared to this latency non-critical training data collection

Moreover, UE-side training data collection is mainly collected to serve for UE-side model, where either UE or UE-side OTT server has a clear understanding and requirement on when and how to collect such data. However, NW is not aware of whether UE has stopped UE-side data collection and may continue to provide the requested configuration. The requested configuration may not be the optimal configuration for NW.

Therefore, UE needs to report its data collection status to NW if UE has stopped its data collection. Upon receiving the indication, NW can release the UE-side training data collection configuration.

Proposal 3: When UE stops UE sided training data collection, UE indicates NW to release the corresponding training data collection configuration.

UE sided data transfer

During RAN #xxx meeting, RAN has sent LS to SA WGs asking SA to start study UE-side data collection methods in [2]. It is observed that, for the method of data transfer over UP or NAS, there are some dependency on SA study. To avoid duplicate discussion, RAN2 can wait for SA progress and check whether there is any impact to RAN2 based on SA conclusion.

Proposal 4: For UE sided data transfer over UP or NAS, RAN2 waits for other WG’s progress.

Another data transfer option is data transfer over RRC, where scalability aspects of RRC segment number should be studied by RAN2. In current specification, UL RRC message can support at most 16 segments and each segment has a maximum size of 9000 bytes.

ULDedicatedMessageSegment message -- ASN1START -- TAG-ULDEDICATEDMESSAGESEGMENT-START ULDedicatedMessageSegment-r16 ::= SEQUENCE { criticalExtensions CHOICE { ulDedicatedMessageSegment-r16 ULDedicatedMessageSegment-r16-IEs, criticalExtensionsFuture SEQUENCE {} } } ULDedicatedMessageSegment-r16-IEs ::= SEQUENCE { segmentNumber-r16 INTEGER (0..15), rrc-MessageSegmentContainer-r16 OCTET STRING, rrc-MessageSegmentType-r16 ENUMERATED {notLastSegment, lastSegment}, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL }

## 4.3 Services ### 4.3.1 Services provided to upper layers The PDCP layer provides its services to the RRC or SDAP layers. The following services are provided by PDCP to upper layers: - transfer of user plane data; - transfer of control plane data; - header compression; - uplink data compression; - ciphering; - integrity protection. The maximum supported size of a PDCP SDU is 9000 bytes. The maximum supported size of a PDCP Control PDU is 9000 bytes.

Based on above observation, the maximum size of one RRC message supported in UL is 144 KB. However, the size of training data could be very large. According to [3], RAN1 indicated the size of training data for CSI can be 225 MB and even larger if the dataset consisting of supporting multiple configurations. Similar size of training dataset is expected for beam management and positioning use case. Assuming 225MB is considered as an average size for UE-side training data collection, it requires 1500 segments or even more if data size is larger. It’s impossible for single RRC message to carry the UE sided data in this massive number of segments.

Observation 1: Thousands of segments are required to transfer UE sided data over RRC message.

Even if the number of RRC segments can be extended, RRC message transmission would require a much longer time. Handover may occur during the RRC message transmission. In current specification, SRB doesn’t support service continuity during handover, where UE has to retransmit the data from the first RRC segment again in target cell, which results in significant signalling overhead and unnecessary waste of air interface resources.

Observation 2: Significant signalling overhead and waste of air interface resource may be expected after handover, as service continuity in SRB is not supported and UE needs to retransmit all the collected data.

Furthermore, current SRB0/1/2 may have higher priority than DRB, and SRB1 has even higher priority than SRB2. Considering the size of training data collection, the data transfer over RRC message may not be finished in a short time and further block the transmission of other RRC message transmission . Transmission of other RRC message or user plane data may be delayed. To avoid the blocking issue, new SRB with lower priority may be required for training data transfer.

Observation 3: New SRB with lower priority is needed to support UE sided data transfer over RRC.

Based on the above observations, it’s challenging to resolve all above listed issues in R19. Considering the time and work load, we propose RAN2 do not support UE sided data transfer over RRC in R19.

Proposal 5: RAN2 do not support UE sided data transfer over RRC in R19.

Conclusion

Based on the discussion in section 2, we have following proposals:

NG-RAN involvement in the control and configuration:

Proposal 1: For UE side data collection, if current configuration is not aligned with the desired configuration, UE requests NW to provide desired configuration for data collection. If NW has provided the desired configuration, UE can start data collection directly without additional indication.

Proposal 2: In UE-side model training data collection request, UE can include associated ID(s) of the requested functionalities. (UAI)

Proposal 3: When UE stops UE sided training data collection, UE indicates NW to release the corresponding training data collection configuration.

UE sided data transfer

Proposal 4: For UE sided data transfer over UP or NAS, RAN2 waits for other WG’s progress.

Observation 1: Thousands of segments are required to transfer UE sided data over RRC message.

Observation 2: Significant signalling overhead and waste of air interface resource may be expected after handover, as service continuity in SRB is not supported and UE needs to retransmit all the collected data.

Observation 3: New SRB with lower priority is needed to support UE sided data transfer over RRC.

Proposal 5: RAN2 do not support UE sided data transfer over RRC in R19.

Reference

[1] RP-243307 Way forward on AIML UE sided data collection

[2] RP-243316 LS on AIML UE sided data collection

[3] R1-2410922 LS on signalling feasibility of dataset and parameter sharing