3GPP TSG-RAN WG2 Meeting #129	R2-2500803

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	 8.6.3

Source:	 Huawei, HiSilicon

Title:	 Discussion on L1 event-triggered measurement reporting

Document for:  Discussion and Decision

1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18. The network receives L1 measurement report to determine the LTM target cell and target beam. Three types of report are defined, namely, periodic, aperiodic and semi-persistent L1 report.

For R19 mobility enhancements, one of the goals outlined in the WID is to define the required improvements for enabling event-triggered L1 measurements.

RP-242356, Revised Work Item: NR mobility enhancements Phase 4 - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

In this contribution, we provide a comprehensive analysis of event-triggered L1 measurement reports, covering the event evaluation and reporting procedures for the key aspects.

2	Discussion

2.1	Event evaluation

2.1.1	LTM Candidate cell

Agreements on L1 event triggered MR 8. For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

According to the agreement in RAN2#127-bis, the structure of the Rel-18 LTM-CSI-ReportConfig should be reused as much as possible. In this legacy structure, there is a parameter called spCellInclusion, which can only be configured in some specific scenarios. For example, during a subsequent LTM cell switch, if the serving cell is configured as one of the candidate cells and spCellInclusion is set to true, the measurement results for the serving cell should be included in the L1 measurement report. Thus, the configuration of the serving cell as one of the SpCells in the LTM candidate configuration can also be applied in the Rel-19 LTM-CSI-ReportConfig.

spCellInclusion This field indicates whether the UE shall include a L1 measurement report associated to the current SpCell. This field can only be configured if the current SpCell is configured as an SpCell of an LTM candidate configuration and the LTM-CSI-ResourceConfig IE associated to the LTM-CSI-ReportConfig IE includes resources for the current SpCell.

Proposal 1: Follow the R18 configuration to include the serving cell configured as an SpCell in the LTM candidate configuration for subsequent LTM operations.

Additionally, if the serving cell is configured as an SpCell in the LTM candidate configuration, the LTM-CSI-SSB-ResourceSet will include SSBs/CSI-RSs from both the serving cell and the other candidate cells. Since all the candidate beams, including those from the serving cell and candidate cells, are combined into one LTM resource set, the UE needs to perform event evaluation for all these beams in the LTM-CSI-SSB-ResourceSet. Especially for EventLTM3, EventLTM4, and EventLTM5, the UE may compare the current beam with the beams from the serving cell. Furthermore, even if the serving cell’s beams satisfy the event conditions, they could also trigger the event reporting procedure for those LTM events. Apparently, these scenarios should be avoided. Therefore, the beams of the serving cell should not undergo the event evaluation procedure. This action can be easily achieved with the current LTM resource set structure, as the UE can distinguish the serving cell's beams in the RRC configuration using the ltm-CandidateIdList.

Proposal 2: If the serving cell is configured as an LTM candidate cell in the LTM candidate configuration, for this candidate cell, there is no need to perform event evaluation for EventLTM3, EventLTM4, and EventLTM5.

2.2	Event reporting procedures

2.2.1	Report contents

It is possible that multiple event triggering conditions are met or for a single event, multiple beams satisfying the event triggering condition within the period of TTT simultaneously. In such cases, generating a separate measurement report for each event or each beam satisfying the event, will lead to frequent transmission of the MAC CE or frequent triggering of SR for MR MAC CE. When one of the events is satisfied, we think only one MAC CE should be enough to send the measurement results. This measurement report can carry all measurement results and triggering event information to save the signalling overhead.

Proposal 3: When multiple events are triggered simultaneously, they can be reported within a single MR MAC CE.

In legacy L3 measurement reports, there is no specific tag to distinguish between entering condition or leaving condition, since the network is only concerned with the beams satisfying the event. Besides, the network can infer whether the MRs are triggered by an entering or leaving condition based on the contents of the cellsTriggeredList. Consequently, the same MAC CE format is used for both entering and leaving condition-triggered MRs.

Proposal 4: A unified MAC CE format should be used for entering or leaving condition-triggered MRs in Rel-19 LTM event-triggered measurement reporting procedure.

Agreements on L1 event triggered MR 8. For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

According to the agreements reached in the last meeting, it has been confirmed that beams satisfying the event can be reported in the MR MAC CE. However, the phrase "beams satisfying the event" is ambiguous and open to multiple interpretations. It could refer to:

- Beams satisfying the entering condition,

- Beams satisfying the entering condition for the whole duration of TTT,

- Beams satisfying the leaving condition, or

- Beams satisfying the leaving condition for the whole duration of TTT.

From the network's perspective, the primary intention is to identify beams suitable as target beams, which aligns with the second interpretation. Therefore, the term "beams satisfying the event" should be defined as "beams that satisfy the entering condition for the entire duration of TTT."

Proposal 5: It needs to be clarified that the term "beams satisfying the event" specifically refers to "beams that satisfy the entering condition for the whole duration of TTT".

Additionally, based on the agreements made in the last meeting, it is proposed that beams not satisfying the event can also be reported in the MR MAC CE, according to the N beams in the MR MAC CE. Consequently, the report content will include both "beams satisfying the event" and "beams not satisfying the event". Thus, an extra bit for each SSBRI/CRI can be used to differentiate between these two types of beams.

Proposal 6: Use 1 bit to indicate whether the reported SSBRI/CRI satisfies the event, in cases beams not satisfying the event are also reported.

It is essential to discuss whether the serving cell should be included in the measurement report. In legacy R18 LTM, the serving cell can be added into a candidate configuration for subsequent cell switch. In such cases, the network can configure the UE to measure the beams of the serving cell and include them in the event-triggered L1 measurement report. This approach aligns with the existing R18 LTM configuration, thus should be reused. The maximum number of beams that can be reported per cell is 4, hence, up to four beams from the serving cell should be included in the report.

Proposal 7: Multiple serving beams can be reported in the measurement report MAC CE.

2.2.2	Truncated MAC CE

Agreements on L1 event-driven MR 10. Support the truncated measurement report MAC CE.

According to the agreements supported in the last meeting, if the UL grant is insufficient to transmit the entire MR, a truncated measurement report MAC CE can be used to carry a portion of the MR. For BFR truncated MAC CE, each has its own eLCID, distinct from normal (un-truncated) BFR MAC CE. This allows the network to easily differentiate between normal MAC CEs and truncated MAC CEs based on the eLCID.

When the transmitted MR is a truncated MAC CE, which is identified by an eLCID different from normal (un-truncated) MAC CE, the network recognizes that there is a remaining portion of the MR to be sent. In such cases, the network will send a DCI to allocate UL resources for the subsequent MR transmission, eliminating the need for the UE to send an SR. As a result, it can be reused for the mechanism of truncated measurement report MAC CE.

Proposal 8: Different eLCIDs should be used for the MR MAC CE and the MR truncated MAC CE.

Moreover, if the LTM event is satisfied, the corresponding event enters a triggering state, awaiting a UL grant to transmit the measurement report. When the UL grant is limited and an event-triggered truncated MAC CE is sent, the event's triggering state should not be canceled to ensure the remaining part of the MR can be sent later. The event's triggering state should only be canceled once the complete MR is successfully transmitted.

If multiple MRs with different ReportConfigId are included in the same MAC CE, and only some are transmitted while others are not, the UE should cancel the triggering status only for the events whose MRs have been fully transmitted. The triggering state for the remaining events should remain on to ensure their transmission at a later time.

Proposal 9: If only part of the MR for the corresponding ReportConfigId is transmitted in the truncated MAC CE, the triggering status of the corresponding event should not be canceled. The triggering state should only be canceled once the complete MR for the corresponding ReportConfigId has been transmitted.

Since the network can deduce that there will be an event-triggered MR MAC CE following a truncated MR MAC CE, it becomes challenging to determine the size of the subsequent event-triggered MR MAC CE and allocate an appropriate uplink grant. To solve this, the number of reported SSB/CSI-RS beams can be explicitly included in the MR. For example, if 8 beams satisfy the event and this total is reported in the MR, but only 5 beams are included in the truncated MAC CE due to limited uplink grant, the network can infer that the remaining 3 beams will be reported in subsequent MR. As a result, the network can allocate a suitable uplink grant without requiring the UE to send a SR. This ensures precise information about the total number of beams satisfying the event, which is available to the network, in the case of a truncated MAC CE. The number of satisfied SSB/CSI-Rs can be also included in the normal (un-truncated) event-triggered MAC CE. This allows for the direct provision of the quantity of satisfied beams at the beginning of the MR.

Proposal 10: The number of reported SSB/CSI-RS should be explicitly indicated in the event-triggered MAC CE, if only the beam(s) satisfying the event could be reported.

2.3	RRC impacts

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS. This field is not applicable to L1 measurements configured with ltm-CSI-ResourceConfigToAddModList-r18. This field is not configured to a UE configured with event(s) related to the L2 U2N Relay UE.

In the RRC specification, the definition of the parameter s-MeasureConfig is not applicable to L1 measurements configured with ltm-CSI-ResourceConfigToAddModList-r18. This restriction applies specifically to L1 measurements, including periodic, semi-persistent, and aperiodic LTM reporting. With the introduction of event-triggered LTM reporting in Rel-19, RAN2 should discuss whether the s-MeasureConfig parameter remains not applicable to event-triggered LTM reporting.

Proposal 11: S-measureConfig is not applicable to in the L1 event-triggered MR.

In the [POST128] RRC running CR, a company has suggested that we should consider adjusting the actual value of a3-Offset/a6-Offset to be the field value * 1 dB, instead of the current 0.5 dB. From our perspective, there is no technical reason to change the granularity, as the current approach works fine and does not present any issues.

CondTriggerConfig field descriptions

a3-Offset Offset value(s) to be used in NR conditional reconfiguration triggering condition for cond event a3. The actual value is field value * 0.5 dB.

EventTriggerConfig field descriptions

a3-Offset/a6-Offset Offset value(s) to be used in NR measurement report triggering condition for event a3/a6. The actual value is field value * 0.5 dB.

Proposal 12: It is suggested to keep the current granularity for a3-Offset/a6-Offset in EventTriggerConfig field, where the actual value is calculated as the field value * 0.5 dB.

2.3	Measurement reports triggering

2.3.1	Scheduling Request for MR

Agreements on L1 event triggered MR 11. The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. 12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

Based on the RAN2 agreements made before, the network should configure a dedicated SR configuration for transmitting the MAC CE report. It means that if multiple measurement reports are triggered simultaneously, UE can send a dedicated SR to request the UL resources.

Based on the RAN2 agreements, the network should configure a dedicated SR for transmitting the MAC CE report. This ensures that if multiple measurement reports are triggered simultaneously, the UE can send a dedicated SR to request UL resources.

According to the RRC configuration of LTM CSI report, the maximum number of the measurement reports could reach 48. As a result, the size of the MR MAC CE can become quite large, making it challenging for the network to allocate appropriate UL resources since the exact size of the MR MAC CE is uncertain.

Observation 1: It is hard for the network to allocate the appropriate UL resource size due to the unpredictable MR MAC CE size.

To address this issue, the number of triggered events can be linked to the PUCCH resources used for SR selection. For example: If the number of triggered events exceeds a certain threshold, the network selects SR Configuration A. Otherwise, it selects SR Configuration B.

Proposal 13: The PUCCH of SR selection is based on the number of triggered events or based on the estimated MAC CE size. E.g. If the number of triggered events is greater than a threshold, the select the SR configuration A; else, select SR configuration B.

# 3	Conclusion

In this paper, we have addressed the objective regarding improvements in event-triggering reports for LTM. There are the following proposals:

Event evaluation

Proposal 1: Follow the R18 configuration to include the serving cell configured as an SpCell in the LTM candidate configuration for subsequent LTM operations.

Proposal 2: If the serving cell is configured as an LTM candidate cell in the LTM candidate configuration, for this candidate cell, there is no need to perform event evaluation for EventLTM3, EventLTM4, and EventLTM5.

Event reporting procedures

Proposal 3: When multiple events are triggered simultaneously, they can be reported within a single MR MAC CE.

Proposal 4: A unified MAC CE format should be used for entering or leaving condition-triggered MRs in Rel-19 LTM event-triggered measurement reporting procedure.

Proposal 5: It needs to be clarified that the term "beams satisfying the event" specifically refers to "beams that satisfy the entering condition for the whole duration of TTT".

Proposal 6: Use 1 bit to indicate whether the reported SSBRI/CRI satisfies the event, in cases beams not satisfying the event are also reported.

Proposal 7: Multiple serving beams can be reported in the measurement report MAC CE.

Proposal 8: Different eLCIDs should be used for the MR MAC CE and the MR truncated MAC CE.

Proposal 9: If only part of the MR for the corresponding ReportConfigId is transmitted in the truncated MAC CE, the triggering status of the corresponding event should not be canceled. The triggering state should only be canceled once the complete MR for the corresponding ReportConfigId has been transmitted.

Proposal 10: The number of reported SSB/CSI-RS should be explicitly indicated in the event-triggered MAC CE, if only the beam(s) satisfying the event could be reported.

RRC impacts

Proposal 11: S-measureConfig is not applicable to in the L1 event-triggered MR.

Proposal 12: It is suggested to keep the current granularity for a3-Offset/a6-Offset in EventTriggerConfig field, where the actual value is calculated as the field value * 0.5 dB.

Measurement reports triggering

Proposal 13: The PUCCH of SR selection is based on the number of triggered events or based on the estimated MAC CE size, e.g. If the number of triggered events is greater than a threshold, the select the SR configuration A; else, select SR configuration B.

# 4	Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4;

- RAN2 #127bis chairman notes;

- RAN2 #128 chairman notes;

- TS 3GPP 38.331 V18.4.0