3GPP TSG-RAN WG2 Meeting #129	R2-2500807

Athens, Greece, 17th – 21st Feb, 2025

Agenda item:	8.7.6

Source: 	Huawei, HiSilicon

Title: 	Discussion on XR rate control

Document for:	Discussion and Decision

1. Introduction

During the last meeting, RAN2 agreed to support QoS flow level recommended bit rate indication. In this contribution, we will further discuss the details such as the signalling design. Besides, there is a reply LS from SA4 clarifying the required range and granularity for XR rate control. We will also analyse the potential enhancements considering the requirements.

2. Discussion

2.1 Per QoS flow MAC CE v.s per DRB MAC CE

Although RAN2 has agreed on the QoS flow level rate control, there was controversy on the details of the indication signalling design. As shown by the following agreement, there are two options. One is to introduce per QoS flow MAC CE directly. The other is to indicate the QoS flows for rate control via RRC signalling and provide the recommended bit rate in per DRB MAC CE.

Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

Comparing the two options, we think the first one is more aligned with the purpose of QoS flow level rate control, while the second one may have some problems. Specifically, if there are more than one QoS flow configured with rate control with the same DRB, when the gNB indicates the recommended bit rate in the DRB level MAC CE, the UE is not sure how to apply the recommended rate to the multiple QoS flows. It depends on the UE behaviour and may be out of network expectation. In the NR QoS mechanism, NG-RAN is responsible to ensure the QoS requirement on a per QoS flow level. If the gNB cannot control the rate of each QoS flow accurately, it would be hard to monitor and guarantee the QoS requirements.

Observation 1: Per DRB MAC CE is inaccurate for QoS flow level rate control and may cause some risk in QoS guarantee.

Some companies opposing the first option have concern on whether the gNB can provide the QoS flow level recommended bit rate in case of CU-DU split when multiple QoS flows are mapped to the same DRB. However, we think there is no issue. Actually, it should be feasible and up to the gNB-DU implementation to provide the QoS flow level recommended bit rate no matter how the QoS flows are mapped to the DRBs.

First, it is noted that F1AP is already able to transfer the QoS flow level QoS parameters to the gNB-DU via the UE context management messages, as the following content excerpted from TS 38.473 [1] shows. So that the gNB-DU can derive the recommended rate for each QoS flow based on the available resource and the QoS requirements. For example, assuming there are two QoS flows with their GFBR equal to 10Mbps and 20Mbps respectively, if the gNB-DU can provide up to 45Mbps for the QoS flows in total, then it can determine the recommended bit rate as 15Mbps and 30Mbps for each of the QoS flows.

### 9.2.2 UE Context Management messages #### 9.2.2.1 UE CONTEXT SETUP REQUEST This message is sent by the gNB-CU to request the setup of a UE context. Direction: gNB-CU gNB-DU. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. *** skipped part ***. >>>>>>QoS Flow Level QoS Parameters; M; 9.3.1.45; -. 9.2.2.7 UE CONTEXT MODIFICATION REQUEST This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU. Direction: gNB-CU gNB-DU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. *** skipped part ***. >>>>>>QoS Flow Level QoS Parameters; M; 9.3.1.45; -.

Another potential implementation is that the gNB-DU may monitor the actual bit rate of each QoS flow by taking statistics of the received DL or UL packets. Specifically, if there is only one QoS flow mapping to the DRB, the gNB-DU can measure the data rate for the DRB, which equals to the rate of the QoS flow. If there are multiple QoS flows mapping to the DRB, the gNB-DU can distinguish the packets belong to different QoS flows via the QFI carried in the GTP-U header (for DL) or the SDAP header (for DL/UL) so that the rate of each QoS flow can be measured. It is worth noting that the CU can configure the DL/UL SDAP header to be included if N to 1 mapping will be performed for the QoS flow which supports bit rate control. With monitoring the actual rate for the QoS flows, the gNB-DU can determine the recommended rate for each QoS flow by taking the available resource, measured date rate per QoS flow and the QoS requirement of each Qos flow into account.

Observation 2: It is feasible and up to implementation for the gNB-DU to provide the QoS flow level recommended bit rate.

With the above analysis and observations, it should be confirmed that there is no technical difficulty to provide the QoS flow level recommended bit rate. So it is reasonable to have per QoS flow MAC CE to indicate the rate directly, to achieve the purpose of QoS flow level rate control.

Proposal 1: RAN2 to introduce per QoS flow MAC CE for UL recommended rate indication.

2.2 Range and granularity for recommended bit rate

SA4 has replied the questions on the required range and granularity for XR rate control in the LS in S4-242182 [2]. The main part of the reply LS is excerpted as below.

# 1. Overall Description: In R2-2409273, a question from RAN2 is: RAN2 are discussing MAC-layer signaling enhancements to enable XR applications to adapt bit rates in response to uplink congestion. To proceed with the design, RAN2 would like to request input from SA4 on the appropriate range and granularity of uplink bit rates that need to be supported for the signaling enhancements. RAN2 respectfully requests SA4 to provide input on the appropriate range and granularity of uplink bit rates that need to be supported in the aforementioned signaling enhancements. The SA4 reply is as follows: The range varies from 100kbps to 40Mbps. The numbers are based on the SA4 traffic characteristics study for XR services documented in 3GPP TR 26.926. Specifically, for XR split rendering and cloud gaming, a typical uplink data rate is 200 kbps. For AR conversational, a typical uplink data rate is 20Mbps. The granularity varies among existing data rate adaptation algorithms, and it can be as small as 1% for some algorithms (e.g., transport-wide congestion control as documented in 3GPP TR 26.822), yet also coarser for other algorithms (e.g., video codecs that can adapt to a given target bit rate constraint). From an application perspective, finer granularity, e.g., 1%, is generally preferred, if possible, but coarser granularity may also be sufficient, e.g., 10%. # 2. Actions: # To RAN2: ACTION: SA4 kindly asks RAN2 to take the above reply into account and provide feedback if any.

From the reply, it can be seen that the required granularity for XR rate control is 1% to 10% with a range from 100kbps to 40Mbps.

Observation 3: The requirement from SA4 for XR data rate includes:

- The range of the data rate varies from 100kbps to 40Mbps.

- The granularity of 1% is preferred.

Such requirement cannot be satisfied by reusing the current bit rate indication design directly. As specified in TS 38.321 [3], currently the network can indicate the bit rates listed in the following table, from 0 to 8Mbps, via the six-bit Bit Rate filed. And the indicated rate can be extended by multiplying with a configured multiplier, which can be 40, 70, 100 or 200.

Table 6.1.3.20-1: Values (kbit/s) for Bit Rate field Index; NR Recommended Bit Rate value [kbit/s]; Index; NR Recommended Bit Rate value [kbit/s]. 0; Note 1; 32; 700. 1; 0; 33; 800. 2; 9; 34; 900. 3; 11; 35; 1000. 4; 13; 36; 1100. 5; 17; 37; 1200. 6; 21; 38; 1300. 7; 25; 39; 1400. 8; 29; 40; 1500. 9; 32; 41; 1750. 10; 36; 42; 2000. 11; 40; 43; 2250. 12; 48; 44; 2500. 13; 56; 45; 2750. 14; 72; 46; 3000. 15; 88; 47; 3500. 16; 104; 48; 4000. 17; 120; 49; 4500. 18; 140; 50; 5000. 19; 160; 51; 5500. 20; 180; 52; 6000. 21; 200; 53; 6500. 22; 220; 54; 7000. 23; 240; 55; 7500. 24; 260; 56; 8000. 25; 280; 57; Reserved. 26; 300; 58; Reserved. 27; 350; 59; Reserved. 28; 400; 60; Reserved. 29; 450; 61; Reserved. 30; 500; 62; Reserved. 31; 600; 63; Reserved. Note 1: For bit rate recommendation message this index is used for indicating that no new recommendation on bit rate is given..

The required range provided by SA4 can be covered by using the multiplier 40, making the maximum value for rate indication become 320Mbps. However, the granularity of the above bit rate table is too coarse comparing to SA4 requirement. The granularity of the above table is about 6%~30%, and majority of the step sizes is larger than 10% for most of the indexes. To meet the requirement for XR rate control, RAN2 should consider enhancements to refine the granularity for rate indication.

Some potential solutions were discussed during the previous meetings, as listed in following agreement from RAN#127bis. We can analyze the following solutions to see how to satisfy the required granularity.

RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - Extend the Bit Rate field - Define a new bit rate table to provide sufficient granularity for XR traffic - Introduce new values for the bitRateMultiplier

To converge the issue, we analyze the pros and cons for each approach.

- Alternative 1: Extend the Bit Rate field.

Currently, the Bit Rate field in the recommended bit rate MAC CE is represented with 6 bits, corresponding to 56 valid code points. A simple way to refine the granularity is to extend the Bit Rate field with more bits so that it can indicate the bit rate in a finer way.

To support the solution, a new bit rate table with more code points would be needed. However, to meet the requirement of 1% granularity from 100kbps to 40Mbps, up to 603 code points are needed if exponential distribution is used, which corresponds to 10 bits. Defining such a large table in the specification seems a bit improper.

Besides, such solution is not future-proof. It closely relies on the requirements of XR applications. For now, it may be feasible to figure out the new code points based on the feedback from SA4, which considered the existing applications and codecs. But it is hard to provide future proof considering the possibility that new applications and codecs with different requirements for rate control may appear.

- Alternative 2: Define a new recommended bit rate table for XR.

This alternative actually works similarly to alternative 1. Both of them need to introduce a new bit rate table. So that alternative 2 has the same disadvantage as alternative 1, i.e., the usage is limited to specific applications and may become unsuitable with the development of new applications.

The difference may be that the bit length of the Bit Rate indication in the MAC CE is not extended for alternative 2, which means there are at most 64 code points, so that the granularity may be larger.

- Alternative 3: Introduce new values for the bit rate multiplier.

The multiplier can be used to extend the range of the recommended bit rate indication. However, it does not change the granularity of the referred table. Considering the legacy table cannot meet the granularity required by SA4, we do not think introducing new values for multiplier is useful without other enhancements.

In our view, alternative 3 can be used together with alternative 1 or 2 to compensate the disadvantages. For example, a new recommended bit rate table with range from 100kbps to 1000kbps and with granularity of 1% can be defined, which costs about 230 code points considering exponential distribution (corresponding to 8 bits). And the range can be extended with some multipliers, e.g., to extend the upper limit to 40Mbps by multiplying 40. To support different range for bit rate recommendation, different multipliers can be used.

But when the multiplier is used, it would be unable to indicate the lower values with the required granularity. For example, assuming the first code point in the new table is 100kbps, when the multiplier 40 is used, the network cannot indicate the bit rates lower than 4Mbps with 1% granularity. To address the issue, dynamic multipliers can be considered. Specifically, in the above example, the network can dynamically use multipliers 1/10/40 to indicate the bit rate in ranges 100~1000kbps, 1000~10000kbps or 10000~40000kbps, while 1% granularity can be always satisfied. To reduce the signaling overhead, the network can pre-configure the multipliers, and indicate which one is used in the recommended bit rate MAC CE.

Based on the above analysis, it can be seen that alternatives 1 and 2 are simple, but they are not future-proof. Once new requirements for range or granularity emerge, the R19-specific table may be unsuitable. Meanwhile, combining alternative 3 with alternative 1 or 2 and supporting dynamic multipliers can be well extended to new scenarios, but it seems a bit more complex. In our view, the additional complexity is not substantial and is justified by the extra flexibility that it brings. We provide TPs for both alternative 1 and the proposed approach to compare the specification impacts based on the above examples.

Proposal 2: RAN2 to refine the recommended bit rate granularity by introducing a new bit rate table and using dynamic multipliers to extend the range of the indication.

Some details of the MAC CE used for XR rate control can be considered based on the above proposals. First, to indicate per QoS flow level recommended bit rate, the QoS flow ID should be included. However, the QoS flow ID is not unique within the UE. To avoid ambiguity, the PDU session ID is also needed to help identify the QoS flow.

To indicate the recommended bit rate based on the proposed approach, several bits can be used to indicate the data rate with 1% granularity within a small range, and a multiplier can be included to extend the indicated bit rate. The multiplier can be an index of the pre-configured multipliers. Values to extend the upper limit of the recommended bit rate to 40Mbps can be considered.

Proposal 3: Recommended bit rate indication MAC CE for XR should consist of the following fields:

- PDU session ID;

- QoS Flow ID;

- Recommended bit rate indication for indicating bit rate with 1% granularity;

- Multiplier for recommended bit rate indication to extend the range.

2.3 UE assistance information

RAN2 has agreed that the gNB can be indicated for which QoS flows it is possible to perform rate control, but it is not clear whether the information comes from CN or UE. The corresponding agreement is excerpted below.

gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN

The motivation to introduce such indication is because it depends the application implementation whether the data rate can be adjusted according to the recommended bit rate provided by the network. It is possible that some applications do not support such functions and cannot take actions based on the network recommendation. For QoS flows associated with such applications, rate control is not supported actually. The gNB should be aware of this so that it only provides the recommended bit rate for rate control capable QoS flows to adapt the network situation.

Since we are considering rate control for UL traffics, it is natural to obtain the information from UE. It depends on the APP developed at the UE side to apply the recommended bit rate. So the UE can be aware of whether rate control is possible or not for a specific QoS flow by checking with the corresponding APP. Then the UE can provide the information to the network via UAI.

Besides, between the two options of indication from UE and CN, we think that it is more proper to indicate the support of a feature on the Uu interface by the Uu interface itself. Besides, the RAN data rate exposure to CN and RAN recommended bit rate indication to UE are two independent features and it is not proper to couple them together. It is also simpler to let UE provide the above information rather than CN from the specification modelling perspective, because UAI is within RAN2 scope. If CN-based solution is considered, both SA2 and RAN3 would be involved, making the work more complicated.

Proposal 3: UE indicates which QoS flows are applicable for rate control via UAI.

2.4 UL MAC CE for UE desired bit rate

In the legacy ANBR procedure, besides the DL recommended bit rate MAC CE sending by the gNB to the UE, there is also an UL MAC CE which is used to indicate the UE desired bit rate. The UL MAC CE has the same format as the DL MAC CE.

For R19 XR rate control, a similar mechanism can be adopted so that the UE can send the desired bit rate to the gNB. This would make the solution more complete and useful. For example, in case of congestion, the gNB may ask the application to decrease the source rate. After a period of time, the application may want to resume the rate, but it is not sure about whether the network congestion has disappeared or not. A proper way is that the UE notifies the gNB about its desired bit rate, then the gNB can evaluate whether the desired rate can be provided and indicate a new recommended bit rate to the UE as the response.

The UE application may also want to change the bit rate due to some user behaviors, such as adjustments on the resolution of the uploaded video. To better coordinate with the network, the UE can indicate the desired rate to the gNB so that the gNB can prepare the scheduling accordingly, or further negotiate the bit rate with the UE.

Observation 4: UL MAC CE for UE desired bit rate can be applicable for UE-initiated change of data rate in the application layer.

To support desired bit rate indication, the UL MAC CE should be enhanced in the same way as the DL MAC CE for bit rate recommendation, so that both MAC CEs can fit XR applications.

Proposal 4: The UL MAC CE with the same enhancements as the DL MAC CE should be supported to allow the UE to indicate the desired bit rate for XR applications.

3. Conclusion

In this contribution, we analyzed the potential enhancements for XR rate control. The following observations and proposals were made:

Per QoS flow v.s per DRB MAC CE:

Observation 1: Per DRB MAC CE is inaccurate for QoS flow level rate control and may cause some risk in QoS guarantee.

Observation 2: It is feasible and up to implementation for the gNB-DU to provide the QoS flow level recommended bit rate.

Proposal 1: RAN2 to introduce per QoS flow MAC CE for UL recommended rate indication.

Range and granularity for recommended bit rate:

Observation 3: The requirement from SA4 for XR data rate includes:

- The range of the data rate varies from 100kbps to 40Mbps.

- The granularity of 1% is preferred.

Proposal 2: RAN2 to refine the recommended bit rate granularity by introducing a new bit rate table and using dynamic multipliers to extend the range of the indication.

Proposal 3: Recommended bit rate indication MAC CE for XR should consist of the following fields:

- PDU session ID;

- QoS Flow ID;

- Recommended bit rate indication for indicating bit rate with 1% granularity;

- Multiplier for recommended bit rate indication to extend the range.

UE assistance information:

Proposal 3: UE indicates which QoS flows are applicable for rate control via UAI.

UL MAC CE for UE desired bit rate:

Observation 4: UL MAC CE for UE desired bit rate can be applicable for UE-initiated change of data rate in the application layer.

Proposal 4: The UL MAC CE with the same enhancements as the DL MAC CE should be supported to allow the UE to indicate the desired bit rate for XR applications.

4. Reference

[1]. 3GPP TS 38.473: "NG-RAN F1 application protocol (F1AP) protocol specification".

[2]. S4-242182, TSG SA WG4, Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications.

[3]. 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".