3GPP TSG-RAN2#129 	R2-2500823

Athens, Greece, 17th Feb – 21th Feb, 2025

Agenda item:		8.18 (TEI19)

Source:	LG Electronics Inc.

Title: 	BWP restriction in MT-SDT

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 agreed to address the issue that TB size of DL transmission in MT-SDT may be small due to CORESET#0 restriction in initial DL BWP, which results in a large number of subsequent transmission.

R2-2410542 Quality Indication in Msg3 for SDT Ericsson discussion Rel-19 TEI19 Observation 1 MT-SDT operation over initial BWP restricts DL MT-SDT transmissions to the bandwidth of CORESET 0 in RRC_INACTIVE state. Observation 2 MT-SDT BWP restriction with no DL quality feedback results in smaller, more conservative TB allocations in the downlink at low MCS values. For larger data volumes, smaller TBS translate to a higher number of subsequent transmissions. Proposal 1 RAN2 addresses the BWP restriction in MT-SDT by introducing a Channel Quality Indication (CQI) in Msg3 or MsgA. Option 1: Remove the CORESET 0 restriction for DL BWP for SDT from/after first DL MT-SDT data Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or MsgA - ZTE has sympathy on the problem statement but not sure about the solution, we should just remove the restriction. The CQI solution is not addressing the problem identified. - Qualcomm thinks that removing the BWP restriction is not a good choice. If we don’t reports CQI in msg3 it will impact RAN4 as well. - LG thinks that the BWP restriction is the root cause and we should remove it. Nokia agrees. - Sony explains that a similar issues was identified in MBS and we agreed to expand coreset 0, introduce CFR. We can reuse that solution. RAN2 will address this issue. FFS the exact solution Noted

In this contribution, we present our view on the issue and propose a solution.

2.	Discussion

Currently, MT-SDT data is transmitted over initial DL BWP only and initial DL BWP is restricted to CORESET#0 in RRC_INACTIVE. Thus, a low number of PRBs can be allocated within CORESET#0.

Meanwhile, when the network receives a request for MT-SDT, i.e. RRCResumeRequest, the network does not know downlink channel quality, so the network may use a conservative MCS (i.e. low MCS).

Due to low PRBs and low MCS, the network schedules small TB size for initial DL transmission, which result the issue of a large number of subsequent transmission. In the last meeting, many companies had some sympathy for this issue and RAN2 agreed to address this issue.

In our view, the root cause of this issue are: 1) the network uses a low MCS, 2) initial DL BWP is restricted to CORESET#0 in RRC_INACTIVE and 3) MT-SDT operation is restricted to initial DL BWP. Therefore, this issue can be addressed by eliminating one of root causes, and we think there are three options.

Option 1. CQI reporting with RRCResumeRequest

In Option 1, CQI reporting is transmitted with RRCResumeRequest. In this solution, the network can know downlink channel quality and can use a better MCS at good downlink channel quality.

In our understanding, currently, UE does not transmit CSI reporting in RRC_INACTIVE (including SDT procedure) until the UE receives DCI to request aperiodic CSI reporting. Thus, the new configuration for CSI reporting in RRC_INACTIVE needs to be introduced, and this brings RAN1 impact.

Even if the new configuration for CSI reporting in RRC_INACTIVE is introduced, Option 1 may impact on RAN4 specification such as CSI reporting requirement upon MT-SDT initiation, e.g., until when the UE should be capable to transmit valid CQI reporting. Similar requirement is SCell Activation delay requirement and this is specified in RAN4 specification [1] as a time requirement within which the UE should be capable to transmit valid CSI reporting upon receiving SCell activation command.

Moreover, even if CQI reporting solution is introduced, initial DL BWP is still restricted to CORESET#0 in RRC_INATIVE, so TB size may not be scheduled large enough.

Therefore, we think that CQI reporting solution is not desirable.

Option 2. Remove CORESET#0 restriction in initial DL BWP for MT-SDT

In Option 2, CORESET#0 restriction in initial DL BWP is not applied if MT-SDT is initiated. By removing CORESET#0 restriction, more number of PRBs can be allocated by using CORESETs other than CORESET#0 and the network can schedule MT-SDT data.

However, Option 2 may bring significant impact on RAN 1 specifications. This is because the UE behaviour on initial DL BWP in RRC_INACTIVE is already specified based on CORESET#0 restriction. Thus, RAN1 may need much time to discuss removing CORESET#0 restriction, and we think that this is also not desirable option.

Option 3. Remove initial BWP restriction for MT-SDT

In Option 3, the UE performs MT-SDT on a separate BWP other than initial BWP. By removing initial BWP restriction, the network can schedule MT-SDT data via the separate BWP that is configured with CORESETs other than CORESET#0 and more number of PRBs can be allocated.

Generally, MT-SDT is considered to be performed on initial BWP only. However, strictly speaking, for RedCap UE, MT-SDT can be performed on a separate BWP other than initial BWP.

For Redcap UE, both RA based MT-SDT and CG-SDT based MT-SDT can be performed using RedCap initial BWP, i.e., initialDownlinkBWP-RedCap and initialUplinkBWP-RedCap. Although the name has the format of initial BWP, technically, RedCap initial BWP is additional BWP that is separately configured from initial BWP.

Thus, similar to RedCap, if the separate BWP is allowed for MT-SDT, MT-SDT can be performed using the separate BWP, and if the separate BWP is configured with CORESETs other than CORESET#0, and large TB size can be scheduled.

In details, for RA/CG-SDT based MT-SDT, the configuration for RA procedure and CG-SDT are configured for separate DL and UL BWP. Upon RA/CG-SDT based MT-SDT initiation, UE performs RA/CG-SDT based MT-SDT using separate DL and UL BWP.

Proposal 1. In order to address the issue of small size TB in MT-SDT, RAN2 agrees to remove the restriction of initial BWP in MT-SDT.

If initial BWP restriction is removed for MT-SDT, MO-SDT on the separate BWP can be also considered.  Both RA-SDT and CG-SDT, i.e. Rel-17 MO-SDT, are allowed for RedCap initial BWP, so we think MO-SDT can be performed on the separate BWP.

Proposal 2. RAN2 agrees to remove the restriction of initial BWP in MO-SDT.

One thing to be confirmed by RAN1 is whether the configuration for RA procedure, RA-SDT and CG-SDT can be configured for the separate BWP. In Rel-17 MO-SDT, RAN2 agreed to introduce separate BWP for MO-SDT, but at that time, RAN1 did not reach to consensus whether CG-SDT can be configured to the separate BWP and RAN2 did not introduce the separate BWP.

However, as mentioned above, for RedCap, the configuration for RA procedure, RA-SDT and CG-SDT is already allowed to be configured for additional BWP, i.e., RedCap initial BWP. Thus, we believe that there is no big technical problem to configure the configuration to the separate BWP.

Anyway, since RAN1 confirmation is needed, it would better to send LS to ask whether the configuration for RA procedure, RA-SDT and CG-SDT is allowed to be configured the separate BWP.

Proposal 3. RAN2 sends LS to ask whether the configuration for RA procedure, RA-SDT and CG-SDT is allowed to be configured the separate BWP.

3.	Conclusion

In this document, we discuss the issue that that TB size of initial DL transmission is small in MT-SDT and made proposals as follows.

Proposal 1. In order to address the issue of small size TB in MT-SDT, RAN2 agrees to remove the restriction of initial BWP in MT-SDT.

Proposal 2. RAN2 agrees to remove the restriction of initial BWP in MO-SDT.

Proposal 3. RAN2 sends LS to ask whether the configuration for RA procedure, RA-SDT and CG-SDT is allowed to be configured the separate BWP.

4.	Reference

- R2-2410542	Quality Indication in Msg3 for SDT	Ericsson

- TS 38.133	Requirements for support of radio resource management	V18.8.0