3GPP TSG-RAN2#129 	R2-2500824

Athens, Greece, 17th Feb – 21th Feb, 2025

Agenda item:		8.7.3 (NR_XR_Ph3-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on enhancement for MG skipping

Document for:	Discussion and Decision

1.	Introduction

In RAN2#127bis, RAN2 agreed to discuss whether RRC-based MG skipping is needed.

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

In this contribution, we present our view on RRC-based MG skipping.

2.	Discussion

According to MAC specification [1], the UE shall not perform the data transmission/reception during a measurement gap except for RACH procedure.

## 5.14 Handling of measurement gaps During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]: 1> not perform the transmission of HARQ feedback, SR, and CSI; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.5, and 5.7. 1> else: 2> not monitor the PDCCH; 2> not receive on DL-SCH.

Thus, in Rel-19 XR, RAN1 made a working assumption to introduce DCI-based MG skipping to enable data transmission/reception during the measurement gap, as follows.

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

In DCI-based MG skipping solution, the network indicates to skip the measurement gap based on UE’ status, e.g. mobility status, delay status of data and etc.

However, the network may not know the UE’s status for the urgent uplink data, e.g. DSR triggering or delay critical data. Thus, we think that an additional method is needed other than DCI-based MG skipping and the additional method can be RRC-based MG gap skipping. For example, the network configures the measurement gap that can be skipped for the urgent uplink data, i.e. skippable measurement gap, and the UE is allowed to transmits the urgent uplink data during the skippable measurement gap if UL resource and the skippable measurement gap is overlapped.

Proposal 1. RAN2 supports RRC-based MG gap skipping method for the urgent uplink data, e.g. DSR triggering or delay critical data.

Meanwhile, the UE does not always transmit the urgent uplink data during the skippable measurement gap. Thus, the network only needs to decode the uplink channel when there is the urgent uplink data. However, there is no way to know whether there is urgent uplink data in the network side.

Therefore, the UE should indicate to the network if the UE has the urgent uplink data. By receiving the indication, the network decodes the uplink channel during the skippable measurement gap, otherwise, the network does not decode the uplink channel.

We think that whether the UE has the urgent uplink data can be indicated by using existing signaling. For example, if the UE has the urgent uplink data which needs to be transmitted within the skippable measurement gap but the UE does not have UL grant, the UE transmits SR prior to the skippable measurement gap. Upon receiving the SR, the network indicates MG skipping and schedules UL grant to allow to transmit the urgent data during the skippable measurement gap.

The other example is that if CG occasion is overlapped with the skippable measurement gap and the UE has the urgent data, the UE indicates the use of the CG occasion by using UTO-UCI prior to the skippable measurement gap and transmits the urgent uplink data.

Proposal 2. UE sends an indication prior to the skippable measurement gap to indicate that there is the urgent uplink data.

However, we don't think taht the measurement gap should be always skippable for the urgent uplink data. This is because it may impact on the mobility performance, e.g. handover. Thus, it would be desirable to allow MG skipping at good radio channel conditions. If radio condition is good, the measurement may not be indispensable, but if radio condition is bad, it makes sense that the UE performs the measurement even in the skippable measurement gap.

Proposal 3. Whether to perform MG skipping is determined based on the radio channel condition.

3.	Conclusion

In this document, we discuss RRC-based MG skipping and make proposals as follows.

Proposal 1. RAN2 supports RRC-based MG gap skipping method for the urgent uplink data, e.g. DSR triggering or delay critical data.

Proposal 2. UE sends an indication prior to the skippable measurement gap to indicate that there is the urgent uplink data.

Proposal 3. Whether to perform MG skipping is determined based on the radio channel condition.

4.	Reference

- TS 38.321	Medium Access Control (MAC) protocol specification	V18.4.0