3GPP TSG-RAN2#129 	R2-2500825

Athens, Greece, 17th Feb – 21th Feb, 2025

Agenda item:		8.12.2 (NR_MIMO_Ph5-Core)

Source:	LG Electronics Inc.

Title: 	Consideration on Asymmetric DL sTRP/UL mTRP

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 discussed issues related to asymmetric DL sTRP/UL mTRP and made some progress.

In this contribution, we discuss additional RAN2 impacts of asymmetric DL sTRP/UL mTRP.

2.	Discussion

In the last meeting, for MAC CE for PL offset update, RAN2 made some agreements as follows.

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB.

In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

In addition to above agreements, some discussion points are still remained for the completion of MAC CE design as follows.

- Point 1. How to indicate the association PL offset and joint/UL TCI state in the new MAC CE

- Point 2. Other contents (e.g. CORESET POOL ID, D/U field and BWP ID field)

Point 1. How to indicate the association PL offset and joint/UL TCI state in the new MAC CE

In the last meeting, RAN2 agreed that PL offset value can be updated for any configured joint/UL TCI state with RRC configured PL offset.

Thus, the new MAC CE should include the information of configured joint/UL TCI state IDs and PL offset values and indicate which joint/UL TCI state’s PL offset is updated to which value, i.e. the association between the configured joint/UL TCI state ID and PL offset value.

In order to indicate the information of the configured joint/UL TCI state ID and PL offset value and the association, we think there is two options.

- Option 1. Bitmap mapping for TCI state ID + PL offset value

- Option 2. Configured TCI state ID + PL offset value

Figure 1. Example of New MAC CE design

In Option 1, Ti field indicates TCI state ID. If Ti is set to 1, a PL offset value of the corresponding TCI state ID is present and PL offset of TCI state ID is updated to the corresponding value.

In Option 2, TCI sate IDi field includes TCI state ID. If TCI state ID is included, a corresponding PL offset value is present and PL offset of TCI state ID is updated to the corresponding value.

Comparing Option 1 and Option 2, if a large number of PL offsets are expected to be updated at the same time, Option 1 is more desirable, otherwise, less than for 16 TCI states or 8 TCI states, Option 2 seems more desirable.

Proposal 1. RAN2 discusses how to indicate TCI state ID considering following options

- Option 1. Bitmap mapping for TCI state ID + PL offset value

- Option 2. Configured TCI state ID + PL offset value

Point 2. Other contents (e.g. CORESET POOL ID, D/U field and BWP ID field)

To discuss which contents should be included in the new MAC CE, it would be helpful to refer contents of (Enhanced) Unified TCI state activation MAC CE.

Contents of (Enhanced) Unified TCI state activation MAC CE and our analysis are as follows:

| Contents | Analysis |
| --- | --- |
| Serving Cell ID | Serving Cell ID is needed in the new MAC CE to indicate the identity of Serving Cell for which the new MAC CE applies. Thus, One Serving Cell ID field is needed. |
| CORESET POOL ID | For asymmetric DL sTRP/UL mTRP scenario, sDCI method is used, while CORESET POOL ID is used for mDCI method. Thus, CORESET POOL ID is not needed. |
| UL/DL BWP ID | For joint mode, TCI-State IE is configured in DL BWP, and for separate mode, TCI-UL-State IE is configured in UL BWP. Considering unified TCI state mode (i.e. unifiedTCI-StateType) is configured per Serving Cell, TCI state to which PL offset is applied is configured either DL BWP or UL BWP. Thus, one BWP field is needed, i.e. two BWP field for DL BWP ID and UL BWP ID are not needed. |
| D/U | For joint mode, PL offset value is associated with joint TCI state, so D/U field is not used for joint mode. For separate mode, PL offset is associated with UL TCI State and PL offset for DL TCI state is not present, so D/U field is not needed. Therefore, D/U field is not needed. |

Based on above analysis, we propose to design the new MAC CE considering follows.

- One Serving Cell field and one BWP field are included

- CORESET Pool ID and D/U field are not included.

Proposal 2. RAN2 design the new MAC CE considering follows.

- One Serving Cell field and one BWP field are included

- CORESET Pool ID field and D/U field are not included

In addition, RAN1 also agreed to support applying PL offset on PDCCH-order based PRACH as follows.

[RAN1#116bis] Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification. - Above is subject to a separate UE capability signaling. [RAN1#118] Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3: - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission. - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not.

When one of joint/UL TCI state is indicated by PDCCH-order, a PL offset associated with the indicated joint/UL TCI state is applied on PDCCH order PRACH transmission and Tx power is calculated based on the PL offset.

However, it is not clear whether it refers to both PDCCH-order CBRA and PDCCH-order CFRA or not. In our understanding, the UE does not know RACH preamble is transmitted to UL/DL TRP or UL-only TRP because there is no association between RA configuration and TRP.

Thus, in order to indicate PL offset only for PRACH transmission towards UL-only TRP, PDDCH-order PRACH should be CFRA.

Proposal 3. RAN2 confirms that PL offset for PDCCH-order PRACH transmission is provided only for PDCCH-order CFRA.

For PL offset on PDCCH-order PRACH, from RAN2 point of view, we think no further RAN2 work is needed except RRC configuration to associate PL offset with joint/UL TCI state.

Proposal 4. RAN2 assumes that no RAN2 work is needed for PL offset on PDCCH-order based PRACH, except RRC configuration.

In addition, for PHR calculation related to PL offset, RAN1 made agreements as follows.

[RAN1#118] Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH. Agreement Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states. - Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

Based on RAN1 agreements, PL offset is applied to Type 1 PHR calculation and uplink power can be changed with PL offset value. Thus, in the last meeting, some companies proposed a new PHR triggering condition considering PL offset update.

According to the current MAC specification, if pathloss for PL RS (PathLoss Reference Signal) has changed more than a threshold, PHR is triggered. In our understanding, PL offset is one of factor to calculate pathloss and the pathloss for PL RS is calculated in the physical layer. Thus, the MAC does not know that pathloss has changed more than the threshold.

Therefore, in RAN2 point of view, a new PHR triggering condition considering PL offset is not needed.

Proposal 5. New PHR triggering condition considering PL offset is not needed.

Meanwhile, according to [1], one of objectives is to apply 2-TAs for asymmetric DL sTRP/UL mTRP scenario reusing Rel-18 specification. In Rel-18, 2-TAs are introduced only for mDCI case, but for asymmetric DL sTRP/UL mTRP, only sDCI is possible, so 2-TAs method needs to be extended considering sDCI.

In order to apply 2-TAs for asymmetric DL sTRP/UL mTRP, RAN1 made agreements and sent LS [2] as follows.

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped. “RAN2 could check whether this RAN1 agreement would cause any MAC impact”

According to RAN1 agreement, almost things are inherited from Rel-18 2-TAs principle. However, for sDCI, coresetPoolIndex is not configured and SRS resource set is used to distinguish TAG, so RAN1 agreed to remove the restriction of of coresetPoolIndex.

In the current RRC specification, 2-TAs can be configured only when two coresetPoolIndex is configured, so the field description needs to be fixed for asymmetric DL sTRP/UL mTRP.

tag2 This field to indicate the second TAG information for the serving cell, it is optionally configured in a serving cell if and only if the serving cell is configured with more than one value for the coresetPoolIndex.

Considering the restriction is still needed for mDCI case, RAN2 needs to discuss how to remove the restriction of coresetPoolIndex in the field description of tag2.

Proposal 6. RAN2 discusses how to remove the restriction of coresetPoolIndex in the field description of tag2.

3.	Conclusion

In this document, we analyse RAN2 impacts from asymmetric DL sTRP/UL mTRP, and made proposals as follows.

Proposal 1. RAN2 discusses how to indicate TCI state ID considering following options

- Option 1. Bitmap mapping for TCI state ID + PL offset value

- Option 2. Configured TCI state ID + PL offset value

Proposal 2. RAN2 design the new MAC CE considering follows.

- One Serving Cell field and one BWP field are included

- CORESET Pool ID field and D/U field are not included

Proposal 3. RAN2 confirms that PL offset for PDCCH-order PRACH transmission is provided only for PDCCH-order CFRA.

Proposal 4. RAN2 assumes that no RAN2 work is needed for PL offset on PDCCH-order based PRACH, except RRC configuration.

Proposal 5. New PHR triggering condition considering PL offset is not needed.

Proposal 6. RAN2 discusses how to remove the restriction of coresetPoolIndex in the field description of tag2.

4.	Reference

- RP-242394	WID revision: NR MIMO Phase 5	Samsung

- R2-2500008	LS to RAN2 on RRC and MAC impacts for Rel-19 NR MIMO Ph5	RAN1