3GPP TSG-RAN2#129 	R2-2500826

Athens, Greece, 17th Feb – 21th Feb, 2025

Agenda item:		8.12.3 (NR_MIMO_Ph5-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on UEI beam reporting impact

Document for:	Discussion and Decision

1.	Introduction

Based on the Rel-19 MIMO WID [1], in the last meeting, RAN2 started to discuss UE-initiated/event-driven beam management (UEI beam management).

In this contribution, we analyse RAN2 impacts of UEI beam reporting based on [1] and RAN1 agreements.

2.	Discussion

According to [1], one of objectives for Rel-19 MIMO WI is to support UEI beam management and RAN1 made following agreements for this aspect.

[RAN1#116] Agreement On UE-initiated/event-driven beam report, at least of following aspects should be included: - Trigger-event detection for beam reporting by UE - UE monitors RS to assess if a beam-reporting trigger condition has been met - FFS: Trigger condition for declaring beam-reporting event - Beam-report transmission by UE - Signaling contents in the beam report - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission - MAC-CE - UCI - Others are not precluded. On UE-initiated/event-driven beam report, the following aspects may be included: - UE requesting UL resource(s) for the beam report - UE notifying transmission of beam report - gNB preconfigured resources Other procedure(s) as required

Based on the above agreement, we can see that UEI beam management is modeled in two steps, i.e., Step A) event detection/trigger and Step B) beam reporting transmission.

Step A) event detection/trigger

For Step A) event detection/trigger, some points need to be determined: Which event triggers beam reporting, Which layer evaluates event for beam reporting, Which RS is used to detect event(s), How to measure RS and Which RS/parameter needs to be configured/updated.

Regarding Which event triggers beam reporting, Which RS is used to detect event(s) and How to measure RS, we think that this is RAN1 scope, and RAN1 already make some progress. Thus, RAN2 doesn’t need to discuss these points again.

Regarding Which layer evaluates event for beam reporting, in the last meeting, some companies think that the event evaluation procedure should capture in MAC and RAN2 should design a common event evaluation procedure for both UEI beam reporting and LTM L1 MR. This is because RAN2 agreed that MAC layer already handles the event evaluation procedure for LTM and both procedures are based on L1 measurement.

Agreements on L1 MR event evaluation - Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. 6. MAC layer handles the event evaluation and measurement report triggering.

We think that the layer that evaluates events for the beam reporting is related to which signaling is used for the beam reporting in Step B).

In the past several meetings, RAN1 agreed to support Mode-A and Mode-B for the beam reporting as follows.

[RAN1#116bis] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode-A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This option is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode-B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all options For above procedures, cross-CC beam reporting is supported for both options. - FFS: Details. [RAN1#118] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least type 1 CG-PUSCH. - FFS: PUCCH as the second channel - FFS: Whether the PUSCH can be with UL data [RAN1#118bis] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least option3 is supported: - Option-1: The same Type-1 CG PUSCH can carry UL-SCH, any other UCI, and the beam report. - Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI. - Option-3: The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can carry any other UCI. FFS: whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report. [RAN1#119] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH does NOT carry the beam report, for the second UL channel in Mode-B, Option-2 is supported: - Option-2: Type-1 CG PUSCH can NOT be transmitted

According to RAN1 agreement, if the beam reporting is triggered based on events, a new UCI is transmitted for the first PUCCH channel. The new UCI is not RAN2 signaling, and generally MAC does not trigger UCI signaling. Thus, we think that there is no impact of the event evaluation procedure on RAN2 specification and it would be not desirable to capture the event evaluation procedure in MAC specification.

In addition, even if the event evaluation procedure is captured in MAC specification, it is not sure whether a common event evaluation procedure can be designed for both UEI beam reporting and LTM. This is because the RS configuration for L1 measurement is different, and reporting signaling is different, i.e., UCI in the second UL channel (PUSCH) for UEI beam reporting and MAC CE for LTM L1 MR.

Observation 1. For both the first PUCCH and the second UL channel, L1 signaling is used, i.e. UCI.

However, whether the event evaluation procedure for UEI beam reporting is in MAC or PHY is still remained as FFS in RAN1. Thus, although we think that the event evaluation procedure should be captured in RAN1 specification, it would be better to wait RAN1 further progress whether the event evaluation procedure is captured in MAC specification or PHY specification.

Therefore, we propose to wait RAN1 further progress on UEI beam reporting before starting to discuss MAC impact by UEI beam reporting.

[RAN1#117] Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

Proposal 1. RAN2 waits for RAN1 further progress on UEI beam reporting before starting to discuss MAC impact by UEI beam reporting.

Step B) beam reporting transmission

Based on the above RAN1 agreements, Mode-A and Model-B can be modeled as follows:

- Mode-A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (new UCI) to request a resource for a second UL channel to carry beam report.

  - Step 2: UE detects the UL-grant DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted on second UL channel – PUSCH (CSI reporting).

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (new UCI) notifying a second UL channel to carry beam report.

  - Step 2: UE transmits the beam report in the second UL channel - Type 1 CG PUSCH. (CSI reporting)

For Step 1 of Mode-A and Mode-B, as mentioned above, RAN1 agreed to use a new UCI and we think that generally MAC does not trigger UCI signaling. Thus, there is no impact of the event evaluation procedure on RAN2 specification, except RRC configuration of the first PUCCH configuration.

Observation 2. For Step 1 of Mode-A and Mode-B, there is no RAN2 impact except RRC configuration of first PUCCH configuration.

For Step 2 and Step 3 of Mode-A, the UE receives a DCI to indicate to transmit beam reporting, then the UE transmits beam reporting. This is similar to the legacy aperiodic CSI reporting and we think there is no RAN2 impact.

Observation 3. For Step 2 and Mode 3 of Mode-A, there is no RAN2 impact except RRC configuration.

For Step 2 of Mode-B, since the beam reporting is transmitted on CG-PUSCH, it may be thought that RAN2 may need to discuss how to handle the overlapping case between the CG-PUSCH and other PUSCH/PUCCH.

However, RAN1 agreed that CG PUSCH cannot carry UL-SCH and can carry the beam reporting and other UCI. This means that CG PUSCH for the beam reporting can be handled in the same way as the legacy overlapping case between the legacy CSI reporting on PUSCH and other PUSCH/PUCCH. Thus, we think there is no RAN2 impact, except RRC configuration for CG-PUSCH.

Observation 4. For Step 2 of Mode-B, there is no RAN2 impact except RRC configuration of CG-PUSCH.

Based on Observation 2, 3 and 4, we propose that RAN2 doesn’t start to discuss beam reporting transmission until there is RAN1 input.

Proposal 2. RAN2 does not start the discussion on beam reporting transmission until there is RAN1 input.

For the RRC configuration for the beam reporting, RAN1 made some agreements and sent LS [2] to inform of RRC parameter list in the last meeting.

However, the RRC parameter list for UEI beam reporting is incomplete and many things are still under discussion in RAN1. We think that it is sufficient to start RAN2 discussion on RRC configuration after RAN1 makes further progress.

Proposal 3. RAN2 starts the discussion of the RRC configuration for UEI beam reporting after RAN1 makes further progress.

3.	Conclusion

In this document, we analysis RAN2 impacts of UEI beam reporting, and made observations and proposals as follows.

Observation 1. For both the first PUCCH and the second UL channel, L1 signaling is used, i.e. UCI.

Observation 2. For Step 1 of Mode-A and Mode-B, there is no RAN2 impact except RRC configuration of first PUCCH configuration.

Observation 3. For Step 2 and Mode 3 of Mode-A, there is no RAN2 impact except RRC configuration.

Observation 4. For Step 2 of Mode-B, there is no RAN2 impact except RRC configuration of CG-PUSCH.

Proposal 1. RAN2 waits for RAN1 further progress on UEI beam reporting before starting to discuss MAC impact by UEI beam reporting.

Proposal 2. RAN2 does not start the discussion on beam reporting transmission until there is RAN1 input.

Proposal 3. RAN2 starts the discussion of the RRC configuration for UEI beam reporting after RAN1 makes further progress.

4.	Reference

- RP-242394	WID revision: NR MIMO Phase 5	Samsung

- R2-2500008	LS to RAN2 on RRC and MAC impacts for Rel-19 NR MIMO Ph5	RAN1