3GPP TSG-RAN WG2 Meeting #129	R2-2500835

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.1.2.2

Title:	Continuous Discussion on LCM for UE side Model

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In RAN2#128[2], following agreements were reached:

Agreements

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

The network can decide when to start/stop the data collection and send configuration.

The network can configure whether UE is allowed to initiate request for data collection.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

We provide further consideration based on the previous RAN2 agreements and RAN1 LS[2].

Discussion

  - On applicability reporting

In RAN2, it is still FFS if inference configuration can be signaled in step 3. According to RAN1 LS, RAN1 has agreed following agreement regarding the content in step 3, step 4 and step 5[2]:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig Any specification impact is a separate discussion

As to what can be included in step 3, RAN1 came to an agreement with options: A) and B). For option A), one or more inference configurations are provided in step 3. Regarding option B), step 3 encompasses one or more set of inference related parameters for applicability report only.

Observation 1: According to RAN1 LS, there are two options on what can be included in step 3 for applicability reporting:

(A) one or more of CSI-ReportConfig for inference configuration

(B) One set or multiple sets of inference related parameters for applicability report only

We understand these two options are suitable for different scenarios. For example, if network prefers a functionality to be activated immediately after UE report applicability, option A can be used. Otherwise, network can choose option B and configure the inference configuration in step 5. After all, it is up to network implementation to decide which option is selected.

However, in option (A), if periodic CSI report configuration is provided in step 3 for inference, network has to prepare resources before step 3 because the UE may activate it after reporting applicability in step 4. But not all functionalities configured in step 3 are applicable. So when the NW receives non-applicable report for a functionality, the NW has to release or deactivate the non-applicable periodic CSI report configuration with additional signaling. This is different from legacy periodic CSI report configuration which is assumed activated upon configuration.

As for the semi-persistent and aperiodic CSI report configuration, it will not be activated autonomously by UE. It is fine to be configured in step 3 and NW activate it explicitly after UE report applicability.

Observation 2: If periodic CSI report for inference configuration is provided in step 3, the NW has to reserve resource for the functionality. If the functionality is non-applicable, the NW has to release or deactivate the non-applicable periodic CSI report configuration with additional signaling.

Based on above consideration, we propose that for option (A), only semi-persistent and aperiodic CSI report configuration for inference are supported. The periodic CSI report configuration for inference can only be configured in step 5.

Proposal 1: For option (A), only semi-persistent and aperiodic CSI report configuration are supported in step 3. Periodic CSI report for inference configuration can only be provided in step 5.

In last RAN2 meeting, one FFS issue related to applicability reporting is whether UE reports explicitly “non-applicable” functionality.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability. Verify this aligns with RAN1 configuration design

Many companies have pointed out that one of the reasons for explicitly reporting non-applicable functionality is to convey the cause of non-applicability to the NW. We think this is an valid argument. Once the NW receives the indicated reason for non-applicability, it can behave accordingly.

For example, if the reason is that the UE has no available model and training data is requested, the NW can decide to configure training data collection configuration for the UE. If the reason is mobility status is not applicable, the NW may decide to not release inference configuration in case the functionality will become applicable later.

Observation 3: It is beneficial for the NW to be informed of the reason why a functionality is non-applicable when UE reports its applicability. The NW can react accordingly according to the reason.

If non-applicable functionality is reported implicitly, all applicable functionalities of all configured functionalities need to be reported to the NW. And based on that, the NW derives which functionalities are non-applicable.

This method is obviously inefficient when the number of changed functionality is smaller than the number of configured functionality. For example, when UAI is triggered when there is change of applicability, it is most likely the number of changed functionalities is one. There is no reason to report all applicable functionalities just because one functionality is changed.

Even for the applicability reporting in step 3/step 4, all applicable functionalities, including those are configured via previous RRCReconfiguration needs to be reported as well as those are configured in this RRCReconfiguration. This is because in step 3/step 4, a previous applicable functionality may becomes non-applicable due to the activation of new functionality results in short of hardware capacity. Thus the UE needs to either report all applicable functionalities or report non-applicable functionality explicitly.

So, for both RRCReconfigurationComplete and UAI case, to report all applicable functionality each time instead of report non-applicable functionality explicitly will results in inefficiency in termof signaling overhead.

Observation 4: If non-applicable functionalities are reported implicitly, whenever there is a change of applicability all applicable functionalities that are configured to the UE must be reported. This is not efficient in terms of signaling overhead.

Based on above analysis, we propose to have a common signaling design for applicability reporting via RRCReconfigurationComplete and UAI: to report the change of applicability of configured functionalities by indicating applicable and non-applicable functionalities explicitly.

Proposal 2: UE can report change of applicability by explicitly including the functionalities that becomes “applicable” and “non-applicable” in RRCReconfigurationComplete and UAI.

It was agreed that both UAI and RRCReconfigurationComplete can be used to report applicability functionality. In addition, we think RRCResume/RRCResumeComplete can also be used for applicability reporting which can bring up some benefits to quick activate AI/ML based feature when UE is transited from RRC_INACTIVE to RRC_CONNCETED.

During the RRC resume procedure, network can request applicability reporting based on UE capability. The UE can response in RRCResumeComplete immediately. This procedure is similar to RRCReconfiguration and RRCReconfigurationComplete. To report applicable functionality in RRCResumeComplete is beneficial to have fast configuration and activation of AI/ML functionalities during RRC resume procedure.

One difference between RRCResume and RRCReconfiguration message is that there is no otherConfig IE in RRCResume message. But the network can transmit another RRCReconfiguration which includes otherConfig if the network wants the UE to do UAI reporting.

Proposal 3: RRCResume/RRCResumeComplete can be used for applicability reporting.

  - On functionality activation/deactivation

If periodic CSI report configuration is provided in step 3 for inference configuration, it is considered as activated only if applicability is reported in step 4. The non-applicable CSI report configuration is not considered as activated. Another similar case is that when a functionality becomes non-applicable from applicable, the periodic CSI report configuration for inference configuration will also be deactivated by NW.

Observation 5: There are two cases that periodic CSI report configuration is configured but is non-applicable for inference:

Case 1: Periodic CSI report configuration is configured in step 3 but is non-applicable.

Case 2: Periodic CSI report configuration is configured in step 3 or step 5, but becomes non-applicable later.

In both of the above cases, there are two options for the NW to handle the non-applicable periodic CSI report configuration, and how to activate the functionality when it becomes applicable:

- Option 1: To activate/deactivate the functionality by configuring/releasing the periodic CSI report configuration.

With option 1, we don’t need to introduce activation/deactivation mechanism for periodic CSI report configuration. If NW wants the UE to keep applicability evaluation, it can configure the UE with inference related parameters for applicability report only (not for inference). The drawback is that another reconfiguration with periodic CSI report configuration for inference is needed when the functionality becomes applicable.

- Option 2: To activate/deactivate the functionality by activating/deactivating the periodic CSI report configuration without release it.

With option 2, the deactivated periodic CSI report configuration can still be used for applicability evaluation. And with a simple activation indication, the functionality can be activated again. However, we need to introduce activation/deactivation mechanism for periodic CSI report configuration which is not introduced in legacy CSI framework.

Proposal 4: RAN2 to decide how to activate/deactivate the functionality that is configured with periodic CSI report configuration for inference:

- Option 1: To activate/deactivate the functionality by configuring/releasing the periodic CSI report configuration.

- Option 2: To activate/deactivate the functionality by activating/deactivating the periodic CSI report configuration without release it.

It was agreed in RAN2#128 that “When a functionality configured by the network to be reported via UAI becomes from non-applicable to applicable, the UE can reports it to the network.”

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

However, It is unclear whether UE can autonomously activate the inference configuration when the functionality becomes applicable, especially for the case that periodic CSI report configuration is configured for inference and it can be activate/deactivate by the NW.

- Option 1: UE activates autonomously the functionality after it reports applicability to NW via UAI.

Option 1 is similar to the activation of periodic CSI report configuration that is provided in step 3. According to RAN1 LS[2], it is considered as activated if UE report applicability in step 4.

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

These two cases are similar: if periodic CSI report configuration is configured to UE, it can be activated by UE autonomously after UE reports applicability to the NW.

However, the drawbacks are also the same: the network side resources needs to be reserved even when the functionality is non-applicable in case the UE will activate it autonomously. It is obviously not efficient for resource usage.

- Option 2: UE keeps the functionality as deactivated after UE reports applicability to NW via UAI and waits for NW’s indication to activate it.

In option 2, the UE expects the NW to activate the functionality. The network needs not to reserve resources before receive applicability reporting. And option 2 is aligned with the following agreement that is reached in RAN2#128 for the case when a functionality becomes non-applicable:

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Based on above consideration, we have following proposal:

Proposal 5: When a functionality becomes applicable from non-applicable, the UE doesn’t autonomously activate the functionality. NW is expected to activate the functionality when it receives report from UE that it is applicable.

Training Data Collection

It was agreed that for data collection configuration for UE-side model training, the UE can send a request for data collection. It is still FFS what the request contains.

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

First of all, the configuration for training data collection should be aligned with the potential inference configuration that the NW will configure for the UE. This is because the UE side model is trained to be applied in the same network in which the training data is collected. Thus it is important for the UE to train the model according to what network will configure for inference. This will ensure that the trained model aligns closely with the actual operational needs of the network, and maximize its effectiveness and utility. Please note that not all set A and set B combination will be configured for inference by the NW.

Observation 6: The UE side model is trained to be applied in the same NW in which the training data is collected. Consequently, the training data collection configuration should be also determined by the NW to ensure the trained model aligns closely with the actual operational needs of the NW.

Secondly, from network point of view, too much flexibility in requesting for training data collection configuration, e.g. UE provides requested combination of set A and set B, will increase network implementation complexity. This is because the NW has to accommodate all kinds of request from different UEs.

And because not all UE provided combination will be configured for inference by the NW, the model that is trained according to requested combination may be useless in the end.

Observation 7: Excessive flexibility in UE request for training data collection configuration, such as UE provides combination of set A and set B, will lead to unnecessary increase in NW side complexity. And not all UE requested configuration will be configured for model inference.

Thus, we preferred the approach that the network provides candidate training data collection configurations for UE to request. The training data collection may include measurement resource configuration and associated ID, or only necessary parameters such as set A and set B information and associated ID. The exact details can be further discussed.

In addition to candidate training data collection configuration, the UE should also be able to request for data collection based on an inference configuration, e.g. when the inference configuration is non-applicable because has no suitable UE side model and UE wants to collect data for UE side model training.

As to the associated ID, it is still unclear in RAN1 on the content of NW-side additional condition. So it is unclear how UE can determine to request data collection configuration that is associated with which associated ID.

Therefore, the candidate configuration could be one or more of following options:

- Option 1: A list of candidate training data collection configuration provided by NW, including measurement resource configuration for set A and set B, and associated ID.

- Option 2: A list of candidate data collection related parameters provided by NW, e.g. set A and set B combination, and associated ID.

- Option 3: A list of inference configuration that can also be requested for data collection.

- Option 4: FFS: a list of associated ID.

The UE request the training data collection configuration by indicating the ID of the candidate configuration. And the NW configure the training data collection configuration based on UE’s request.

Proposal 6: The NW provides a list of candidate configurations and UE perform request based on the candidate configurations. RAN2 to decide which following option(s) are adopted as the candidate configurations:

- Option 1: A list of candidate training data collection configuration, including measurement resource configuration for set A and set B, and associated ID.

- Option 2: A list of candidate training data collection related parameters, e.g. set A and set B combination, and associated ID.

- Option 3: A list of inference configuration that can also be requested for training data collection.

- Option 4: FFS: a list of candidate associated ID.

As to the signaling used to provide the candidate configuration, we think RRCReconfiguration message can be used. FFS whether SI message can also be used.

Proposal 7: RRCReconfiguration can be used to provide the candidate configuration for data collection. FFS whether SI message can also be used.

As to the signaling used to request for the data collection configuration, we think both RRCReconfigurationComplete and UAI can be used. When the candidate configurations are provided in RRCReconfiguration, the UE can request the configuration via RRCReconfigurationComplete as response. And the UE should also be able to request the configuration later via UAI.

Further RRCReconfigurationComplete and UAI can be also used to request to stop data collection to the NW.

Proposal 8: Both RRCReconfigurationComplete and UAI can be used by UE to request to start and stop data collection.

In the last RAN2 meeting, one FFS issue related to training data collection configuration is as following:

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

In our understanding, there are following cases in which the UE cannot perform data collection based on received configuration. In either case, it is necessary for NW to be indicated that the configuration can’t be performed. Thus the NW can release the configuration to save network resources.

Case 1: The UE request training data collection configuration and network configures according to UE request. But later, the UE can’t perform data collection due to some reasons, e.g. short of hardware capacity. For this case, the UE should be able to indicate to NW via UAI to stop the data collection.

Case 2: NW configures training data collection via RRCReconfiguration without UE request. And the UE cannot perform data collection based on the received configuration. For this case, the UE should be able to indicate in RRCReconfigurationComplete to stop the data collection.

Based on above considerations, we think UE can indicate to NW to stop data collection via RRCReconfigurationComplete or UAI when it can’t perform data collection based on the received configuration. FFS whether the detailed reasons are needed.

Proposal 9: When UE can’t perform data collection based on received configuration it should indicate NW to stop data collection via RRCReconfigurationComplete or UAI. FFS whether detailed reasons are needed.

In last RAN2 meeting, it was agreed that NW can control the initiation and configuration for data collection:

- The following methods for network control of the initiation and configuration for data collection:

    - The network can decide when to start/stop the data collection and send configuration.

    - The network can configure whether UE is allowed to initiate request for data collection.

Besides, we think it is necessary for the NW to be able to control when the UE can initiate another request to start data collection.

One option is to reuse prohibit timer mechanism. With this mechanism, the UE is configured with prohibit timer value. The prohibit timer is started when UE initiates a request for a data collection configuration via UAI or RRCReconfigurationComplete. The UE should not initiate another UAI to request for the same data collection configuration before the prohibit timer expires.

On the other hand, it seems there is no need to configure prohibit timer for the request to stop data collection.

Proposal 10: UE can be configured with a prohibit timer which is started after UE initiates request for a data collection configuration via RRCReconfigurationComplete or UAI. UE will not initiate UAI for the same data collection configuration again before the prohibit timer expires.

Conclusion

Observation 1: According to RAN1 LS, there are two options on what can be included in step 3 for applicability reporting:

(A) one or more of CSI-ReportConfig for inference configuration

(B) One set or multiple sets of inference related parameters for applicability report only

Observation 2: If periodic CSI report for inference configuration is provided in step 3, the NW has to reserve resource for the functionality. If the functionality is non-applicable, the NW has to release or deactivate the non-applicable periodic CSI report configuration with additional signaling.

Observation 3: It is beneficial for the NW to be informed of the reason why a functionality is non-applicable when UE reports its applicability. The NW can react accordingly according to the reason.

Observation 4: If non-applicable functionalities are reported implicitly, whenever there is a change of applicability all applicable functionalities that are configured to the UE must be reported. This is not efficient in terms of signaling overhead.

Observation 5: There are two cases that periodic CSI report configuration is configured but is non-applicable for inference:

Case 1: Periodic CSI report configuration is configured in step 3 but is non-applicable.

Case 2: Periodic CSI report configuration is configured in step 3 or step 5, but becomes non-applicable later.

Observation 6: The UE side model is trained to be applied in the same NW in which the training data is collected. Consequently, the training data collection configuration should be also determined by the NW to ensure the trained model aligns closely with the actual operational needs of the NW.

Observation 7: Excessive flexibility in UE request for training data collection configuration, such as UE provides combination of set A and set B, will lead to unnecessary increase in NW side complexity. And not all UE requested configuration will be configured for model inference.

Proposal 1: For option (A), only semi-persistent and aperiodic CSI report configuration are supported in step 3. Periodic CSI report for inference configuration can only be provided in step 5.

Proposal 2: UE can report change of applicability by explicitly including the functionalities that becomes “applicable” and “non-applicable” in RRCReconfigurationComplete and UAI.

Proposal 3: RRCResume/RRCResumeComplete can be used for applicability reporting.

Proposal 4: RAN2 to decide how to activate/deactivate the functionality that is configured with periodic CSI report configuration for inference:

- Option 1: To activate/deactivate the functionality by configuring/releasing the periodic CSI report configuration.

- Option 2: To activate/deactivate the functionality by activating/deactivating the periodic CSI report configuration without release it.

Proposal 5: When a functionality becomes applicable from non-applicable, the UE doesn’t autonomously activate the functionality. NW is expected to activate the functionality when it receives report from UE that it is applicable.

Proposal 6: The NW provides a list of candidate configurations and UE perform request based on the candidate configurations. RAN2 to decide which following option(s) are adopted as the candidate configurations:

- Option 1: A list of candidate training data collection configuration, including measurement resource configuration for set A and set B, and associated ID.

- Option 2: A list of candidate training data collection related parameters, e.g. set A and set B combination, and associated ID.

- Option 3: A list of inference configuration that can also be requested for training data collection.

- Option 4: FFS: a list of candidate associated ID.

Proposal 7: RRCReconfiguration can be used to provide the candidate configuration for data collection. FFS whether SI message can also be used.

Proposal 8: Both RRCReconfigurationComplete and UAI can be used by UE to request to start and stop data collection.

Proposal 9: When UE can’t perform data collection based on received configuration it should indicate NW to stop data collection via RRCReconfigurationComplete or UAI. FFS whether detailed reasons are needed.

Proposal 10: UE can be configured with a prohibit timer which is started after UE initiates request for a data collection configuration via RRCReconfigurationComplete or UAI. UE will not initiate UAI for the same data collection configuration again before the prohibit timer expires.

References

- RAN2#128 chairman notes

- R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model